| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0848 |
| --- | --- |

| Title: | CE10 related Document: Speedups for Uniform Directional Diffusion Filters For Video Coding (JVET-M0042) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Anastasia Henkel, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de, anastasia.henkel@hhi-extern.fraunhofer.de. |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

New speedups for the uniform diffusion filters described in JVET-M0042 are investigated.

# Proposed Speedups

The document refers to CE10 contribution JVET-M0042, [2], more specifically speed ups for test CE10.4.2. In the inter case, less merge candidates are tested with the diffusion filters and the search is stopped earlier. Additionally, the filter is also tested in case of the affine inter mode.

# Results

According to the CE10 description in JVET-L1030 [1] and the common test conditions in JVET-L1010 [5], the proposed method is evaluated for intra-only, random-access and low delay configuration with VTM 3.0.

In the following table an overview over the scheduled CE tests with their corresponding config parameters is shown.

| Test |  |
| --- | --- |
| All Intra, Random Access and Low Delay, | #Diffusion filtering DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0,60% | -0,81% |  | -0,34% |  |
| Class A2 | -0,48% | -0,56% |  | -0,31% |  |
| Class B | -0,52% | -0,63% |  | -0,67% |  |
| Class C | -0,20% | -0,27% |  | -0,12% |  |
| Overall | -0,44% | -0,56% |  | -0,38% |  |
| Enc Time [%] | 110% |  |  |  |  |
| Dec Time [%] | 97% |  |  |  |  |
| Class D | -0,12% |  | -0,30% |  | 0,01% |
| Class F | -0,16% |  | -0,14% |  | 0,01% |

|  | VTM, Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B |  |  |  |  |  |
| Class C | -0,10% | 0,49% |  | 0,49% |  |
| Class E | 0,00% | 1,14% |  | 1,74% |  |
| Overall |  |  |  |  |  |
| Enc Time [%] | 112% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0,03% |  | 1,00% |  | 0,64% |
| Class F |  |  |  |  |  |

# References

- C.W. Hsu, M. Winken, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” JVET-L1030, Macao, CN, 3–12 Oct. 2018.

- J. Rasch et al., “CE10: Uniform Directional Diffusion Filters For Video Coding (Test 10.4.1-10.4.2)” JVET-M0042, Marrakech, MA, 9–18 Jan. 2019.

- F.Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).