| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0854-r2 |
| --- | --- |

| Title: | CE4-related: Combination of CE4.4.4a and CE4.4.5b |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai Li Jingya Ru-Ling Liao Chong Soon Lim | Email: | +81-43-299-8526 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp +65-65505346 jingya.li@sg.panasonic.com ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Sharp Corporation Panasonic |  |  |

_____________________________

# Abstract

This contribution reports results of combining CE4.4.4.a and CE4.4.5.b. It shows 0.31% B-D rate reduction with 102% encoding time and 100% decoding time in RA condition and 0.08% B-D rate reduction with 103% encoding time and 100% decoding time in LDB condition.

# Proposed Methods

## Directional candidates in CE4.4.4.a

Directional candidates are used in addition to horizontal / vertical candidates of VTM-3.0. The directional candidates are shown as below.

Table 1: Motion direction

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +2 | –2 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +2 | –2 | +1 | -1 | +1 | -1 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

## Adaptive distance table in CE4.4.5.b

Use adaptive distance table based on picture resolution, i.e., if picture resolution is not larger than 2K, i.e., 1920×1080, Table 2 is used as base distance table; Otherwise, Table 3 is used as base distance table.

Table 2: MMVD distance table candidate

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Table 3: MMVD distance table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

## Encoder optimization algorithm

The early termination for MMVD which is included in VTM-3.0 is disabled for both CE4.4.4.a and CE4.4.5.b, (the early termination used for MMVD in VTM3.0 is: when the SATD cost of a MMVD candidate with short distance is smaller than a threshold, the MMVD candidate with larger distance will not be tested.)

In CE4.4.4.a, one encoder fast algorithm is added.

To reduce the fixed number of the searched candidates without loss as much as possible, a fast algorithm like “Hexagon Search” is applied. In this algorithm, the searching for directions is nested in the searching for distances. At first, all directions (8 directions) are searched with the smallest distance. After this first searching loop, only 3 directions, that are the best direction in previous distance and 2 neighbors, are searched. By using this algorithm, the number of searched candidates in each base MV is equal to 8 + 3 * 7 = 29 candidates.

In CE4.4.5.b, one encoder fast algorithm is added.

This fast algorithm includes 2 parts (1) During pre-selection of MMVD candidate using SATD cost, BDOF is disabled for motion compensation. After best MMVD candidates are selected, BDOF is applied to those candidates. (2) During pre-selection of MMVD candidate using SATD cost, chroma motion compensation is skipped. After best MMVD candidates are selected, chroma motion compensation is calculated for to those candidates.

# Experimental Results

There are three tests evaluated.

Test 1: Combined test of CE4.4.4.a and CE4.4.5.b

Test 2: Test 1 with signaling PPS flag to indicate which distance table is used for current picture

Test 3: Applied encoder optimization algorithm used in CE4.4.5.b to VTM-3.0.

Table 1. Results of Test 1: combined test of CE4.4.4.a and CE4.4.5.b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.54% | -0.69% | 99% | 100% |
| Class A2 | -0.52% | -0.68% | -0.73% | 100% | 101% |
| Class B | -0.17% | -0.05% | -0.10% | 102% | 100% |
| Class C | -0.19% | -0.14% | -0.09% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.34% | 102% | 100% |
| Class D | -0.31% | -0.25% | -0.11% | 105% | 100% |
| Class F | -0.10% | -0.09% | -0.08% | 107% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.37% | -0.28% | 103% | 100% |
| Class C | -0.06% | 0.13% | 0.26% | 104% | 100% |
| Class E | -0.11% | -0.01% | 0.02% | 103% | 100% |
| Overall | -0.08% | -0.11% | -0.03% | 103% | 100% |
| Class D | -0.19% | 0.05% | -0.24% | 104% | 100% |
| Class F | -0.05% | 0.06% | -0.39% | 104% | 100% |

Table 2. Results of Test 2: combined test of CE4.4.4.a and CE4.4.5.b with signaling PPS flag

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.54% | -0.69% | 99% | 100% |
| Class A2 | -0.52% | -0.68% | -0.73% | 100% | 100% |
| Class B | -0.17% | -0.05% | -0.10% | 102% | 99% |
| Class C | -0.19% | -0.14% | -0.09% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.34% | 102% | 100% |
| Class D | -0.31% | -0.25% | -0.11% | 105% | 100% |
| Class F | -0.10% | -0.09% | -0.08% | 108% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.37% | -0.28% | 103% | 100% |
| Class C | -0.06% | 0.13% | 0.26% | 104% | 100% |
| Class E | -0.11% | -0.01% | 0.02% | 103% | 100% |
| Overall | -0.08% | -0.11% | -0.03% | 103% | 100% |
| Class D | -0.19% | 0.05% | -0.24% | 104% | 99% |
| Class F | -0.04% | 0.06% | -0.39% | 105% | 100% |

Table 3. Results of Test 3: applied encoder optimization algorithm to VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.15% | -0.09% | 98% | 100% |
| Class A2 | -0.19% | -0.20% | -0.19% | 100% | 100% |
| Class B | -0.10% | -0.12% | -0.26% | 101% | 100% |
| Class C | -0.11% | -0.12% | -0.14% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.14% | -0.18% | 101% | 100% |
| Class D | -0.16% | -0.43% | -0.26% | 105% | 100% |
| Class F | -0.12% | -0.17% | -0.17% | 106% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.31% | 0.06% | 104% | 100% |
| Class C | -0.13% | -0.06% | -0.30% | 104% | 100% |
| Class E | -0.04% | 0.16% | -0.12% | 102% | 100% |
| Overall | -0.08% | -0.11% | -0.10% | 104% | 100% |
| Class D | -0.12% | -0.22% | -0.30% | 105% | 100% |
| Class F | -0.18% | -0.28% | -0.24% | 104% | 100% |

# Conclusion

This contribution reports the results of combined test of CE4.4.4.a and CE4.4.5.b.

# References

[1] H. Yang, S. Liu and K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-L1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

[2] T. Hashimoto, E. Sasaki and T. Ikai, “CE4: Enhanced Merge with MVD (Test 4.4.4),” JVET-M0060, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

[3] J. Li, R.-L. Liao and C. S. Lim, “CE4: MMVD improvement (test 4.4.5),” JVET-M0312, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).