| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0890 |
| --- | --- |

| Title: | CE9-related: BDOF buffer reduction and enabling VPDU based application |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen, Xiang Ma, Semih Esenlik, Haitao Yang, Jianle Chen Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki | Tel: Email: | semih.esenlik@huawei.com, jianle.chen@huawei.com, kenji.kondo@sony.com, teruhiko.s@sony.com |
| Source: | Huawei Technologies. Co., Ltd, Sony Corporation |  |  |

_____________________________

# Abstract

During the BoG on CE9 related contributions, three issues have been identified regarding the current design of BDOF.

- Not being able to process BDOF in VPDU basis.

- Buffering latency of SAD calculation for early termination before BDOF.

- Delayed processing of 1 row and 1 column of BDOF subblocks at the VPDU boundaries.

The contribution tries to address the 3 problems by disabling SAD based early termination and implicit splitting of BDOF application region into VPDU units (if the CU exceeds VPDU size).

The simulation results show 0.0X% luma BD-rate reduction with X% increase in decoding time. Additionally results of only disabling SAD based early termination reported, where -0.05% Luma BD-rate is reported with 2% increase in decoding time.

# Introduction

During the 12th JVET meeting in Macau, the concept of VPDU was introduced into the VVC draft specification. The goal of VPDU is to guarantee completion of the processing of one 64x64 square region before starting the processing of other 64x64 square regions. This method is intended to reduce the memory footprint of the pipelined hardware implementations.

During the same meeting BDOF was included in the VVC draft specification. It is noted that in the current specification the maximum luma CU size that is allowed to apply BDOF is set to be equal to 128x128.

# Problem Statement

Assume a CTU is split horizontally into 2 leaf CUs, and the top CU applies BDOF. Since BDOF is applied to coding blocks that are larger than 64 luma samples in one dimension, two problems exist:

- The CU-based early termination decision requires that SAD matching cost is computed for the whole CU. Accordingly in order the finish the processing of the top-left VPDU, top-right VPDU also needs to be reconstructed. Hence the intended goal of sequential processing of VPDUs cannot be achieved.

- BDOF is applied on a 4x4 sub-block basis where gradients are computed for the 6x6 extended window surrounding the sub-block. Similarly to the first problem, the top-right VPDU needs to be reconstructed in order to enable the computation of gradients along the VPDU boundary, before the processing of top-left VPDU is finished. The problem is depicted in figure below, where the CU size is 128x128.

Figure 1: Delayed application of BDOF around the VPDU boundary.

# Proposed Solution

It is proposed to;

- Disable SAD based early termination completely.

- Implicitly split the BDOF application region into VPDU units if the CU exceeds VPDU one VPDU. Each application region (with maximum size of 64x64) is considered as an individual CU for BDOF operation.

It is noted that the SAD early termination can help reduce the decoding time for software implementations, whereas the complexity is increased for hardware implementations.

# Experimental Results

The simulation results for the proposed solution are presented in the tables below.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.05% | 0.02% | 0.00% | 102% | 105% | 100% | 104% |
| Class C | -0.03% | -0.02% | -0.02% | 102% | 104% | 100% | 102% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.10% | 0.06% | 102% | 103% | 100% | 101% |
| Class F (optional) | -0.03% | -0.06% | -0.04% | 102% | 110% | 102% | 112% |

Table 1: Simulation results for combination of proposed modifications. First column of decoding times are obtained by Huawei, second column obtained by Sony.

|  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncT | DecT |
| Class A1 | -0.06% | 0.01% | 0.04% | 102% | 104% | 100% | 102% |
| Class A2 | -0.05% | 0.02% | 0.06% | 102% | 104% | 101% | 102% |
| Class B | -0.07% | -0.01% | -0.01% | 102% | 105% | 100% | 102% |
| Class C | -0.02% | 0.03% | 0.01% | 102% | 104% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.05% | 0.01% | 0.02% | 102% | 105% | 100% | 102% |
| Class D | 0.01% | -0.11% | 0.05% | 102% | 104% | 100% | 100% |
| Class F | -0.03% | -0.06% | -0.02% | 102% | 110% | 102% | 111% |

Table 2: Simulation results disabling SAD based early termination only. First set of Enc/Dec times are obtained by Huawei, second set obtained by Sony.

# Conclusion

It is proposed to disable SAD based early termination of BDOF operation and implicit splitting of BDOF application region into VPDU units (if the CU exceeds one VPDU). It is suggested to include the proposed changes in the next version of the draft specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).