| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0057 |
| --- | --- |

| Title: | AHG12: On tile group design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com stewe@stewe.org shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

We propose a constraint that handling of in-loop filtering (and potentially any inter-prediction) operations across non-rectangular tile group boundaries is disallowed. Hence, turning on/off filtering or inter-prediction operations across boundaries is enabled only for rectangular partitions (e.g. tile, rectangular tile, and potentially sub-picture), because of two reasons: 1) Non-rectangular region is never independently decoded by a decoder. So, handing of filtering process across non-rectangular regions boundaries is not useful for parallel processing. 2) Disabling adaptive loop filter (or motion compensation) operations across non-rectangular boundaries is very complicated in terms of low-level processing.

Introduction

In VVC, the same tile design was inherited from HEVC, but a new scheme, so-called tile group, was adopted to support multiple use cases; 1) partial delivery and decoding, 2) parallel decoding process, 3) MTU size matching with tile granularity. For parallel processing, in Marrakesh meeting, two syntax elemeents (loop_filter_across_tiles_enabled_flag and loop_filter_across_tile_groups_enabled_flag) were adopted, to selectively allow in-loop filtering process across tile group boundary.

An observed problem with the syntax elements is that disabling in-loop filtering across tile group boundaries while enabling the filtering across tile boundaries may complicate the filtering process, when the tile group is non-rectangular. Figure 1 illustrates the case. When the adaptive loop filter (ALF) is processed with a 7x7 diamond filter across tile group boundary. If the tile group is non-rectangular, its filtering process is a kind of chaos. For each pixel, each filter coefficient, a boundary checking process is mandatory to identify which pixels belong to the current tile group. The cases of temporal motion compensation and current picture reference are not different. If a boundary extension is applied at tile group boundary for motion compensation, its padding process is also complex. In this contribution, we simply propose a constraint that tile-group-level loop filtering (or any other operations) control at boundaries shall be allowed only for rectangulary tile groups.

Figure 1. Illustration of ALF processing at non-rectangular tile group boundaries

Another minor issue about the flag (loop_filter_across_tile_groups_enabled_flag) is that the flag need not to be signaled when single_tile_per_tile_group_flag is equal to 1.

Proposed syntax elements

Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| .. |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| if( loop_filter_across_tiles_enabled_flag && rec_tile_group_flag && !single_tile_per_tile_group_flag) |  |
| loop_filter_across_tile_groups_enabled_flag | u(1) |
| … |  |
| } |  |

loop_filter_across_tile_groups_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile group boundaries in pictures referring to the PPS. loop_filter_across_tile_group_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile group boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tile_groups_enabled_flag is inferred to be equal to the value of loop_filter_across_tiles_enabled_flag.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).