| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0058 |
| --- | --- |

| Title: | AHG17: On decoded picture buffer management |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

The contribution proposes the following items to improve the decoded picture buffer management, by addressing the temporal sub-layer adaptation:

Proposal 1: TRAIL_NUT, STSA_NUT, RASL_NUT and RADL_NUT are re-specified as (TRAIL_N, TRAIL_R), (STSA_N, STSA _R), (RASL_N, RASL_R) and (RADL_N, RASL_R) to indicate non-referenced pictures in the same sub-layer. Accordingly, non-referenced pictures having the temporal ID equal to that of the current picture are included in the RefPicList[ i ].

Proposal 2: sps_max_dec_pic_buffering_minus1 is signaled for each highest temporal id in SPS.

Proposal 3: The list of unused reference pictures for each highest temporal id is signaled in tile group header.

Proposal 4: All reference pictures with TemporalId greater than HighestTid are marked as "unused for reference", when the value of HighestTid is not equal to sps_max_sub_layers_minus1.

Proposal-1

The NAL unit types TRAIL_NUT, STSA_NUT, RASL_NUT and RADL_NUT are split to (TRAIL_N, TRAIL_R), (STSA_N, STSA _R), (RASL_N, RASL_R) and (RADL_N, RASL_R) to indicate non-referenced pictures in the same sub-layer.

In order to discard unnecessary VCL NAL units for bit-rate adaptation, indicating VCL NAL units of non-refenced pictures. Accordingly, non-referenced pictures having the temporal ID equal to that of the current picture are included in the RefPicList[ i ]. Then, non-referenced pictures will be marked as “unused reference picture” and can be promptly removed from the decoded picture buffer (DPB).

Proposed text of proposal-1

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded tile group of a non- STSA trailing picture tile_group_layer_rbsp( ) | VCL |
| 2 3 | STSA_N STSA_R | Coded tile group of an STSA picture tile_group_layer_rbsp( ) | VCL |
| 4 5 | RASL_N RASL_R | Coded tile group of a RASL picture tile_group_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RASL_R | Coded tile group of a RADL picture tile_group_layer_rbsp( ) | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded tile group of an IDR picture tile_group_layer_rbsp( ) | VCL |

If a picture has nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, the picture is an SLNR picture. Otherwise, the picture is a sub-layer reference picture.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ] are constructed as follows:

for( i = 0; i < 2; i++ ) {
		for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with PicOrderCntVal equal to RefPicPocList[ i ][ j ] 
				&& the reference picA is not an SLNR picture with TemporalId equal to that of the current picture)
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"					(8-5)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
					if( there is a reference picA in the DPB with PicOrderCntVal & ( MaxPicOrderCntLsb − 1 )
										equal to poc_lsb_lt[ i ][ RplsIdx[ i ] ][ j ]

&& the reference picA is not an SLNR picture with TemporalId equal to that of the current picture)
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
				} else {
					if( there is a reference picA in the DPB with PicOrderCntVal equal to
										FullPocLt[ i ][ RplsIdx[ i ] ][ j ]

&& the reference picA is not an SLNR picture with TemporalId equal to that of the current picture )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
			}
				k++
		}
	}
}

It is a requirement of bitstream conformance that the following constraints apply:

- There shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] for which one or more of the following are true:

– The entry is equal to "no reference picture".

– The entry is an SLNR picture and has TemporalId equal to that of the current picture.

Proposal-2

sps_max_dec_pic_buffering_minus1 is signaled for each highest temporal id in SPS.

The value of the variable HighestTid can be determined an external means, if the external means is available. Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1. Then, the decoder can estimate the maximum required size of the decoded picture buffer for the given HighestTid value.

Proposed text of proposal-2

| seq_parameter_set_rbsp( ) { |  |
| --- | --- |
| … |  |
| for( i = 0; i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

sps_max_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i.  The value of sps_max_dec_pic_buffering_minus1 [ i ] shall be in the range of 0 to MaxDpbSize − 1, inclusive, where MaxDpbSize is as specified somewhere else.

Proposal-3

The list of unused reference pictures for each highest temporal id is signaled in tile group header.

Depending on the value of HighestTid, some reference pictures of each temporal sub-layer may not be used as reference for the subsequent pictures. By explicitly signaling unused reference pictures corresponding to each HightesTid value in tile group header, the unused decoded pictures as reference can be promptly removed from the DPB.

Proposed text of proposal-3

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| unused_ref_pic_signaling_enabled_flag | u(1) |
| … |  |
| } |  |

unused_ref_pic_signaling_enabled_flag equal to 0 specifies that num_unused_ref_pic and delta_poc_unused_ref_pic[ i ] are not present in tile group header, and the removal timing of a decoded picture from the DPB is implicitly determined. unused_ref_pic_signaling_enabled_flag equal to 1 specifies that num_unused_ref_pic and delta_poc_unused_ref_pic[ i ] are present in tile group header, and the removal timing of a decoded picture from the DPB is explicitly determined by parsing delta_poc_unused_ref_pic[ i ].

| tile_group_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if(unused_ref_pic_signaling_enabled_flag) { |  |
| num_unused_ref_pic | ue(v) |
| for( I = 0; i < num_unused_ref_pic; i++) { |  |
| delta_poc_unused_ref_pic [ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

num_unused_ref_pic specifies the number of unused reference picture entries. If not present, the value of this field is set equal to 0.

delta_poc_unused_ref_pic [ i ] specifies the absolute difference between the picture order count values of the current picture and the unused decoded picture referred to by the i-th entry. The value of delta_poc_unused_ref_pic [ i ] shall be in the range of 0 to 215 − 1, inclusive.

If unused_ref_pic_signaling_enabled_flag is equal to 1, the following applies:

for( i = 0; i < num_unused_ref_pic[ HighestTid ]; i++ )
	if( there is a reference picture picX in the DPB
			with PicOrderCntVal equal to (the current picture PicOrderCntVal - delta_poc_unused_ref_pic [ HighestTid ][ i ])
		picX is marked as “unused for reference”.

Proposal-4

All reference pictures with TemporalId greater than HighestTid are marked as "unused for reference", when the value of HighestTid is not equal to sps_max_sub_layers_minus1.

The HighestTid value can be changed by an external means, instantly. The sub-bitstream extraction process is applied with the HighestTid as input.

Proposed text of proposal-4

Decoding process for reference picture marking

This process is invoked once per picture, after decoding of a tile group header and the decoding process for reference picture list construction for the tile group as specified in clause 8.3.2, but prior to the decoding of the tile group data. This process may result in one or more reference pictures in the DPB being marked as "unused for reference" or "used for long-term reference".

A decoded picture in the DPB can be marked as "unused for reference", "used for short-term reference" or "used for long-term reference", but only one among these three at any given moment during the operation of the decoding process. Assigning one of these markings to a picture implicitly removes another of these markings when applicable. When a picture is referred to as being marked as "used for reference", this collectively refers to the picture being marked as "used for short-term reference" or "used for long-term reference" (but not both).

STRPs are identified by their PicOrderCntVal values. LTRPs are identified by the Log2( MaxLtPicOrderCntLsb ) LSBs of their PicOrderCntVal values.

If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1, all reference pictures currently in the DPB (if any) are marked as "unused for reference".

All reference pictures with TemporalId greater than HighestTid are marked as "unused for reference", when the value of HighestTid is not equal to sps_max_sub_layers_minus1.

Otherwise, the following applies:

- For each LTRP entry in RefPicList[ 0 ] or RefPicList[ 1 ], when the referred picture is an STRP, the picture is marked as "used for long-term reference".

- Each reference picture in the DPB that is not referred to by any entry in RefPicList[ 0 ] or RefPicList[ 1 ] is marked as "unused for reference".

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).