| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0078-v1 |
| --- | --- |

| Title: | CE2-5.6: Simplifications for inherited affine merging candidates at CTU row boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, cw.hsu, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | MediaTek Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes simplifications for inherited affine merging candidates at coding tree unit (CTU) row boundaries. Compared with the inherited affine merging candidates in VTM4.0, it is asserted that this proposal can remove the “coding unit (CU) width and x position information” line buffer. At CTU row boundaries, it is proposed to use two subblock motion vectors (MVs) with a fixed distance between the two subblocks for affine candidate inheritance, where an accurate distance is used to derive the affine motion and “CU width and x position information” are not needed anymore. Results are summarized as follows.

Proposed method with the fixed distance equal to 4
VTM4.0-RA: {Y-BD-rate=0.02%, EncT=100%, DecT=100%}
VTM4.0-LB: {Y-BD-rate=0.01%, EncT=99%, DecT=101%}

Proposed method with the fixed distance equal to 8
VTM4.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM4.0-LB: {Y-BD-rate=0.02%, EncT=100%, DecT=99%}

Proposed method with the fixed distance equal to 16
VTM4.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=99%}
VTM4.0-LB: {Y-BD-rate=-0.02%, EncT=100%, DecT=101%}

# Introduction

In JVET-L meeting, the line buffer reduction methods for inherited affine merging candidates at CTU row boundaries presented in JVET-L0045 [1] and JVET-L0694 [2] were adopted. In JVET-L0045, it is proposed to use the 4-parameter affine motion model with the bottom-left and bottom-right control point motion vectors (CPMVs) for affine motion inheritance from a neighbouring CU located at a CTU row boundary, regardless whether the neighbouring CU uses the 4- or 6-parameter affine motion model. In JVET-L0694, the bottom-left and bottom-right subblock MVs are used for the affine motion inheritance if the candidate block is from the upper CTU row. No line buffer is required for CPMVs. For 4K video for example, both JVET-L0045 and JVET-L0694 reduce the line buffer requirement for the affine mode from 9,216 bytes to 256 bytes.

There are still two shortcomings. First, additional 256-byte line buffer is needed for storing CU width information. Second, the distance between bottom-left and bottom-right subblock MVs for affine inheritance is not accurate, and the accurate distance should be the width of the CU minus 4. This contribution proposes to use two subblock MVs with a fixed distance between the two subblocks for affine inheritance to deal with these two problems. It was presented in JVET-M0168 [3] and JVET-M0270 [4] and planned as CE2-5.6 in JVET-M1022 [5]. More details are introduced in the next sections.

# Proposed methods

## Method 1: Two subblock MVs with a fixed distance between the two subblocks for affine inheritance at CTU row boundaries (CE2-5.6a)

Figure 1.  Neighbouring 4×4 blocks involved to perform affine inheritance in CE2-5.6a

In CE2-5.6a, two adjacent above neighbouring 4×4 blocks as shown in Figure 1 are used to perform affine inheritance when above neighbouring blocks are from the upper CTU row. Firstly, Block B0, B1 and B2 are checked in order to find the first affine coded block denoted as Bx. Secondly, the 4×4 block right adjacent to Bx denoted as By is accessed, to check if By is also affine coded and share the same reference index as Bx. If both Bx and By are valid, the affine model can be inherited from MVs of Bx and By with a fixed distance between Bx and By, where the distance is equal to 4. In the worst case, the proposed method needs to check the affine_flag in four 4×4 blocks and check the equality of reference indices once.

With the proposed method, the CU width and x position in line buffers are totally removed. For one CTU, it only needs to access 35 4×4 blocks from the above CTU row at most, which is much lower than 96 in VTM4.0. Table 1 summarizes the memory comparison. The local buffer storage for affine inheritance is not changed in CE2-5.6a.

Table 1 Comparison of memory requirement

|  | Line buffer memory for affine inheritance (bits per 8×8 block) | Number of required 4×4 blocks in the above CTU row |
| --- | --- | --- |
| HEVC | 0 | 32+2 = 34 |
| VTM4.0 | 8 | 32*3 = 96 |
| CE2-5.6a | 0 | 32+3 = 35 |

## Method 2: Two subblock MVs with a fixed distance between the two subblocks for affine inheritance at CTU row boundaries (CE2-5.6b and CE2-5.6c)

Two subblock MVs with a fixed distance between the two subblocks are used for the affine motion inheritance if the candidate block is from the upper CTU row. As shown in Figure 2, B0 and B0’, B1 and B1’, and B2 and B2’ are used for deriving 4-parameter inherited affine merging candidates, and the distance between these two subblocks is N. The CU width line buffer is not needed because the distance between two subblocks is fixed. To check the availability, in addition to checking whether B0 (or B1, B2) is affine coded, it is necessary to check whether B0’ (or B1’, B2’) is affine coded and has same reference index as B0 (or B1, B2) in both reference lists. Table 2 summarizes the buffer usage comparison between proposed method and VTM4.0. The “CU width and x position information line buffer” is never needed.

Figure 2.  Inherited affine merging candidate derivation at CTU row boundaries

Table 2.  Buffer usage comparison between the proposed method and VTM4.0

|  | Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer (bits per 8x8) | Additional logics |
| --- | --- | --- | --- |
| VTM4.0 | 7689 (33*(36*3*2+3*2+10+1)) | 8 (width, x) |  |
| Proposed method | Same | 0 | No |

# Experiment results

The proposed inherited affine merging candidate derivation methods have been implemented on top of VTM4.0. The common test conditions [6] are used for evaluation. Table 3 summarizes the results of the proposed method 1. Table 4 summarizes the results of the proposed method 2 with a fixed distance equal to 8. Table 5 summarizes the results of the proposed method 2 with a fixed distance equal to 16.

Table 3.  Inherited affine merging candidate derivation at CTU row boundaries using two subblock MVs with a fixed distance equal to 4

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.11% | 0.06% | 100% | 101% |
| Class A2 | 0.01% | 0.00% | 0.03% | 100% | 101% |
| Class B | 0.01% | -0.03% | -0.08% | 99% | 100% |
| Class C | 0.02% | -0.06% | -0.12% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.05% | -0.04% | 100% | 100% |
| Class D | 0.00% | -0.10% | -0.05% | 99% | 95% |
| Class F | 0.01% | 0.05% | 0.02% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.37% | 0.12% | 98% | 101% |
| Class C | -0.01% | 0.23% | -0.01% | 100% | 101% |
| Class E | 0.02% | -0.17% | -0.09% | 100% | 100% |
| Overall | 0.01% | -0.12% | 0.03% | 99% | 101% |
| Class D | -0.01% | -0.13% | -0.19% | 100% | 103% |
| Class F | -0.17% | -0.15% | 0.31% | 100% | 103% |

Table 4.  Inherited affine merging candidate derivation at CTU row boundaries using two subblock MVs with a fixed distance equal to 8

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.09% | 0.09% | 100% | 101% |
| Class A2 | 0.00% | -0.11% | 0.06% | 100% | 99% |
| Class B | 0.00% | -0.06% | 0.08% | 100% | 99% |
| Class C | 0.00% | -0.09% | -0.08% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.08% | 0.04% | 100% | 100% |
| Class D | 0.01% | 0.03% | -0.06% | 100% | 100% |
| Class F | 0.00% | 0.03% | 0.01% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | 0.17% | 100% | 100% |
| Class C | -0.01% | 0.11% | 0.00% | 100% | 100% |
| Class E | 0.06% | -0.03% | 0.12% | 100% | 96% |
| Overall | 0.02% | -0.02% | 0.10% | 100% | 99% |
| Class D | 0.03% | 0.29% | -0.29% | 101% | 101% |
| Class F | -0.10% | 0.01% | -0.28% | 100% | 99% |

Table 5.  Inherited affine merging candidate derivation at CTU row boundaries using two subblock MVs with a fixed distance equal to 16

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.07% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | 0.00% | 100% | 98% |
| Class B | -0.01% | -0.07% | -0.05% | 100% | 100% |
| Class C | 0.02% | 0.00% | -0.03% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class F | 0.00% | 0.05% | -0.02% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.15% | -0.33% | 100% | 102% |
| Class C | -0.02% | -0.13% | -0.06% | 100% | 100% |
| Class E | -0.06% | 0.07% | -0.37% | 100% | 99% |
| Overall | -0.02% | 0.03% | -0.25% | 100% | 101% |
| Class D | 0.01% | -0.14% | 0.18% | 101% | 101% |
| Class F | -0.09% | -0.33% | 0.00% | 100% | 100% |

# Conclusion

In this contribution, methods are proposed to simplify inherited affine merging candidates effectively. The coding efficiency impact is minor. Compared with the inherited affine merging candidates in VTM4.0, the buffer usage can be reduced.

# References

[1] M. Zhou, “CE4: Test results of CE4.1.11 on line buffer reduction for affine mode,” Document of Joint Video Experts Team, JVET-L0045.

[2] H. Chen, H. Yang, J. Chen, H. Huang, and W.-J. Chien, “CE4-related: Combination of affine mode clean up and line buffer reduction,” Document of Joint Video Experts Team, JVET-L0694.

[3] Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, and S.-M. Lei, “CE2-related: Simplifications for inherited affine candidates,” Document of Joint Video Experts Team, JVET-M0168.

[4] K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, and D. Hong, “CE2-related: An alternative storing method for affine inheritance,” Document of Joint Video Experts Team, JVET-M0270.

[5] C.-C. Chen, Y. He, and H. Liu, “Description of Core Experiment 2 (CE2): Sub-block based Motion Prediction,” Document of Joint Video Experts Team, JVET-M1022.

[6] F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).