| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0088-v1 |
| --- | --- |

| Title: | CE5-1: Adaptive loop filter with virtual boundary processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Tzu-Der Chuang, Chen-Yen Lai, Zhi-Yi Lin, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, peter.chuang, jenny.lai, zhi-yi.lin, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, CE5.1 tests for removing the required line buffer of adaptive loop filter (ALF) are reported. In ALF of VTM4.0, seven luma line buffers and four chroma line buffers are required because of using 7x7 diamond filters with 4x4 block-based classification for luma component and 5x5 diamond filter for chroma component. In order to totally remove the line buffer requirement, ALF with virtual boundary (VB) processing is proposed: when one sample located at one side of a VB is filtered, accessing samples located at the other side of the VB is forbidden. The originally required samples at the other side of the VB are replaced with padded samples. To accommodate the deblocking filter (DF) and sample adaptive offset (SAO) in VTM4.0, the VBs are set as four luma lines and two chroma lines above CTU row boundaries. After the proposed VB processing, weighted averaging is applied to filtered samples and ALF input samples to produce final ALF output samples near VBs. An implicit filter on/off control is also applied. Simulation results are summarized as follows.

ALF with VB processing:
Luma BD-rate = 0.08% (AI), 0.08% (RA), 0.10% (LB)

ALF with VB processing and weighted averaging
Luma BD-rate = 0.07% (AI), 0.09% (RA), 0.09% (LB)

ALF with VB processing, weighted averaging, and implicit filter on/off control
Luma BD-rate = 0.07% (AI), 0.10% (RA), 0.11% (LB)

# Introduction

The ALF line buffer requirement in VTM4.0 [1] is analyzed in the following two paragraphs. VB processing is described in Section 2 for removing all the ALF line buffers.

For luma, 7x7 diamond filters with 4x4 block-based classification are used. For a 4x4 block, a 10x10 window is required to perform the block-based classification and filtering. As shown in Figure 1, each square of green boundaries is a luma sample, the black bold lines are CTU boundaries, and the 10x10 samples in the brown dashed window are the required samples to filter the 4x4 blue block. In a typical hardware decoder, when the current CTU (containing the 4x4 blue block) is processed in ALF, the CTU below the current CTU is not yet processed. Therefore, the four luma lines above the CTU row boundary are unavailable for ALF because DF and SAO cannot be applied to these four luma lines without the CTU below the current CTU (and note that DF, SAO, and ALF are applied in sequential order). Consequently, the 4x4 blue block (and other 4x4 blocks on the same row) cannot be filtered by ALF, either. This introduces seven luma line buffers to store SAO output samples for filtering the 4x4 blue block when the below CTU row is processed. For the samples above the 4x4 blue block, they can be filtered by ALF without waiting for the next CTU row.

Figure 1.  Illustration for describing ALF luma line buffer requirement.

For chroma, a 5x5 diamond filter without classification is used. As shown in Figure 2, the two chroma lines above the CTU row boundary are unavailable for ALF because DF and SAO cannot be applied to these two chroma lines without the CTU below the current CTU. Consequently, the red sample (and other samples on the same row or below) cannot be filtered by ALF, either. This introduces four chroma line buffers to store SAO output samples for filtering the red sample when the below CTU row is processed. For the samples above the red sample, they can be filtered by ALF without waiting for the next CTU row.

Figure 2.  Illustration for describing ALF chroma line buffer requirement.

# ALF with virtual boundary processing

ALF with VB processing was proposed in JVET-M0164 [2] and planned as CE5.1. The concept is to avoid data access across VBs. Figure 3 and Figure 4 are two examples for luma. The VB is set as four lines above CTU row boundary. When the samples above the VB (region A in Figure 3) are filtered, samples below the VB (region B in Figure 3) are unavailable and are replaced with generated samples by padding. Similarly, when the samples below the VB (region B in Figure 4) are filtered, samples above the VB (region A in Figure 4) should be avoided (for removing line buffers) and are replaced with generated samples by padding.

Figure 3.  The concept of filtering luma samples above the VB.

Figure 4.  The concept of filtering luma samples below the VB.

For chroma, VBs are set as two chroma lines above CTU row boundaries. An example of the chroma VB is the blue dashed line in Figure 2.

As described above, padding is applied in VB processing. When a sample at one side of the VB is filtered, samples at the other side of the VB are replaced with padded samples during gradient calculation and filtering process. Figure 5 is an example for gradient calculation. The gradients are used in block-based classification and calculated by using four 3-tap gradient filters. In Figure 5, the center sample is the to-be-gradient-filtered sample, and a VB is right below the to-be-gradient-filtered sample. Each of the three samples below the VB is replaced with its nearest sample above the VB during gradient calculation.

Figure 5.  Modification for gradient calculations in VB processing.

The padding technique of accumulating gradients for a 4x4 block is different. As shown in Figure 6, the 4x4 green block needs to accumulate subsampled gradients in the 8x8 region, and the VB is right below the 4x4 green block. Instead of using padded samples to generate gradients below the VB, the gradients below the VB are replaced with the nearest gradient on the same column above the VB.

Figure 6.  Modification of accumulating gradients for a 4x4 block in VB processing.

A weighted averaging process is added after the filtering process. A final ALF output sample is a weighted averaging of the filtered sample and the ALF input sample. The weight for the filtered sample is dependent on the number of padded lines in the filtering process, and the sum of the two weights is 1.0. If there is no padded lines in the filter footprint, the weight of the filtered sample is equal to 1.0. If there is one padded line in the filter footprint, the weight of filtered sample is 0.5. If there are two padded lines in the filter footprint, the weight of filtered sample is 0.25. If there are three padded lines in the filter footprint, the weight of filtered sample is 0.125.

Furthermore, the implicit filter on/off control presented in JVET-M0301 [3] is also combined with the proposed method. The implicit on/off decision is made according to the ratio between the absolute sum of filter coefficients applied to padded samples and the absolute sum of filter coefficients applied to real samples. If the ratio is greater than 0.6, then ALF is not applied to this sample. Otherwise, ALF with the proposed method is applied to this sample.

# Experimental results

The ALF with VB processing has been implemented on VTM4.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [4]. Table 1 shows the simulation results of CE5.1.1, which only enables ALF with VB processing but disables the weighted averaging. Table 2 shows the simulation results of CE5.1.2, which enables ALF with VB processing and weighted averaging. Table 3 shows the simulation results of CE5.1.3, which includes the implicit filter on/off control on top of ALF with VB processing and weighted averaging.

Table 1.  Simulation results of ALF with VB processing but disabling weighted averaging

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.08% | 0.08% | 100% | 104% |
| Class A2 | 0.10% | 0.07% | 0.05% | 100% | 103% |
| Class B | 0.05% | 0.03% | 0.03% | 99% | 102% |
| Class C | 0.05% | 0.03% | 0.05% | 100% | 101% |
| Class E | 0.11% | 0.02% | 0.05% | 100% | 102% |
| Overall | 0.08% | 0.04% | 0.05% | 100% | 102% |
| Class D | 0.03% | 0.02% | 0.03% | 100% | 101% |
| Class F | 0.06% | 0.02% | 0.02% | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.13% | 0.08% | 100% | 105% |
| Class A2 | 0.10% | -0.07% | 0.02% | 100% | 106% |
| Class B | 0.07% | 0.00% | -0.03% | 100% | 101% |
| Class C | 0.07% | 0.03% | -0.02% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.03% | 0.01% | 100% | 103% |
| Class D | 0.00% | -0.20% | 0.00% | 99% | 101% |
| Class F | 0.05% | 0.05% | 0.08% | 100% | 104% |
|  |  |  | LB |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.09% | 0.11% | 100% | 103% |
| Class C | 0.02% | 0.16% | -0.06% | 101% | 103% |
| Class E | 0.25% | 0.13% | -0.79% | 100% | 103% |
| Overall | 0.10% | 0.05% | -0.17% | 100% | 103% |
| Class D | -0.01% | -0.11% | -0.69% | 100% | 102% |
| Class F | -0.01% | 0.31% | 0.23% | 100% | 102% |

Table 2.  Simulation results of ALF with VB processing and weighted averaging

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.21% | 0.16% | 100% | 102% |
| Class A2 | 0.07% | 0.13% | 0.08% | 100% | 106% |
| Class B | 0.06% | 0.07% | 0.07% | 100% | 102% |
| Class C | 0.05% | 0.07% | 0.10% | 100% | 102% |
| Class E | 0.10% | 0.11% | 0.19% | 100% | 103% |
| Overall | 0.07% | 0.11% | 0.11% | 100% | 103% |
| Class D | 0.03% | 0.06% | 0.06% | 100% | 103% |
| Class F | 0.06% | 0.06% | 0.05% | 100% | 103% |
|  |  |  | RA |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.06% | 0.09% | 100% | 101% |
| Class A2 | 0.11% | -0.07% | 0.02% | 100% | 102% |
| Class B | 0.10% | 0.09% | 0.12% | 100% | 101% |
| Class C | 0.07% | -0.04% | -0.17% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.02% | 0.02% | 100% | 102% |
| Class D | 0.03% | -0.02% | -0.02% | 100% | 105% |
| Class F | 0.04% | 0.06% | 0.06% | 100% | 101% |
|  |  |  | LB |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.05% | -0.09% | 100% | 101% |
| Class C | 0.04% | 0.29% | 0.27% | 101% | 105% |
| Class E | 0.19% | 0.27% | -0.32% | 102% | 103% |
| Overall | 0.09% | 0.14% | -0.03% | 101% | 103% |
| Class D | 0.02% | 0.23% | -0.17% | 100% | 103% |
| Class F | -0.02% | -0.25% | -0.31% | 102% | 106% |

Table 3.  Simulation results of ALF with VB processing, weighted averaging, and implicit filter on/off control

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.21% | 0.16% | 100% | 101% |
| Class A2 | 0.07% | 0.13% | 0.08% | 100% | 105% |
| Class B | 0.06% | 0.07% | 0.07% | 100% | 102% |
| Class C | 0.05% | 0.07% | 0.10% | 100% | 102% |
| Class E | 0.10% | 0.11% | 0.19% | 101% | 107% |
| Overall | 0.07% | 0.11% | 0.11% | 100% | 103% |
| Class D | 0.03% | 0.06% | 0.06% | 100% | 102% |
| Class F | 0.06% | 0.06% | 0.05% | 100% | 102% |
|  |  |  | RA |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.11% | 0.13% | 0.02% | 100% | 104% |
| Class A2 | 0.11% | -0.07% | 0.08% | 100% | 103% |
| Class B | 0.10% | 0.13% | 0.10% | 100% | 103% |
| Class C | 0.07% | -0.04% | -0.17% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.04% | 0.01% | 100% | 103% |
| Class D | 0.03% | -0.03% | -0.02% | 99% | 103% |
| Class F | 0.04% | 0.06% | 0.06% | 100% | 101% |
|  |  |  | LB |  |  |
|  |  |  | VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.06% | 0.14% | 100% | 104% |
| Class C | 0.08% | 0.47% | 0.20% | 100% | 105% |
| Class E | 0.20% | 0.19% | -0.21% | 100% | 103% |
| Overall | 0.11% | 0.18% | 0.07% | 100% | 104% |
| Class D | 0.03% | 0.20% | -0.12% | 100% | 107% |
| Class F | -0.04% | 0.09% | 0.42% | 100% | 104% |

# Conclusions

This contribution reports the results of CE5.1 tests: applying VB processing to ALF in order to remove the line buffer requirement of ALF. The VBs are set as four lines above CTU row boundaries for luma and two lines above CTU row boundaries for chroma. The proposed method can remove all the line buffers for ALF with less than 0.1% BD-rates.

# References

- VTM4.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- C.-Y. Chen, T.-D. Chuang, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, and S.-M. Lei, “Adaptive loop filter with virtual boundary processing,” Document of Joint Video Experts Team, JVET-M0164.

- A. M. Kotra, S. Esenlik, B. Wang, H. Gao, and J. Chen, “Non-CE: Loop filter line buffer reduction,” Document of Joint Video Experts Team, JVET-M0301.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).