| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0173-v1 |
| --- | --- |

| Title: | Non-CE8: IBC Reference Area Rearrange |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao Semih Esenlik Biao Wang Anand Meher Kotra Jianle Chen | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

In VVC Draft 4.0 [1], IBC mode reference area is extended to left CTU. However, in some case the IBC reference area is not continues. An IBC reference Area rearrange method is proposed in this contribution. Compare with CTC [2] and TGM [3] sequences (IBC and HashME on for all classes), the proposed method achieves -0.06%/0.00%/xx% in AI/RA/LDB coding gain for CTC overall, -0.22%/-0.17%/-0.25% AI/RA/LDB coding gain for ClassF and -0.51%/-0.56%/-0.24% AI/RA/LDB coding gain for TGM sequences.

# Introduction

VVC Draft 4.0 is using both the samples in the current CTU and left CTU to predict the IBC mode. In the current CTU, all reconstructed samples can be used as reference of the current IBC block. In the left CTU of the current CTU, the CTU is divided into 4 64x64 area, if the collocated 64x64area in the current CTU is not yet reconstructed, the 64x64 area in the left CTU is considered as reference of the current IBC block. Figure 1 shows the CTU level using QT split or Horizontal BT split case, and Figure 2 shows the CTU level using Vertical BT split case.

# Problem Statement

As shown in the Figure 1 and Figure 2, the bold lines indicate continues reference 64x64 areas. If the CTU level split mode of the current CTU is using QT or horizontal BT, and current block falls into the top-right or bottom left 64x64 area of the current block as shown in Figure 1 (b) and (c). The less continues area in the reference area there is, the less IBC mode possibility can be achieved.

Fig 1: IBC reference area when QT or HBT is used for the current CTU level.

Fig 2: IBC reference area when VBT is used for the current CTU level.

# Proposed Method

Since the IBC mode is mostly used in Intra slice, the proposed method focus on the current CTU level split is QT case (Figure 1)

To avoid the discontinues area in Figure 1 (b) and Figure 1(c), the proposed method rearranged the IBC left CTU reference are by changing the QT scan order. In VVC Draft 4.0, the QT split scan order is left-top, right-top, left bottom, right-bottom for all splitting level, while in the proposed method, the QT split scan order is changes as left-top, left-bottom, right top, right bottom for all splitting levels

With the proposed method, when the CTU level split of the current CTU is QT split, the IBC reference are is same as shown in Figure 2. The proposed method increases continues IBC reference area.

# Simulation results

Table 1 shows the simulation results using VTM 4.0, both in the anchor and test with IBC and HashME on for all sequence.

Table 1, rearrange IBC reference area of the left CTU by changing QT scan order

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.07% | 101% | 101% |
| Class A2 | 0.00% | -0.03% | -0.03% | 101% | 100% |
| Class B | -0.13% | -0.19% | -0.16% | 101% | 100% |
| Class C | -0.08% | -0.08% | -0.01% | 101% | 100% |
| Class E | -0.04% | -0.10% | -0.01% | 101% | 100% |
| Overall | -0.06% | -0.09% | -0.06% | 101% | 100% |
| Class D | -0.01% | -0.13% | -0.12% | 100% | 98% |
| Class F | -0.22% | -0.28% | -0.23% | 101% | 100% |
| TGM | -0.51% | -0.56% | -0.53% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.04% | 0.03% | 0.08% | 100% | 100% |
| Class B | -0.04% | -0.13% | -0.28% | 100% | 100% |
| Class C | 0.02% | 0.36% | 0.21% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | -0.02% | 100% | 100% |
| Class D | 0.04% | 0.23% | 0.22% | 100% | 100% |
| Class F | -0.17% | -0.23% | -0.33% | 100% | 100% |
| TGM | -0.56% | -0.33% | -0.38% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.06% | -0.45% | -0.40% | 100% | 100% |
| Class E | 0.00% | -0.16% | 1.32% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.06% | 1.32% | 0.88% | 100% | 100% |
| Class F | -0.25% | -0.39% | -0.17% | 100% | 100% |
| TGM | -0.24% | -0.25% | -0.26% | 100% | 100% |

# Draft specification changes

The draft text changes are based on [1], the proposed method is highlighted as follow:

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| cbSubDiv = 2 * cqtDepth |  |
| if( cbSize > 64 ) { |  |
| … |  |
| x1 = x0 + ( cbSize / 2 ) |  |
| y1 = y0 + ( cbSize / 2 ) |  |
| dual_tree_implicit_qt_split( x0, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x0, y1, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y0, cbSize / 2, cqtDepth + 1 ) |  |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  |
| dual_tree_implicit_qt_split( x1, y1, cbSize / 2, cqtDepth + 1 ) |  |
| } else { |  |
| … } |  |
| } |  |

| coding_tree( x0, y0, cbWidth, cbHeight, qgOn, cbSubDiv, cqtDepth, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( split_cu_flag ) { |  |
| … |  |
| } else { |  |
| x1 = x0 + ( cbWidth / 2 ) |  |
| y1 = y0 + ( cbHeight / 2 ) |  |
| coding_tree( x0, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubDiv, cqtDepth + 1, 0, 0, 0, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubDiv, cqtDepth + 1, 0, 0, 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples ) |  |
| coding_tree( x0, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubDiv, cqtDepth + 1, 0, 0, 2, treeType ) |  |
| if( x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y0, cbWidth / 2, cbHeight / 2, qgOn, cbSubDiv, cqtDepth + 1, 0, 0, 1, treeType ) |  |
| if( y1 < pic_height_in_luma_samples && x1 < pic_width_in_luma_samples ) |  |
| coding_tree( x1, y1, cbWidth / 2, cbHeight / 2, qgOn, cbSubDiv, cqtDepth + 1, 0, 0, 3, treeType ) |  |
| } |  |
| } else |  |
| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

# Conclusion

The proposed method are addressing discontinues of the IBC reference area of Intra slice. By changing the QT scan order to rearrange the IBC reference area, the proposed method achieves -0.06%/0.00%/xx% in AI/RA/LDB coding gain for CTC overall, -0.22%/-0.17%/-0.25% AI/RA/LDB coding gain for ClassF and -0.51%/-0.56%/-0.24% AI/RA/LDB coding gain for TGM sequence. The method is proposed to be included in the next version of VVC draft.

# References

- “Versatile Video Coding (Draft 4)”, JVET-M1001, B. Bross, J. Chen, S. Liu

- „JVET common test conditions and software reference configurations for SDR video “, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

- „Description of Core Experiment 8 (CE8): Screen Content Coding Tools “, JVET-M1028, X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).