| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0191 |
| --- | --- |

| Title: | [AHG17/AHG12] On APS id for bitstream merging for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Eric Nassor Jonathan Taquet Gerald Kergourlay Frederic Maze | Email: | nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr jonathan.taquet@crf.canon.fr gerald.kergourlay@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes to add an optional extension identifier (aps_extension_id) in the APS and associates a tile group extension identifier (tile_group_extension_id) with tile groups ids in the PPS. It is reported that the new identifiers simplify handling of APS identifier when combining or merging tile groups originated from different bistreams. In particular, it avoids rewriting the APS identifier specified in the tile groups header in the merged bitstream. It is asserted that the merging at Systems level would be simplified since it would mainly consist in modifying Parameter Set NAL units.

# Introduction

During the creation of a new video stream with regions taken from different video streams (for OMAF viewport dependent streaming for instance), same APS identifiers are likely to be used in the video streams. Indeed, the APS identifier is a value between 0 and 31 and it may change at each frame. The figure below illustrates the generation of a new bitstream from two input bitstreams.

Figure 1: Tile groups and bitstream merging

In this example, we want to merge the TG (Tile Group) 3 from first video and the TG 1, 2 and 4 from second video in a new video (see Figure 1). It is possible for the TG 3 to reference a first APS with id 0 in first video and for TG 4 to reference a second different APS with same id 0 in the second video. In this case, in order to create the merged video, we will need to change the id of one of the APS and to change also the TG header of one of the TG to change the reference during the rewriting process.

The first operation (changing the APS id) requires reading the non-VCL units in the video streams, decoding the NAL unit headers and rewriting the APS units. This operation is not too complex because the non-VCL units are relatively small and the headers are encoded with mostly fixed length values. But we also need to read all the VCL units to modify the APS id inside the TG headers. This second operation is complex because it requires reading all the video bitstream, decoding all the TG headers which contain many variable length fields and rewriting the complete bitstream.

# Proposal

A better structure for the APS would allow an efficient merge of several videos allowing easy resolution of APS id conflicts without requiring modification of the VCL NAL units (and in particular the TG headers).

## Adaptation parameter set syntax

In order to solve the problem of conflicting APS id when merging videos, we propose to add a new ID (the aps_extension_id called APS extension id) inside the APS.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_ id_extension_flag | u(1) |
| if(aps_id_extension_flag) { |  |
| aps_extension_id | u(5) |
| alf_data( ) |  |
| … |  |
| } |  |

adaptation_parameter_set_id provides an identifier for the APS to use as a reference by other syntax elements. The value of adaptation_parameter_set_id shall be in the range of 0 to 31.

aps_id_extension_flag equal to 1 specifies the presence of aps_extension_id in the APS. aps_extension_flag equal to 0 specifies the absence of the aps_extension_id.

aps_extension_id when present, provides extended identifier for the APS to use as reference by other syntax elements. The value of aps_extension_id shall be in the range of 0 to 31. When not present, the value of aps_extension_id is inferred to be equal 0.

## Picture parameter set RBSP syntax

### Option 1

The PPS contains an association for each tile group of the associated APS extension identifier.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_id[ i ] | u(v) |
| } |  |
| if( rect_tile_group_flag ) { |  |
| signalled_aps_id_extension_flag | u(1) |
| if(signalled_aps_id_extension_flag) { |  |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) |  |
| tile_group_aps_extension_id[ i ] | u(5) |
| } |  |

signalled_aps_id_extension_flag equal to 1 specifies the presence of tile_group_aps_extension_id[ i ] in the APS. signalled_aps_id_extension_flag equal to 0 specifies the absence of the tile_group_aps_extension_id[ i ] in the PPS.

tile_group_aps_extension_id[ i ] specifies the tile group extension ID of the i-th tile group, when present. When not present, the tile_group_aps_extension_id[ i ] is inferred equal to 0, for each i in the range of 0 to num_tile_group_in_pic_minus1 inclusive.

### Option 2

In order to simplify the parsing of the PPS, the different loops of the PPS could be merged as illustrated by the following syntax:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( rect_tile_group_flag ) { |  |
| signalled_tile_group_id_flag | u(1) |
| if( signalled_tile_group_id_flag ) { |  |
| signalled_tile_group_id_length_minus1 | ue(v) |
| } |  |
| signalled_aps_id_extension_flag | u(1) |
| for( i = 0; i <= num_tile_groups_in_pic_minus1; i++ ) { |  |
| if( signalled_tile_group_id_flag ) |  |
| tile_group_id[ i ] | u(v) |
| if(signalled_aps_id_extension_flag) |  |
| tile_group_aps_extension_id[ i ] | u(5) |
| } |  |
| } |  |

## General tile group header syntax

No modification of the tile group header is contemplated. The presence of the APS identifier when combined with the APS extension identifier obtained from the PPS allows determining the right APS in all configurations.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| tile_group_aps_id | ue(v) |
| } |  |
| ... | u(1) |
| } |  |

The semantics of some syntax elements of the tile group header are the following:

tile_group_aps_id specifies the identifier of the APS in use.

The variable tileGroupExtensionIdx which specifies the index of the tile group APS extension identifier is derived as follows:

if( rect_tile_group_flag ) {
	tileGroupExtensionIdx = 0
	while( tile_group_address  !=  tile_group_id[ tileGroupExtensionIdx ] )
		tileGroupExtensionIdx ++

}

else {

tileGroupExtensionIdx = 0;

}

The APS in use is the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id and the aps_extension_id equal to tile_group_aps_extension_id[tileGroupExtensionIdx].

The TemporalId of the APS NAL unit having adaptation_parameter_set_id equal to tile_group_aps_id and the aps_extension_id equal to tile_group_aps_extension_id[tileGroupExtensionIdx] shall be less than or equal to the TemporalId of the coded tile group NAL unit.

When multiple APSs with the same value of adaptation_parameter_set_id and aps_extension_id are referred to by two or more tile groups of the same picture, the multiple APSs with the same value of adaptation_parameter_set_id and aps_extension_id shall have the same content.

# Example

The figure above is an application example of the proposal. As in previous example, two bitstreams with respective tile groups 3 and 4 to be merged, each referring to an APS with id equal 0.

In the resulting bitstream after rewriting, both tile groups NAL units 3 and 4 are unmodified and continue to refer to the associated APS using the APS identifier with a value of 0. What changed is that now, the APS originated from the first bitstream specifies both an APS identifier with a value 0 and an APS extension identifier with a value of 0. The APS from the second bitstream comprises an APS identifier with a value 0 and an APS extension identifier with a value 1.

The PPS associates the tile group 3 with the APS extension identifier 0 and the tile group 4 with the APS extension identifier 1. At decoding, the decoder is therefore able to decode the tile group 3 with the correct identification of the associated APS based on the APS identifier stored in the tile group 3 and the associated APS extension identifier form the PPS. The same is true for the decoding of the tile group 4.

It is to be noted that this mechanism works even if the APS identifier changes from a picture to another for the tile groups with the same identifier. The APS extension identifier will stay the same, still allowing the identification of the right APS.

This proposal fixes the APS identifier collisions while keeping the tile group structure intact and the merge process of tile groups from different original bitstreams is simplified.

In addition, since the proposal modifies only Parameter Sets NAL units and keeps unchanged tile groups, it simplifies the merging of two ISO Base Media Format Files..

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).