| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0228 |
| --- | --- |

| Title: | CE3: Reduced number of reference samples for CCLM parameter calculation (CE3-1.4.1 and CE3-1.4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi, Jin Heo, Sunmi Yoo, Ling Li, Jungah Choi, Jaehyun Lim, Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, sunmi.yoo, l.li, jungah.choi, jaehyun.lim, seunghwan3.kim }@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of CE3-1.4, which is about the reduced number of reference samples for CCLM parameter calculation. In CE3-1.4.1, half of reference samples are used for CCLM parmeter calculation. In CE3-1.4.2, on top of CE3-1.4.1, maximum number of pixels is restricted to eight. In both tests, the number of reference sample pairs, which are used in the CCLM parameter determination, is reduced to half in the worst case (i.e., 2x2 CCLM prediction) so that the luma downsampling operation and the pixel comparison complexity are reduced by half.

Experimental results of CE3-1.4.1 show 0.00%, 0.07%, and 0.03% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration, and the results of CE3-1.4.2 show 0.00%, 0.04%, and 0.01% BD-rates in All Intra configuration.

Proposed method

As proposed in JVET-M0219[1], the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM (multi-directional CCLM) so that the required computational complexity can be reduced significantly. In the second test (CE3-1.4.2), the maximum number of reference sample pairs are limited to 8, as shown in Table 1.

With the proposed method, the number of sample pairs for CCLM parameter calculation is reduced by half for worst case (i.e. 2x2 block), and it brings the reduced number of downsampling operation in luma reference pixel generation and the pixel comparison operation in CCLM parameter derivation, as shown in Table 2 and 3.

Table 1 Number of sample pairs for CCLM parameter calculation in VTM4 and proposed CE test

| Chroma block size | VTM4 | CE3-1.4.1 | CE3-1.4.2 |
| --- | --- | --- | --- |
| 2 x n / n x 2 | 4 | 2 | 2 |
| 4 x n / n x 4 (n >=4) | 8 | 4 | 4 |
| 8 x n / n x 8 (n >=8) | 16 | 8 | 8 |
| 16 x n / n x 16 (n >=16) | 32 | 16 | 8 |
| 32 x 32 | 64 | 32 | 8 |

Table 2 Number of pixels for luma downsampling process in VTM4 and proposed CE test

| Chroma block size | VTM4 | CE3-1.4.1 | CE3-1.4.2 |
| --- | --- | --- | --- |
| 2 x n / n x 2 | 4 | 2 | 2 |
| 4 x n / n x 4 (n >=4) | 8 | 4 | 4 |
| 8 x n / n x 8 (n >=8) | 16 | 8 | 8 |
| 16 x n / n x 16 (n >=16) | 32 | 16 | 8 |
| 32 x 32 | 64 | 32 | 8 |

Table 3 Number of pixels comparison operation for CCLM parameter calculation in VTM4 and proposed CE test

| Chroma block size | VTM4 | CE3-1.4.1 | CE3-1.4.2 |
| --- | --- | --- | --- |
| 2 x n / n x 2 | 8 | 4 | 4 |
| 4 x n / n x 4 (n >=4) | 16 | 8 | 8 |
| 8 x n / n x 8 (n >=8) | 32 | 16 | 16 |
| 16 x n / n x 16 (n >=16) | 64 | 32 | 16 |
| 32 x 32 | 128 | 64 | 16 |

In this proposal, the subsampling pixels for the CCLM parameter calculation can be selected in the same way as introduced in [1].

Experimental results

The proposed method has been implemented using VTM4.0 as software base according to the common test conditions defined in JVET-M1010 [2] and Core Experiment description in JVET-M1023 [3].

CE3-1.4.1

In this CE test, the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM. Table 4 and Table 5 show the test results in AI and RA configuration.

Table 4 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM4

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.05% | -0.11% | 100% | 100% |
| Class A2 | 0.04% | 0.27% | 0.11% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.04% | 101% | 101% |
| Class C | 0.02% | 0.12% | 0.17% | 100% | 101% |
| Class E | 0.00% | 0.07% | -0.08% | 100% | 101% |
| Overall | 0.00% | 0.07% | 0.03% | 100% | 100% |
| Class D | 0.01% | 0.26% | 0.19% | 101% | 104% |
| Class F | 0.07% | 0.22% | 0.17% | 101% | 100% |

Table 5 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM4

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.30% | -0.22% | 101% | 101% |
| Class A2 | 0.00% | 0.34% | 0.17% | 101% | 100% |
| Class B | -0.02% | 0.08% | -0.08% | 100% | 101% |
| Class C | 0.02% | 0.17% | -0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.08% | -0.04% | 101% | 101% |
| Class D | 0.04% | -0.38% | 0.27% | 100% | 101% |
| Class F | 0.02% | 0.12% | 0.11% | 100% | 101% |

CE3-1.4.2

In this test, the half number of reference samples in the determination of the CCLM parameters is used for CCLM and MDLM, and the maximum number of reference sample pairs is limited to 8. Table 6 and Table 7 show the test results in AI and RA configuration.

Table 6 Experimental results of Test1 for all-intra (AI) test condition; anchor is VTM4

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.23% | 100% | 99% |
| Class A2 | 0.03% | 0.26% | 0.15% | 100% | 99% |
| Class B | 0.00% | -0.10% | -0.06% | 101% | 100% |
| Class C | 0.02% | 0.13% | 0.23% | 100% | 100% |
| Class E | 0.00% | -0.01% | -0.05% | 101% | 101% |
| Overall | 0.00% | 0.04% | 0.01% | 101% | 100% |
| Class D | 0.01% | 0.30% | 0.20% | 101% | 104% |
| Class F | 0.05% | 0.28% | 0.15% | 101% | 101% |

Table 7 Experimental results of Test1 for random-access (RA) test condition; anchor is VTM4

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | -0.14% | 101% | 100% |
| Class A2 | 0.00% | 0.18% | 0.09% | 101% | 100% |
| Class B | -0.01% | -0.15% | -0.34% | 101% | 101% |
| Class C | 0.03% | 0.28% | 0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | -0.12% | 101% | 101% |
| Class D | 0.01% | -0.05% | 0.48% | 100% | 101% |
| Class F | 0.02% | 0.08% | 0.04% | 101% | 101% |

Conclusion

To reduce the complexity of CCLM parameter derivation, the limitation of reference sample number for CCLM parameter calculation is proposed in this CE tests. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- J. Choi, et al., “CE3-related : Reduced number of reference samples for CCLM parameter calculation”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0219, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- G. Van der Auwera, et al., “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1023, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Specification of CE3-1.4.1

(section 8.4.4.2.8 in VVC specification)

Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << 1, yTbC << 1 )		(8-155)

The variables availL, availT and availTL are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC, yTbC − 1 ) as inputs, and the output is assigned to availT.

- The availability of top-left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC − 1 ) as inputs, and the output is assigned to availTL.

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC + x, yTbC − 1 ) as inputs, and the output is assigned to availableTR

- When availableTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC + y ) as inputs, and the output is assigned to availableLB

- When availableLB is equal to TRUE, numLeftBelow is incremented by one.

The number of available neighbouring chroma samples on the top and top-right numTopSamp and the number of available neighbouring chroma samples on the left and left-below nLeftSamp are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT  ?  nTbW  :  0		(8-156)

numSampL = availL  ?  nTbH  :  0		(8-157)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM )  ?  ( nTbW + numTopRight )  :  0	(8-158)

numSampL = ( availL  &&  predModeIntra = =  INTRA_L_CCLM )  ?  ( nTbH + numLeftBelow )  :  0	(8-159)

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC & ( 1 << ( CtbLog2SizeY − 1 ) − 1 )  = =  0 )  ?  TRUE  :  FALSE. 	(8-160)

The variable nS, xS, and yS are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

nS = ( ( availL && availT ) ? Min( nTbW, nTbH ) >> 1 : ( availL ? nTbH : nTbW ) >> 1 ) 	(8-xxx)

xS = numSampT >> Log2( nS )		(8-xxx)

yS = numSampL >> Log2( nS ) 		(8-xxx)

- Otherwise if predModeIntra is equal to INTRA_L_CCLM, the following applies:

nS = numSampL >> 1		(8-xxx)

xS = 2				(8-xxx)

yS = 2				(8-xxx)

- Otherwise (predModeIntra is equal to INTRA_T_CCLM), the following applies:

nS = numSampT >> 1		(8-xxx)

xS = 2				(8-xxx)

yS = 2				(8-xxx)

- If nS is equal to 1 and availL && availT is equal to FALSE, the following applies:

nS = 2				(8-xxx)

xS = 1				(8-xxx)

yS = 1			(8-xxx)

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both numSampL and numSampT are equal to 0, the following applies:

predSamples[ x ][ y ] = 1 << ( BitDepthC − 1 )		(8-161)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * 2 − 1, y= 0..nTbH * 2 − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples samples pY[ x ][ y ] are derived as follows:

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..2 * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..2 * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x ][ 2 * y − 1 ] +
	 pY[ 2 * x − 1 ][ 2 * y ] + 4 * pY[ 2 * x ][ 2 * y ] + pY[ 2 * x + 1 ][ 2 * y ] + 	(8-162)
		pY[ 2 * x ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] +
	 pY[ −1 ][ 2 * y ] + 4 * pY[ 0 ][ 2 * y ] + pY[ 1 ][ 2 * y ] + 	(8-163)
		pY[ 0 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] + 2 * pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 2 ) >> 2	(8-164)

    - If availT is equal to TRUE, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x ][ −1 ] +
	 pY[ 2 * x − 1 ][ 0 ] + 4 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 	(8-165)
		pY[ 2 * x ][ 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x − 1 ][ 0 ] + 2 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 2 ) >> 2	(8-166)

    - If availL is equal to TRUE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] +
	 pY[ −1 ][ 0 ] + 4 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 	(8-167)
		pY[ 0 ][ 1 ] + 4 ) >> 3

    - Otherwise if availL is equal to TRUE and availT is equal to FALSE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ −1 ][ 0 ] + 2 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 2 ) >> 2	(8-168)

    - Otherwise if availL is equal to FALSE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] + 2 * pY[ 0 ][ 0 ] + pY[ 0 ][ 1 ] + 2 ) >> 2	(8-169)

    - Otherwise (availL is equal to FALSE and availT is equal to FALSE), pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = pY[ 0 ][ 0 ]		(8-170)

    - Otherwise, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x − 1 ][ 2 * y ] + 	pY[ 2 * x − 1 ][ 2 * y + 1 ] + 
	2* pY[ 2 * x ][ 2 * y ] +	 2*pY[ 2 * x ][ 2 * y + 1 ] + 	(8-171)
		pY[ 2 * x + 1 ][ 2 * y ] + 	pY[ 2 * x + 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ 0 ][ 2 * y ] + 2*pY[ 0 ][ 2*y + 1 ] + 	(8-172)
		pY[ 1 ][ 2 * y ] +	 pY[ 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 1 ) >> 1	(8-173)

  - When numSampL is greater than 0, the down-sampled neighbouring left luma samples pLeftDsY[ y ] with y = 0..numSampL − 1 are derived as follows:

    - If ( y + 1) % yS is equal to 0, the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pLeftDsY[ y ] with y = 1..nTbH − 1 is derived as follows:

pLeftDsY[ y ] = ( pY[ −2 ][ 2 * y − 1 ] +
		 pY[ −3 ][ 2 * y ] + 4 * pY[ −2 ][ 2 * y ] + pY[ −1 ][ 2 * y ] + 	(8-174)
		pY[ −2][ 2 * y + 1 ] + 4 ) >> 3

        - If availTL is equal to TRUE, pLeftDsY[ 0 ] is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −2 ][ −1 ] +
		 pY[ −3 ][ 0 ] + 4 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 	(8-175)
		pY[ −2 ][ 1 ] + 4 ) >> 3

        - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −3 ][ 0 ] + 2 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 2 ) >> 2	(8-176)

      - Otherwise, the following applies:

pLeftDsY[ y ] = ( pY[ −1 ][ 2 * y ] + pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ −2 ][ 2 * y ] + 2*pY[ −2 ][ 2 * y + 1 ] + 	(8-177)
		pY[ −3 ][ 2 * y ] + 	pY[ −3 ][ 2 * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the down-sampled neighbouring top luma samples pTopDsY[ x ] with x = 0..numSampT − 1 are specified as follows:

    - If ( x + 1 ) % xS is equal to 0, the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

          - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x ][ −3 ] +
		pY[ 2 * x − 1 ][ −2 ] + 4 * pY[ 2 * x ][ −2 ] + pY[ 2 * x + 1 ][ −2 ] + 	(8-178)
		pY[ 2 * x ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-179)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

        - pTopDsY[ 0 ] is derived as follows:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] +
		pY[ −1 ][ −2 ] + 4 * pY[ 0 ][ −2 ] + pY[ 1 ][ −2 ] + 	(8-180)
		pY[ 0 ][ −1 ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ −1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-181)
		pY[ 1 ][ −1 ] + 2 ) >> 2

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] + 2 * pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 2 ) >> 2	(8-182)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]	(8-183)

      - Otherwise, the following applies:

        - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

          - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −2 ] + pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −2 ] +	 2*pY[ 2 * x ][ −1 ] + 	(8-184)
		pY[ 2 * x + 1 ][ −2 ] + pY[ 2 * x + 1 ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-185)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

        - pTopDsY[ 0 ] is derived as follows:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −2 ] + pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −2 ] + 2*pY[ 0 ][ −1 ] + 	(8-186)
		pY[ 1 ][ −2 ] + pY[ 1 ][ −1 ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-187)
		pY[ 1 ][ −1 ] + 2 ) >> 2

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 1 ) >> 1	(8-188)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]		(8-189)

  - The variables nS, xS, yS are derived as follows:

    - If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

nS = ( ( availL && availT ) ? Min( nTbW, nTbH ) : ( availL ? nTbH : nTbW ) ) 	(8-190)

xS = 1 << ( ( ( nTbW > nTbH ) && availL && availT ) ? ( Log2( nTbW) − Log2( nTbH ) ) : 0 ) 	(8-191)

yS = 1 << ( ( ( nTbH > nTbW ) && availL && availT ) ? ( Log2( nTbH) − Log2( nTbW ) ) : 0 ) 	(8-192)

    - Otherwise if predModeIntra is equal to INTRA_L_CCLM, the following applies:

nS = numSampL		(8-193)

xS = 1		(8-194)

yS = 1		(8-195)

    - Otherwise (predModeIntra is equal to INTRA_T_CCLM), the following applies:

nS = numSampT		(8-196)

xS = 1		(8-197)

yS = 1		(8-198)

  - The variables minY, maxY, minC and maxC are derived as follows:

    - The variable minY is set equal to 1 << (BitDepthY) + 1 and the variable maxY is set equal to −1.

    - If availT is equal to TRUE, the variables minY, maxY, minC and maxC with x = 0..nS − 1 are derived as follows:

    - The variable subx is set equal to ( x + 1) * xS - 1

    - If minY is greater than pTopDsY[ subx x * xS ], the following applies:

minY = pTopDsY[ subx x * xS ] 	(8-199)

minC = p[ subx x * xS ][ −1 ] 	(8-200)

    - If maxY is less than pTopDsY[ subx x * xS ], the following applies:

maxY = pTopDsY[ subx x * xS ] 	(8-201)

maxC = p[ subx x * xS ][ −1 ] 	(8-202)

    - If availL is equal to TRUE, the variables minY, maxY, minC and maxC with y = 0..nS − 1 are derived as follows:

    - The variable suby is set equal to ( y + 1) * yS - 1

    - If minY is greater than pLeftDsY[ suby y * yS ], the following applies:

minY = pLeftDsY[ suby y * yS ] 	(8-203)

minC = p[ −1 ][ suby y * yS ] 	(8-204)

    - If maxY is less than pLeftDsY[ suby y * yS ], the following applies:

maxY = pLeftDsY[ suby y * yS ] 	(8-205)

maxC = p[ −1 ][ suby y * yS ] 	(8-206)

  - The variables a, b, and k are derived as follows:

    - If numSampL is equal to 0, and numSampT is equal to 0, the following applies:

k = 0		(8-207)

a = 0		(8-208)

b = 1 << ( BitDepthC − 1) 		(8-209)

    - Otherwise, the following applies:

diff = maxY − minY	(8-210)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-211)

x = Floor( Log2( diff ) )	(8-212)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-213)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-214)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-215)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-216)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-217)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-218)

b = minC − ( ( a * minY ) >> k )	(8-219)

where divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-220)

    - Otherwise (diff is equal to 0), the following applies:

k = 0	(8-221)

a = 0	(8-222)

b = minC	(8-223)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-224)

Specification of CE3-1.4.2

(section 8.4.4.2.8 in VVC specification)

Specification of INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM intra prediction mode

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a sample location ( xTbC, yTbC ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- chroma neighbouring samples p[ x ][ y ], with x = −1, y = 0..2 * nTbH − 1 and x = 0.. 2 * nTbW − 1, y = − 1.

Output of this process are predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The current luma location ( xTbY, yTbY ) is derived as follows:

( xTbY, yTbY )  =  ( xTbC << 1, yTbC << 1 )		(8-155)

The variables availL, availT and availTL are derived as follows:

- The availability of left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC ) as inputs, and the output is assigned to availL.

- The availability of top neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC, yTbC − 1 ) as inputs, and the output is assigned to availT.

- The availability of top-left neighbouring samples derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC, yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC − 1 ) as inputs, and the output is assigned to availTL.

- The number of available top-right neighbouring chroma samples numTopRight is derived as follows:

- The variable numTopRight is set equal to 0 and availTR is set equal to TRUE.

- When predModeIntra is equal to INTRA_T_CCLM, the following applies for x = nTbW..2 * nTbW − 1 until availTR is equal to FALSE or x is equal to 2 * nTbW − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC + x, yTbC − 1 ) as inputs, and the output is assigned to availableTR

- When availableTR is equal to TRUE, numTopRight is incremented by one.

- The number of available left-below neighbouring chroma samples numLeftBelow is derived as follows:

- The variable numLeftBelow is set equal to 0 and availLB is set equal to TRUE.

- When predModeIntra is equal to INTRA_L_CCLM, the following applies for y = nTbH..2 * nTbH − 1 until availLB is equal to FALSE or y is equal to 2 * nTbH − 1:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xTbC , yTbC ) and the neighbouring chroma location ( xTbC − 1, yTbC + y ) as inputs, and the output is assigned to availableLB

- When availableLB is equal to TRUE, numLeftBelow is incremented by one.

The number of available neighbouring chroma samples on the top and top-right numTopSamp and the number of available neighbouring chroma samples on the left and left-below nLeftSamp are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

numSampT = availT  ?  nTbW  :  0		(8-156)

numSampL = availL  ?  nTbH  :  0		(8-157)

- Otherwise, the following applies:

numSampT = ( availT  &&  predModeIntra  = =  INTRA_T_CCLM )  ?  ( nTbW + numTopRight )  :  0	(8-158)

numSampL = ( availL  &&  predModeIntra = =  INTRA_L_CCLM )  ?  ( nTbH + numLeftBelow )  :  0	(8-159)

The variable bCTUboundary is derived as follows:

bCTUboundary = ( yTbC & ( 1 << ( CtbLog2SizeY − 1 ) − 1 )  = =  0 )  ?  TRUE  :  FALSE. 	(8-160)

The variable nS, xS, and yS are derived as follows:

- If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

nS = ( ( availL && availT ) ? Min( 4, Min( nTbW, nTbH ) >> 1 ) :
Min( 8, ( availL ? nTbH : nTbW ) >> 1 ) ) 		(8-xxx)

xS = numSampT >> Log2( nS )		(8-xxx)

yS = numSampL >> Log2( nS ) 		(8-xxx)

- Otherwise if predModeIntra is equal to INTRA_L_CCLM, the following applies:

nS = Min( 8, numSampL >> 1 )		(8-xxx)

xS = 2				(8-xxx)

yS = nS > 8 ? ( numSampL >> 3 ) : 2		(8-xxx)

- Otherwise (predModeIntra is equal to INTRA_T_CCLM), the following applies:

nS = Min( 8, numSampT >> 1 )		(8-xxx)

xS = nS > 8 ? ( numSampT >> 3 ) : 2		(8-xxx)

yS = 2				(8-xxx)

- If  nS is equal to 1 and availL && availT is equal to FALSE, the following applies:

nS = 2				(8-xxx)

xS = 1				(8-xxx)

yS = 1			(8-xxx)

The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If both numSampL and numSampT are equal to 0, the following applies:

predSamples[ x ][ y ] = 1 << ( BitDepthC − 1 )		(8-161)

- Otherwise, the following ordered steps apply:

  - The collocated luma samples pY[ x ][ y ] with x = 0..nTbW * 2 − 1, y= 0..nTbH * 2 − 1 are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x, yTbY + y ).

  - The neighbouring luma samples samples pY[ x ][ y ] are derived as follows:

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..2 * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..2 * numSampT − 1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When availTL is equal to TRUE, the neighbouring top-left luma samples pY[ x ][ y ] with x = −1, y = −1, −2, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

  - The down-sampled collocated luma samples pDsY[ x ][ y ] with x = 0..nTbW − 1,  y = 0..nTbH − 1 are derived as follows:

    - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 1..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x ][ 2 * y − 1 ] +
	 pY[ 2 * x − 1 ][ 2 * y ] + 4 * pY[ 2 * x ][ 2 * y ] + pY[ 2 * x + 1 ][ 2 * y ] + 	(8-162)
		pY[ 2 * x ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] +
	 pY[ −1 ][ 2 * y ] + 4 * pY[ 0 ][ 2 * y ] + pY[ 1 ][ 2 * y ] + 	(8-163)
		pY[ 0 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 1..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y − 1 ] + 2 * pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 2 ) >> 2	(8-164)

    - If availT is equal to TRUE, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x ][ −1 ] +
	 pY[ 2 * x − 1 ][ 0 ] + 4 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 	(8-165)
		pY[ 2 * x ][ 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pDsY[ x ][ 0 ] = ( pY[ 2 * x − 1 ][ 0 ] + 2 * pY[ 2 * x ][ 0 ] + pY[ 2 * x + 1 ][ 0 ] + 2 ) >> 2	(8-166)

    - If availL is equal to TRUE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] +
	 pY[ −1 ][ 0 ] + 4 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 	(8-167)
		pY[ 0 ][ 1 ] + 4 ) >> 3

    - Otherwise if availL is equal to TRUE and availT is equal to FALSE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ −1 ][ 0 ] + 2 * pY[ 0 ][ 0 ] + pY[ 1 ][ 0 ] + 2 ) >> 2	(8-168)

    - Otherwise if availL is equal to FALSE and availT is equal to TRUE, pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = ( pY[ 0 ][ −1 ] + 2 * pY[ 0 ][ 0 ] + pY[ 0 ][ 1 ] + 2 ) >> 2	(8-169)

    - Otherwise (availL is equal to FALSE and availT is equal to FALSE), pDsY[ 0 ][ 0 ] is derived as follows:

pDsY[ 0 ][ 0 ] = pY[ 0 ][ 0 ]		(8-170)

    - Otherwise, the following applies:

    - pDsY[ x ][ y ] with x = 1..nTbW − 1, y = 0..nTbH − 1 is derived as follows:

pDsY[ x ][ y ] = ( pY[ 2 * x − 1 ][ 2 * y ] + 	pY[ 2 * x − 1 ][ 2 * y + 1 ] + 
	2* pY[ 2 * x ][ 2 * y ] +	 2*pY[ 2 * x ][ 2 * y + 1 ] + 	(8-171)
		pY[ 2 * x + 1 ][ 2 * y ] + 	pY[ 2 * x + 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - If availL is equal to TRUE, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ −1 ][ 2 * y ] + 	pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ 0 ][ 2 * y ] + 2*pY[ 0 ][ 2*y + 1 ] + 	(8-172)
		pY[ 1 ][ 2 * y ] +	 pY[ 1 ][ 2 * y + 1 ] + 4 ) >> 3

    - Otherwise, pDsY[ 0 ][ y ] with y = 0..nTbH − 1 is derived as follows:

pDsY[ 0 ][ y ] = ( pY[ 0 ][ 2 * y ] + pY[ 0 ][ 2 * y + 1 ] + 1 ) >> 1	(8-173)

  - When numSampL is greater than 0, the down-sampled neighbouring left luma samples pLeftDsY[ y ] with y = 0..numSampL − 1 are derived as follows:

    - If ( y + 1) % yS is equal to 0, the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pLeftDsY[ y ] with y = 1..nTbH − 1 is derived as follows:

pLeftDsY[ y ] = ( pY[ −2 ][ 2 * y − 1 ] +
		 pY[ −3 ][ 2 * y ] + 4 * pY[ −2 ][ 2 * y ] + pY[ −1 ][ 2 * y ] + 	(8-174)
		pY[ −2][ 2 * y + 1 ] + 4 ) >> 3

        - If availTL is equal to TRUE, pLeftDsY[ 0 ] is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −2 ][ −1 ] +
		 pY[ −3 ][ 0 ] + 4 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 	(8-175)
		pY[ −2 ][ 1 ] + 4 ) >> 3

        - Otherwise, pDsY[ x ][ 0 ] with x = 1..nTbW − 1 is derived as follows:

pLeftDsY[ 0 ] = ( pY[ −3 ][ 0 ] + 2 * pY[ −2 ][ 0 ] + pY[ −1 ][ 0 ] + 2 ) >> 2	(8-176)

      - Otherwise, the following applies:

pLeftDsY[ y ] = ( pY[ −1 ][ 2 * y ] + pY[ −1 ][ 2 * y + 1 ] + 
	2* pY[ −2 ][ 2 * y ] + 2*pY[ −2 ][ 2 * y + 1 ] + 	(8-177)
		pY[ −3 ][ 2 * y ] + 	pY[ −3 ][ 2 * y + 1 ] + 4 ) >> 3

  - When numSampT is greater than 0, the down-sampled neighbouring top luma samples pTopDsY[ x ] with x = 0..numSampT − 1 are specified as follows:

    - If ( x + 1 ) % xS is equal to 0, the following applies:

      - If sps_cclm_colocated_chroma_flag is equal to 1, the following applies:

        - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

          - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x ][ −3 ] +
		pY[ 2 * x − 1 ][ −2 ] + 4 * pY[ 2 * x ][ −2 ] + pY[ 2 * x + 1 ][ −2 ] + 	(8-178)
		pY[ 2 * x ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-179)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

        - pTopDsY[ 0 ] is derived as follows:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] +
		pY[ −1 ][ −2 ] + 4 * pY[ 0 ][ −2 ] + pY[ 1 ][ −2 ] + 	(8-180)
		pY[ 0 ][ −1 ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ −1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-181)
		pY[ 1 ][ −1 ] + 2 ) >> 2

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −3 ] + 2 * pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 2 ) >> 2	(8-182)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]	(8-183)

      - Otherwise, the following applies:

        - pTopDsY[ x ] with x = 1..numSampT − 1 is derived as follows:

          - If bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −2 ] + pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −2 ] +	 2*pY[ 2 * x ][ −1 ] + 	(8-184)
		pY[ 2 * x + 1 ][ −2 ] + pY[ 2 * x + 1 ][ −1 ] + 4 ) >> 3

          - Otherwise (bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ x ] = ( pY[ 2 * x − 1 ][ −1 ] + 
	2* pY[ 2 * x ][ −1 ] + 	(8-185)
		pY[ 2 * x + 1 ][ −1 ] + 2 ) >> 2

        - pTopDsY[ 0 ] is derived as follows:

          - If availTL is equal to TRUE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −2 ] + pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −2 ] + 2*pY[ 0 ][ −1 ] + 	(8-186)
		pY[ 1 ][ −2 ] + pY[ 1 ][ −1 ] + 4 ) >> 3

          - Otherwise if availTL is equal to TRUE and bCTUboundary is equal to TRUE, the following applies:

pTopDsY[ 0 ] = ( pY[ − 1 ][ −1 ] + 
	2* pY[ 0 ][ −1 ] + 		(8-187)
		pY[ 1 ][ −1 ] + 2 ) >> 2

          - Otherwise if availTL is equal to FALSE and bCTUboundary is equal to FALSE, the following applies:

pTopDsY[ 0 ] = ( pY[ 0 ][ −2 ] + pY[ 0 ][ −1 ] + 1 ) >> 1	(8-188)

          - Otherwise (availTL is equal to FALSE and bCTUboundary is equal to TRUE), the following applies:

pTopDsY[ 0 ] = pY[ 0 ][ −1 ]		(8-189)

  - The variables nS, xS, yS are derived as follows:

    - If predModeIntra is equal to INTRA_LT_CCLM, the following applies:

nS = ( ( availL && availT ) ? Min( nTbW, nTbH ) : ( availL ? nTbH : nTbW ) ) 	(8-190)

xS = 1 << ( ( ( nTbW > nTbH ) && availL && availT ) ? ( Log2( nTbW) − Log2( nTbH ) ) : 0 ) 	(8-191)

yS = 1 << ( ( ( nTbH > nTbW ) && availL && availT ) ? ( Log2( nTbH) − Log2( nTbW ) ) : 0 ) 	(8-192)

    - Otherwise if predModeIntra is equal to INTRA_L_CCLM, the following applies:

nS = numSampL		(8-193)

xS = 1		(8-194)

yS = 1		(8-195)

    - Otherwise (predModeIntra is equal to INTRA_T_CCLM), the following applies:

nS = numSampT		(8-196)

xS = 1		(8-197)

yS = 1		(8-198)

  - The variables minY, maxY, minC and maxC are derived as follows:

    - The variable minY is set equal to 1 << (BitDepthY) + 1 and the variable maxY is set equal to −1.

    - If availT is equal to TRUE, the variables minY, maxY, minC and maxC with x = 0..nS − 1 are derived as follows:

    - The variable subx is set equal to ( x + 1) * xS - 1

    - If minY is greater than pTopDsY[ subx x * xS ], the following applies:

minY = pTopDsY[ subx x * xS ] 	(8-199)

minC = p[ subx x * xS ][ −1 ] 	(8-200)

    - If maxY is less than pTopDsY[ subx x * xS ], the following applies:

maxY = pTopDsY[ subx x * xS ] 	(8-201)

maxC = p[ subx x * xS ][ −1 ] 	(8-202)

    - If availL is equal to TRUE, the variables minY, maxY, minC and maxC with y = 0..nS − 1 are derived as follows:

    - The variable suby is set equal to ( y + 1) * yS - 1

    - If minY is greater than pLeftDsY[ suby y * yS ], the following applies:

minY = pLeftDsY[ suby y * yS ] 	(8-203)

minC = p[ −1 ][ suby y * yS ] 	(8-204)

    - If maxY is less than pLeftDsY[ suby y * yS ], the following applies:

maxY = pLeftDsY[ suby y * yS ] 	(8-205)

maxC = p[ −1 ][ suby y * yS ] 	(8-206)

  - The variables a, b, and k are derived as follows:

    - If numSampL is equal to 0, and numSampT is equal to 0, the following applies:

k = 0		(8-207)

a = 0		(8-208)

b = 1 << ( BitDepthC − 1) 		(8-209)

    - Otherwise, the following applies:

diff = maxY − minY	(8-210)

    - If diff is not equal to 0, the following applies:

diffC = maxC − minC	(8-211)

x = Floor( Log2( diff ) )	(8-212)

normDiff = ( ( diff << 4 ) >> x ) & 15	(8-213)

x += ( normDiff  !=  0 )  ?  1  :  0	(8-214)

y = Floor( Log2( Abs ( diffC ) ) ) + 1	(8-215)

a = ( diffC * ( divSigTable[ normDiff ] | 8 ) + 2y − 1 ) >> y	(8-216)

k = ( ( 3 + x − y ) < 1 )  ?  1  :  3 + x − y	(8-217)

a = ( ( 3 + x − y ) < 1 )  ?  Sign( a ) * 15  :  a	(8-218)

b = minC − ( ( a * minY ) >> k )	(8-219)

where divSigTable[ ] is specified as follows:

divSigTable[ ] = { 0, 7, 6, 5, 5, 4, 4, 3, 3, 2, 2, 1, 1, 1, 1, 0 }	(8-220)

    - Otherwise (diff is equal to 0), the following applies:

k = 0	(8-221)

a = 0	(8-222)

b = minC	(8-223)

  - The prediction samples predSamples[ x ][ y ] with x = 0..nTbW − 1, y = 0.. nTbH − 1 are derived as follows:

predSamples[ x ][ y ] = Clip1C( ( ( pDsY[ x ][ y ] * a ) >> k ) + b )	(8-224)