Text for test 1

      - Derivation process for motion vector arrays from affine control point motion vectors

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- two variables cbWidth and cbHeight specifying the width and the height of the luma coding block,

- the number of control point motion vectors numCpMv,

- the control point motion vectors cpMvLX[ cpIdx ], with cpIdx = 0..numCpMv − 1 and X being 0 or 1,

- the reference index refIdxLX and X being 0 or 1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY.

Outputs of this process are:

- the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1,

- the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

The following assignments are made for x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

CpMvLX[ x ][ y ][ 0 ] = cpMvLX[ 0 ]		(8-763)

CpMvLX[ x ][ y ][ 1 ] = cpMvLX[ 1 ]		(8-764)

CpMvLX[ x ][ y ][ 2 ] = cpMvLX[ 2 ]		(8-765)

The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-766)

log2CbH = Log2( cbHeight )		(8-767)

The variables mvScaleHor, mvScaleVer, dHorX and dVerX are derived as follows:

mvScaleHor = cpMvLX[ 0 ][ 0 ] << 7		(8-768)

mvScaleVer = cpMvLX[ 0 ][ 1 ] << 7		(8-769)

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-770)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-771)

The variables dHorY and dVerY are derived as follows:

- If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-772)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-773)

- Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-774)

dVerY = dHorX		(8-775)

The variable pelScale is set equal to (1<<7)<< 4.

The variable use8x8SubBlock is set to true if predFlagL0[xCb][yCb] and predFlagL1[xCb][yCb] are both equal to 1 and any of the following is false.

- 4 * dHorX > -4 * pelScale and 4 * dHorX < pelScale

- 4 * dHorY > - pelScale and 4 * dHorY < 4 * pelScale

- 4 * dVerX > - pelScale and 4 * dVerX < 4 * pelScale

- 4 * dVerY > -4 * pelScale and 4 * dVerY < pelScale

- 4 * dHorX + 4 * dVerX > -4 * pelScale and 4 * dHorX + 4 * dVerX < pelScale

- 4 * dHorY + 4 * dVerY > -4 * pelScale and 4 * dHorY + 4 * dVerY < pelScale

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The luma motion vector mvLX[ xSbIdx ][ ySbIdx ] is derived as follows :

xPosCb = 2 + ( xSbIdx << 2 )		(8-776)

yPosCb = 2 + ( ySbIdx << 2 )		(8-777)

xPosCb = use8x8SubBlock ? (xPosCb >> 3) << 3 + 4

yPosCb = use8x8SubBlock ? (yPosCb >> 3) << 3 + 4

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = ( mvScaleHor + dHorX * xPosCb + dHorY * yPosCb )	(8-778)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = ( mvScaleVer + dVerX * xPosCb + dVerY * yPosCb )	(8-779)

- The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to  mvLX[ xSbIdx ][ ySbIdx ], rightShift set equal to 7, and leftShift set equal to 0 as inputs and the rounded  mvLX[ xSbIdx ][ ySbIdx ] as output.

- The motion vectors mvLX[ xSbIdx ][ ySbIdx ] are clipped as follows:

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 0 ] )	(8-780)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = Clip3( −217, 217 − 1, mvLX[ xSbIdx ][ ySbIdx ][ 1 ] )	(8-781)

For xSbIdx = 0..numSbX − 1 and ySbIdx = 0..numSbY − 1, the following applies:

- The average luma motion vector mvAvgLX is derived as follows:

mvAvgLX = mvLX[ ( xSbIdx >> 1 << 1) ][ (ySbIdx>>1<<1) ] + 	(8-782)	mvLX[ ( xSbIdx >> 1 << 1 ) + 1 ][ (ySbIdx>>1<<1) + 1 ]

mvAvgLX[ 0 ] = mvAvgLX[ 0 ] >= 0  ?  ( mvAvgLX[ 0 ] + 1 ) >> 1  :  	(8-783)	−( ( −mvAvgLX[ 0 ] + 1 ) >> 1 )

mvAvgLX[ 1 ] = mvAvgLX[ 1 ] >= 0  ?  ( mvAvgLX[ 1 ] + 1 ) >> 1  :  	(8-784)	−( ( −mvAvgLX[ 1 ] + 1 ) >> 1 )

- The derivation process for chroma motion vectors in clause 8.5.2.13 is invoked with mvAvgLX and refIdxLX as inputs, and the chroma motion vector mvCLX[ xSbIdx ][ ySbIdx ] as output.

[Ed. (BB): This way four 2x2 chroma subblocks (4x4 chroma block) share the same motion vector which is derived from the average of two 4x4 luma subblock motion vectors. In the decoding process motion compensation is still performed on 2x2 chroma blocks which is however a motion compensation on a chroma 4x4 block because all chroma MVs inside a 4x4 chroma block are the same. I would prefer an editorial change that makes it more clear that affine chroma MC is performed on 4x4 chroma blocks.]

Text for test 2

##### Luma sample 4-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

–	a list padVal[ dir ] with dir = 0,1 specifying reference sample padding direction and amount.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to 2*xFracL or  2*yFracL are specified in Table 8-10.

The luma interpolation filter coefficients fPadL[ 0 ] are set equal to fL[ 2 * xFracL ] and fPadL[ 1 ] are set equal to fL[ 2 * yFracL ] and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-841)

fPadL[ dir ][ 0 ] = 0		(8-842)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ dir ][ 2 ] + fPadL[ dir ][ 3 ]		(8-843)

fPadL[ dir ][ 3 ] = 0		(8-844)

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) .

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

xInti = sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWL, xIntL + i − 1 ) :	(8-845)
	 Clip3( 0, picWL − 1, xIntL + i − 1 )

yInti = Clip3( 0, picHL − 1, yIntL + i − 1 )		(8-846)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracL and yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt1 ][ yInt1 ] << shift3	(8-847)

–	Otherwise if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=03fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yInt1 ]   >>  shift1	(8-848)

–	Otherwise if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=03fPadC[ 1 ][ i ] * refPicLXL[ xInt1 ][ yInti ]   >>  shift1 	(8-849)

–	Otherwise if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..3, is derived as follows:

temp[ n ] =  i=03fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-850)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =( fPadL[ 1][ 0 ] * temp[ 0 ] +
					 fPadL[ 1 ][ 1 ] * temp[ 1 ] +
					 fPadL[ 1 ][ 2 ] * temp[ 2 ] +		(8-851)
					 fPadL[ 1 ][ 3 ] * temp[ 3 ] ) >> shift2

### 8.5.7Decoding process for inter blocks

#### 8.5.7.1General

This process is invoked when decoding a coding unit coded in inter prediction mode.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- variables numSbX and numSbY specifying the number of luma coding subblocks in horizontal and vertical direction,

- the motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the refined motion vectors refMvL0[ xSbIdx ][ ySbIdx ] and refMvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0 .. numSbX − 1, and ySbIdx = 0 .. numSbY − 1,

- the bi-prediction weight index gbiIdx,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an array predSamples of prediction samples.

The variable use4TapFilter is set equal to false.

If  MotionModelIdc[ xCb ][ yCb ] is equal to 1 and predFlagL0[xCb][yCb] and predFlagL1[xCb][yCb] are both equal to 1, the following applies:

- The variables log2CbW and log2CbH are derived as follows:

log2CbW = Log2( cbWidth )		(8-766)

log2CbH = Log2( cbHeight )		(8-767)

- The variables dHorX and dVerX are derived as follows:

dHorX = ( cpMvLX[ 1 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbW ) 	(8-770)

dVerX = ( cpMvLX[ 1 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbW ) 	(8-771)

- The variables dHorY and dVerY are derived as follows:

  - If numCpMv is equal to 3, the following applies:

dHorY = ( cpMvLX[ 2 ][ 0 ] − cpMvLX[ 0 ][ 0 ] ) << ( 7 − log2CbH ) 	(8-772)

dVerY = ( cpMvLX[ 2 ][ 1 ] − cpMvLX[ 0 ][ 1 ] ) << ( 7 − log2CbH ) 	(8-773)

  - Otherwise ( numCpMv is equal to 2), the following applies:

dHorY = − dVerX		(8-774)

dVerY = dHorX		(8-775)

  - The variable pelScale is set equal to (1<<7)<< 4.

  - The variable use4TapFilter is set equal to true if any of the following is false.

    - 4 * dHorX > -4 * pelScale and 4 * dHorX < pelScale

    - 4 * dHorY > - pelScale and 4 * dHorY < 4 * pelScale

    - 4 * dVerX > - pelScale and 4 * dVerX < 4 * pelScale

    - 4 * dVerY > -4 * pelScale and 4 * dVerY < pelScale

    - 4 * dHorX + 4 * dVerX > -4 * pelScale and 4 * dHorX + 4 * dVerX < pelScale

    - 4 * dHorY + 4 * dVerY > -4 * pelScale and 4 * dHorY + 4 * dVerY < pelScale

#### 8.5.7.3 Fractional sample interpolation process

##### 8.5.7.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-811)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-812)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-813)

xFracL = refMvLX[ 0 ] & 15		(8-814)

yFracL = refMvLX[ 1 ] & 15		(8-815)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.7.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - bdofFlag is equal to TRUE.

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + .

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

If use4TapFilter is equal to false the following applies:

- The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.7.3.2 with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, and padVal as inputs.

Otherwise the following applies:

- The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 4-tap interpolation filtering process as specified in clause 8.5.7.3.x with ( xIntL, yIntL ), ( xFracL, yFracL ), refPicLX, and padVal as inputs.