| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0256 |
| --- | --- |

| Title: | CE2: Worst-case Memory Bandwidth Reduction for affine (Test 2-4.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Luong Pham Van, Wei-Jung Chien and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, lphamvan, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the results of CE2 test 2-4.5. Three approaches to reduce worst-case memory bandwidth of affine motion compensation are tested. The summary of Y, U, V BD-rate change in common test conditions are shown in the following:

- Switch subblock size:  0.04%, -0.06%, -0.03% in RA, 0.00%, 0.21%, -0.22% in LDB

- Switch interpolation filter:  0.07%, 0.05%, 0.07% in RA, 0.02%, -0.09%, -0.10% in LDB

- Encoder bitstream conformance:  0.03%, 0.01%, 0.06% in RA, 0.04%, 0.09%, -0.28% in LDB

Two supplemental test results for switch subblock size and switch interpolation filter are also reported, in which the affine restriction formula in CE2 test 2.4.8 is used. The results are:

- Switch subblock size:  0.00%, 0.00%, 0.02% in RA, 0.00%, -0.06%, -0.07% in LDB

- Switch interpolation filter:  -0.01%, -0.01%, -0.01% in RA, -0.01%, 0.06%, 0.12% in LDB

# Introduction

In JVET-M0400 [1], two different approaches are proposed to reduce the worst-case memory bandwidth of motion compensation for affine motion model. The first solution (switch block size) is to restrict bi-predictions that have memory bandwidth exceeding a predefined level. If the memory bandwidth exceeds the predefined level, a larger subblock is used. In the second solution (switch filter), the luma interpolation may be performed with the chroma interpolation filter to ensure the memory bandwidth requirement. These two approaches and an encoder bitstream conformance approach for bandwidth reduction of affine motion compensation are studied in CE2 test 2-4.5.

# CE2 test 2-4.5

In CE2 test 2-4.5, the memory bandwidth of affine motion compensation is restricted to bi-prediction of a 9x9 block. Since the motion vectors of every 4 4x4 subblocks (formed an 8x8 block) in an affine block have the same relation that follow the affine motion model, the restriction can be described as to 4 4x4 subblocks yet it applies to the affine block.

Considering the 4 4x4 subblocks in an 8x8 affine coded block, vi=vix,viy, i=0, 1,2 are the 3 control-point MVs and

Normv1x-v0x=v1x-v0x*128/w

Normv1y-v0y=v1y-v0y*128/w

Normv2x-v0x=v2x-v0x*128/h

Normv2x-v0x=v2x-v0x*128/h

w and h are the width and height of the block, respectively.

To ensure that the bandwidth of 4 4x4 subblocks’ motion compensation is less than or equal to that of a 9x9 block, the motion vectors needs to be confined. Let the motion vectors of the top-left 4x4 subblock be the reference motion vector, the motion vector differences of the reference motion vector and the motion vectors of other 3 4x4 subblocks can be derived. The motion differences define the bounding box, which is a 9x9 block in this proposal, to guarantee the targeted memory bandwidth.

the following constrain are imposed when it’s bi-prediction:

- The top-right 4x4 subblock should not move to the left and reach the left boundary and should not move to the right and reach one sample beyond the right boundary, that is:

(4*Normv1x-v0x> -4*pel &&+4*Normv1x-v0x < pel)

- The top-right 4x4 subblock should not move to the top and reach one sample beyond the above boundary and should not move to the bottom and reach the bottom boundary, that is:

(4*Normv1y-v0y > -pel && 4*Normv1y-v0y < 4*pel)

- The bottom-left 4x4 subblock should not move to the left and reach the left boundary and should not move to the right and reach one sample beyond the right boundary, that is:

4*Normv2x-v0x> -pel &&4*Normv2x-v0x < 4*pel

- The bottom-left 4x4 subblock should not move to the top and reach the above boundary and should not move to the bottom and reach one sample beyond the bottom boundary, that is:

4*Normv2y-v0y> -4*pel && 4*Normv2y-v0y < pel

- The bottom-right 4x4 subblock should not move to the left and reach the left boundary and should not move to the right and reach one sample beyond the right boundary, that is:

(4*Normv1x-v0x + 4*Normv2x-v0x> -4*pel&&

(4*Normv1x-v0x + 4*Normv2x-v0x< pel))

- The bottom-right 4x4 subblock should not move to the top and reach the above boundary and should not move to the bottom and reach one sample beyond the bottom boundary, that is:

(4*Normv1y-v0y + 4*Normv2y-v0y> -4*pel&&

(4*Normv1y-v0y + 4*Normv2y-v0y< pel))

If the affine coded block is bi-prediction and the above condition is not true, 3 approaches are tested in CE2 test 2-4.5. In the first approach, the subblock size for affine motion compensation is set to 8x8. In the second approach, the interpolation filter for affine motion compensation is switched to 4-tap chroma filter. The rationale behind this is: the correlation between pixels is less under high deformed motion, therefore, it’s not necessary to use long tap interpolation filter. In the third approach, the affine prediction mode is discarded at encoder.

# Complexity analysis

Table 1 Complexity analysis of proposed method

| Bandwidth measurement for uni/bi-prediction | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size |
| --- | --- | --- | --- |
| - /11.06 | - Same - Same - Same | - Same - Same - Same | 13 comparisons per CU (1 for uni/bi + 12 for restriction) |

# Test results

Three tests using the 3 approaches are conducted. Two supplemental results are also provided, in which the constrain condition in CE2 test 2-4.8 [2] is used for the first and second approaches. The summary result is shown in Table 2. Detailed test results are shown in Table 3 to Table 7. Runtime is not accurate for Class A in RA.

Table 2 Summary results of CE2 test 2-4.5

| Tests | RA BD-rate (Y, U, V) | LDB BD-rate (Y, U, V) |
| --- | --- | --- |
| 1. switch subblock size | 0.04%, -0.06%, -0.03% | 0.00%, 0.21%, -0.22% |
| 2. Switch filter | 0.07%, 0.05%, 0.07% | 0.02%, -0.09%, -0.10% |
| 3. Encoder bitstream conformance | 0.03%, 0.01%, 0.06% | 0.04%, 0.09%, -0.28% |
| 4. 1+ CE2.4.8 equation | 0.00%, 0.00%, 0.02% | 0.00%, -0.06%, -0.07% |
| 5. 2 + CE2.4.8 equation | -0.01%, -0.01%, -0.01% | -0.01%, 0.06%, 0.12% |

Table 3 Test 1 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.04% | 109% | 116% |
| Class A2 | 0.13% | 0.04% | 0.08% | 101% | 101% |
| Class B | 0.01% | -0.15% | -0.07% | 102% | 102% |
| Class C | 0.01% | -0.11% | -0.11% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.06% | -0.03% | 102% | 104% |
| Class D | -0.01% | -0.07% | -0.09% | 97% | 97% |
| Class F | 0.07% | 0.12% | 0.06% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.12% | 100% | 101% |
| Class C | -0.03% | 0.27% | 0.00% | 100% | 95% |
| Class E | 0.03% | 0.42% | -0.69% | 101% | 103% |
| Overall | 0.00% | 0.21% | -0.22% | 100% | 100% |
| Class D | -0.02% | 0.13% | -0.49% | 101% | 98% |
| Class F | -0.18% | 0.23% | -0.67% | 102% | 102% |

Table 4 Test 2 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.02% | 0.13% | 110% | 115% |
| Class A2 | 0.21% | 0.25% | 0.31% | 93% | 100% |
| Class B | 0.03% | 0.03% | -0.02% | 102% | 102% |
| Class C | 0.02% | -0.04% | -0.02% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.05% | 0.07% | 101% | 104% |
| Class D | 0.01% | -0.04% | -0.01% | 99% | 101% |
| Class F | 0.08% | 0.10% | 0.08% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.19% | -0.02% | 100% | 101% |
| Class C | 0.00% | 0.27% | -0.03% | 100% | 101% |
| Class E | 0.06% | -0.41% | -0.33% | 101% | 101% |
| Overall | 0.02% | -0.09% | -0.10% | 100% | 101% |
| Class D | -0.06% | 0.28% | -0.34% | 101% | 101% |
| Class F | -0.11% | -0.10% | -0.12% | 102% | 103% |

Table 5 Test 3 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | 0.11% | 112% | 118% |
| Class A2 | 0.05% | 0.02% | 0.13% | 100% | 100% |
| Class B | 0.03% | 0.07% | 0.01% | 102% | 102% |
| Class C | 0.02% | -0.05% | 0.05% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.01% | 0.06% | 103% | 104% |
| Class D | -0.01% | -0.02% | 0.01% | 98% | 98% |
| Class F | 0.02% | 0.03% | 0.00% | 102% | 102% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.02% | -0.27% | 100% | 97% |
| Class C | 0.03% | 0.23% | 0.02% | 100% | 97% |
| Class E | 0.09% | 0.09% | -0.67% | 101% | 102% |
| Overall | 0.04% | 0.09% | -0.28% | 100% | 98% |
| Class D | 0.01% | 0.18% | -0.39% | 101% | 105% |
| Class F | -0.16% | -0.05% | 0.02% | 102% | 104% |

Table 6 Test 4 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 102% | 100% |
| Class A2 | -0.01% | -0.03% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.03% | 0.04% | 102% | 102% |
| Class C | 0.01% | -0.05% | 0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.02% | 101% | 102% |
| Class D | -0.01% | -0.02% | 0.01% | 100% | 102% |
| Class F | 0.01% | 0.00% | -0.02% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.21% | -0.16% | 100% | 100% |
| Class C | -0.03% | 0.22% | 0.02% | 101% | 100% |
| Class E | 0.05% | -0.18% | -0.06% | 101% | 101% |
| Overall | 0.00% | -0.06% | -0.07% | 100% | 100% |
| Class D | -0.08% | -0.39% | -0.64% | 100% | 98% |
| Class F | -0.12% | -0.14% | 0.17% | 101% | 103% |

Table 7 Test 5 results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.10% |  | 0.04% |  | 102% | 100% |  |
| Class A2 | -0.02% | 0.05% | 0.00% | 101% | 101% |
| Class B | -0.01% | -0.01% | -0.03% | 102% | 103% |
| Class C | -0.01% | 0.01% | -0.04% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Class D | 0.00% | -0.01% | -0.04% | 100% | 101% |
| Class F | -0.01% | -0.05% | -0.05% | 101% | 101% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.05% | 0.08% | 100% | 99% |
| Class C | -0.03% | 0.23% | 0.37% | 101% | 101% |
| Class E | -0.02% | 0.00% | -0.14% | 100% | 99% |
| Overall | -0.01% | 0.06% | 0.12% | 100% | 100% |
| Class D | 0.01% | -0.15% | -0.27% | 100% | 100% |
| Class F | -0.11% | -0.19% | -0.66% | 101% | 104% |

# References

- W.-J. Chien, L. Pham Van, H. Huang, et al. “CE2-related: Worst-case memory bandwidth reduction for VVC”, JVET-M0400, the 13th JVET meeting, Jan. 2019, Marrakesh, MA.

- M. Zhou, “CE2: On restriction of memory bandwidth consumption of affine mode (CE2-4.8)”, JVET-N0068, the 14th JVET meeting, Mar. 2019, Geneva, CH.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).