| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0270 |
| --- | --- |

| Title: | CE9-2.3: Modifications on Bi-Directional Optical Flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Jizheng Xu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of CE9-2.3. In current VTM [1], Bi-directional Optical Flow (BDOF) could be performed for large CUs like 128*128, 128*64 and 64*128 CUs, which requires large memory. Meanwhile, in early termination stage of BDOF, Sum of Absolute Difference (SAD) between two reference blocks shall be calculated, which increases the worst-case computational complexity. This contribution proposes to modify BDOF in two aspects: first, when a CU contains more than 256 luma pixels, it is split into sub-blocks and each sub-block contains 256 luma pixels. BDOF is performed for each sub-block. Second, SAD is calculated only using four corner pixels of each 4*4 sub-block. It is asserted that the memory required in BDOF is reduced by 99.7% and number of operations required in SAD calculation is also reduced by approximately 75%. Simulation results reportedly show negligible impact on compression performance.

# Introduction

Current VTM employs BDOF to improve prediction of bi-predicted CUs. BDOF could be applied to large CUs like 128*128, 128*64 and 64*128 CUs, which requires large memory. Meanwhile, an early termination method is used in BDOF to reduce the average decoding complexity. SAD between two reference blocks of the CU is calculated as follows: first, SAD is calculated for each 4*4 sub-block, then SAD of all 4*4 sub-blocks is summed up. If SAD between the two reference blocks is smaller than a first threshold, BDOF is skipped for the entire CU; otherwise, for each 4*4 sub-block, if SAD between the two reference 4*4 blocks is smaller than a second threshold, BDOF is skipped for the 4*4 sub-block. Though the early termination method reduces the average decoding complexity, it increases the worst-case computational complexity.

# Proposed method

To address the problems mentioned above, two methods are proposed [2]:

- Method A: If a CU contains more than 256 luma pixels, it is split into sub-blocks and each sub-block contains 256 luma pixels. BDOF is performed for each sub-block.

- Method B: SAD is calculated using the 4 corner pixels instead of all 16 pixels for each 4*4 sub-block.

In this way, the memory required in BDOF is dramatically reduced by 99.7%. Meanwhile, operations required in SAD calculation is also dramatically reduced, by approximately 75%.

Note that when both DMVR and BIO are allowed, both will be performed at sub-block level in current VTM. Therefore, proposed method is applied only when DMVR is disallowed and BIO is allowed.

# Experimental results

The proposed methods were implemented on top of reference software VTM-4.0 [1], and simulations were performed following the common test conditions [3]. Results are provided for the following three tests:

- Test #1: Method A.

- Test #2: Method B.

- Test #3: Test #1 + Test #2.

The results are shown in Table 1~Table 3.

Table 1: Coding performance of Test #1

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | 102% | 102% |
| Class A2 | 0.00% | 0.01% | -0.01% | 101% | 100% |
| Class B | 0.02% | -0.02% | -0.01% | 100% | 102% |
| Class C | 0.01% | -0.02% | -0.06% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.02% | 100% | 100% |
| Class D | 0.09% | -0.07% | 0.04% | 98% | 94% |
| Class F | 0.00% | 0.01% | 0.01% | 98% | 98% |

Table 2: Coding performance of Test #2

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class B | 0.01% | -0.03% | -0.01% | 99% | 100% |
| Class C | 0.02% | -0.06% | -0.07% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.03% | 99% | 100% |
| Class D | 0.03% | -0.04% | -0.11% | 96% | 94% |
| Class F | 0.01% | -0.01% | 0.02% | 97% | 97% |

Table 3: Coding performance of Test #3

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.01% | 102% | 101% |
| Class A2 | 0.02% | 0.04% | 0.03% | 100% | 100% |
| Class B | 0.04% | -0.04% | -0.03% | 99% | 100% |
| Class C | 0.02% | -0.03% | -0.07% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | -0.02% | 99% | 99% |
| Class D | 0.09% | -0.08% | -0.01% | 97% | 91% |
| Class F | -0.01% | -0.02% | 0.01% | 102% | 101% |

# Conclusions

This contribution reports results of CE9-2.3, which can dramatically reduce the memory required in BDOF and the operations required in SAD calculation. Meanwhile, it has negligible impact on compression performance. It is recommended to adopt this proposal into VVC.

# References

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “Non-CE9: Modifications on Bi-Directional Optical Flow”, H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong, Doc. JVET-M0249, Jan. 2019.

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).