| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0354 |
| --- | --- |

| Title: | AHG12: conformance of temporally independent tile groups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Valeri George Karsten Suehring Thomas Schierl | Email: | {forename.surname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes a bitstream extraction process for temporally independent tile groups (or MCTS in HEVC terms), the corresponding HRD text for extracted temporally independent tile groups and the relevant syntax and semantics that define output tile sets for which HRD parameters for conformance of independent tile groups are indicated.

# Introduction

HEVC specifies conformance of MCTS through means of the MCTS extraction process as defined in the MCTS extraction information set SEI message semantics. This extraction process relies on adapting slice header (slice segment addresses and byte alignment) and replacing parameter sets and SEI messages through the ones encapsulated in the related MCTS-nested SEI messages.

Conformance of temporally independent tile groups (or MCTS in HEVC terms) is a desirable feature for VVC and the current proposal defines conformance in a similar manner as for layered video coding in HEVC, i.e. by means of output tiles sets (similar to output layer sets).

# Proposal

This document proposes to specify tile output sets in VVC for which individual sets of HRD parameters are signalled in the parameter sets. By doing so, VVC will allow conformance signalling for temporally independent tile groups in similar fashion than HEVC handles conformance for layer-wise coding.

Similar to HEVC; it is envisioned that extraction of temporally independent tiles should only be possible at NAL unit granularity. For VVC; this means that extraction is to be carried out on tile group granularity.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| [ … ] |  |
| tile_output_sets_present_flag | u(1) |
| if( tile_output_sets_present_flag ) { |  |
| tile_group_id_length_minus1 | ue(v) |
| num_ots_format_sets | ue(v) |
| for( i = 0; i < num_ots_format_sets – 1 ; i++ ){ |  |
| ots_width_in_luma_samples[ i ] | ue(v) |
| ots_height_in_luma_samples[ i ] | ue(v) |
| profile_tier_level(sps_max_sub_layers_minus1)[ i ] |  |
| } |  |
| num_output_tile_sets | ue(v) |
| for( i = 0; i < num_output_tile_sets – 1 ; i++ ){ |  |
| num_tiles_in_output_tile_set[ i ] | ue(v) |
| ots_format_set_idx[ i ] | ue(v) |
| for( j = 0; j < num_tile_groups_in_output_tile_set – 1 ; j++ ){ |  |
| tile_group_id_in_sps[ i ][ j ] | u(v) |
| } |  |
| hrd_parameters(1, sps_max_sub_layers_minus1) |  |
| } |  |
| [ … ] |  |
| rbsp_trailing_bits( ) |  |
| } |  |

tile_output_sets_present_flag specifies the presences of tile output sets. When tile_output_sets_present_flag is equal to 1, the SPS contains output tile set formats and HRD parameter for each output tile set. When tile_output_sets_present_flag is equal to 0, no output tile sets are defined.

tile_group_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax element tile_group_id_in_sps[ i ][ j ]. The value of tile_group_id_length_minus1 shall be in the range of 0 to 15, inclusive.

num_ots_format_sets specifies the number of output tile set format sets present in the bitstream. The value of num_ots_format_sets shall be in the range of 1 to 512. When not present, num_ots_format_sets is inferred to be equal to 0.

ots_width_in_luma_samples[ i ] specifies the width in luma samples of the i-th output tile set format.

ots_height_in_luma_samples[ i ] specifies the height in luma samples of the i-th output tile set format.

profile_tier_level()[ i ] specifies the profile, tier and level of the i-th output tile set format.

num_output_tile_sets specifies the number of output tile sets present in the bitstream. The value of num_output_tile_sets shall be in the range of 1 to 512. When not present, num_output site_sets is inferred to be equal to 0.

num_tile_groups_in_output_tile_set[ i ] specifies the number of tile groups in the i-th output tile set present in the bitstream. The value of num_tile_groups_in_output_tile_set[ i ] shall be in the range of 1 to 512. When not present, num_output site_sets is inferred to be equal to 0.

output_tile_set_format_set_idx[ i ] specifies the index of the output tile set format of the i-th output tile set. The value of num_output_tile_sets shall be in the range of 1 to num_ots_format_sets – 1

tile_group_id_in_sps[ i ][ j ] specifies the tile group identifier in the tile group header of the j-th tile group belonging to the i-th tile output set.

## Extraction process

Inputs to this process are a bitstream, and a target highest TemporalId value tIdTarget and a target tile group identifier list tileGroupIdListTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, tIdTarget equal to any value in the range of 0 to 6, inclusive, and tileGroupIdListTarget either equal to the tile group identifier list associated with a tile output set specified in the active SPS or consisting of all the tile_group_id values of the VCL NAL units present in the input bitstream as inputs, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with tile_group_id equal to every value in tileGroupIdListTarget.

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded tile group NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget or tile_group_id not equal to any value in tileGroupIdListTarget.

## HRD

The HRD relevant changes compared to the HRD starting point text proposed in JVET-N0350 are shown below:

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this clause:

- An operation point under test, denoted as TargetOp, is selected by selecting a tile identifier list OpTileGroupIdList and a target highest TemporalId value OpTid. The tile identifier list OpTileGroupIdList of TargetOp either consists of all the tile_group_id values of the VCL NAL units present in entireBitstream or consists of all the tile_group_id values of a output tile set specified by the active SPS. The value of OpTid is in the range of 0 to sps_max_sub_layers_minus1, inclusive. The values of OpTileGroupIdList and OpTid are such that the sub-bitstream BitstreamToDecode that is the output by invoking the sub-bitstream extraction process as specified in clause 10 with entireBitstream, OpTid and OpTileGroupIdList as inputs satisfy both of the following conditions:

–	There is at least one VCL NAL unit in BitstreamToDecode with tile_group_id equal to each of the tile_group_id values in OpTileGroupIdList.

–	There is at least one VCL NAL unit with TemporalId equal to OpTid in BitstreamToDecode.

- TargetDecTileGroupIdList is set equal to OpTileGroupIdList of TargetOp and HighestTid is set equal to OpTid of TargetOp.

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected. If TargetDecTileGroupIdList contains all tile_group_id values present in entireBitstream, the hrd_parameters( ) syntax structure in the active SPS (or provided through an external means not specified in this Specification) is selected. Otherwise, the hrd_parameters( ) syntax structure in the active SPS (or provided through some external means not specified in this Specification) that applies to TargetOp is selected. Within the selected hrd_parameters( ) syntax structure, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) is selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- An access unit associated with a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as access unit 0.

- For each access unit in BitstreamToDecode starting from access unit 0, the buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and the picture timing SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected. The selected buffering period, picture timing and decoding unit information SEI messages shall be either non-scalable-nested SEI messages or provided by external means.

- A value of SchedSelIdx is selected. The selected SchedSelIdx shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where cpb_cnt_minus1[ HighestTid ] is found in the hrd_parameters( ) syntax structure as selected above.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 *  n2 , where the values of n0, n1 and n2 are specified as follows:

–	n0 is derived as follows:

–	If BitstreamToDecode is a Type I bitstream, n0 is equal to 1.

–	Otherwise (BitstreamToDecode is a Type II bitstream), n0 is equal to 2.

–	n1 is equal to cpb_cnt_minus1[ HighestTid ] + 1.

–	n2 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp

# Discussion

Note that currently the tiling syntax is indicated in the PPS. In HEVC, when MCTS was used and MCTS extraction was enabled, tiles_fixed_structure_flag was required to be equal to 1 in the VUI. Similar constraints would be desirable for VVC.

It is asserted that a cleaner option is to include the tiling syntax into the SPS for use-cases where MCTS and extraction thereof is desired. This is not part of the contribution but discussion is encouraged on whether such a direction would be desirable.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).