| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0364-v2 |
| --- | --- |

| Title: | Non-CE6: Further cost reduction for primary transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes several methods to reduce the worst-case multiplications per coefficient for primary transform in VVC, which is 32x32 and 32x64 transform using DCT-2. With the proposed method, the worst-case multiplication per coefficient is reportedly reduced by 25%. Experimental results reports 0.02%, 0.03% and 0.xx% luma BD rate for AI, RA and LDB configurations under Common Test Condition (CTC).

# Problem Statement

In Marrakesh meeting, a zero-out scheme for 32-point DST-7 and DCT-8 [1] was adopted which reduces the worst-case multiplications per coefficient on primary transform. According to the analysis in JVET-M0046 [2], for an MxN transform unit with zero-out, if only the top-left mxn region kept non-zero transform coefficients for, the number of multiplications per coefficient using the direct matrix multiply can be computed by

Muls_per_coeffdirectMatrixMultiply=mMn+m

The number of multiplications per coefficient for DCT-2 using half butterfly and half direct matrix multiply can be computed by

Muls_per_coeffhalfButterFly=mM(n2+log2n2)+(m2+log2m2)

Based on the above analysis, in VVC Draft 4, the worst-case multiplications per coefficient becomes 32x32 and 32x64 DCT-2, which requires around 25% more multiplications than the second worst-case (16x32 transform using DST-7 and DCT-8 combination). Since 32x32 DCT-2 and 32x64 DCT-2 are only two specific cases of all supported transform size and type combinations, it is further investigated in this contribution whether the worst-case multiplication per sample can be further reduced without big impact on coding performance.

# Proposal

## Method 1

For 32x64 transform using DCT-2, it is proposed to switch the horizontal and vertical transform, such that 64-length vertical DCT-2 transform using zero-out is performed first, followed by the 32-length full size DCT-2.

For the 32x32 transform using DCT-2, it is proposed to keep the non-zero coefficients for the top half 32x16 region, i.e., a zero-out transform (keeps the first 16 coefficients) is applied only for the vertical transform, while the horizontal transform is still a full-size transform.

## Method 2

For 32x64 transform using DCT-2, it is proposed to keep the non-zero coefficients for the top half 32x16 region, i.e., a zero-out transform (keeps the first 16 coefficients) is applied only for the vertical 64-length DCT-2 transform, while the horizontal transform is still a full-size transform.

For the 32x32 transform using DCT-2, the same approach defined in above method 1 is applied.

# Experiment results

The number of multiplications per coefficient for 32x32 and 32x64 is calculated according to the formula in Section 1, it is reportedly shown that, with proposed changes, the worst-case multiplication per coefficient of primary transform is reduced from 64 to 48 based on direction matrix multiply, and 48 to 32 based on half butterfly, which is 25% reduction.

Table 1: Multiplications per coefficient for 32x32 and 32x64 DCT-2.

|  | Multiplications per coefficient (Direct matrix multiply) |  |  | Multiplications per coefficient (Half butterfly) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VTM-4.0 | Method 1 | Method 2 | VTM-4.0 | Method 1 | Method 2 |
| 32x32 DCT-2 | 64 | 48 |  | 40 | 30 |  |
| 32x64 DCT-2 | 64 | 48 | 48 | 40 | 30 | 31 |

Three experiments are performed using different configuration of max transform size after applying the proposed fixes on VTM-4.0. The experiment is performed under CTC condition [3] except for the different settings of max transform size. The anchor for all the experiments is VTM-4.0. Detailed results for each experiment are reported in Table 1, Table 2 and Table 3, respectively.

Table 1: Coding performance of Method 1

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.00% | 0.01% | 100% | 98% | 0.01% | -0.01% | 0.03% | 101% | 102% |  |  |  |  |  |
| Class A2 | 0.03% | 0.16% | 0.08% | 101% | 101% | 0.02% | 0.10% | -0.04% | 102% | 102% |  |  |  |  |  |
| Class B | 0.02% | 0.08% | 0.06% | 100% | 100% | 0.04% | -0.01% | -0.15% | 101% | 101% |  |  |  |  |  |
| Class C | 0.01% | 0.03% | 0.02% | 100% | 101% | 0.05% | -0.02% | -0.05% | 101% | 102% | 0.08% | -0.07% | -0.23% | 100% | 102% |
| Class E | 0.01% | 0.04% | -0.05% | 100% | 100% |  |  |  |  |  | -0.08% | -0.25% | 0.28% | 101% | 103% |
| Overall | 0.02% | 0.06% | 0.03% | 100% | 100% | 0.03% | 0.01% | -0.06% | 101% | 102% |  |  |  |  |  |
| Class D | 0.03% | 0.03% | 0.01% | 103% | 104% | 0.06% | -0.24% | -0.12% | 101% | 103% | 0.07% | 0.10% | -0.71% | 101% | 100% |
| Class F | 0.00% | -0.03% | -0.05% | 101% | 101% | 0.02% | 0.11% | -0.04% | 102% | 102% | -0.03% | 0.06% | 0.19% | 100% | 103% |

Table 2: Coding performance of Method 2

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.00% | 0.01% | 102% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.03% | 0.16% | 0.08% | 101% | 101% | 0.03% | 0.06% | -0.05% | 101% | 100% |  |  |  |  |  |
| Class B | 0.02% | 0.08% | 0.05% | 101% | 101% | 0.05% | -0.04% | -0.10% | 100% | 100% |  |  |  |  |  |
| Class C | 0.01% | 0.03% | 0.02% | 104% | 103% | 0.03% | 0.10% | -0.07% | 101% | 103% | 0.10% | -0.03% | -0.03% | 98% | 99% |
| Class E | 0.01% | 0.04% | -0.05% | 99% | 99% |  |  |  |  |  | 0.00% | -0.08% | 0.76% | 104% | 104% |
| Overall | 0.02% | 0.06% | 0.03% | 101% | 101% | 0.03% | 0.02% | -0.06% | 101% | 101% |  |  |  |  |  |
| Class D | 0.03% | 0.03% | 0.01% | 102% | 104% | 0.06% | -0.18% | -0.10% | 101% | 103% | 0.06% | -0.15% | -1.17% | 104% | 104% |
| Class F | 0.00% | -0.03% | -0.05% | 102% | 102% | 0.02% | 0.15% | -0.03% | 101% | 102% | -0.10% | -0.32% | -0.04% | 98% | 101% |

# Conclusions

This proposal provides further cost reduction on primary transform. With the proposed method, it is reportedly shown that the worst-case multiplication per sample is reduced by 25% comparing to VTM-4.0. It is recommended to adopt the proposed changes into next version of VVC draft.

# Spec text

#### 8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are both equal to 0 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, trTypeHor and trTypeVer are specified in Table 8-15 depending on intraPredMode.

- Otherwise, if cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 8-14 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (sps_explicit_mts_intra_enabled_flag and sps_explicit_mts_inter_enabled_flag are equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1030)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1031)

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-13 depending on tu_mts_idx[ xTbY ][ yTbY ].

The variables nonZeroW and nonZeroH are derived as follows:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(8-1032)

nonZeroH = Min( nTbH, ( trTypeVer > 0  | | ( nTbW= =32 && nTbH > = 32 ) )  ?  16  :  32 )	(8-1033)

# References

- M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6-related: 32 point MTS based on skipping high frequency coefficients,” JVET-M0297, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- M. Zhou, Y. Hu, “CE6-related: A study of primary transforms,” JVET-M0046, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).