| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0424-v1 |
| --- | --- |

| Title: | CE6-related: Simplification of MTS for intra-CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao-Hsiung Hung, Hilmi E. Egilmez, Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-658-1293 chaohsiu@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to replace DCT-8 in MTS by identity transform (IDT) or DCT-2 in intra-CUs based on a pre-defined transform length T. Also, a fast encoding method is introduced. Over VTM-4.0, with common test configuration, the proposed methods show 0.04%, 0.xx% and 0.xx% luma BD-rate, 90%, % and % encoding time in AI, RA and LDB with similar decoding time. When inter MTS is enabled, the proposed methods show 0.xx%, 0.xx% and 0.xx% luma BD-rate, %, % and % encoding time in AI, RA and LDB with similar decoding time over VTM-4.0.

# Introduction

In VTM 4.0, multiple transform selection (MTS) [1] consists of the following 6 transform modes:

- mode 0: DCT 2(H) × DCT 2(V)

- mode 1: Transform skip (TS) = IDT (H) × IDT (V)

- mode 2: DST 7(H) × DST 7(V)

- mode 3: DCT 8(H) × DST 7(V)

- mode 4: DST 7(H) × DCT 8(V)

- mode 5: DCT 8(H) × DCT 8(V)

where H and V denote horizontal and vertical transform, respectively.

The MTS mode 3, 4 and 5 above all include DCT-8. According to JVET-M0366 [2], these 3 modes will be used with low frequency. To simplify MTS, it is proposed to replace DCT-8, that is less frequently used in coding intra-CUs, by other transform cores.

# Proposed methods

It is proposed to replace DCT-8 in modes from 3 to 5 depending on transform length T. Specifically, if the transform length T ≤ 16, DCT-8 is replaced by IDT. Otherwise, DCT-8 is replaced by DCT-2. This approach is only applied to intra CUs.

In addition, two encoder-only methods (see Sections 2.1 and 2.2) are proposed to improve the trade-off between coding performance and encoding complexity.

## Size and mode-dependent scaling factor for L1 norm

In VTM 4.0, for each MTS mode sum of absolute values (L1-norm) of transformed coefficients are calculated for fast encoding. Let S be the calculated L1-norm corresponding to mode 0. For MTS mode of 1 to 5, if the L1 norm is less than X × S, this mode will be included in the candidate list for full R-D checking, where X is a scaling factor that is set as X = 1 for mode 1, and it is in the range of [1.2, 1.5] defined based on maximum transform length of the TU for the rest of the modes

This contribution the threshold X scaled as (X/Y). For MTS mode 1, Y = 1. Otherwise, Y is determined as follows.

If horizontal (resp. vertical) transform is DST-7 or DCT-2 after DCT-8 replacement, YH (resp. YV) is set to1. Otherwise, horizontal (resp. vertical) transform is IDT after DCT-8 replacement and YH (resp. YV) is determined as in Table 2-1. Then Y is set as YH × YV.

Table 2-1. Definition of length dependent YH and YV or

| horizontal or vertical transform length | YH or YV |
| --- | --- |
| 4 | 1.0 |
| 8 | 1.3 |
| 16 | 1.7 |
| 32 | 2.0 |

## Additional RD checks for MTS modes

In VTM 4.0, after checking RD cost of mode 0 and mode 1, if mode 1 is better than mode 0, RD cost of other modes will not be checked. In this contribution, this speed-up algorithm is disabled.

# Experimental Results

The experimental results are presented in the following sections. We show the results of all proposed methods in Section 3.1. and those of only DCT-8 replacement in Section 3.2.

## Experimental results of all proposed methods

Table 3-1-1, Table 3-1-2, and Table 3-1-3 show the results with CTC over VTM-4.0. Table 3-1-4 and Table 3-1-5 show the results with InterMTS enabled. Since AI results of InterMTS enabled are the same as those of CTC, we skip these results.

Table 3-1-1, all proposed methods, CTC, AI

Table 3-1-2, all proposed methods, CTC, RA

Table 3-1-3, all proposed methods, CTC, LDB

Insert table

Table 3-1-4, all proposed methods, InterMTS enabled, RA

Insert table

Table 3-1-5, all proposed methods, InterMTS enabled, LDB

Insert table

## Experimental results of DCT-8 replacement only

Table 3-2-1 shows the AI results of DCT-8 replacement with CTC.

Table 3-2-1, DCT-8 replacement only, CTC, AI

Insert table

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip”, JVET-M0464, Jan. 2019.

- C. Hollman, D. Saffar, P. Wennersten, J. Ström, “CE-6 related: Transform Simplification”, JVET-M0366, Jan, 2019.