| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0489 |
| --- | --- |

| Title: | CE1: Bilateral filter tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jacob Ström, Per Wennersten, Jack Enhorn, Torshamnsgatan 23 164 80 Stockholm Sweden Kevin Reuze, Dmytro Rusanovskyy, Marta Karczewicz Qualcomm Technologies, Inc., 10160 Pacific Mesa Blvd San Diego, CA 92121 (USA) | Tel: Email: Email: | +46702672192 Jacob.strom (at) ericsson.com dmytror@qti.qualcomm.com |
| Source: | Ericsson, Qualcomm |  |  |

_____________________________

# Abstract

This document describes the core experiment tests CE1-1.1 through CE1-1.7 related to bilateral filtering. Test tests span different placements of the filter; as a post-reconstruction filter (tests CE1-1.1 through CE1-1.4, and CE1-1.7) as well as a loop filter (CE1-1.5). It also spans different amounts of precision in the calculations (CE1-1.1 versus CE1-1.2) and parametrization aspects (CE1-1.7).

# Introduction

At the Marrakech meeting it was decided to revisit post-reconstruction filters in CE1. During the meeting implementers were sharing information on desired features of such a filter:

- Intra blocks should not be filtered. Nor should intra-like blocks such as intra block copy (IBC) blocks or combined intra/inter blocks (CIIB) be filtered.

- The smallest filtered block should be no smaller than 64 samples.

- It was preferable that a filter that did not copy samples from intra blocks, IBC blocks or CIIB blocks.

It was decided that all post-reconstruction and post-prediction filters should obey 1 and 2 above, and that there should be three categories to give information on how much it was worth to copy pixels from surrounding blocks. Therefore, any post-reconstruction/post-prediction filter should be evaluated under these three restrictions:

- No restriction on using samples to the left and above the block (mandatory)

- Samples to the left and above can be used, but only if they come from an inter block (i.e., not from an intra block or a CIIP block or an IBC/CPR block). (mandatory)

- No samples outside the block can be used. (non-mandatory)

For loop-filter tests, there would be no restriction on the smallest processed block, no restriction on applying filtering on intra blocks and no restriction on using samples outside the block inside the same CTU.

This document will go through each CE test and describe the results. When the above restrictions apply, the tests are named accordingly. Hence, as an example, CE1-1.1c uses no samples outside the block.

# CE1-1.1a, CE1-1.1b and CE1-1.1c

These three tests use a bilateral filter as a post-reconstruction filter. Hence after the reconstruction is formed by adding the prediction to the residual and clipping, the filter is applied. To filter the intensity IC of the current sample, the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used.

|  | IA |  |
| --- | --- | --- |
| IL | IC | IR |
|  | IB |  |

Figure 1. Samples used in the weighted sum.

The filtered value IF is then calculated as a weighted sum of the surrounding delta-intensity values as

IF=IC+wAΔIA+wBΔIB+wLΔIL+wRΔIR+c1≫c2,     (Eqn 2.1)

where the deltas are the difference against the center sample, ΔIA=IA-IC etc. The weights are calculated as

wA=max0, m-k*dNLA.      (Eqn 2.2)

Here, k and m depend on the qp and block size, and dNLA is a non-local average of absolute differences in a 3×3 neighborhood as described in JVET-J0021 section 4.

In JVET-J0021, k could be 816. This was changed in JVET-M0885 so that k always equals a three-bit number 1ab where the first bit is always one. Hence k*dNLA can be implemented in hardware using two additions

k*dNLA=dNLA≪2+a & dNLA≪1+b & dNLA.     (Eqn 2.3)

Alternatively, one can see this as a two-bit multiplication and an addition

k*dNLA=dNLA≪2 +ab*dNLA.       (Eqn 2.4)

If dNLA>227, the weight resulting from Equation 2.2 is guaranteed to be zero. Hence it is sufficient with a multiplication of size 2 bits times 8 bits for Equation 2.4. The value of k is fetched from an array

k_tab[63-18+1] = {5, 5, 7, 5, 4, 7, 6, 5, 5, 4, 4, 7, 7, 6, 6, 5, 5, 5, 5, 4, 4, 4, 4, 4, 7, 7, 7, 6, 6, 6, 6, 6, 5, 5, 5, 5, 5, 5, 5, 4, 4, 4, 4, 4, 4, 4};

where qp-18 is used to index the array. The values c1 and c2 are calculated as

c1=8192≫s       (Eqn 2.5)

c2=14-s,           (Eqn 2.6)

where the number of shifts s is fetched from another table also indexed by qp-18:

s_tab[63-18+1] = {6, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1};

## Hardware complexity

Note that the values in k_tab only contain values between 4 and 7, i.e., four different values. Hence two bits per qp are needed, in total (63-18+1)*2 = 92 bits = 12 bytes for the entire k_tab. Likewise, shift_tab contain only values between 1 and 6, i.e., 5 different values, so three bits per qp are sufficient, giving (63-18+1)*3=138 bits or 18 bytes. In total 30 bytes are sufficient for the two tables.

The variable m is calculated as m=1769≫s, where s is the value from the shift tab. Since s is always at least 1, this will also affect the maximum weight from max0, k*0+1769=1769 to max0, k*0+1769≫1=884. For the multiplication

wA*ΔIA        (Eqn 2.7)

this means that 10 bits are sufficient for the first factor and 11 bits are sufficient for the second factor. The computational complexity to calculate the result in Equation 2.6 thus becomes

Multiplications: one 2-bit by 8-bit multiplication (Equation 2.5) and one 10-bit by 11 bit multiplications (Equation 2.7)

Additions: 7 for calculating sNLA (see JVET-L0172_v2 section 5.2), 1 for calculating k*sNLA, and 1 for the subtraction m-k*sNLB

Shifts: 1 for calculating sNLA and 1 for k*sNLB (all fixed number of steps)

Checks: 3 for calculating sNLA and 1 for the max in Equation 2.2

We only need to calculate Equation 2.7 for the right and below neighbor – the calculation result for the above and left neighbor can be obtained from neighboring filtered sample. We thus multiply all the above results by two to get the number of operations per filtered sample. To this we must add the operations of Equation 2.1, i.e., five additions and a shift. In total we get per filtered sample:

- 2 multiplications of 2-bits by 8-bits and 2 multiplications of 6-bits by 9-bits

- 23 additions

- 5 shifts (4 fixed and one variable)

- 8 checks

Per block we also need to do one look-up to get k, one look-up to get s, one shift each for c1 and m and one addition to get c1.

## Results

### Results for CE1-1.1a

Random access results for CE1-1.1a. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.13% | -0.06% | 101% | 100% |
| Class A2 | -0.40% | -0.26% | -0.12% | 103% | 100% |
| Class B | -0.47% | -0.17% | -0.11% | 102% | 101% |
| Class C | -0.28% | 0.01% | 0.03% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.13% | -0.06% | 102% | 100% |
| Class D | -0.32% | 0.05% | 0.14% | 105% | 104% |
| Class F | -0.13% | 0.04% | -0.03% | 114% | 102% |

Low delay results for CE1-1.1a. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | 0.68% | 0.96% | 102% | 103% |
| Class C | -0.30% | 0.62% | 0.67% | 98% | 99% |
| Class E | -0.40% | 0.66% | 0.73% | 101% | 99% |
| Overall | -0.35% | 0.65% | 0.81% | 101% | 101% |
| Class D | -0.52% | 1.05% | 0.53% | 104% | 104% |
| Class F | -0.17% | 0.31% | 0.30% | 95% | 95% |

### Results for CE1-1.1b

Random access results for CE1-1.1b. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.08% | -0.04% | 101% | 97% |
| Class A2 | -0.40% | -0.23% | -0.07% | 102% | 102% |
| Class B | -0.45% | -0.12% | -0.07% | 101% | 101% |
| Class C | -0.27% | 0.03% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.10% | -0.05% | 101% | 100% |
| Class D | -0.35% | 0.18% | 0.03% | 100% | 101% |
| Class F | -0.11% | 0.06% | -0.03% | 99% | 100% |

Low delay results for CE1-1.1b. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | 0.63% | 0.83% | 102% | 102% |
| Class C | -0.31% | 0.64% | 0.64% | 99% | 97% |
| Class E | -0.36% | 0.80% | 0.02% | 102% | 97% |
| Overall | -0.34% | 0.68% | 0.56% | 101% | 99% |
| Class D | -0.54% | 0.81% | 0.39% | 106% | 105% |
| Class F | -0.18% | -0.13% | -0.22% | 94% | 92% |

### Results for CE1-1.1c

Random access results for CE1-1.1c. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.15% | -0.11% | 101% | 98% |
| Class A2 | -0.36% | -0.21% | -0.12% | 102% | 102% |
| Class B | -0.43% | -0.04% | -0.02% | 102% | 102% |
| Class C | -0.23% | 0.02% | 0.01% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.08% | -0.05% | 102% | 102% |
| Class D | -0.31% | 0.01% | 0.06% | 104% | 101% |
| Class F | -0.11% | 0.05% | 0.00% | 114% | 101% |

Low delay results for CE1-1.1c. No SIMD code has been used.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | 0.63% | 0.92% | 101% | 97% |
| Class C | -0.28% | 0.52% | 0.58% | 97% | 97% |
| Class E | -0.38% | 0.81% | 0.67% | 101% | 98% |
| Overall | -0.31% | 0.64% | 0.74% | 100% | 98% |
| Class D | -0.56% | 0.89% | 0.96% | 105% | 104% |
| Class F | -0.10% | 0.09% | 0.66% | 97% | 96% |

# CE1-1.2a, CE1-1.2b and CE1-1.2c

The next test CE1-1.2 is equivalent to CE1-1.1 except for the fact that the factors of the multiplications have been shifted to obtain a better balance between BD-rate gain and hardware complexity. The main difference is that the multiplication in Equation 2.7 has been reduced from 10-bit × 11-bit to 6-bit × 9-bit. To accomplish this, Equation 2.1 has been replaced by

IF=IC+wAΔsIA+wBΔsIB+wLΔsIL+wRΔsIR+c1≫c2,     (Eqn 3.1)

where ΔsIA through ΔsIR are shifted versions of the sample differences ΔIA=IA-IC:

ΔsIA=ΔIA+4≫3.      (Eqn 3.2)

The weight wA is calculated as

wA=max0, m-k*sNLA.      (Eqn 3.3)

and ditto for wB through wR. The value sNLA is a shifted version of dNLA: sNLA=dNLA≫3. Instead of a three-bit value as in CE1-1.1, the value k is now a two-bit value 1abinwhere the most significant bit is always 1. It is obtained from a table indexed by qp-18:

k_tab[63-18+1]{2, 2, 3, 2, 2, 3, 3, 2, 2, 2, 2, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2};

The value m in Equation 3.3 is obtained as

m=55≫s,      (Eqn 3.5)

where s is obtained from the following table indexed by qp-18:

s_tab[63-18+1] ={5, 4, 3, 3, 3, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, };

The values c1 and c2 in Equation 3.1 are obtained as

c1=32≫s      (Eqn 3.6)

c2=6-s.     (Eqn 3.7)

## Hardware complexity

Examining Equation 3.3,

wA=max0, m-k*sNLA.      (Eqn 3.3)

the second term of the subtraction k*sNLA can be calculated as a conditional add sNLA≪1+a & sNLA, where a is the least significant bit in k=1abin.

The first term in Equation 3.1 is

wA*ΔsIA.     (Eqn 3.8)

Since the smallest possible weight is wA=0 and the largest possible weight is wA=m≤55, this means that the first factor in Equation 3.8 is a 6-bit number. The second factor ΔsIA is a sample difference shifted three steps according to Equation 3.2. The largest possible such value is 1023+4≫3=128 and the smallest such value is -1023+4≫3=-127, and hence 9 bits are sufficient for the second factor. The multiplication in Equation 3.8 can therefore be implemented using 6-bit by 9-bit multiplier. The computational complexity to calculate the result from Equation 3.8 thus becomes

Multiplications: one 6-bit by 9-bit multiplications (Equation 3.8)

Additions: 7 for calculating sNLA (see JVET-L0172_v2 section 5.2), 1 for calculating k*sNLA, 1 for the subtraction m-k*sNLB, and 1 for Equation 3.2

Shifts: 1 for calculating sNLA, 1 for Equation 3.2 and 1 for k*sNLB (all fixed number of steps)

Checks: 3 for calculating sNLA and 1 for the max in Equation 3.3

We do not need to calculate Equation 3.8 all four terms in Equation 3.1, only for the right and below neighbor. The calculation result for the above and left neighbor can be obtained from neighboring filtered samples. We thus double all the above results to get the number of operations for the weighted terms of Equation 3.1. To this we must add the remaining operations of Equation 3.1, i.e., five additions and a shift. In total we get the following operations per filtered sample:

- 2 multiplications of 6-bits by 9-bits

- 25 additions

- 7 shifts (6 fixed and one variable)

- 8 checks

The memory requirements are 1 bit per qp for k_tab, i.e., 63-18+1*1=46 bits or 6 bytes, and 4 bits per qp for s_tab, i.e., 63-18+1*3=138 bits or 18 bytes. In total 24 bytes of storage.

## Results

### Results for CE1-1.2a

Random access results for CE1-1.2a. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.20% | -0.03% | 101% | 100% |
| Class A2 | -0.37% | -0.33% | -0.14% | 102% | 100% |
| Class B | -0.44% | -0.15% | -0.12% | 102% | 102% |
| Class C | -0.27% | 0.10% | -0.09% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.13% | -0.10% | 102% | 100% |
| Class D | -0.35% | -0.02% | 0.08% | 103% | 101% |
| Class F | -0.10% | 0.09% | 0.00% | 103% | 100% |

Low delay results for CE1-1.2a. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | 0.75% | 0.47% | 101% | 99% |
| Class C | -0.21% | 0.76% | 0.69% | 97% | 96% |
| Class E | -0.19% | 0.70% | 0.36% | 102% | 97% |
| Overall | -0.23% | 0.74% | 0.52% | 100% | 98% |
| Class D | -0.45% | 0.80% | 0.44% | 106% | 107% |
| Class F | -0.08% | -0.11% | -0.50% | 97% | 92% |

### Results for CE1-1.2b

Random access results for CE1-1.2b. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.14% | -0.06% | 102% | 98% |
| Class A2 | -0.33% | -0.26% | -0.17% | 102% | 100% |
| Class B | -0.43% | -0.15% | -0.18% | 104% | 103% |
| Class C | -0.23% | 0.02% | -0.06% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.13% | -0.12% | 103% | 102% |
| Class D | -0.35% | 0.02% | 0.26% | 105% | 103% |
| Class F | -0.10% | 0.04% | -0.07% | 114% | 102% |

Low delay results for CE1-1.2b. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | 0.79% | 0.68% | 101% | 100% |
| Class C | -0.17% | 0.74% | 0.81% | 97% | 98% |
| Class E | -0.22% | 1.05% | 0.51% | 102% | 97% |
| Overall | -0.20% | 0.84% | 0.68% | 100% | 98% |
| Class D | -0.44% | 1.39% | 0.39% | 106% | 107% |
| Class F | -0.10% | -0.14% | 0.02% | 96% | 94% |

### Results for CE1-1.3a

Random access results for CE1-1.2c. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.16% | 100% | 97% |
| Class A2 | -0.33% | -0.26% | -0.15% | 103% | 99% |
| Class B | -0.39% | -0.10% | -0.15% | 101% | 101% |
| Class C | -0.20% | 0.05% | 0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.12% | -0.11% | 102% | 100% |
| Class D | -0.33% | 0.06% | 0.14% | 102% | 100% |
| Class F | -0.07% | 0.03% | -0.03% | 97% | 101% |

Low delay results for CE1-1.2c. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | 0.65% | 0.59% | 102% | 99% |
| Class C | -0.17% | 0.57% | 0.60% | 99% | 99% |
| Class E | -0.27% | 0.95% | 0.46% | 102% | 99% |
| Overall | -0.19% | 0.70% | 0.56% | 101% | 99% |
| Class D | -0.41% | 1.35% | 0.57% | 105% | 105% |
| Class F | 0.06% | 0.93% | 0.76% | 96% | 93% |

# CE1-1.3a, CE1-1.3b and CE1-1.3c

It was stated at the Macau meeting that the calculation of the dNLA value gave rise to an extra cycle of latency, meaning that the solution would have a latency of three cycles rather than two. Therefore, in CE1-1.3, it is investigated what happens if this calculation is removed, and ΔIA is used instead of dNLA. In other regards it follows CE1-1.2: Just as in CE1-1.2, the sample is filtered using

IF=IC+wAΔsIA+wBΔsIB+wLΔsIL+wRΔsIR+c1≫c2,     (Eqn 4.1)

where ΔsIA through ΔsIR are shifted versions of the sample differences ΔIA=IA-IC:

ΔsIA=ΔIA+4≫3.      (Eqn 4.2)

However, the weight wA is now calculated as

wA=max0, m-k*sΔIA.      (Eqn 4.3)

and ditto for wB through wR. Just as for CE1-1.2, k*sΔIA can be calculated using one conditional addition and a shift as

sΔIA≪1+a*sΔIA.      (Eqn 4.4)

Here a is the lowest significant bit of k. Just as for CE1-1.2, k is fetched from a table using qp-18,

k_tab[63-18+1] = {3, 3, 2, 3, 2, 2, 2, 3, 3, 2, 2, 2, 2, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2}.

The value m in Equation 3.3 is obtained as

m=66≫s,      (Eqn 4.5)

where s is obtained from the following table indexed by qp-18:

s_tab[63-18+1] = {5, 4, 4, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0};

Just as for CE1-1.2, the values c1 and c2 in Equation 3.1 are obtained using

c1=32≫s      (Eqn 4.6)

c2=6-s.     (Eqn 4.7)

## Hardware complexity

The number of operations is the same as for CE1-1.2 with the following differences:

The largest value for the weight wA=max0, m-k*ΔIA is now m=66 instead of m=55. Hence the multiplication

wA*ΔIA        (Eqn 4.8)

is now 7 bits wide in the first factor and 11 bits wide in the second factor.

The operations needed to calculate dNLA are now replaced by a single absolute value to calculate ΔsIA from ΔsIA. The operations necessary to calculate Equation 4.8 thus become

Multiplications: one 7-bit by 9-bit multiplications (Equation 4.8)

Additions: 1 for calculating k*ΔsIA, 1 for the subtraction m-k*sNLB, and 1 for Equation 4.2

Shifts: 1 for Equation 4.2 and 1 for k*ΔsIA (both fixed number of steps)

Checks: 1 for the absolute value ΔsIA and 1 for the max in Equation 4.3

Together with the remaining operations of Equation 3.1, i.e., five additions and a shift we get the following number of operations per filtered sample:

- 2 multiplications of 7-bits by 9-bits

- 11 additions

- 5 shifts (4 fixed and one variable)

- 4 checks

The memory requirements are 1 bit per qp for k_tab, i.e., 63-18+1*1=46 bits or 6 bytes, and 4 bits per qp for s_tab, i.e., 63-18+1*3=138 bits or 18 bytes. In total 24 bytes of storage.

## Results

### Results for CE1-1.3a

Random access results for CE1-1.3a. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.14% | -0.04% | 102% | 99% |
| Class A2 | -0.29% | -0.29% | -0.03% | 104% | 105% |
| Class B | -0.33% | -0.07% | -0.18% | 101% | 102% |
| Class C | -0.18% | 0.07% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.09% | -0.07% | 102% | 101% |
| Class D | -0.28% | 0.07% | 0.05% | 102% | 100% |
| Class F | -0.22% | 0.09% | 0.02% | 114% | 102% |

Low delay results for CE1-1.3a. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | 0.84% | 0.51% | 100% | 98% |
| Class C | -0.26% | 0.90% | 0.75% | 98% | 97% |
| Class E | 0.28% | 0.94% | -0.19% | 101% | 97% |
| Overall | -0.09% | 0.88% | 0.41% | 100% | 97% |
| Class D | -0.50% | 0.83% | 0.65% | 105% | 105% |
| Class F | -0.46% | 0.28% | 0.17% | 97% | 93% |

### Results for CE1-1.3b

Random access results for CE1-1.3b. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.12% | -0.02% | 102% | 100% |
| Class A2 | -0.30% | -0.23% | -0.06% | 104% | 103% |
| Class B | -0.32% | -0.12% | -0.11% | 103% | 101% |
| Class C | -0.19% | 0.08% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.09% | -0.07% | 103% | 101% |
| Class D | -0.30% | -0.04% | 0.16% | 102% | 102% |
| Class F | -0.21% | 0.13% | 0.04% | 101% | 100% |

Low delay results for CE1-1.3b. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | 0.61% | 0.50% | 101% | 99% |
| Class C | -0.24% | 0.88% | 0.80% | 97% | 97% |
| Class E | 0.25% | 0.78% | 0.38% | 100% | 97% |
| Overall | -0.09% | 0.74% | 0.57% | 100% | 98% |
| Class D | -0.51% | 1.04% | 0.28% | 105% | 103% |
| Class F | -0.44% | 0.24% | 0.18% | 96% | 94% |

### Results for CE1-1.3c

Random access results for CE1-1.3c. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.11% | -0.02% | 100% | 99% |
| Class A2 | -0.26% | -0.20% | -0.05% | 101% | 102% |
| Class B | -0.28% | -0.06% | -0.13% | 100% | 101% |
| Class C | -0.16% | 0.01% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -0.08% | -0.09% | 100% | 100% |
| Class D | -0.32% | 0.04% | -0.02% | 101% | 98% |
| Class F | -0.17% | 0.11% | 0.09% | 110% | 102% |

Low delay results for CE1-1.3c. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | 0.77% | 0.68% | 100% | 99% |
| Class C | -0.21% | 0.96% | 0.56% | 98% | 96% |
| Class E | 0.28% | 1.14% | 0.53% | 99% | 95% |
| Overall | -0.04% | 0.93% | 0.60% | 99% | 97% |
| Class D | -0.47% | 0.82% | 0.00% | 104% | 102% |
| Class F | -0.31% | 0.02% | -0.09% | 95% | 92% |

# CE1-1.4

No test was conducted in this category.

# CE1-1.5

The filters in CE1-1.1 through CE1-1.3 are post-reconstruction filters, i.e., they are invoked right after the residual has been added to the prediction and clipped. It was decided to also test the bilateral filter as a loop-filters. Hence in CE1-1.5, the filter is applied right before deblocking.

As mentioned in the CE-description, the restrictions that apply for post-reconstruction and post-prediction filters do not apply to loop-filters, since all samples are already available. There is also no restriction on filtering intra blocks or smaller blocks such as 4×4 blocks. There is however a restriction not to read outside the CTU.

To filter the intensity IC of the current sample, the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used, as shown in Figure 1 (repeated here for the convenience of the reader).

|  | IA |  |
| --- | --- | --- |
| IL | IC | IR |
|  | IB |  |

Figure 1. Samples used in the weighted sum.

The filtered value IF is then calculated as a weighted sum of the surrounding delta-intensity values as

IF=IC+wAΔsIA+wBΔsIB+wLΔsIL+wRΔsIR+c1≫c2,     (Eqn 5.1)

where ΔsIA through ΔsIR are shifted versions of the sample differences ΔIA=IA-IC:

ΔsIA=ΔIA+4≫3.      (Eqn 5.2)

The filter is applied to both intra and inter blocks, but they are handled slightly differently.

## Inter block filtering

For inter blocks, the filtering follows exactly that of CE1-1.2, the only difference being that blocks smaller than 64 samples are allowed to be filtered.

## Intra block filtering

For intra blocks, the filtering is more similar to CE1-1.3. The weight wA is calculated as

wA=max0, m-k*sΔIA.      (Eqn 5.3)

and ditto for wB through wR. Just as for CE1-1.3, k*sΔIA can be calculated using one conditional addition and one shift using

sΔIA≪1+a*sΔIA.      (Eqn 5.4)

Here a is the lowest significant bit of k. Just as for CE1-1.3, k is fetched from a table using qp-18. However, in CE1-1.5 this table has two rows. Which row to use is depends on the size of the block. Let Dmin=minblockwidth, blockheight. Then if Dmin<16, the first row is used, otherwise the second row is used:

k_tab[2][63-18+1] = {{3, 3, 2, 3, 2, 2, 3, 3, 2, 2, 2, 2, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2},
                                    {2, 2, 3, 2, 2, 3, 3, 2, 2, 2, 2, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2}};

Note that the second row is the same as the one used for inter filtering (it is described in CE1-1.2). The value m in Equation 4.3 is calculated as

m=66≫s        if Dmin<1655≫s        otherwise                   (Eqn 5.5)

where s is obtained using qp-18 from the following table, where again the first row is used if Dmin<16 and the second row otherwise:

s_tab[2][63-18+1] = {{ 5, 4, 3, 3, 3, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0},
                                   { 5, 4, 4, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0}};

Note that the second row is identical to the row used for inter filtering. The values c1 and c2 in Equation 5.1 are obtained using

c1=32≫s      (Eqn 5.6)

c2=6-s.     (Eqn 5.7)

## Hardware complexity

Apart from the memory requirements, the hardware complexity for intra filtering is equivalent to that of CE1-1.3, with the following operations per filtered sample:

- 2 multiplications of 7-bits by 9-bits

- 11 additions

- 5 shifts (4 fixed and one variable)

- 4 checks

For inter filtering we also need to calculate dNLA which takes an additional 14 additions, 2 shifts and 4 checks:

- +14 additions

- +2 shifts (fixed)

- +4 checks

The memory requirements for intra filtering is as follows: For k_tab we now need 1 bit per qp per row, i.e., 63-18+1*1*2=92 bits or 12 bytes, and 3 bits per qp and row for s_tab, i.e., 63-18+1*3*2=276 bits or 35 bytes. In total 47 bytes of storage.  Inter filtering uses the second row of the intra filter tables and thus requires no extra storage.

## Results

All intra results for CE1-1.5. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.25% | 0.24% | 108% | 112% |
| Class A2 | -0.15% | 0.31% | 0.15% | 106% | 108% |
| Class B | -0.10% | 0.33% | 0.39% | 107% | 109% |
| Class C | -0.33% | 0.11% | 0.12% | 109% | 105% |
| Class E | -0.16% | 0.22% | 0.22% | 104% | 105% |
| Overall | -0.16% | 0.25% | 0.24% | 107% | 108% |
| Class D | -0.28% | 0.17% | 0.14% | 104% | 102% |
| Class F | -0.39% | 0.10% | 0.02% | 105% | 110% |

Random access results for CE1-1.5. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.26% | -0.08% | 106% | 99% |
| Class A2 | -0.44% | -0.28% | -0.17% | 105% | 105% |
| Class B | -0.52% | 0.03% | -0.27% | 106% | 102% |
| Class C | -0.48% | 0.04% | 0.10% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -0.09% | -0.11% | 105% | 102% |
| Class D | -0.52% | -0.04% | 0.19% | 103% | 103% |
| Class F | -0.36% | 0.06% | -0.01% | 102% | 102% |

Low delay results for CE1-1.5. No SIMD code has been used for the filter.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.23% | 0.57% | 0.61% | 106% | 106% |
| Class C | -0.27% | 1.20% | 0.74% | 103% | 103% |
| Class E | -0.30% | 1.43% | 0.57% | 102% | 99% |
| Overall | -0.26% | 0.99% | 0.64% | 104% | 103% |
| Class D | -0.57% | 0.82% | 0.45% | 107% | 108% |
| Class F | -0.10% | 0.38% | 0.13% | 101% | 100% |

# CE1-1.6

No test was conducted in this category.

# CE1-1.7

Test CE1-1.7 targeted more accurate PWL modeling of BIF parameters. It is proposed to parametrize weight function wi∆Ii  through a piece-wise linear model with 2 ranges, in comparison to the single range linear approximation employed by the proponent of CE1-1.

Similarly to the base design, CE1-1.1, bilateral filter is used as a post-reconstruction filter. To filter the intensity IC of the current sample (see Figure 1), the surrounding four-neighbors IA (above) IB (below) IL (left) IR (right) are used (further jointly referred as Ii). The filtered value IF is calculated as a weighted sum of the surrounding delta-intensity values as:

IF=IC+wAΔIA+wBΔIB+wLΔIL+wRΔIR+c1≫c2,     (Eqn 7.1)

where the deltas are the difference between sample value Ii (sample at i position) against the center sample, ΔIi=Ii-IC etc. The delta-value dependent weights factors wi are determined by a piece-wise linear function W(ΔI), derived from parameters of bilateral filter, QP value and parameters of mode, as presented in JVET-L0406.

Side information (ROM) of less then 120 bytes of data defines the weighting function W(ΔI):

gen_pivot[10] = {42950, 37903, 26050, 13944, 5813, 1887, 477, 94, 14, 2}

sc[5] = { 30195, 19316, 26068, 19316, 19316};

num[45] = { 65535, 43690, 32768, 26214, 21845, 18724, 16384, 14563, 13107, 11915, 10922, 10082, 9362, 8738, 8192, 7710, 7281, 6898, 6553, 6241, 5957, 5698, 5461, 5242, 5041, 4854, 4681, 4519, 4369, 4228, 4096, 3971, 3855, 3744, 3640, 3542, 3449, 3360, 3276, 3196, 3120, 3048, 2978, 2912, 2849};

Utilized for filtering W(ΔI) weight function initialized as following:

High accuracy W’(ΔI, QP,mode) LUT consisting of 10 ranges is initialized for each QP and block sizes by a linear interpolation of W(ΔI) values from gen_pivot[10] entries.

Following this, a coarse PWL approximation of the W’(ΔI, QP,sizeID) with 2 pieces is conducted. The size of the first piece is set to be equal to a power of 2, thus it can calculated as R = 1<< rShift, and PWL model is defined through scalePWL and offsetPWL values determined through each of the piece. Figure below visualize the coarse approximation described above.

Figure 3. Results of approximation of a weight function W’(ΔI, QP,sizeID)

Initialized BIF parameters of 285 bytes (RAM):

Effectively, prior to coding/decoding, BIF weighting function in CE1-7.a c are initialized  with the following parameters (normalized to the bitdepth of the of CE1-1 base design) which are estimated on fly and stored in RAM of size of approximately 285 Bytes.

shiftPWL[46] = [ 0, 1, 1, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5 ];

scalePWL[2][2][46] =

[[250, 125, 83, 62, 50, 41, 35, 31, 27, 25, 22, 20, 19, 17, 18, 15, 14, 14, 13, 12, 12, 11, 10, 10, 10, 9,9, 9, 9, 9, 8, 7, 7, 7, 7, 7, 6, 6, 6, 6, 6,6,5,5,5,5],
[531, 278, 171, 139, 107, 86, 73, 69, 60, 53, 48, 43, 39, 36, 37, 35, 32, 30, 28, 26, 25, 24, 22, 21, 20, 19, 19, 18, 19, 18, 18, 17, 16, 16, 15, 15, 14, 14, 13, 13, 13, 12, 12, 12, 11, 11 ]],
[186, 93, 62, 46, 37, 31, 26, 23, 20, 18, 16, 15, 14, 13, 13, 11, 11, 10, 9, 9, 8, 8, 8, 7, 7, 7, 6, 6, 7, 6, 6, 5, 5, 5, 5, 5, 5, 4, 4, 4, 4, 4, 4, 4, 4, 4 },
[394, 206, 127, 103, 79, 64, 54, 51, 45, 39, 35, 32, 29, 27, 27, 25, 24, 22, 21, 19, 18, 17, 16, 16, 15, 14, 14, 13, 14, 13, 13, 13, 12, 12, 11, 11, 10, 10, 10, 9, 9, 9, 9, 8, 8, 8]]

offsetPWL0[2] = [2135, 1582]

offsetPWL1[2][46] =

[1884, 1884, 1968, 1884, 1934, 1968, 1992, 1885, 1912, 1934, 1953, 1968, 1981, 1992, 1845, 1885, 1898, 1911, 1923, 1934, 1943, 1952, 1960, 1967, 1974, 1980, 1986, 1991, 1823, 1845, 1865, 1885, 1891, 1897, 1905, 1911, 1917, 1923, 1929, 1933, 1939, 1943, 1947, 1951, 1955, 1959 ]

[1396, 1396, 1458, 1396, 1433, 1458, 1475, 1396, 1416, 1433, 1446, 1458, 1467, 1475, 1367, 1396, 1406, 1416, 1425, 1433, 1440, 1446, 1452, 1458, 1462, 1467, 1471, 1475, 1352, 1368, 1382, 1396, 1400, 1406, 1410, 1416, 1420, 1424, 1428, 1432, 1436, 1440, 1442, 1446, 1448, 1452 ]

5 BIF parameters are required for 1 block (48 bytes): scalePWL[2], offsetPWL[2], shiftPWL.

Filtering process:

Two implementation approaches are feasible for the proposed scheme.

First is LUT-based approach, where weights WΔI are tabulated for all non-zero weights and accessed by ΔI:

WΔI=sigmaDistTabWΔI. 					(Eqn 7.3)

This approach may be beneficial for large block size processing due to reduction in number of operations or for memory non-constrained platforms.

Second is LUT-free approach fir weight derivation for given delta value ΔI:

T=1≪shiftPWL

indx=(ΔI<T) ? 1:0

n=ΔI<T? -ΔI : T-ΔI

k=scalePWLindx

m=offsetPWLindx

WΔI=max0, m-k*n .						(Eqn 7.2)

This derivation is similar to the implementation of CE1-1, with addition of determining value of T, indx and n variables which adds 1 comparator, 1 shift and 1 add to complexity estimate of CE1-1.

In the current document, simulation results for LUT-based implementation is reported. Additional test results for proposed method with LUT-free weight derivation by Eq.(7.2) (and a bug-fix) are reported in CE1-related document JVET-N0482.

### Hardware complexity

In this section, the complexity of the most critical component – the complexity of weight derivation and filtering process is assessed.

For LUT based implementation CE1-7 tests, reported in this document , the weight of the BIF is derived from the LUT as shown in Eq(7.2) and wi*ΔIi  from Eq. 7.3 is implemented as a multiplication of the two 16 bits number.

Taking into account parallel calculation of the Eq (7.1) for neighbouring samples of the block, term wi*ΔIi is calculated only for the right and below samples, thus total number of operations for LUT-based implementation is assessed as:

- 2 mults       Eq.7.1

- 13 adds      7 for non-local sum + 5 Eq. 7.1

- 1 shift

- 1 comp

The memory requirements for LUT-based design are assessed as:

ROM: 					120 bytes

RAM: 					5449 words for LUT W’(ΔI, QP,sizeID)

For LUT-free approach, weights are derived by group of calculations in Eq. 7.2.

For the Equation 7.2 for any n > 172, the resulting weight is equal to 0, thus variable n can be represented with 8 bits. With max scale value k is 531, the Equation 7.2 can be implemented with 8-bit by 10-bit multiplication and one subtraction. Maximum value of the product in Equation 7.2 is below 2394, which can be accommodated by a 12 bit register following by 1 comp. Additionally, 1 comparator, 1 shift and 1 add operations are needed to derive k, m, n parameters. Another 8-bit by 12-bit multiplication is needed for term wi*ΔIi in Eq (7.1).

Multiplications: 2         8-bit by 10-bit mult + one 8-bit by 12 bit mult

Additions: 23                  (Eq. 7.2)

Shifts: 1                        (Eq. 7.2)

Checks: 2                      (Eq. 7.2)

Taking into account parallel calculation of the Eq (7.1) for neighbouring samples of the block, term wi*ΔIi is calculated only for the right and below samples, thus total number of operations for on-fly approach is assessed as:

- 4 mults

- 9 adds

- 3 shifts

- 4 comps

The memory requirements for LUT-free approach is assessed as:

ROM: 									285 bytes

Params at block level: 					48 bytes (5 params)

## Results

Simulation results for CE1-1.7 design for constrains a and c with LUT-based implementation are shown in tables below. Additional results for CE1-1.7 with LUT-free based implementation are reported in CE-related document JVET-N0482.

### CE1-1.7.a

Random Access results for CE1-1.7a. No SIMD code has been used, reported time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.14% | -0.01% | 111% | 116% |
| Class A2 |  |  |  |  |  |
| Class B | -0.44% | -0.07% | 0.01% | 87% | 101% |
| Class C | -0.29% | 0.16% | -0.01% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.33% | 0.01% | 0.14% | 102% | 104% |
| Class F |  |  |  |  |  |

Lowdelay B results for CE1-1.7 a. No SIMD code has been used, reported time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.42% | 0.37% | 0.51% | 84% | 89% |
| Class C | -0.37% | 0.71% | 0.80% | 85% | 92% |
| Class E | -0.38% | 0.57% | 0.10% | 100% | 103% |
| Overall | -0.39% | 0.54% | 0.50% | 88% | 94% |
| Class D | -0.53% | 0.80% | 0.01% | 101% | 101% |
| Class F |  |  |  |  |  |

### CE1-1.7.c

Random access results for CE1-1.7c. No SIMD code has been used, reported time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.08% | 0.03% | 88% | 85% |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.05% | -0.06% | 84% | 89% |
| Class C | -0.25% | 0.12% | 0.01% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.30% | 0.16% | 0.19% | 81% | 91% |
| Class F |  |  |  |  |  |

Lowdelay B results for CE1-1.7 c. No SIMD code has been used, reported time est. are inaccurate.

|  |  |  | Over VTM-4.0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | 0.47% | 0.25% | 83% | 88% |
| Class C | -0.39% | 0.70% | 0.59% | 81% | 93% |
| Class E | -0.47% | 0.56% | 0.33% | 83% | 102% |
| Overall | -0.40% | 0.57% | 0.38% | 83% | 93% |
| Class D | -0.53% | 0.75% | 0.25% | 82% | 93% |
| Class F |  |  |  |  |  |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).