| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0504 |
| --- | --- |

| Title: | CE2-related: Simplified prediction refinement for affine motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, a 2x2 subblock based prediction refinement method is proposed for affine motion compensation. The prediction refinement is performed by using the optical flow equation to derive an offset for each pixel in a 4x4 subblock. The delta motion vectors are calculated in a 2x2 subblock basis, and the gradient can be calculated in parallel for each 4x4 subblock. The test results show that the BD-rate changes are -0.28%, -0.08%, -0.01% in RA and -0.18%, -0.06%, -0.11% in LDB.

# Introduction

In current VTM, the affine motion compensation is performed in 4x4 subblock basis. In JVET-M0268 [1], the interweaved prediction for affine motion compensation provides coding gains by taking advantage of more dense motion field and weighted prediction. In JVET-N0273 [2], it’s proposed to adaptively using 2x2 subblock for affine uni-prediciton without increasing worst case bandwidth and computational complexity. In JVET-N0236 [3], a prediction refienment with optical flow method is proposed for affine mode. The luma prediction refinement is calculated as

ΔI(i,j)= gxi,j*Δvx(i,j)+gyi,j*Δvy(i,j).

wherein the Δv(i,j) is the difference between pixel MV at location (i,j) and the sub-block MV of the subblock to which pixel (i,j) belongs. gxi,j and gyi,j are the spatial gradients at location (i,j).

In the proposed method, the prediction refienment method is combined with 2x2 subblock based affine motion compenstion to simplified the refinement process. The 2x2 subblock motion field is used for deriving the refinement offset. The gradient is calculated in the 4x4 subblock basis and without refering the prediction sample outside the 4x4. In this way, the motion compensation and the prediction refinement can be done in parallel for all 4x4 subblocks. The proposed method is applied to both uni-prediction and bi-prediction, but it’s applied to luma only. For each affine mode coding unit, it requires 8 additions for delta MV calculation. For each 4x4 subblock in the affine mode coding unit, the proposed method requires, 16 additions for gradient calculation, 32 multiplications and 32 addtitions for the refinement process. In total, the additional operations are 32 multiplications and 48 additions for each 4x4 subblock.

# Derivation of 2x2 subblock motion field

The 2x2 subblock motion field is derived based on the control-point motion vectors (CPMVs) of current block. The normalized affine parameters c, d, e and f are first calculated as:

c=v1x-v0xwd=v2x-v0xhe=v1y-v0ywf=v2y-v0yh

Wherein (vix, viy), i=0,1,2 are the CPMVs. the Then for the 4 2x2 in a 4x4 subblock, the delta MV relative to the center of the 4x4 subblock is calculated as follows:

Top-left 2x2:

Δvxx,y=-c-dΔvyx, y=-e-f

Top-right 2x2:

Δvxx,y=c-dΔvyx, y=e-f

Bottom-left 2x2:

Δvxx,y=-c+dΔvyx, y=-e+f

Bottom-right 2x2:

Δvxx,y=c+dΔvyx, y=e+f

In total, 8 additions are required for each coding unit. The 4x4 subblocks in a coding unit share the same calculated delta MVs.

# Derivation of spatial gradients

The spatial gradients are calculated using the prediction samples within each 4x4 subblock. Similar to the gradient calculation in JVET-N0263, a 3-tap filter [-1, 0, 1] is used. If the prediction samples outside current 4x4 subblock, it’s padded by the boundary samples. Therefore, for the fist column of the 4x4, i=0, the gradient at x direction is calculated as Ii+1, j-Ii, j. For the last column of the 4x4, i=3, the gradient at x direction is calculated as Ii, j-Ii-1, j. For the first row of the 4x4, j=0, the gradient at y direction is calculated as Ii, j+1-Ii, j. For the last row of the 4x4, j=0, the gradient at y direction is calculated as Ii, j-Ii, j-1.  In total, 16 additions are required for each 4x4 subblock.

# Test result

The proposed method is implemented on top of VTM-4.0. The refinement process is implemented without SIMD optimization. The test results under random access and low delay B common test conditions are shown in Table 1. The overall Y, U, V, BD-rate changes are -0.28%, -0.08%, -0.01% in RA and -0.18%, -0.06%, -0.11% in LDB. The runtime increase at encoder are 1% in RA and 2% in LDB. The decoder runtime doesn’t change in both RA and LDB.

Table 1 Test results

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.10% | -0.01% | 101% | 100% |
| Class A2 | -0.46% | 0.02% | 0.01% | 97% | 98% |
| Class B | -0.38% | -0.25% | -0.02% | 103% | 101% |
| Class C | -0.15% | 0.06% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.08% | -0.01% | 101% | 100% |
| Class D | -0.32% | -0.06% | -0.11% | 100% | 98% |
| Class F | -0.35% | -0.07% | -0.12% | 102% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | -0.53% | -0.23% | 102% | 101% |
| Class C | -0.11% | 0.33% | -0.20% | 101% | 98% |
| Class E | -0.20% | 0.21% | -0.45% | 103% | 101% |
| Overall | -0.18% | -0.06% | -0.27% | 102% | 100% |
| Class D | -0.37% | 0.36% | -0.87% | 102% | 98% |
| Class F | -0.69% | -0.58% | -0.72% | 102% | 101% |

# Conclusion

This contribution proposes a simplified prediction refinement method using 2x2 motion field and 4x4 based gradient calculation. The results show -0.28% in RA and in -0.18% LDB without significant run time increase. It’s suggested to study the proposed method together with other related methods in CE.

# References

- K. Zhang, L. Zhang, H. Liu, et al. “Non-CE2: Interweaved Prediction for Affine Motion Compensation”, JVET-M0268, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

- H. Huang, W.-J Chien, M. Karczewicz. “CE2-related: Affine motion compensation using 2x2 subblock”, JVET-N0273, the 14th JVET meeting, Mar. 2019, Geneva, CH

- L. Luo, Y. He, “CE2-related: Prediction refinement with optical flow for affine mode”, JVET-N0236, the 14th JVET meeting, Mar. 2019, Geneva, CH

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).