| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0535 |
| --- | --- |

| Title: | Crosscheck of JVET-N0160 (Non-CE6: Simplification on transform selection for Intra Sub-Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-N0160 which is related to transform selection of Intra Sub-Partitioning (ISP) mode. In JVET-N0160, three methods on transform selection for ISP mode were proposed. The software implementation as provided by the proponent of JVET-N0160 has been investigated and confirmed to be aligned with the proposal description. Available simulations results are reportedly matching with the results provided by proponent of JVET-N0160.

# Crosscheck report

In JVET-N0160, three methods on transform selection for ISP mode are proposed, as described below:

Method #1

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 2.

Table 1 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar | DST7 | DST7 |
| DC | DCT2 | DCT2 |
| Ang. [2-34] && even Ang. [35-66] && odd | DST7 | DCT2 |
| Ang. [2-34] && odd Ang. [35-66]&& even | DCT2 | DST7 |

Method #2

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 3.

Table 2 – Transform selection depends on intra mode

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar Ang. [2-66] && odd | DST7 | DST7 |
| DC | DCT2 | DCT2 |
| Ang. [2-34] && even | DST7 | DCT2 |
| Ang. [35-66] && even | DCT2 | DST7 |

Method #3

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>16, tH = DCT2

- If h =2 or h >16, tV = DCT2

- Otherwise, the transform is selected as in Table 4.

Table 3 – Transform selection depends on ISP sub-block shape

| Sub-block shape | tH | tV |
| --- | --- | --- |
| W=H | DST7 | DST7 |
| W>H | DCT2 | DST7 |
| W<H | DST7 | DCT2 |

The anchor used for the simulations is VTM-4.0, and simulations are performed under common test condition (CTC) [1]. Below figure summarizes the results.

Table 4: Coding performance of Method 1 versus VTM-4.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.03% | 100% | 99% | -0.01% | -0.07% | -0.01% | 102% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.08% | -0.03% | 100% | 101% | -0.03% | 0.00% | 0.01% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.12% | 99% | 100% | -0.01% | 0.06% | -0.07% | 100% | 100% |  |  |  |  |  |
| Class C | 0.00% | -0.02% | 0.00% | 101% | 102% | 0.02% | -0.17% | -0.18% | 101% | 102% | 0.00% | 0.05% | 0.00% | 99% | 99% |
| Class E | -0.01% | 0.02% | -0.04% | 98% | 101% |  |  |  |  |  | -0.04% | -0.07% | 0.81% | 101% | 103% |
| Overall | 0.00% | 0.02% | 0.02% | 100% | 100% | -0.01% | -0.04% | -0.07% | 101% | 101% |  |  |  |  |  |
| Class D | -0.01% | -0.02% | 0.07% | 104% | 104% | 0.03% | -0.30% | -0.20% | 102% | 103% | 0.02% | 0.99% | 0.02% | 101% | 101% |
| Class F | 0.00% | 0.03% | -0.06% | 100% | 100% | -0.01% | -0.10% | 0.00% | 102% | 103% | -0.10% | -0.44% | -0.15% | 100% | 101% |

Table 5: Coding performance of Method 2 versus VTM-4.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | -0.05% | 100% | 100% | -0.01% | -0.23% | -0.05% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.02% | 0.05% | -0.06% | 100% | 100% | -0.01% | -0.07% | 0.05% | 101% | 100% |  |  |  |  |  |
| Class B | -0.01% | -0.02% | 0.01% | 100% | 99% | 0.01% | -0.07% | -0.22% | 102% | 101% | 0.00% | -0.49% | -0.60% | 101% | 99% |
| Class C | 0.00% | -0.06% | -0.14% | 102% | 101% | 0.03% | -0.08% | -0.13% | 101% | 102% | 0.29% | -0.03% | 0.05% | 101% | 99% |
| Class E | -0.05% | -0.10% | -0.17% | 99% | 99% |  |  |  |  |  | -0.23% | 0.35% | -0.61% | 101% | 100% |
| Overall | -0.02% | -0.02% | -0.07% | 100% | 100% | 0.01% | -0.10% | -0.11% | 101% | 101% | 0.04% | -0.12% | -0.39% | 101% | 99% |
| Class D | 0.02% | -0.01% | -0.17% | 104% | 104% | -0.01% | -0.31% | -0.43% | 102% | 104% | 0.48% | 0.07% | 0.31% | 98% | 103% |
| Class F | 0.01% | 0.00% | -0.20% | 100% | 102% |  |  |  |  |  | 1.76% | 0.98% | 0.90% | 98% | 98% |

Table 6: Coding performance of Method 3 versus VTM-4.0.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | -0.04% | 100% | 97% | 0.01% | 0.01% | -0.02% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.02% | 0.13% | 0.00% | 100% | 101% | 0.00% | 0.06% | -0.07% | 102% | 101% |  |  |  |  |  |
| Class B | 0.03% | 0.09% | 0.14% | 99% | 98% | 0.03% | 0.10% | -0.03% | 101% | 101% |  |  |  |  |  |
| Class C | 0.07% | 0.03% | 0.08% | 100% | 100% | 0.07% | 0.08% | 0.05% | 101% | 102% | -0.02% | 0.34% | -0.12% | 102% | 102% |
| Class E | 0.13% | 0.16% | 0.13% | 99% | 99% |  |  |  |  |  | 0.08% | 0.44% | 0.19% | 101% | 101% |
| Overall | 0.05% | 0.09% | 0.07% | 99% | 99% | 0.03% | 0.07% | -0.02% | 101% | 101% |  |  |  |  |  |
| Class D | 0.05% | 0.01% | -0.04% | 101% | 103% | 0.03% | -0.09% | -0.05% | 102% | 103% | -0.07% | -0.37% | -0.57% | 103% | 103% |
| Class F | 0.00% | 0.13% | 0.09% | 102% | 100% | 0.01% | 0.00% | -0.09% | 102% | 103% | -0.08% | -0.07% | -0.37% | 101% | 101% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.