| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0536-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0372 (Non-CE3: ISP with independent sub-partitions for certain block sizes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-N0372 which is related to Intra Sub-partitions with independent sub-partition for certain block sizes. The software implementation was provided by the proponent of JVET-N0372. Available simulation results as provided in this contribution are reportedly matching with the results provided by proponent of JVET-N0372.

# Crosscheck report

In JVET-N0372, an extension of the Intra Sub-Partitions (ISP) coding mode to 4×4 blocks is proposed, i.e., enabling 4x1 and 1x4 sub-partitioning with reference samples from CU boundaries. In addition, a modified sub-partitioning for 4x8 and 8x4 blocks is proposed by performing intra prediction using reference samples from CU boundary.

The SW implementation was provided by the proponent of JVET-N0372 and simulations are performed under common test condition (CTC) [1]. Below figure summarizes the results, the anchor used for the simulations is VTM-4.0.

Three tests are crosschecked:

Table 1: Coding performance of JVET-N0372 Test 1 versus VTM-4.0 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.03% | 0.05% | 99% | 100% | 0.00% | 0.02% | 0.05% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | 0.02% | 101% | 100% | 0.01% | 0.08% | -0.07% | 100% | 99% |  |  |  |  |  |
| Class B | -0.05% | -0.05% | -0.13% | 101% | 101% | -0.01% | 0.10% | 0.00% | 100% | 100% | -0.02% | 0.10% | -0.43% | 99% | 100% |
| Class C | -0.22% | -0.14% | -0.20% | 102% | 100% | -0.04% | 0.11% | 0.15% | 100% | 100% | 0.03% | -0.15% | -0.26% | 100% | 99% |
| Class E | -0.19% | -0.13% | -0.11% | 103% | 101% |  |  |  |  |  | -0.04% | -1.21% | -1.07% | 101% | 98% |
| Overall | -0.10% | -0.07% | -0.09% | 101% | 100% | -0.01% | 0.08% | 0.04% | 100% | 100% | -0.01% | -0.31% | -0.53% | 100% | 99% |
| Class D | -0.22% | -0.20% | -0.22% | 101% | 102% | -0.10% | -0.20% | -0.07% | 100% | 99% | 0.01% | -0.44% | -0.19% | 102% | 102% |
| Class F | -0.47% | -0.44% | -0.32% | 99% | 101% | -0.25% | -0.07% | -0.23% | 100% | 100% | -0.04% | 0.22% | 0.22% | 100% | 98% |

Table 1: Coding performance of JVET-N0372 Test 2 versus VTM-4.0 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.03% | 97% | 99% | -0.01% | -0.09% | -0.06% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.01% | -0.04% | -0.01% | 99% | 100% | 0.04% | -0.04% | 0.05% | 99% | 100% |  |  |  |  |  |
| Class B | 0.06% | 0.08% | 0.09% | 99% | 100% | 0.03% | 0.26% | 0.10% | 100% | 100% | 0.04% | 0.22% | -0.36% | 99% | 100% |
| Class C | 0.21% | 0.21% | 0.18% | 99% | 98% | 0.09% | 0.27% | 0.42% | 99% | 99% | 0.00% | -0.26% | -0.20% | 100% | 99% |
| Class E | 0.42% | 0.35% | 0.26% | 98% | 99% |  |  |  |  |  | 0.03% | -0.84% | 0.29% | 102% | 100% |
| Overall | 0.14% | 0.12% | 0.10% | 98% | 99% | 0.04% | 0.13% | 0.14% | 100% | 100% | 0.02% | -0.20% | -0.14% | 100% | 99% |
| Class D | 0.25% | 0.28% | 0.12% | 99% | 98% | 0.13% | 0.13% | -0.19% | 100% | 101% | 0.01% | 0.69% | -0.58% | 100% | 102% |
| Class F | 0.31% | 0.16% | 0.22% | 95% | 98% | 0.23% | 0.13% | 0.17% | 100% | 101% | 0.14% | 0.42% | 0.17% | 98% | 98% |

Table 1: Coding performance of JVET-N0372 Test 3 versus VTM-4.0 under CTC.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.02% | 97% | 100% | 0.01% | -0.08% | 0.09% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.01% | -0.04% | 0.00% | 100% | 98% | 0.02% | 0.15% | 0.01% | 100% | 99% |  |  |  |  |  |
| Class B | -0.02% | 0.06% | -0.01% | 102% | 101% | 0.00% | 0.22% | 0.16% | 101% | 100% | -0.01% | 0.11% | -0.36% | 99% | 99% |
| Class C | -0.13% | -0.05% | 0.00% | 103% | 102% | -0.04% | 0.20% | 0.32% | 100% | 101% | -0.01% | -0.36% | -0.34% | 99% | 98% |
| Class E | -0.08% | -0.01% | -0.07% | 103% | 100% |  |  |  |  |  | -0.03% | -0.23% | 0.18% | 101% | 100% |
| Overall | -0.05% | 0.00% | -0.01% | 101% | 100% | 0.00% | 0.14% | 0.16% | 100% | 100% | -0.02% | -0.13% | -0.22% | 100% | 99% |
| Class D | -0.19% | -0.10% | -0.11% | 101% | 103% | -0.04% | -0.03% | -0.23% | 101% | 100% | 0.00% | 0.68% | -0.21% | 100% | 101% |
| Class F | -0.44% | -0.41% | -0.27% | 97% | 101% | -0.24% | -0.18% | -0.36% | 100% | 100% | -0.09% | 0.60% | -0.05% | 101% | 100% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.