| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0776 |
| --- | --- |

| Title: | BoG Report on CE2-related Contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Yuwen He | Email: | chunchic@qti.qualcomm.com, yuwen.he@interdigital.com |
| Source: | BoG |  |  |

_____________________________

# Summary

The sessions were held 1915~2345 on March 21 and xxxx~xxxx on March xx for discussing 18 technical contributions in 5 categories:

- Shared merging candidate list region for sub-block merge modes (1)

- Memory bandwidth and complexity reduction (3)

- Affine motion compensation (10)

- Storage reduction for sub-block modes (2)

- Others (2)

## Recommendations

### Recommended adoptions to VTM

Normative changes

JVET-N0481 (Method 3): Non-CE2: Gbi inheritance for constructed affine merge candidate

Non-normative changes

### Recommended CE2 tests from following contributions

#### Affine motion compensation

JVET-N0236 (Test 1 and Test 5): CE2-related: Prediction refinement with optical flow for affine mode

JVET-N0273: CE2-related: Affine motion compensation using 2x2 subblock

JVET-N0399 (Test 1 and 2): CE2-related: Simplifications of interweaved affine mode

JVET-N0504: CE2-related: Simplified prediction refinement for affine motion compensation

JVET-N0510: CE2-related: Phase-variant affine subblock motion compensation

## Open issues

### Requested review in track

### Revisit in track

JVET-N0752: CE2-related: Supplementary results for CE2-5.5.c

### Contribution not reviewed yet (7)

JVET-N0199, JVET-N0268, JVET-N0334, JVET-N0335, JVET-N0336, JVET-N0709, JVET-N0737.

# Shared merging candidate list region for sub-block merge modes

JVET-N0079 CE2-related: Applying SMR to subblock-based merging candidate list [Y.-C. Lin, C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, it is proposed to apply shared merging candidate list region (SMR) to subblock-based merging candidate list. The test results are reported below.

When SMR area threshold is set equal to 128:
VTM4.0-RA: {BD-rate=0.01% (Y), 0.01% (Cb), -0.07% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.01% (Y), 0.20% (Cb), -0.01% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 256:
VTM4.0-RA: {BD-rate=0.02% (Y), -0.05% (Cb), -0.08% (Cr); EncT=100%; DecT=102%}
VTM4.0-LB: {BD-rate=0.03% (Y), 0.12% (Cb), -0.16% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 512:
VTM4.0-RA: {BD-rate=0.06% (Y), -0.02% (Cb), -0.05% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.05% (Y), 0.11% (Cb), -0.23% (Cr); EncT=100%; DecT=102%}

It is asserted that the proposed method can increase hardware decoder throughput when subblock-based merging candidates are enabled.

Q. Why the threshold value equal to 32 is chosen for SMR in the last meeting.

A. A participant responded that the decision was made based on less coding loss.

Whether there is really a bottleneck in affine candidate derivation is still questionable.

Test 1 (threshold=128): 0.01%/100%/103% for RA; 0.01%/100%/101% for LB.

Test 2 (threshold=256): 0.02%/100%/102% for RA; 0.03%/100%/101% for LB.

Test 3 (threshold=512): 0.06%/100%/103% for RA; 0.05%/100%/102% for LB.

2-3% decoding time increase is observed, due mainly to non-optimized software implementation.

An expert commented that 8x8 affine may not be a problem and has no throughput issue when smaller CU size for affine merge is 8x8.

Suggestion: No action.

# Memory bandwidth and complexity reduction

JVET-N0199 CE2-related: combined test of affine memory bandwidth complexity reduction (Test2.4.6+Test2.4.8) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic), M. Zhou (Broadcom)]

TBP

JVET-N0266 CE2-related: Disabling bi-prediction or inter prediction for small blocks [H. Liu, L. Zhang (Bytedance)]

Since this proposal was adopted during the discussion of CE2 in Track B, the proponents agreed to not present for this proposal.

JVET-N0369 CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant) [M. Zhou (Broadcom)]

This contribution provides supplemental results for CE2-4.8 (JVET-N0068). Different from algorithm tested in JVET-N0068, in this variant the reference block bounding box size is computed for 2x2 sub-block vectors for both bi-directional and unidirectional affine mode.

For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  2*17*17 for 2x2 sub-block vectors of unidirectional affine mode, the average BD-differences compared to the VTM4.0 are as follows:

- For Test1 in which the fallback mode is not triggered, the BD-rate difference is 0.01% in RA and 0.00% in LD_B, respectively.

- For Test2 in which the fallback mode is triggered, the BD-rate difference is -0.01% in RA and -0.01% in LD_B, respectively.

In CE2-4.8, bounding box is of size 17*17 for 2x2 sub-block vectors of bi-directional affine modes and  17*13 for 2x1 and 1x2 sub-block vectors of unidirectional affine mode.

In this proposal, the setting of bounding box for unidirectional affine mode becomes 2*17*17 for 2x2 subblock vectors.

Method 1: encoder control to ensure affine model parameter meet the memory bandwidth requirement.

Method 2: Test 1 + fallback mode (8x8 subblocks)

The proponent stated that this contribution is proposed just for information.

# Affine motion compensation

JVET-N0236 CE2-related: Prediction refinement with optical flow for affine mode [J. Luo, Y. He (InterDigital)]

This contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, prediction sample is refined by adding a difference derived by the optical flow equation, which is referred as prediction refinement with optical flow (PROF). The proposed method can achieve inter prediction in pixel level granularity without increasing the memory access bandwidth. The simulation result reportedly shows on average X.XX% and X.XX% BD rate saving for RA and LD, respectively, with X.XX% encoding time and X.XX% decoding time increase.

This proposal extends the notion of BDOF to Affine sub-block based motion compensation. It was later confirmed that that BDOF is not applied to sub-block modes in the current draft standard.

Boundary padding of BDOF for regular inter modes is applied only to CU boundaries, while this proposal applies it to subblock boundaries.

The proponent stated that text edges are improved in visual quality.

Test 1: 4x4 Uni/Bi sub-blocks (-0.48%/103%/101% for RA)

Boundary padding process is not aligned with the current BDOF’s design. This proposal takes the nearest integer samples, while BDOF clips MV and takes the corresponding integer samples for padding.

Test 2: Test 1 + Replace 4x4 Bi by 8x8 Bi (-0.33%/102%/100% for RA.)

Test 3: Same as Test 1 but used for Huawei’s sequences (-1.77% gain for RA; -1.02% for LB; Peak -2.62% SpincalendarPart)

Test 4: 4x4 Uni/Bi sub-blocks + Alternative gradient method (-0.52%/105%/102% for RA)

The padding process is applied to each boundary of the 11x11 reference samples of each 4x4 subblock by 1 pixel at each side, resulting in 13x13.

Test 5: Test 4 + Replace 4x4 Bi by 8x8 Bi + Alternative gradient method (-0.38%/ 103%/ 101% for RA)

Test 6: Test 1 + Replace 4x4 Bi by 8x4 Bi (-0.40%/ 103%/ 101% for RA)

One non-proponent recommended Test 1 could be considered for adoption, while several other non-proponents recommend to further study this proposal in a context of CE.

As bandwidth can already be addressed in CE2-4.8, 8x8 Bi sub-block configuration as proposed may not be necessary for testing.

It was also commented that there are similar approaches (e.g. interweave prediction) that can achieve similar coding performance-wise benefit and are suggested to study them together.

Some participants showed interest to see combination testing results of this proposed method and other related proposals (e.g. interweave prediction)

Suggestion: Further study Test 1 (with at most 8x8 subblock size for Bi) and Test 5 in a CE.

JVET-N0273 CE2-related: Affine motion compensation using 2x2 subblock [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

In this contribution, it’s proposed to adaptively use 2x2 or 4x4 subblock based motion compensation for affine uni-prediction.  The adaptation is based on the affine parameters and ensure that the proposed method doesn’t increase worst case memory bandwidth. The proposed method is only applied to uni-prediction, it also doesn’t increase the worst-case computational complexity for interpolation. The test results show that the BD-rate changes are -0.34%, -0.05%, 0.02% in RA and -0.18%, -0.01%, -0.19% in LDB

In current VTM, the worst-case computational complexity for motion compensation is 4x4 bi-prediction. For 4x4 uni-prediction, it requires 4*(4+7) interpolations in horizontal direction and 4*4 interpolations in vertical direction. Therefore, the total number of interpolations for 4x4 bi-prediction is (4*11+4*4) *2 = 120, i.e. 7.5 interpolations / sample. For 2x2 uni-prediction, it requires 2*(2+7) interpolations in horizontal direction and 2*2 interpolations in vertical direction, 22 in total, i.e. 5.5 interpolations / sample. A 4x4 subblock using 2x2 uni-prediction needs 88 (22*4) interpolations.

Each interpolation requires 8 multiplications and 7 additions. Therefore, 4x4 bi-prediction requires 960 multiplications and 840 additions. The 2x2 uni-prediction for a 4x4 block requires 704 multiplications and 616 additions.

(Chaired by Y. He)

2x2 uni-prediction is less complex than 4x4 bi-prediction.

Discussions:

The method will switch 2x2 interpolation and 4x4 interpolation for uni-prediction. The bounding box size is 15x15 for each 8x8 region (equivalent to 8x8 MC).

The storage for MV field is still 4x4. 4x4 sub-block MV is derived by averaging two diagonal 2x2 sub-block MVs. The method is only applied to luma component.

In order to pre-fetching memory for MC, the bounding box needs to be calculated before MC.

The MV used in the MC (2x2 sub-block) is not aligned with the MV (4x4 sub-block) used for MV prediction.

The MV used in the strength derivation of deblocking is not aligned with the MV used in MC.

2x2 MC is not friendly for SIMD implementation.

Suggestion: Further study in CE; (1) validate the bounding box calculation; (2) SIMD implementation.

JVET-N0399 CE2-related: Simplifications of interweaved affine mode [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

Interweaved affine mode was proposed to improve coding efficiency. It converts uni-predicted affine block to bi-predicted block by introducing a uni-predicted auxiliary affine block. The final prediction is obtained by weighted averaging the two uni-predicted blocks. In this contribution, it is proposed to regard interweaved affine mode as a special bi-predicted mode so that weighted bi-prediction process is reused. Furthermore, it is proposed to simplify the weighting process. It is reported that minor impact on coding performance is observed by the proposed methods.

Improvement to Interweave CE2-1.1b.

Method 1: Use high-precision bi-prediction for interweave

Method 2: Apply 0.5 weight to non-boundary samples to achieve bi-prediction

[Test 1]  Method 1

- Relative to VTM-4.0: -0.32%/103%/102% for RA; -0.12%/104%/104% for LB.

- Relative to CE2-1.1b: 0.00%/102%/103% for RA; 0.02%/100%/101% for LB.

[Test 2]  Method 1 + Method 2

- Relative to VTM-4.0: -0.30%/103%/101% for RA; -0.14%/104%/103% for LB.

- Relative to CE2-1.1b: 0.02%/102%/103% for RA; 0.00%/101%/100% for LB.

The proponent stated that the runtime is not reliable in this test.

A non-proponent and crosschecker suggest that this simplification could benefit the design of the interweave prediction (CE2-1.1) in terms of complexity.

Suggestion: Further study Test 1 and Test 2 in a CE.

JVET-N0456 CE2-related: Affine motion model derivation method for affine merge mode [W. Chen, J. Luo, Y. He (InterDigital)]

Regression based motion vector field (RMVF) proposed in JVET-L0171 provides a new motion vector representation for merge mode. This tool derives the motion vectors of each sub-block in the CU based on the spatially neighboring motion vectors. The current contribution proposes a new affine motion model derivation method without using any least mean square error calculation. When affine merge mode is selected and the proposed model derivation method is enabled in VTM-4.0, the affine model estimated from MVs of neighboring coding blocks can be used as a predictor of the affine model to be used for current CU.

The proposed method provides on average -0.xx% and -0.xx% luma BD-Rate impact in RA and LDB configurations, respectively. The encoding runtimes of xxx% and xxx % and decoding runtimes of xxx% and xxx% in RA, LDB, and LDP configurations were observed, respectively.

Simplification of RMVF to avoid using the least-mean square for parameter derivation. This mode is signalled as an alternative merge mode with 4-paramenter and 6-parameters modes.

In VTM-3.0, it is mentioned from the proponent that the coding gain of RMVF (without simplification) is around 0.09%.

A non-proponent commented that the RMVF’s complexity characteristics should be evaluated and compared with that of affine merge (inherited and constructed modes) to justify the coding performance-wise improvement with the additionally increase of computational complexity.

-0.07%/-0.06% for Class B/C with +2%/+2% encoding/decoding time increase .

Suggestion: No action.

JVET-N0481 Non-CE2: Gbi inheritance for constructed affine merge candidate [J. Chen, R.-L. Liao, Y. Ye (Alibaba)]

In current VVC, Bi-prediction with CU weights (BCW, or GBi) and affine merge are both employed. With BCW, each inter-coded block is further assigned a bi-prediction weight index in addition to the motion information. In the current affine merge design, for inherited affine merge candidate, the BCW of neighboring block is inherited, while for constructed affine merge candidate, the BCW index is set to the default value (i.e. equal weight). To align the design of BCW index derivation between these two types of affine merge candidates, it is proposed to derive a BCW index for constructed affine merge candidate instead of always setting to the default value. Simulation results reportedly show that the harmonization achieves 0.02% and 0.05% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

Gbi index inheritance alignment for constructed affine merge.

Method 1: if at least 2 out of the 3 CPMVs (top-left, top-right and bottom-left) are sharing the same GBi index, this GBi index will be set to all three CPMVs; otherwise, default GBi index will be set -> at most 3 comparisons.

RA: -0.02%/100%/99%; LB: 0.05%/101%/102%, where Class E is observed to be with 0.05% loss w/ peak loss 0.16%.

Method 2: if 2 of the CPMVs' GBi index forms a pair (e.g. index 0 and 4 which correspond to -1/4 and 5/4), then the default GBi index will be set; otherwise, Method 1 applies -> at most 3 additions and 3 comparisons + 3 comparison from Method 1.

RA: -0.02%/102%/101%; LB: -0.02%/101%/102%.

Method 3: divide GBi index into groups of {0}, {1, 2, 3} and {4}. If all the GBi indices are from the same group, method 1 will be used; otherwise, the default GBi index will be set.

The intention of index grouping is to prevent from falling into extreme cases.

RA: -0.02%/101%/101%; LB: -0.05%/101%/102%.

The proponent stated preference for this method and was later supported by several non-proponents.

It is stated this method achieves better design consistency between inheritance and constructed affine merge candidates.

Suggestion: Adopt Method 3.

JVET-N0504 CE2-related: simplification of 2x2 subblock based affine motion compensation [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

In this contribution, a 2x2 subblock based prediction refinement method is proposed for affine motion compensation. The prediction refinement is performed by using the optical flow equation to derive an offset for each pixel in a 4x4 subblock. The delta motion vectors are calculated in a 2x2 subblock basis, and the gradient can be calculated in parallel for each 4x4 subblock. The test results show that the BD-rate changes are -0.28%, -0.08%, -0.01% in RA and -0.18%, -0.06%, -0.11% in LDB.

(Chaired by Y. He)

Difference between this proposal and JVET-N0236: (1) 2x2 sub-block based optical flow; (2) the gradient calculation at 4x4 sub-block boundary.

Discussion:

It is commented that the refinement is still sample based operation though the optical flow is based on 2x2 sub-block level.

The gradient calculation is not aligned with that in BDOF at 4x4 sub-block boundary.

The reduction on optical flow calculation for each CU is from 32 additions to 8 additions.

Suggestion: Further study in CE; test 2x2 sub-block level optical flow.

JVET-N0510 CE2-related: Phase-variant affine subblock motion compensation [T.-D. Chuang, C.-Y. Chen, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

In this contribution, phase-variant affine subblock motion compensation (MC) is proposed. The conventional two stage horizontal-vertical interpolation is applied. However, unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows and different sample columns in the affine subblock. The used interpolation filter sets in the proposed method are the same as those in VTM4.0. The number of filtering operations for each affine subblock in the proposed method is also the same as that in VTM4.0. Simulation results reportedly show that the luma BD-rates are -0.33% and -0.23% in RA and LB cases, respectively.

This proposal applies difference horizontal filters associated with different phases as suggested by affine parameters to generating intermediate filter results from horizontal filtering. Therefore, multiple interpolation filters (11) associated different phases are applied to respective rows on the (4+7)x(4+7) reference samples.

The same approach is further applied to vertical filtering to generate the resulting 4x4 affine sub-block prediction signals.

Subjective comparisons are provided in the proposal, showing better visual quality particularly in object edges.

It is noted that additional logics (one add, right-shift, clip operations) are required in MC stage, compared with current affine motion compensation.

It was asked whether loading multiple set of interpolation filters for 4x4 subblock is doable. It was later confirmed from the proponent that in hardware implementation, 11 sets of filter coefficients are loaded simultaneously (one for a row of 4x4 subblock‘s reference samples) and then horizontal or vertical interpolation can be done parallelly in a pipeline stage.

RA: -0.33%/102%/101%; LB: -0.23/101%/100%.

Suggestion: Further study in CE.

JVET-N0709 CE2-related: Computation reductions of interweaved affine mode [K. Kawamura, S. Naito (KDDI)] [late]

TBP

JVET-N0737 Non-CE2: Combination of affine MV clip and prediction refinement with optical flow [X. Li, G. Li, X. Xu, S. Liu (Tencent)] [late]

TBP

JVET-N0752 CE2-related: Supplementary results for CE2-5.5.c [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)] [late]

In this contribution, supplementary results for CE2-5.5.c are provided. Two tests are conducted based on CE2-5.5.c. In supplementary test 1, at most two merge candidates can be inherited from the history table or neighbouring blocks. In supplementary test 2, five affine constructed merge candidates are removed. Simulation results reportedly show that the BD-rate changes are -0.04% and -0.01% under the RA test configurations in test 1 and test 2, respectively.

This proposed method introduces 3 new potential candidates with removing 5 constructed candidates, so the total potential candidate is reduced from 8 to 6.

Suggestion: To be revisited in Track B together with CE2-5.5c.

# Storage reduction for sub-block modes

JVET-N0268 CE2-related: Remove above-left affine inherited motion vector predictor [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

TBP

JVET-N0336 CE2/4-related: On motion field storage reduction for spatial motion prediction [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

TBP

# Others

JVET-N0334 CE2/4/9-related: Overflow prevention in motion field storage [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

TBP

JVET-N0335 CE2/4-related: unification of MV rounding [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

TBP