| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0010 |
| --- | --- |

| Title: | JVET AHG report: Encoding algorithm optimizations (AHG10) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Alberto Duenas Alexis Michael Tourapis C. Helmrich Sergey Ikonin Andrey Norkin Rickard Sjöberg | Email: | alberto.duenas@outlook.com atourapis@apple.com christian.helmrich@hhi.fraunhofer.de Sergey.Ikonin@huawei.com anorkin@netflix.com rickard.sjoberg@ericsson.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

The document summarizes the activities of the AHG on Encoding algorithm optimizations between the 14th meeting in Geneva, CH (19-27, March 2019) and the 15th meeting in Gothenburg, SE (3-12, July 2019).

# Introduction

At the 14th JVET meeting, the AHG on Encoding algorithm optimizations was established with the following mandates:

- Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization.

- Study the impact of adaptive quantization on individual tools in the test model.

- Study the quantization adaptation tool in the test model.

- Investigate the feasibility of adding a CTC test category in which adaptive quantization is turned on.

- Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures, adaptive quantization without signalling, and multi-pass encoding.

- Study methods of rate control and their impact on performance, subjective and objective quality.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Overview of input documents related to the AHG

The following input documents were identified to be related to the AHG:

## JVET-O0256: Non-CE: Fast encoder with adjusted threshold in dependent quantization

In the current VVC design, reverse scanning of coefficients is applied. To save the encoding time, the determination of last non-zero coefficient in the dependent quantization process is based on a given threshold. With this scheme, several high frequency coefficients can be directly set to zero without quantization. However, as a bundle of advanced prediction and transform technologies have been adopted, the frequency domain energy becomes more compact. Therefore, a larger threshold is proposed, which is asserted to be more effective on saving the encoding time. Simulation results reportedly show that the proposed threshold setting is able to bring 6% and 4% encoding time saving with 0.02% and 0.00% BD-rate loss under the AI and RA configurations, respectively.

## JVET-O0432: AHG10: LMCS encoder improvement

This contribution proposes non-normative encoder algorithm optimizations for LMCS to improve coding performance for SDR and enable LMCS for HLG. The specific changes proposed are:

- support LMCS for HDR-HLG CTC content in RA and AI test conditions;

- replace the fixed 5x5 neighborhood size used to calculate local spatial variance with a variable neighborhood size that increases or decreases based on picture resolution;

- simplify the method of allocating codewords based on histogram and local spatial variance; and,

- provide LMCS encoder configuration parameters in accordance with “Guidelines for VVC reference software development.”

The coding performance for SDR CTC using the proposed encoder implementation compared to VTM5.0 anchor (LMCS enabled) in terms of BDRate for PSNR Y, U, and V and processing times is:

- AI     {-0.30%,   0.33%,  0.32%}, Enc: 101%, Dec: 100%

- RA    {-0.16%, -0.11%, -0.12%}, Enc:  98%, Dec:  99%

- LDB  {-0.20%, -0.37%, -0.59%}, Enc: 100%, Dec: 99%

The coding performance for HLG CTC using the proposed encoder implementation compared to VTM5.0 anchor (LMCS disabled) in terms of BDRate for PSNR Y, U, and V and processing times is:

- AI     {-0.17%, 2.04%, 1.99%}, Enc: 103%, Dec: 103%

- RA    {-1.03%, 2.28%, 1.94%}, Enc: 100%, Dec: 103%

## JVET-O0549: AHG10: Encoder-only GOP-based temporal filter

Unavailable

## JVET-O0635: AHG10-Related:optimized CTU level bit allocation in inter frames

Unavailable

## JVET-O0888: AHG10: Adaptive Coding Sub-set for encoder optimization

This contribution introduces an additional flexibility capacity at the encoder level. This proposal tries to optimize the coding cost of specific parameters in case of underuse of the full standard capacity. Two scenarios are considered: underuse due to specific content at a given encoding point and desired underuse of modes due to a specific encoder implementation. While several mechanisms are already available to handle these underuses in previous standards and in current VVC draft, including high level flags and CABAC probability update, it is asserted that higher flexibility might bring a better trade-off for encoder design. The contribution proposes a subset signalization of specific coding modes at PPS or Slice level. This is presented in this contribution for two coding elements: the SAO offsets and the intra coding modes. Three way of sub-set signaling are evaluated in the contribution and it is asserted that early results show a potential to be further investigated.

# Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.