| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0013 |
| --- | --- |

| Title: | JVET AHG report: Tool reporting procedure (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jill Boyce (co-chairs) Yi-Wen Chen, Roman Chernyak, Kiho Choi, Ryoji Hashimoto, Yu-Wen Huang, Hyeongmun Jang, Ru-Ling Liao, Shan Liu, Daniel Luo (vice-chairs) | Email: | wchien@qti.qualcomm.com jill.boyce@intel.com yiwenchen@kwai.com chernyak.roman@huawei.com kiho14.choi@samsung.com ryoji.hashimoto.te@renesas.com yuwen.huang@mediatek.com hm.jang@lge.com ruling.lrl@alibaba-inc.com shanl@tencent.com jiancong.luo@alibaba-inc.com |
| Source: | AHG13 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 14th Meeting in Geneva, CH (19–27 March 2019) and the 15th meeting in Gothenburg, SE (3–12 July 2019). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-N1005.

# Mandates

The AHG was established with the following mandates:

- Prepare output document JVET-N1005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers.

- Provide configurations files, bitstreams, and results of the tool-on/tool-off testing.

- Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing.

- Prepare a report with results of the tests.

# Activities

The initial version of JVET-N1005 “Methodology and reporting template for tool testing” was provided on April 11th.  The document contained a reporting template.

All tests described in JVET-N1005 were conducted. VTM tool tests were conducted on VTM-5.0 software with VTM configuration by switching off or on specific tool either in configuration files or macros.

The tested tools, testers, and cross-checkers are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | RA | LD | Tester | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma separate tree | CST | JVET-K0230, JVET-K0556, JVET-N0137 | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Dependent quantization* | DQ | JVET-M0173, JVET-M0251, JVET-M0470, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Cross-component linear model | CCLM | JVET-L0340, JVET-N0271 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| multiple transform set | MTS | JVET-M0297, JVET-M0464, JVET-M0497, JVET-N0866 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Adaptive loop filter | ALF | JVET-M0132, JVET-N0242, JVET-N0415, JVET-N0180 | X | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Daniel Luo (daniel.luo@interdigital.com) |
| Affine motion model | AFF | JVET-M0145, JVET-M0192, JVET-N0068, JVET-N0194, JVET-N0334, JVET-N0481 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; guichunli@ tencent.com) |
| subblock-based temporal merging candidates | SbTMVP | JVET-M0273 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Adaptive motion vector resolution | AMVR | JVET-M0246 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Triangular partition mode | TPM | JVET-M0328, JVET-M0883, JVET-N0340, JVET-N0483 | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| Bi-directional optical flow | BDOF | JVET-M0487, JVET-N0444, JVET-N0146, JVET-N0325 |  | X |  | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Combined intra/inter prediction | CIIP | JVET-L0100, JVET-N0302 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Merge with MVD | MMVD | JVET-M0068, JVET-M0171, JVET-M0255, JVET-N0127, JVET-N0332, JVET-N0448, JVET-N0380 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Bi-predictive with CU weights | BCW | JVET-M0111, JVET-M0264, JVET-N0481 |  | X | X | Daniel Luo (daniel.luo@interdigital.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Multi-reference line prediction | MRLP | JVET-L0283 | X | X | X | Shan Liu (shanl; leolzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Intra block copy mode** | IBC | JVET-M0407, JVET-M0483, JVET-N0383, JVET-N0175, JVET-N0251, JVET-N0384, JVET-N0317, JVET-N0383, JVET-N0251, JVET-N0318, JVET-N0467, JVET-N0843 | X | X | X | Shan Liu (shanl; xiaozhongxu@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Intra sub-partitioning | ISP | JVET-M0102, JVET-N0308 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Decoder motion vector refinement | DMVR | JVET-M0147, JVET-N0407, JVET-N0178, JVET-N0146, JVET-N0153, JVET-N0442, JVET-N0162, JVET-N0262, JVET-N0440, JVET-N0086 |  | X |  | Daniel Luo (daniel.luo@interdigital.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Sub-block transform | SBT | JVET-M0140 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Luma mapping with chroma scaling | LMCS | JVET-M0427, JVET-N0220, JVET-N0477, | X | X | X | Taoran Lu (tlu@dolby.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Symmetric motion vector difference | SMVD | JVET-M0444 |  | X | X | Yi-Wen Chen(yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Quantized residual DPCM*** | RDPCM | JVET-N0413 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Matrix based intra prediction | MIP | JVET-N0217 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Low frequency non-separable transform | LFNST | JVET-N0105, JVET-N0193 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Joint coding of chrominance residuals | JCCR | JVET-N0054 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Temporal motion vector predictor | TMVP |  |  | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) |  |

* Test was conducted by disabling DQ and enabling Sign Data Hiding

** Test was conducted by enabling IBC for anchor

*** Test was conducted by enabling RDPCM

# Results

The results of the tests are summarized in Table 2-6 below. The attached spreadsheet provides additional data. Table 7 shows tool test results across several VTM versions. The combined BD-Rate is computed based on (BD-Rate_Y*8+ BD-Rate_U+ BD-Rate_V)/10. Scatter plots are also provided for the tested tools in random access configuration, comparing PSNR-Y based bd-rate on the Y axis vs. each of Enc runtime ratio, Dec runtime ratio, and a weighted average of Enc and Dec runtime ratio, (Enc + a*Dec)/(a+1), with a configurable weight, a. The exemplary weighting is set to 6 and can be adjusted in the spreadsheet attached to this report.

Full experimental results and configuration files can be found at the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-5.0/

There was no bitrate or PSNR differences between testers and cross-checkers.

Encoder and Decoder runtime ratios provided by both the testers and cross-checkers are included in the reporting template, to identify if there were significant runtime differences.

Table 3 Simulation results in all intra configuration (AI) of VTM tool tests. (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.41% | 12.73% | 13.50% | 170% | 103% | 172% | 103% |
| DQ | 1.90% | -0.94% | -0.95% | 96% | 108% | 91% | 105% |
| CCLM | 1.85% | 17.20% | 17.54% | 99% | 100% | 98% | 98% |
| MTS | 0.84% | 0.37% | 0.42% | 87% | 99% | 87% | 98% |
| ALF | 2.42% | 2.73% | 3.06% | 98% | 87% | 97% | 87% |
| MRLP | 0.36% | 0.16% | 0.18% | 96% | 99% | 95% | 99% |
| IBC | 0.63% | 0.63% | 0.66% | 68% | 100% | 69% | 97% |
| LMCS | 0.81% | -1.20% | -0.98% | 100% | 99% | 99% | 99% |
| RDPCM on | -0.05% | -0.05% | -0.06% | 104% | 100% | 106% | 100% |
| MIP | 0.36% | 0.10% | 0.14% | 87% | 100% | 86% | 97% |
| LFNST | 1.26% | 0.39% | 0.55% | 77% | 99% | 77% | 100% |
| JCCR | 0.29% | 0.58% | 1.40% | 95% | 100% | 98% | 102% |

Table 4 Simulation results in random access configuration (RA) of VTM tool tests. (VTM anchor)

|  |  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.14% | 6.05% | 7.57% | 105% | 98% | 101% | 97% |
| DQ | 1.71% | -0.51% | -0.72% | 101% | 104% | 97% | 101% |
| CCLM | 0.90% | 15.85% | 16.95% | 99% | 100% | 98% | 99% |
| MTS | 0.33% | 0.51% | 0.42% | 91% | 100% | 92% | 101% |
| ALF | 4.91% | 4.56% | 4.07% | 97% | 79% | 97% | 83% |
| AFF | 2.53% | 1.83% | 1.75% | 90% | 99% | 89% | 98% |
| SbTMVP | 0.43% | 0.29% | 0.31% | 100% | 100% | 100% | 98% |
| AMVR | 1.05% | 1.51% | 1.48% | 89% | 101% | 89% | 99% |
| TPM | 0.35% | 0.63% | 0.67% | 92% | 100% | 92% | 99% |
| BDOF | 0.78% | 0.24% | 0.15% | 96% | 93% | 99% | 92% |
| CIIP | 0.38% | 0.08% | 0.02% | 96% | 99% | 99% | 99% |
| MMVD | 0.58% | 0.64% | 0.64% | 91% | 100% | 92% | 101% |
| BCW | 0.43% | 0.53% | 0.58% | 96% | 101% | 98% | 102% |
| MRLP | 0.20% | 0.03% | -0.02% | 99% | 100% | 99% | 100% |
| IBC | -0.01% | 0.33% | 0.25% | 94% | 100% | 95% | 102% |
| ISP | 0.13% | 0.04% | 0.08% | 98% | 100% | 98% | 100% |
| DMVR | 0.82% | 1.05% | 1.07% | 101% | 96% | 100% | 96% |
| SBT | 0.41% | -0.02% | 0.07% | 95% | 100% | 96% | 100% |
| LMCS | 1.39% | -2.83% | -2.55% | 94% | 99% | 94% | 99% |
| SMVD | 0.25% | 0.25% | 0.23% | 96% | 101% | 97% | 101% |
| RDPCM on | -0.01% | -0.07% | -0.05% | 100% | 100% | 101% | 101% |
| MIP | 0.27% | 0.26% | 0.32% | 94% | 100% | 93% | 97% |
| LFNST | 0.79% | 0.39% | 0.81% | 85% | 100% | 86% | 100% |
| JCCR | 0.25% | 0.65% | 0.72% | 97% | 99% | 99% | 102% |
| TMVP | 1.19% | 0.93% | 1.03% | 100% | 97% |  |  |

Table 5 Simulation results in low delay B configuration (LDB) of VTM tool tests. (VTM anchor)

|  |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.08% | 2.44% | 3.56% | 100% | 99% | 99% | 99% |
| DQ | 1.37% | 1.78% | 1.42% | 99% | 102% | 97% | 102% |
| CCLM | 0.12% | 4.53% | 5.62% | 100% | 100% | 100% | 103% |
| MTS | 0.23% | 0.49% | 0.62% | 97% | 99% | 98% | 99% |
| ALF | 4.16% | 4.74% | 3.85% | 95% | 81% | 94% | 83% |
| AFF | 2.18% | 1.54% | 1.73% | 83% | 97% | 83% | 95% |
| SbTMVP | 0.60% | 0.62% | 0.36% | 101% | 98% | 100% | 97% |
| AMVR | 0.39% | 0.82% | 0.73% | 91% | 97% | 91% | 97% |
| TPM | 0.88% | 1.34% | 1.02% | 87% | 101% | 88% | 103% |
| CIIP | 0.46% | 0.93% | 0.81% | 96% | 98% | 96% | 99% |
| MMVD | 0.57% | 0.39% | 0.18% | 94% | 102% | 94% | 104% |
| BCW | 0.31% | 0.32% | 0.25% | 97% | 99% | 98% | 100% |
| MRLP | 0.12% | 0.06% | 0.11% | 100% | 103% | 100% | 102% |
| IBC | -0.03% | 0.04% | 0.35% | 84% | 100% | 86% | 103% |
| ISP | 0.06% | 0.19% | 0.44% | 99% | 100% | 100% | 102% |
| SBT | 0.69% | 0.48% | -0.08% | 92% | 99% | 93% | 98% |
| LMCS | 0.68% | -2.23% | -2.21% | 94% | 99% | 93% | 100% |
| RDPCM on | -0.02% | 0.16% | 0.10% | 100% | 99% | 102% | 101% |
| MIP | 0.07% | 0.20% | 0.06% | 97% | 100% | 96% | 101% |
| LFNST | 0.34% | -2.30% | -2.60% | 86% | 102% | 85% | 98% |
| JCCR | 0.12% | 3.39% | 5.19% | 99% | 99% | 101% | 101% |
| TMVP | 2.16% | 1.85% | 1.82% | 101% | 97% |  |  |

Table 6 Pixel usage and memory bandwidth results of VTM tool “off” test. (VTM anchor)

|  | AI |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | Pixel usage | Pixel usage | Ave mem BW | Max mem BW | Pixel usage | Ave mem BW | Max mem BW |
| CCLM | 48.03% | 3.54% |  |  | 0.92% |  |  |
| ALF | 99.00% | 65.38% |  |  | 60.05% |  |  |
| AFF |  | 13.41% |  |  | 19.38% |  |  |
| SBTMC |  | 6.13% |  |  | 7.64% |  |  |
| AMVR |  | 4.79% |  |  | 2.02% |  |  |
| TAP |  | 2.38% |  |  | 5.62% |  |  |
| BDOF |  | 50.03% |  |  |  |  |  |
| CIIP |  | 1.09% |  |  | 1.57% |  |  |
| MMVD |  | 7.67% |  |  | 9.34% |  |  |
| BPWA |  | 9.66% |  |  | 8.37% |  |  |
| MRLP | 6.99% | 0.80% |  |  | 0.27% |  |  |
| DMVR |  | 42.79% |  |  |  |  |  |
| SBT |  | 2.50% |  |  | 4.06% |  |  |
| SMVD |  | 2.68% |  |  |  |  |  |
| MIP | 13.03% | 3.75% |  |  | 2.10% |  |  |
| LFNST | 10.05% | 2.70% |  |  |  |  |  |
| JCCR | 4.38% | 0.22% |  |  | 0.08% |  |  |

Table 7 test results of VTM tool “off” test on various VTM versions

|  |  | VTM RA |  |
| --- | --- | --- | --- |
| Abbreviation | VTM3 | VTM4 | VTM5 |
| CST | 0.74% | 1.25% | 1.47% |
| DQ | 1.39% | 1.36% | 1.24% |
| CCLM | 4.09% | 4.20% | 4.00% |
| MTS | 1.25% | 0.80% | 0.36% |
| ALF | 3.61% | 3.73% | 4.79% |
| AFF | 2.42% | 2.46% | 2.38% |
| SbTMVP | 0.52% | 0.43% | 0.40% |
| AMVR | 0.98% | 1.13% | 1.14% |
| TPM | 0.43% | 0.43% | 0.41% |
| BDOF | 1.02% | 0.63% | 0.66% |
| CIIP | 0.43% | 0.51% | 0.31% |
| MMVD | 0.81% | 0.52% | 0.59% |
| BiCW | 0.48% | 0.45% | 0.45% |
| MRLP | 0.24% | 0.18% | 0.16% |
| IBC | 0.07% | 0.00% | 0.05% |
| ISP |  | 0.24% | 0.12% |
| DMVR |  | 0.80% | 0.87% |
| SBT |  | 0.33% | 0.33% |
| LMCS |  | 0.62% | 0.57% |
| SMVD |  | 0.26% | 0.24% |
| RDPCM on |  |  | -0.02% |
| MIP |  |  | 0.27% |
| LFNST |  |  | 0.75% |
| JCCR |  |  | 0.34% |
| TMVP |  |  | 1.15% |

Figure 1. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 2. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 3. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (VTM anchor)

# Related contributions

| JVET-0461 | AHG13: On chroma tool evaluation in Dual Tree | T. Poirier, F. Le Léannec, F. Galpin, E. Francois (InterDigital) |
| --- | --- | --- |

| JVET-O0551 | AHG13: Tool-off tests on OpenImage database images | J. Ström, D. Saffar, P. Wennersten, R. Sjöberg |
| --- | --- | --- |

# Recommendations

The AHG recommends the following:

- Consider the reported tool test results during tool adoption decision making

- Review related contributions

- Refine list of tested tools and test methodology for the next meeting cycle

  - Consider the reported tool test results as a benchmark for CE tests

  - Consider including reporting of compute system information for testers and cross-checkers

  - Consider additional performance or complexity metrics