| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0016-v1 |
| --- | --- |

| Title: | JVET AHG report: Implementation studies (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Zhou, J. An, E. Chai, K. Choi, S. Sethuraman, T. Hsieh, X. Xiu | Email: | minhua.zhou@broadcom.com jicheng.ajc@alibaba-inc.com eric.chai@ublnx.com kiho14.choi@samsung.com sriram.sethuraman@ittiam.com thsieh@qti.qualcomm.com xiaoyuxiu@kwai.com |
| Source: | AHG16 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG16: implementation studies, between the 14th JVET meeting in Geneva, CH (19–27 March 2019) and the 15th JVET meeting in Gothenburg, SE (3-12 July 2019).

The AHG was established with the following mandates:

- Study draft and proposed coding tools to identify implementation issues relating to decoder pipelines, decoder throughput, and other aspects of implementation difficulty.

- Solicit hardware analysis of complex tools.

- Provide feedback on potential solutions to address identified issues.

# Activities

There were not many email exchanges on the main JVET email reflector (jvet@lists.rwth-aachen.de) with an [AHG16] indication on message headers. A summary of the AHG activities is provided as follows:

## Comments on the current VTM5.0 design and some of tools being tested

- It is strongly recommended to simply remove the 1xN/2xN sub-partitions from the intra sub-partition prediction (ISP) to alleviate implementation difficulties in both the intra prediction and transform parts of the design. It is not desirable to make the design even more complicated by introducing more small size sub-partitions.

- The CE2 tests aiming for the reduction of the chroma reconstruction latency (caused by the combination of separate tree and CCLM) may not serve the purpose, as the derivation of the cclmEnabled flag is a complex process and the CABAC parsing depends on it. It is hence recommended to seek simplifications outside the CE.

- There are CE tests going on to move the validity check of luma/chroma IBC vectors to the decoder side. While those methods may make decoder designs safer, it should be investigated whether there are enough cycles on the decoder side to do the check, as the operation is down to 4x4 for luma and 2x2 for chroma in the worst case.

- The newly adopted residual coding for transform skip is significantly different from the existing residual coding method, requiring yet another building block.  It codes the coefficients in reverse order, requires three additional loops through all the coefficients for gt3/gt4/gt5 coding, and does not benefit coding of camera captured content. It is desirable to align it with the existing residual coding in terms of coefficient coding order and to reduce the number of coefficient coding passes/loops for CABAC throughput improvement and cost reduction.

- For the baseline VVC design, it is generally not recommended to introduce more context coded bins and specific processing logic for minor gain in SCC without benefiting coding of camera captured content, because doing so not only decreases the CABAC throughput but also adds cost. It might be more appropriate to study those for more complex profiles.

- The maximum number of distinct APSs that a picture can use for the ALF and LMCS should be properly defined, in order to constrain the memory footprint required for storing ALF/LMCS parameters and computational complexity needed for LMCS table derivation (which has divisions in it).  For example, the current design allows up to 32 APSs signaled. It would require roughly 13 Kbytes on-chip memory to buffer those APSs if all of them are used for the ALF of a picture (not counting fixed filters that need roughly 1 Kbytes additional memory).    Each ALF parameter set has about 410 bytes.

- The MV context storage of triangle merge mode (in which the triangle MVs are populated to create 4x4 block vectors that are stored as TMVPs and used as spatial MV neighbors of merge/AMVP modes) seems to be unnecessarily complicated, it should be simplified.

- Removal of small block size (e.g. 2x2) chroma intra-prediction for the shared tree case is essential for high performance decoder implementations (being studied in CE3). The issue was already resolved for the separate (dual) tree case at the last meeting.

- As the project is approaching the CD stage, more attention should be paid to non-CTC and corner cases, and developing solutions to fix the broken parts of design (if any). For example, in the current VTM5.0 design the merge mode is broken, because the decoder behavior is undefined when all the merge flags are set to zero.

- In the current design, the LFNST flag is derived at the end of the CU (need to check total number of coefficients inside and outside the LFNST corner regions).  For a 128x128 CU, the first PB could not be processed without waiting for the LFNST index that comes after the last PB. Although this does not necessarily break the overall 64x64 based decoder pipeline (if the CABAC could be decoupled), it increases the data buffering by 4x for a certain number of decoder pipeline stages. It is costly.  It is desirable to disable the LFNST flag derivation across the 64x64 VDPU boundaries.  Potential simplifications could include e.g. moving it to a per TU index rather than a per-CU index, or disabling it for CU size 128x128.

## Implementation issues identified

- In the newly adopted RDPCM, the residual coefficients could potentially grow significantly larger than the standard 16-bit coefficient range (-32767, 32767) for 10-bit video coding if allowed to grow unbounded across all the columns (or rows). It is desirable to constraint the residual coefficients in 16-bit range.  This was fixed by adding a clamping after each step of residual coefficient reconstruction to both the reference software and text.

- ALF line buffer issue along slice/brick/tile boundary and 360 video virtual boundary

Figure 1. Current text specification of the ALF virtual boundary

The ALF virtual boundary was adopted at the last meeting to remove the ALF line buffer, it is located at 4 luma lines above the bottom CTU boundary (except for those CTUs at the last CTU row of a picture). In the current specification text, the ALF virtual boundary is not present for a CTU if its bottom CTU boundary is also a slice/brick/tile boundary (when the loop-filter across those boundaries is disabled) or a 360 video virtual boundary (see Fig. 1 for example).

It is asserted that the current text specification defeats the purpose of the ALF virtual boundary adoption and has the following consequences:

- For decoders, not knowing the slice/brick/tile boundaries upfront (i.e. low-delay applications) means that the ALF line buffer needs to be restored. To process the ALF filtering of the bottom lines of the current CTU (e.g. luma lines 5, 6 and 7 above the bottom CTU boundary), the decoders need to know whether the current bottom CTU boundary coincides with other types of boundaries. This information, however, is unknown until the next slice/brick/tile is decoded.

- Even if the slice/brick/tile boundaries are known upfront (e.g. the CABAC decoding is fully decoupled at frame level), the decoders need to run the loop-filters (de-blocking, SAO and ALF) at a speed of 68 lines per 64x64 VDPU all the time (an overprovision of roughly 6%) in order to avoid using the ALF line buffer.

It is therefore recommended to specify the ALF virtual boundary at the fixed location of 4 lines above the bottom CTU boundary, regardless of whether the bottom CTU boundary coincides with a slice/brick/tile boundary or a 360 video virtual boundary (see Figure 2 for example). It is also desirable to unify the padding methods along an ALF virtual boundary, a 360 video virtual boundary and a slice/brick/tile boundary.

Figure 2. Proposed fix for the ALF virtual boundary

## Memory bandwidth study for VTM5.0

Broadcom conducted a memory bandwidth study by running both the VTM5.0 and VTM4.0 for CTC with a commercial motion compensation cache model integrated. The summary results of the random access configuration are provided in the Table 1 (for informational purposes).

The changes which may cause the difference in memory bandwidth consumption between the VTM5.0 and VTM4.0 include: removing the 4x4 uni-pred and 8x4/4x8 bi-pred PUs, using the 6-tap filters for affine MC and restricting the sub-block motion vector spread of the affine mode. Both the average (see ABW_diff column) and the peak memory bandwidth consumption (see MBW_diff column) are slightly decreased.

|  | Random Access Main 10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |  |  |  |
|  | Y | U | V | TCM_diff | ABW_diff | MBW_diff | EncT | DecT |
| Class A1 | -3.29% | -1.44% | -1.58% | -2.10% | -2.21% | -1.28% | 118% | 108% |
| Class A2 | -2.56% | -3.40% | -1.19% | -0.92% | -0.99% | -0.95% | 123% | 117% |
| Class B | -2.09% | -1.63% | -3.70% | -0.56% | -0.50% | -4.10% | 124% | 112% |
| Class C | -1.82% | 0.26% | -3.17% | -0.14% | -0.34% | -2.32% | 135% | 112% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.35% | -1.44% | -2.63% | -0.93% | -1.01% | -2.16% | 125% | 112% |
| Class D | -0.52% | 0.42% | -2.76% | 0.28% | 0.15% | -3.30% | 148% | 107% |
| Class F (optional) | -6.09% | -4.39% | -6.63% | 1.53% | 1.81% | 1.24% | 141% | 111% |

Table 1. Memory bandwidth comparison (VTM5.0 vs.VTM4.0, RA)

Where

- TCM_diff : Total cache misses (over all the frames coded), percentage difference relative to VTM4.0.

- ABW_diff: Average memory bandwidth (over all the frames coded), percentage difference relative to VTM4.0.

- MBW_diff : Worst case (Max) memory bandwidth (among all the frames coded), percentage difference relative to VTM4.0.

MBW_diff is the most important measure, which shows the worst case memory bandwidth consumption difference of VTM5.0 relative to VTM4.0.

## Bin to bit ratio study for VTM5.0

It has been reported that the bin to bit ratio continues to increase from VTM4.0 to VTM5.0. The summary results are provided in Table 2 for informational purposes (data from JVET-O0068).

|  | Bin2bit ratio (peak, weighted) |  |  |  |  |  |  |  | Bin2bit ratio (peak, unweighted) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 |  | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) |  | HM16.19 | VTM4.0 | VTM5.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.95 | 1.10 |  | 1.13 | 15.72% | 19.71% | 3.99% |  | 1.23 | 1.38 | 1.40 | 12.38% | 14.21% | 1.82% |
| RA | 0.93 | 1.09 |  | 1.12 | 17.39% | 20.88% | 3.49% |  | 1.21 | 1.38 | 1.40 | 13.75% | 15.25% | 1.50% |
| LB | 0.96 | 1.09 |  | 1.13 | 13.34% | 16.99% | 3.64% |  | 1.24 | 1.37 | 1.39 | 11.25% | 12.88% | 1.63% |
| LP | 0.96 | 1.09 |  | 1.13 | 13.24% | 17.30% | 4.06% |  | 1.23 | 1.37 | 1.39 | 11.08% | 13.02% | 1.94% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AI | 0.79 |  | 0.94 | 0.95 | 18.11% | 20.24% |  | 2.12% | 1.19 | 1.31 | 1.33 | 10.45% | 11.40% | 0.95% |
| RA | 0.74 |  | 0.87 | 0.88 | 17.17% | 19.00% |  | 1.83% | 1.18 | 1.28 | 1.28 | 7.63% | 8.39% | 0.76% |
| LB | 0.78 |  | 0.92 | 0.93 | 17.67% | 19.67% |  | 2.00% | 1.18 | 1.30 | 1.31 | 9.54% | 10.28% | 0.74% |
| LP | 0.78 |  | 0.92 | 0.93 | 17.46% | 19.46% |  | 2.00% | 1.18 | 1.30 | 1.31 | 9.50% | 10.24% | 0.74% |
| Note*: Percentage difference between VTM5.0 and VTM4.0 is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2. Bin to bit ratio increase from VTM4.0 to VTM5.0 (Overall)

It was also reported that the weighted bin to bit ratio in class F sequences is already higher than the average numbers presented in Table 2.

In Table 2, a bypass bin is counted as 0.25 context coded bins in the weighted measurements; and a bypass bin and a context coded bin carry an equal weight (1:1) in the unweighted measurements.

# Related contributions

The following contributions are identified for the AHG.

## Contributions related to the luma-chroma latency reduction for the combination of CCLM and dual-tree

- JVET-O0129, “Non-CE2: CCLM with Chroma Separate Tree”, 	C.-W. Kuo, J. Li, C. Lim (Panasonic)

- JVET-O0196, “CE2-related: CCLM for dual tree with 32x32 latency”,	Y. Zhao, H. Yang (Huawei)

- JVET-O0274, “CE2-related: Luma-chroma dependency reduction for chroma separate tree by constraining CCLM usage”,	C.-M. Tsai, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0524, “Non-CE2: alternative solutions for reducing the luma-chroma latency”, T. Poirier, E. François, F. Le Léannec (InterDigital)

- JVET-O0611, “CE2-3 related: On CCLM restriction for 32x32 luma latency”,	K. Kawamura, K. Unno, S. Naito (KDDI)

## Contributions for simplification of IBC vector validity check

- JVET-O0127, “CE8-related: IBC modifications”, J. Li, C.S. Lim (Panasonic)

- JVET-O0258, “Non-CE8: Adaptive single/dual tree with IBC simplification”, W. Zhu, J. Xu, L. Zhang (Bytedance)

- JVET-O0248, “CE8-related: Dedicated IBC reference buffer without bitstream restrictions”, H. Gao, A.Karabutov, S. Esenlik, S. Ikonin, B. Wang, A. M. Kotra, E. Alshina (Huawei)

- JVET-O0482, “CE8-related: Modulo operations for IBC block vectors”, X. Xu, X. Li, S. Liu (Tencent)

## Contributions related to the residual coding of transform skip mode

- JVET-O0295, “CE7-related: Modified subblock coding passes for coding TS residual”, S.-T. Hsiang, S.-M. Lei (MediaTek)

- JVET-O0558, “CE7-related: Reduction of Gtx passes in transform skip residual coding”, M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban (Qualcomm)

- JVET-O0619, “Non-CE7: Simplification of transform skip residual coding”, M. G. Sarwer, Y. Ye (Alibaba)

- JVET-O0623, “CE7-related: Interleaved coefficient coding for transform-skip mode”, M. Karczewicz, H. Wang, M. Coban, Y.-H. Chao (Qualcomm)

## Contributions related to the MV context storage of triangle merge mode

- JVET-O0128, “Non-CE4: Simplification of triangle mode MV context storage”,  	J. Li, C.S. Lim (Panasonic)

- JVET-O0217, “Non-CE4: Simplification of motion vector storing process for triangle merge mode”, Y. Ahn, D. Sim (Digital Insights)

- JVET-O0279, “CE4-related: Simplification of motion vector storing process for triangle prediction mode”,	Y.-L. Hsiao, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0314, “Non-CE4: Modification of MV storage in triangle merge mode”, 	P.-H. Lin (Foxconn)

- JVET-O0329, “Non-CE4: Simplification of motion vector storage process for triangle merge mode”, H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)

- JVET-O0378, “Non-CE4: Simplification of MV context storage for triangle mode”,	F. Chen, L. Wang(Hikvision)

- JVET-O0411, “Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage”,	K. ReuzÃ©, C.-C Chen, W.-J. Chien, H. Huang, M. Karczewicz (Qualcomm)

- JVET-O0418, “CE4-related: Simplification of motion vector storage operation for triangle merge mode”, X.W. Meng (PKU), X. Zheng (DJI), S.S Wang, S.W. Ma (PKU)

- JVET-O0601, “CE4-related: On motion vector population under triangle prediction mode	“, X. Wang, Y.-W. Chen, X. Xiu, T.-C. Ma (Kwai Inc.)

- JVET-O0629, “Non-CE4: Simplification of motion field for triangle partition”,	R.-L. Liao, Y. Ye, J. Chen (Alibaba)

- JVET-O0660, “Simplification of MV storage for blending block in triangle mode”, 	K. Panusopone, L. Wang (Nokia)

## Contributions for the LFNST latency reduction

- JVET-O0213, “Non-CE6: Block size restriction of LFNST”, M. Koo, J. Nam, J. Lim, M. Salehifar, S. Kim (LGE)

- JVET-O0266, “Non-CE6: Simplifications on LFNST”, K. Zhang, L. Zhang, Z. Deng, H. Liu, N. Zhang (Bytedance)

- JVET-O0293, “CE6-related: Latency reduction for LFNST signaling”, M.-S. Chiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0352, “Non-CE6: TU/TB-level LFNST index coding”, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)

- JVET-O0476, “Non-CE6: Syntax change for LFNST”, D. Park, Y.-U. Yoon, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)

- JVET-O0569, “Non-CE6: LFNST Signaling at the TU Level”, A. Nalci, H. E. Egilmez, A. Said, M. Coban, V. Seregin, W.-J. Chien, M. Karczewicz (Qualcomm)

## Contributions for ALF padding unification and line buffer reduction

- JVET-O0625, “Non-CE5: Unified padding method for samples at variant boundaries in ALF”, H. Liu, L. Zhang, K. Zhang, Z. Deng, N. Zhang, J. Xu (Bytedance)

## Other AHG16-related contributions

- JVET-O0046, “AHG16: Changing cu_qp_delta signalling to facilitate VDPU-level processing”, B. Heng, M. Zhou, T. Hellman (Broadcom)

- JVET-O0068, “AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0”, 	M. Zhou (Broadcom)

- JVET-O0428,  “AHG16/AHG17: LMCS related clean-ups”,	T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby), J. Zhao, S.H. Kim (LGE)

- JVET-O0525, “AHG16: Removal of PCM mode	“, T. Hellman (Broadcom)

- JVET-O0526,  “AHG16: Setting the minimum CTU size to 32x32”,	T. Hellman, M. Zhou (Broadcom)

# Recommendations

The AHG recommends reviewing the input contributions.