| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0022-v1 |
| --- | --- |

| Title: | CE2: Summary report on luma-chroma dependency reduction |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Jie Chen Edouard François Peng Yin | Email: | jiechen.cj@alibaba-inc.com Edouard.Francois@interdigital.com pyin@dolby.com |
| Source: | CE2 coordinators |  |  |  |

_____________________________

- Abstract

This document provides a summary report of Core Experiment 2 (CE2) on luma-chroma dependency reduction. This CE comprises 3 categories, as follows:

- CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case (8 tests)

- CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case (7 tests)

- CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case (2 tests)

All the techniques are implemented on top of VTM5.0 and tested against VTM5.0. The simulation results and crosscheck reports are summarized in this contribution

Test conditions, source codes and test results

CTC conditions as agreed at the 14th JVET meeting (JVET-N1010) are used for the evaluation of objective performance. The anchor is the VTM5.0. ‘All Intra’, ‘Low Delay’ and ‘Random Access’ configurations are used for evaluating techniques in CE2-2 and CE2-3, and ‘Low Delay’ and ‘Random Access’ are used for evaluating techniques CE2-1. BD-rates, encoding and decoding time comparisons are reported by all proponents. In addition, results using the HDR sequences in JVET-N1011 may be optionally provided.

Source codes corresponding with the test descriptions contained in this document were uploaded to the CE2 GitLab project https://vcgit.hhi.fraunhofer.de/jvet-n-ce2/VVCSoftware_VTM

The test results and the crosscheck results were uploaded in https://vcgit.hhi.fraunhofer.de/jvet-n-ce2/ce2_results

Timeline

| T1 | April 17. 2019 | CE test description finalized (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | 2 weeks after 5.0 release (May 20. 2019 ) | Initial software of CE tests released; initial study by cross-checkers begins (2 weeks after 5.0 release) |
| T3 | June 5. 2019 | Software of CE tests is finalized; final cross-check begins (4 weeks before next meeting) |
| T4 | June 18. 2019 | Document upload: CE contribution document including spec txt. Full test results, cross-check reports (1 week before regular doc deadline) |

CE2-1: Chroma residual scaling luma-chroma dependency reduction – Single-tree case

Defined tests

In CE2-1 tests, chroma residual scaling (CRS) is not enabled for dual-tree case (separate luma-chroma partitioning tree), which means CRS is disabled in Intra tile groups similar as VTM5.0 anchor.

| CE2-1 – Chroma residual scaling luma-chroma dependency reduction – Single-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Source | Cross-checker |
| CE2-1.1 | Derive the scaling factor from the average value of the top-left 16×16 luma prediction samples | JVET-O0049 (MediaTek) | J. Chen (alibaba) |
| CE2-1.2 | Derivation of the scaling factor from the first (upper-left) sample value of the corresponding luma prediction | JVET-O0098 (Dolby) | C. Chevance (Interdigital) |
| CE2-1.3.a | For intra coded blocks, derivation of the scaling factor from one of the top-left, top or left neighboring reconstructed luma samples, based on their availability | JVET-O0087 (Interdigital) | T. Lu (Dolby) |
| CE2-1.3.b (withdrawn) | For intra coded blocks, derivation of the scaling factor from the average value of 2 available neighboring reconstructed luma samples | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-1.3.c | For intra coded blocks, derivation of the scaling factor from the average value of 4 available neighboring reconstructed luma samples | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-1.3.d | For intra coded blocks, derivation of the scaling factor from the average value of 8 available neighboring reconstructed luma samples | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-1.3.e | For intra coded blocks, derivation of the scaling factor from the average value of all available neighboring reconstructed luma samples | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-1.4.a | For intra coded blocks, derivation of the scaling factor from the top-left reference luma sample used for intra prediction | JVET-O0102 (Alibaba) | T. Lu (Dolby) |
| CE2-1.4.b | For intra coded blocks, derivation of the scaling factor from the averaged value of 2 reference luma samples | JVET-O0102 (Alibaba) | Z.-Y. Lin (MediaTek) |
| CE2-1.4.c | For intra coded blocks, derivation of the scaling factor from the averaged value of 4 reference luma samples | JVET-O0102 (Alibaba) | E. Francois (Technicolor) |
| CE2-1.4.d | For intra coded blocks, derivation of the scaling factor from the averaged value of 8 reference luma samples | JVET-O0102 (Alibaba) | Z.-Y. Lin (MediaTek) |
| CE2-1.4.e | For intra coded blocks, derivation of the scaling factor from the averaged value of reference luma samples used for intra prediction | JVET-O0102 (Alibaba) | E. Francois (Technicolor) |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-1.1 | 0.00% | -0.04% | -0.04% | 100% | 99% | 0.00% | 0.04% | -0.06% | 100% | 100% |
| 2-1.2 | 0.00% | -0.09% | -0.10% | 99% | 99% | 0.02% | 0.05% | -0.03% | 100% | 98% |
| 2-1.3.a | 0.01% | -0.09% | -0.03% | 101% | 100% |  |  |  |  |  |
| 2-1.3.c | 0.00% | 0.00% | -0.07% | 99% | 101% |  |  |  |  |  |
| 2-1.3.d | 0.00% | -0.01% | -0.02% | 100% | 99% |  |  |  |  |  |
| 2-1.3.e | -0.01% | -0.02% | -0.06% | 99% | 101% | 0.00% | 0.11% | -0.13% | 101% | 102% |
| 2-1.4.a | -0.01% | -0.08% | -0.01% | 100% | 100% | -0.01% | 0.28% | -0.22% | 100% | 99% |
| 2-1.4.b | 0.00% | -0.04% | -0.07% | 100% | 100% | -0.01% | 0.04% | -0.06% | 100% | 99% |
| 2-1.4.c | 0.00% | -0.02% | -0.03% | 100% | 100% | -0.01% | 0.14% | 0.03% | 100% | 99% |
| 2-1.4.d | 0.00% | -0.03% | -0.02% | 100% | 100% | -0.01% | 0.10% | -0.10% | 100% | 99% |
| 2-1.4.e | 0.01% | -0.08% | -0.06% | 100% | 100% | 0.02% | 0.23% | 0.06% | 100% | 99% |

Crosscheck reports

| Test# | crosschecker | source codes match description ? (Y/N) | full checking or spot-checking ? | Coding performance Match ? (Y/N) | EncT/DecT match ? (Y/N) | other comments |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1.1 | J. Chen (alibaba) | Y | F | Y | Y |  |
| 2-1.2 | C. Chevance (InterDigital) | Y | F | Y | Y |  |
| 2-1.3.a | T. Lu (Dolby) | Y | F | Y | Y |  |
| 2-1.3.c | J. Chen (Alibaba) | Y | F |  |  |  |
| 2-1.3.d | J. Chen (Alibaba) | Y | F |  |  |  |
| 2-1.3.e | J. Chen (Alibaba) | Y | F | Y | Y |  |
| 2-1.4.a | T. Lu (Dolby) | Y | F | Y | Y |  |
| 2-1.4.b | Z.-Y. Lin (MediaTek) | Y | F | Y | Y |  |
| 2-1.4.c | E. Francois (InterDigital) | Y | F | Y | Y |  |
| 2-1.4.d | Z.-Y. Lin (MediaTek) | Y | F | Y | Y |  |
| 2-1.4.e | E. Francois (InterDigital) | Y | F | Y | Y |  |

Detailed test descriptions

CE2-1.1

In the CE2-1.1, the chroma scaling factor is derived the average value of the top-left 16×16 luma prediction samples.

CE2-1.2

In the CE2-1.2, the chroma scaling factor is derived from the first (upper-left) sample value of the corresponding luma prediction.

CE2-1.3

In the CE2-1.3, for conventional intra coded block, luma reconstructed samples neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-1.3.a, one sample among the top-left, or top or left neighboring luma reconstructed sample is used, based on the samples availability. When none is available, scaling factor is set to neutral value (equivalent tono chroma residual scaling).

- CE2-1.3.b, the averaged value of 2 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-1.3.c, the averaged value of 4 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-1.3.d, the averaged value of 8 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-1.3.e, the averaged value of all available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

CE2-1.4

In the CE2-1.4, for conventional intra coded block, luma reference samples used for intra prediction of the collocated luma block are used to derive the chroma scaling factor.

Five options are tested:

- CE2-1.4.a, the top-left luma reference sample is used.

- CE2-1.4.b, the averaged value of 2 luma reference samples is used.

- CE2-1.4.c, the averaged value of 4 luma reference samples is used.

- CE2-1.4.d, the averaged value of 8 luma reference samples is used.

- CE2-1.4.e, the averaged value of luma reference samples is used.

CE2-2: Chroma residual scaling luma-chroma dependency reduction – Dual-tree case

Defined tests

In CE2-2 tests, chroma residual scaling (CRS) is enabled for dual-tree case (separate luma-chroma partitioning tree) and the following methods are applied to all tile groups.

| CE2-2 – Chroma residual scaling luma-chroma dependency reduction – Dual-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Tester | Cross-checker |
| CE2-2.1.a | CRS on in dual-tree For all intra coded blocks, scaling factor derived from one of the top-left, top or left reconstructed luma samples neighboring the collocated luma block, based on their availability | JVET-O0087 (Interdigital) | T. Lu (Dolby) |
| CE2-2.1.b | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 2 available reconstructed luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) |  |
| CE2-2.1.c | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 4 available reconstructed luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) |  |
| CE2-2.1.d | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 8 available reconstructed luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) |  |
| CE2-2.1.e | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of all available reconstructed luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) |  |
| CE2-2.2.a | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the top-left reference luma sample neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-2.2.b | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 2 reference luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-2.2.c | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 4 reference luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-2.2.d | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of 8 reference luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-2.2.e | CRS on in dual-tree For all intra coded blocks, scaling factor derived from the average of all reference luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |
| CE2-2.3 | CRS on in dual-tree For intra coded blocks in dual-tree, scaling factor derived from the average of available reconstructed luma samples neighboring the collocated luma block For intra coded blocks in single-tree, scaling factor derived from the average of reference luma samples neighboring the collocated luma block | JVET-O0087 (Interdigital) | J. Chen (Alibaba) |

Simulation results

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-2.1.a | 0.03% | -0.46% | -0.62% | 102.00% | 101.00% | 0.03% | -0.71% | -0.59% | 101% | 100% |  |  |  |  |  |
| 2-2.1.b | 0.03% | -0.36% | -0.52% | 101.00% | 101.00% | 0.02% | -0.60% | -0.61% | 100% | 102% |  |  |  |  |  |
| 2-2.1.c | 0.02% | -0.37% | -0.51% | 101.00% | 102.00% | 0.03% | -0.65% | -0.66% | 100% | 102% |  |  |  |  |  |
| 2-2.1.d | 0.02% | -0.37% | -0.53% | 101.00% | 100.00% | 0.02% | -0.64% | -0.59% | 101% | 100% |  |  |  |  |  |
| 2-2.1.e | 0.03% | -0.37% | -0.52% | 100.00% | 101.00% | 0.01% | -0.64% | -0.66% | 101% | 101% | -0.01% | -0.39% | -0.50% | 101% | 99% |
| 2-2.2.a | 0.04% | -0.44% | -0.60% | 102.00% | 99.00% | 0.02% | -0.66% | -0.61% | 101% | 101% |  |  |  |  |  |
| 2-2.2.b | 0.03% | -0.36% | -0.52% | 101.00% | 101.00% | 0.02% | -0.60% | -0.61% | 100% | 102% |  |  |  |  |  |
| 2-2.2.c | 0.02% | -0.37% | -0.51% | 101.00% | 102.00% | 0.03% | -0.65% | -0.66% | 100% | 102% |  |  |  |  |  |
| 2-2.2.d | 0.03% | -0.36% | -0.51% | 101.00% | 101.00% | 0.02% | -0.67% | -0.67% | 101% | 101% |  |  |  |  |  |
| 2-2.2.e | 0.03% | -0.34% | -0.52% | 102.00% | 101.00% | 0.03% | -0.58% | -0.65% | 101% | 100% | 0.00% | -0.48% | -0.50% | 99% | 100% |
| 2-2.3 | 0.03% | -0.37% | -0.52% | 102.00% | 101.00% | 0.03% | -0.61% | -0.68% | 103% | 101% | 0.02% | -0.45% | -0.41% | 101% | 98% |

Crosscheck report

| Test# | crosschecker | source codes match description ? (Y/N) | full checking or spot-checking ? | Coding performance Match ? (Y/N) | EncT/DecT match ? (Y/N) | other comments |
| --- | --- | --- | --- | --- | --- | --- |
| 2-2.1.a | T. Lu (Dolby) | Y | F | Y | Y |  |
| 2-2.1.b | T. Lu (Dolby) |  |  |  |  |  |
| 2-2.1.c | T. Lu (Dolby) |  |  |  |  |  |
| 2-2.1.d | J. Chen (alibaba) | Y | S |  |  |  |
| 2-2.1.e | J. Chen (alibaba) | Y | S |  |  |  |
| 2-2.2.a | J. Chen (alibaba) | Y | F |  |  |  |
| 2-2.2.b | J. Chen (alibaba) | Y | F |  |  |  |
| 2-2.2.c | J. Chen (alibaba) | Y | F |  |  |  |
| 2-2.2.d | J. Chen (alibaba) | Y | F |  |  |  |
| 2-2.2.e | J. Chen (alibaba) | Y | F | Y | Y |  |
| 2-2.3 | J. Chen (alibaba) | Y | F | Y | Y |  |

Detailed test descriptions

CE2-2.1

In the CE2-2.1, for intra coded blocks in single-tree and dual-tree cases, luma reconstructed samples neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-2.1.a, one sample among the top-left, or top or left neighboring luma reconstructed sample is used, based on the samples availability. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-2.1.b, the averaged value of 2 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-2.1.c, the averaged value of 4 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-2.1.d, the averaged value of 8 available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- CE2-2.1.e, the averaged value of all available neighboring luma reconstructed samples is used. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

CE2-2-2

In the CE2-2.2, for intra coded blocks in single-tree and dual-tree cases, luma reference samples used for intra prediction, neighboring the luma block collocated with the top-left sample of the chroma block are used to derive the chroma scaling factor.

Three options are tested:

- CE2-2.2.a, the top-left neighboring luma reference sample is used.

- CE2-2.2.b, the averaged value of neighboring luma reference samples is used.

- CE2-2.2.c, the averaged value of up to 2 neighboring luma reference samples is used.

- CE2-2.2.d, the averaged value of up to 4 neighboring luma reference samples is used.

- CE2-2.2.e, the averaged value of up to 8 neighboring luma reference samples is used.

CE2-2.3

CE2-2.3 tests the following combination:

- For intra coded blocks in dual-tree, the chroma residual scaling factor is derived from the average of all available reconstructed luma samples neighboring the collocated luma block. When none is available, scaling factor is set to neutral value (equivalent to no chroma residual scaling).

- For intra coded blocks in single-tree, the chroma residual scaling factor is derived from the average of reference luma samples neighboring the collocated luma block.

CE2-3: CCLM luma-chroma dependency reduction in Dual-tree case

Defined tests

| CE2-3 - CCLM luma-chroma dependency reduction in Dual-tree case |  |  |  |
| --- | --- | --- | --- |
| Test # | Description | Tester | Cross-checker |
| CE2-3.1 | CCLM enabled if chroma block and its collocated luma blocks are inside the same 16x16/32x32 non-overlapping area | E. Francois (Technicolor) | A. Segall (Sharp) |
| CE2-3.2 | CCLM parameters derived from reconstructed luma/chroma samples neighboring the luma block collocated with the chroma block top-left sample | E. Francois (Technicolor) | A. Segall (Sharp) |

Simulation results

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2-3.1 | 0.28% | 2.37% | 2.53% |  |  | 0.07% | 1.08% | 0.96% |  |  |  |  |  |  |  |
| 2-3.2 | 0.33% | 2.17% | 2.28% |  |  | 0.10% | 1.16% | 0.98% |  |  |  |  |  |  |  |

Crosscheck report

| Test# | crosschecker | source codes match description ? (Y/N) | full checking or spot-checking ? | Coding performance Match ? (Y/N) | EncT/DecT match ? (Y/N) | other comments |
| --- | --- | --- | --- | --- | --- | --- |
| 2-3.1 | A. Segall (Sharp) |  | F |  |  |  |
| 2-3.2 | A. Segall (Sharp) |  | F |  |  |  |

Detailed test descriptions

CE2-3.1

In CE2-3.1, CCLM is enabled only if the chroma block is inside a 16x16 non-overlapping area, and if all its co-located luma samples belong to luma blocks included in the corresponding 32x32 area.

CE2-3.2

In CE2-3.2, CCLM parameters are derived from reconstructed luma/chroma samples neighboring the luma block collocated with the top-left sample of the chroma block.

CE related contributions

| JVET-VC number | Title |
| --- | --- |
| JVET-O0129 | Non-CE2: CCLM with Chroma Separate Tree |
| JVET-O0130 | Non-CE2: CRS with Chroma Separate Tree |
| JVET-O0196 | CE2-related: CCLM for dual tree with 32x32 latency |
| JVET-O0232 | CE2-related: No-latency LMCS Chroma Residual Scaling for Single tree and Dual tree |
| JVET-O0233 | CE2-related: Linear LMCS |
| JVET-O0262 | Non-CE2: Removal of division operations in LMCS |
| JVET-O0271 | CE2-related: Chroma scaling for CIIP |
| JVET-O0272 | CE2-related: Simplified inverse luma mapping |
| JVET-O0273 | CE2-related: Luma-chroma latency reduction for chroma separate tree |
| JVET-O0274 | CE2-related: Luma-chroma dependency reduction for chroma separate tree by constraining CCLM usage |
| JVET-O0427 | Non-CE2: Remove differential coding for ALF coefficients |
| JVET-O0429 | Non-CE2: alternative solutions for chroma residual scaling factors derivation for dualTree |
| JVET-O0448 | Non-CE2: Inverse LUT carriage for LMCS |
| JVET-O0524 | Non-CE2: alternative solutions for reducing the luma-chroma latency |
| JVET-O0550 | CE2-related: Further simplification of chroma residual scaling factors derivation |
| JVET-O0603 | CE2-related: Prediction dependent luma residual scaling for adaptive in-loop re-shaper |
| JVET-O0608 | CE2-related: On latency reduction for chroma residual scaling |
| JVET-O0611 | CE2-3 related: On CCLM restriction for 32x32 luma latency |
| JVET-O0618 | CE2-related: Removal of luma-chroma dependency in LMCS |
| JVET-O0627 | CE2-related: luma-chroma dependency reduction on chroma scaling |

Recommendations

Discuss CE2 tests and review all CE related contributions.

References

- J. Chen, E. François, P. Yin, “Description of Core Experiment 2 (CE2): luma-chroma dependency reduction,” 14th JVET Meeting, Geneva, CH, Mar. 2019, JVET-N1022