| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0023-v3 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com l.li@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools, including mode coding, for potential inclusion into the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3-1: ISP (9 tests)

- CE3-2: Small chroma block size restrictions for shared tree (3 tests)

- CE3-3: CCLM model improvements (6 tests)

This document summarizes the objective results (BD-rates, runtimes), cross-check reports, and related input contributions.

# Test conditions and guidelines

CTC conditions as agreed at the 14th JVET meeting (Geneva, CH) for SDR are used for the evaluation of objective performance (JVET-N1010). The anchor is the ‘VVC Test Model’ or VTM. Results are reported for both the ‘All Intra’ and ‘Random Access’ test configurations for all sub-CEs, and additionally the ‘Low Delay B’ configuration for CE3-2. The VTM version 5.0 software is used with ‘vtm’ configuration files to produce the VTM anchor. Tools are implemented into the VTM version 5.0 software. Proposals are primarily compared with respect to objective compression efficiency while complexity aspects are also assessed.

Source codes corresponding with the test descriptions contained in this document are uploaded to the following CE3 GitLab project:

https://vcgit.hhi.fraunhofer.de/JVET-N-CE3/VVCSoftware_VTM.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any inconsistencies between the test description, the provided final software, and the VVC spec text changes?

- Do the cross-checked results match the proponent results for the test conditions? Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column. The spreadsheets with cross-checked results are attached to this summary report.

The deadline for providing the cross-check reports to the CE coordinators was 22 June 2019. The following cross-check reports were received late or were missing spreadsheets with cross-checked results:

| Test # | Date received |
| --- | --- |
| CE3-1.3 | 28 June 2019 |

# Overview of CE3 test description document changes

The following changes were made to the CE3 test description document [1] after the T1 deadline (17 April 2019) had passed. Besides changes to contact persons and assignment of cross-checkers, the following changes:

- Test CE3-1.5 and its 3 subtests were withdrawn by the proponent.

# CE3-1 on ‘ISP’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-1.1.1 | CU-based prediction is applied for CUs whose sub-partition size is 1xN or 2xN. | JVET-O0080 (KDDI) |
| 3-1.1.2 | CU-based prediction is applied for CUs whose sub-partition size is 1xN. |  |
| 3-1.2.1 | Disable partitions in ISP that can result into either 1xN or 2xN sub-blocks. An 8xN block can be split into 2 4xN blocks | JVET-O0048 (Huawei) |
| 3-1.2.2 | Disable partitions in ISP that can result into 1xN sub-blocks |  |
| 3-1.3 | Apply CU-based low latency ISP to 4×8, 8×4 CUs, and the sub-partition with 1xN or 2xN partition size is disabled. | JVET-O0099 (InterDigital) |
| 3-1.4.1 | Remove ISP CU if it causes 1xN sub-partition | JVET-O0109 (Kwai) |
| 3-1.4.2 | Remove ISP CU if it causes 1xN or 2xN sub-partition |  |
| 3-1.6 | 4xN prediction regions for 4xN and 8xN (N > 4) ISP-coded blocks with vertical subpartitions | JVET-O0106 (Qualcomm, HHI) |
| 3-1.7 | Test 3-1.6 with additional restriction of only allowing vertical prediction mode for 4xN ISP-coded blocks with vertical subpartitions | JVET-O0107 (HHI) |

The following tables provide an overview of the aspects of each test. The data is provided for luma CBs that can potentially be split into narrow 1xN and 2xN vertical sub-partitions (if a 4-way ISP split would be performed), i.e., luma CBs with width equal to 4 and 8. The first table specifies the sizes of dependent sub-partitions that require reconstruction of the neighbouring sub-partition to the left for prediction of the current sub-partition. The second table specifies the transform block sizes that are applied to the dependent sub-partitions. The VTM anchor data is provided for reference. Yellow highlights indicate tested changes with respect to the VTM anchor.

- Sub-partition sizes with prediction dependency (V: vertical splitting; H: horizontal splitting)

| Test# vs. CB size | 4x4 | 4x8 | 4xN (N>=16) | 8x4 | 8xN (N>=8) |
| --- | --- | --- | --- | --- | --- |
| VTM | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.1 | No ISP | V: 4x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 8xN H: 8x(N/4) |
| 3-1.1.2 | No ISP | V: 2x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.2.1 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.2.2 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.3 | No ISP | V: No ISP H: 4x8 | V: No ISP H: 4x(N/4) | V: 8x4 H: 8x4 | V: 4xN H: 8x(N/4) |
| 3-1.4.1 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.4.2 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: No ISP H: 8x(N/4) |
| 3-1.6 | No ISP | V: 4x8 H: 4x4 | V: 4xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.7 | No ISP | V: 4x8 (*) H: 4x4 | V: 4xN (*) H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |

(*) VER pred mode only

- Transform block sizes:

| Test# vs. CB size | 4x4 | 4x8 | 4xN (N>=16) | 8x4 | 8xN (N>=8) |
| --- | --- | --- | --- | --- | --- |
| VTM | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.1 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.1.2 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.2.1 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.2.2 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.3 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 4xN H: 8x(N/4) |
| 3-1.4.1 | No ISP | V: 2x8 H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.4.2 | No ISP | V: No ISP H: 4x4 | V: No ISP H: 4x(N/4) | V: 4x4 H: 8x2 | V: No ISP H: 8x(N/4) |
| 3-1.6 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |
| 3-1.7 | No ISP | V: 2x8 H: 4x4 | V: 1xN H: 4x(N/4) | V: 4x4 H: 8x2 | V: 2xN H: 8x(N/4) |

## CE3-1: Test results

|  | All Intra Main10 - Over VTM-5.0 |  |  |  |  | Random Access Main10 - Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-1.1.1 | 0.03% | 0.02% | 0.02% | 99% | 100% | 0.01% | -0.07% | -0.03% | 100% | 100% |
| 3-1.1.2 | 0.01% | -0.01% | 0.02% | 99% | 100% | 0.02% | 0.01% | -0.10% | 100% | 100% |
| 3-1.2.1 | 0.07% | 0.02% | 0.07% | 98% | 99% | 0.03% | -0.13% | -0.09% | 99% | 98% |
| 3-1.2.2 | 0.01% | -0.02% | 0.03% | 99% | 99% | 0.01% | -0.07% | -0.04% | 99% | 98% |
| 3-1.3 | 0.08% | 0.03% | 0.07% | 100% | 99% | 0.04% | 0.00% | -0.04% | 101% | 100% |
| 3-1.4.1 | 0.01% | -0.02% | 0.03% | 97% | 99% | 0.01% | -0.07% | -0.04% | 99% | 100% |
| 3-1.4.2 | 0.08% | 0.00% | 0.04% | 98% | 101% | 0.02% | -0.10% | -0.05% | 100% | 100% |
| 3-1.6 | 0.03% | -0.01% | 0.01% | 100% | 98% | 0.03% | -0.06% | -0.11% | 99% | 99% |
| 3-1.7 | 0.01% | -0.01% | 0.03% | 99% | 97% | 0.02% | -0.02% | -0.04% | 99% | 100% |

## CE3-1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross-checker comments |
| --- | --- | --- | --- |
| 3-1.1.1 | Eiichi Sasaki (Sharp) | y | Both source code and provided spec text are checked and there is no inconsistency. The results matched with the proponent’s and there are no additional results. |
| 3-1.1.2 |  | y |  |
| 3-1.2.1 | Adarsh K. Ramasubramonian (Qualcomm) | y | Source code matches the test description, spec text. The results matched with those of the proponents. There are no additional results. |
| 3-1.2.2 |  | y | Source code matches the test description, spec text. The results matched with those of the proponents. There are no additional results. |
| 3-1.3 | Kai Zhang (Bytedance) | y | SW matches the description. Spec text was not studied. |
| 3-1.4.1 | Xin Zhao, Liang Zhao (Tencent) | y | Source code matches with the test description and the spec text. The crosscheck results match with the proponents’ results. |
| 3-1.4.2 |  | y |  |
| 3-1.6 | Hua Yang (Interdigital) | y | Source code and spec changes match the test description. The results matched with those of the proponents. There are no additional results. Details in JVET-O0174. |
| 3-1.7 | S. De-Luxán-Hernández (HHI) | y | There are no inconsistencies between the test description, the software and the VVC Spec text changes. Results are identical. |

## CE3-1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-O0311 | 3-1.3 | CE3-related: Intra sub-partition improvement for low decoding latency and hardware implementation efficiency |
| JVET-O0371 | 3-1 | CE3-related: On 1xN and 2xN sub-partition in ISP |

# CE3-2 on ‘Small chroma block size restrictions for shared tree’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-2.1.1 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from the coding mode of the first CU inside SCIPU | JVET-O0050 (MediaTek, Huawei) |
| 3-2.1.2 | Do not split chroma blocks of smallest chroma intra prediction units (SCIPU) - SCIPU determined from a flag in coding_tree() |  |
| 3-2.2 | All coded blocks within a parallel-processable region (PPR) of size larger than or equal to 16 chroma samples to be reconstructed in parallel in the single-tree case by removing the dependency between blocks within the PPR. | JVET-O0118 (Qualcomm, MediaTek) |

The following table provides summary of each test:

| Test | Separate trees | Shared trees |
| --- | --- | --- |
| 3-2.1.1 | Not changed | Smallest chroma intra CB size is 16 chroma samples. SCIPU(*) mode (inter or non-inter) is derived from prediction mode of the first luma CB in the SCIPU (without explicit signalling) |
| 3-2.1.2 | Not changed | Smallest chroma intra CB size is 16 chroma samples. SCIPU mode (inter or non-inter) is explicitly signalled on CU level in P,B slice |
| 3-2.2 | Not changed | Parallel-processable region (PPR) of size 16 or 32 chroma samples, where reference samples within PPR are marked as unavailable. VVC reference sample substitution process is used to construct reference sample array for intra prediction. In case of IBC block, the block vector of luma component that leads to an overlap between the reference block of the chroma and its PPR area is restricted. |

(*) SCIPU: A coding tree node whose chroma block size is larger than or equal to 16 chroma samples (32 or 16 samples) and has at least one child luma block smaller than 64 luma samples. Coding blocks within one SCIPU should be all inter-coded or all non-inter (intra or IBC) coded. Chroma components cannot be further split if the SCIPU is intra/IBC predicted.

## CE3-2: Test results

|  | Random Access Main10 - Over VTM-5.0 |  |  |  |  | Low Delay B Main10 - Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-2.1.1 | 0.07% | -0.08% | -0.11% | 100% | 100% | 0.14% | 0.16% | 0.07% | 100% | 100% |
| 3-2.1.2 | 0.09% | -0.01% | -0.02% | 99% | 101% | 0.19% | 0.19% | 0.10% | 101% | 103% |
| 3-2.2 | 0.07% | 0.02% | 0.09% | 100% | 100% | 0.05% | 0.13% | -0.16% | 100% | 100% |

## CE3-2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-2.1.1 | Chris Rosewarne (Canon) | y | Source code was studied and consistent with proposal description. Although the spec changes are somewhat invasive by necessity, no discrepancies with the description of the respective tests were found. Cross checked results match those provided by the proponent. Just CTC testing (RA/LD) – no further testing done. For test CE3-2.1.1, the cross checker observed some increase in LDB runtime at 104% overall instead of ~100%. |
| 3-2.1.2 |  | y |  |
| 3-2.2 |  | y | Source code was studied and consistent with proposal description. Although the spec changes are somewhat invasive by necessity, no discrepancies with the description of the respective tests were found.Cross checked results match those provided by the proponent, excepting one small mismatch on the case ra_main-BlowingBubbles-27. I reran the case and got the same result as my first run. The magnitude of the mismatch is similar to the parallel/serial mismatch sometimes seen, so I think overall results can be considered matching. I just let you know the tiny discrepancy seen. CTC testing (RA/LD) was performed – no further testing done. |

## CE3-2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-O0536 | 3-2.2 | CE3-related: Redundant bit saving for CE3-2.2 |
| JVET-O0640 | 3-2.1 | CE3-related: On 2x2/2x4/4x2 chroma blocks at the corner of pictures |

# CE3-3 on ‘CCLM model improvements’

## Defined tests [1]

| Test # | Short description | Doc. # |
| --- | --- | --- |
| 3-3.1 | Modified beta parameter derivation in context of the CCLM simplification (JVET-N0271). Luma and chroma averaged values are used to derive the CCLM β parameter. | JVET-O0088 (Sony) |
| 3-3.2.1 | Filtering is applied to both luma and chroma templates to remove outliers (VTM5 filters luma samples only) | JVET-O0095 (Huawei, HHI) |
| 3-3.2.2 | Test CE3-3.2.1 with calculating mean values for more robust parameter β deriving |  |
| 3-3.2.3 | Test CE3-3.2.2 without template filtering |  |
| 3-3.2.4 | Test CE3-3.2.1 with calculating mean values for more robustly deriving both parameters α and β |  |
| 3-3.2.5 | Test CE3-3.2.4 without template filtering |  |

The following table provides an overview for assessing the computational complexity of each test. The changes caused by subtests CE3-3 relate to preparing input data for min / max method, namely: template filtering and mean value calculations for CCLM parameter derivation. In particular, for VTM-5.0, the number of fetched samples equals 28 (4 luma samples after down-sampling x 6 luma samples as input values for 6-tap filter + 4 chroma samples = 24 + 4 =28) and the quantity of add operations is equal to 32 (4 luma samples after down-sampling x 6 add ops per each filtering operation + 2 add ops to calculate min for luma + 2 add ops to calculate min for chroma + 2 add ops to calculate max for luma + 2 add ops to calculate max for chroma = 24 + 8 = 32 add operations).

| Test # | Chroma 4x4 blocks (dual-tree type) |  | Chroma 4x8 blocks (dual-tree type) |  | Chroma 8x4 blocks (dual-tree type) |  | Other chroma blocks (any tree type) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Samples fetched | Add ops | Samples fetched | Add ops | Samples fetched | Add ops | Samples fetched | Add ops |
| VTM-5.0 | 28 | 32 | 28 | 32 | 28 | 32 | 28 | 32 |
| Test 3-3.1 | 28 | 34 | 28 | 34 | 28 | 34 | 28 | 34 |
| Test 3-3.2.1 | 34 | 44 | 35 | 44 | 35 | 44 | 36 | 44 |
| Test 3-3.2.2 | 28 | 46 | 28 | 46 | 28 | 46 | 36 | 46 |
| Test 3-3.2.3 | 8 | 10 | 8 | 10 | 8 | 10 | 28 | 34 |
| Test 3-3.2.4 | 35 | 44 | 35 | 44 | 35 | 44 | 36 | 44 |
| Test 3-3.2.5 | 8 | 8 | 8 | 8 | 8 | 8 | 28 | 32 |

## CE3-3: Test results

| Test # | All Intra Main10 - Over VTM-5.0 |  |  |  |  | Random Access Main10 - Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3-3.1 | -0.01% | -0.11% | -0.07% | 100% | 100% | 0.00% | -0.18% | -0.19% | 99% | 100% |
| 3-3.2.1 | 0.03% | 0.32% | 0.39% | 99% | 99% | 0.02% | 0.44% | 0.48% | 98% | 99% |
| 3-3.2.2 | 0.02% | 0.24% | 0.32% | 99% | 99% | 0.00% | 0.30% | 0.35% | 98% | 99% |
| 3-3.2.3 | 0.00% | 0.04% | 0.08% | 99% | 99% | 0.00% | -0.12% | -0.09% | 98% | 99% |
| 3-3.2.4 | 0.01% | 0.27% | 0.32% | 99% | 99% | 0.01% | 0.31% | 0.39% | 98% | 99% |
| 3-3.2.5 | 0.00% | 0.07% | 0.10% | 99% | 99% | 0.00% | -0.02% | -0.11% | 98% | 99% |

## CE3-3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3-3.1 | Christian Helmrich (HHI) | y | Source code was studied and is consistent with description. Spec text was provided for CE3-3.1 and is OK. AI, RA results match proponent’s results. LB was also tested. |
|  | Vasily Rufitskiy (Huawei) | y | Source code was studied, no inconsistencies with the description and spec text have been found. AI, RA results match proponent’s results. |
| 3-3.2.1 | Steve Keating (Sony) | y | Source code was studied and is consistent with description and spec text. AI, RA results match proponent’s results. |
| 3-3.2.2 |  | y | Source code was studied and is consistent with description and spec text. AI, RA results match proponent’s results. |
| 3-3.2.3 | M. Ikeda (Sony) | y | Source code was studied and is consistent with description and spec text. AI and RA results match proponent’s results. |
| 3-3.2.4 | Steve Keating (Sony) | y | Source code was studied and is consistent with description and spec text. AI, RA results match proponent’s results. |
| 3-3.2.5 | M. Ikeda (Sony) | y | Source code was studied and is consistent with description and spec text. AI and RA results match proponent’s results. |

## CE3-3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-O0343 |  | CE3-related: Simplification on CCLM process |
| JVET-O0377 |  | CE3-related: Context reduction for CCLM syntax element |
| JVET-O0412 | CE3-3.1, CE3-3.2.2-5 | CE3-related: Sampling point extension for CCLM |
| JVET-O0510 | CE3-3.2.1 | CE3-related: Extensions to CCLM reference sample filtering in CE3-3.2.1 |

# References

- G. Van der Auwera, L. Li, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 14th JVET Meeting, Geneva, CH, Mar. 2019, JVET-N1023