| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0024-v2 |
| --- | --- |

| Title: | CE4: Summary report on inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Chun-Chi Chen | Email: | haitao.yang@huawei.com chunchic@qti.qualcomm.com |
| Source: | CE4 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 4 on inter prediction. This CE comprises 3 categories, as follows:

- Switchable interpolation filter,

- Affine motion compensation,

- Combination of LIC and affine MC.

All techniques as planned in the context of CE4 are implemented on top of and tested against VTM-5.0. Simulation results and crosschecking reports of each test specified in this document are provided.

CE4-1: Switchable interpolation filter

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-1.1 | Additional half-pel AMVR mode only (for comparison purposes) | JVET-O0057 (HHI) |
| 4-1.2 | Test 1.1 + one alternative switchable luma half-pel interpolation filter, selected half-pel filter is propagated as part of the spatial merging candidate | JVET-O0057 (HHI) |
| 4-1.3 | Test 1.1 + two alternative switchable luma half-pel interpolation filters, selected half-pel filter is propagated as part of the spatial merging candidate | JVET-O0057 (HHI) |
| 4-1.4 | Test 1.2 without propagation of selected half-pel filter in merge mode | JVET-O0057 (HHI) |
| 4-1.5 | Test 1.3 without propagation of selected half-pel filter in merge mode | JVET-O0057 (HHI) |
| 4-1.6 | Test 1.2 with DMVR disabled for CUs using an alternative half-pel interpolation filter | JVET-O0057 (HHI) |
| 4-1.7 | Test 1.3 with DMVR disabled for CUs using an alternative half-pel interpolation filter | JVET-O0057 (HHI) |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-1.1 | -0.05% | -0.12% | -0.16% | 103% | 100% | 0.01% | 0.04% | -0.26% | 103% | 100% |
| 4-1.2 | -0.26% | -0.81% | -0.84% | 101% | 99% | -0.10% | -0.02% | -0.27% | 103% | 100% |
| 4-1.3 | -0.34% | -0.98% | -0.93% | 104% | 99% | -0.12% | -0.03% | -0.05% | 106% | 99% |
| 4-1.4 | -0.12% | -0.63% | -0.53% | 102% | 99% | -0.06% | -0.04% | -0.09% | 103% | 99% |
| 4-1.5 | -0.18% | -0.70% | -0.69% | 105% | 100% | -0.04% | -0.12% | -0.23% | 108% | 101% |
| 4-1.6 | -0.26% | -0.85% | -0.85% | 102% | 99% | -0.10% | -0.02% | -0.27% | 103% | 100% |
| 4-1.7 | -0.33% | -0.94% | -0.90% | 105% | 99% | -0.12% | -0.03% | -0.05% | 106% | 99% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-1.1 | K. Reuze (Qualcomm) | Y | Y | Y | N* |
| 4-1.2 | K. Reuze (Qualcomm) | Y | Y | Y | N* |
| 4-1.3 | K. Reuze (Qualcomm) | Y | Y | Y | N* |
| 4-1.4 | T. Solovyev (Huawei) | Y | Y | Y | Y |
| 4-1.5 | T. Solovyev (Huawei) | Y | Y | Y | Y |
| 4-1.6 | T. Solovyev (Huawei) | Y | N** | Y | Y |
| 4-1.7 | T. Solovyev (Huawei) | Y | N** | Y | Y |
| * Crosschecker reports ~2% extra runtime increase at encoder side. ** Crosschecker confirmed with the tester that there is a discrepancy between SW and Text in the derivation of HPEL filter index for pairwise merge candidate. SW match CE description: Y SW implementation match CE description, N mismatch. Text match SW: Y text match SW, N mismatch. Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results. EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch. |  |  |  |  |  |

Detailed description of tests

- Test 4-1.1, 4-1.2 and 4-1.3 in JVET-O0057 (HHI)

In JVET-N0309, a switched luma half-pel interpolation filter is proposed. The switching of the half-pel luma interpolation filter is done depending on the motion vector accuracy. In addition to the existing quarter-pel, full-pel, and 4-pel AMVR modes, a new half-pel accuracy AMVR mode is introduced. Only in case of half-pel motion vector accuracy, an alternative half-pel luma interpolation filter can be selected.

- Half-pel AMVR mode: An additional AMVR mode for non-affine non-merge inter-coded CUs is proposed which allows signaling of motion vector differences at half-pel accuracy. The existing AMVR scheme of the current VVC draft is extended straightforward in the following way: Directly following the syntax element amvr_flag, if amvr_flag == 1, there is a new context-modeled binary syntax element hpel_amvr_flag which indicates usage of the new half-pel AMVR mode if hpel_amvr_flag == 1. Otherwise, i.e. if hpel_amvr_flag == 0, the selection between full-pel and 4-pel AMVR mode is indicated by the syntax element amvr_precision_flag as in the current VVC draft.

- Alternative luma half-pel interpolation filters: In JVET-N0309, alternative luma half-pel interpolation filters are proposed. For a non-affine non-merge inter-coded CU which uses half-pel motion vector accuracy (i.e., the half-pel AMVR mode), an alternative luma half-pel interpolation filter is used. For test 4-1.2, one alternative luma half-pel interpolation filter is used. For test 4-1.3, a switching between two alternative half-pel interpolation filters is made based on the value of a new syntax element if_idx. The syntax element if_idx is only signaled in case of half-pel AMVR mode. In case of skip/merge mode using a spatial merging candidate, the information which interpolation filter is applied for the half-pel position is inherited from the neighbouring block.

- Test 4-1.4 and 4-1.5 in JVET-O0057 (HHI)

The tests 4-1.4 and 4-1.5 correspond to tests 4-1.2 and 4-1.3, with the distinction that in tests 4-1.4 and 4-1.5 the information which interpolation filter is applied for the half-pel position is not inherited from the neighbouring block in case of skip/merge mode using a spatial merging candidate.

- Test 4-1.6 and 4-1.7 in JVET-O0057 (HHI)

The tests 4-1.6 and 4-1.7 correspond to tests 4-1.2 and 4-1.3, with the distinction that in tests 4-1.6 and 4-1.7 the DMVR process is not applied to CUs which are encoded in spatial skip/merge mode and which have inherited the information from the neighbouring block that an alternative interpolation filter is applied for the half-pel position.

CE4-2: Affine motion compensation

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-2.1a | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for both uni- and bi-prediction. | JVET-O0070 (InterDigital) |
| 4-2.1b* | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for uni-prediction and 8x8 for bi-prediction. | JVET-O0070 (InterDigital) |
| 4-2.1c* | Prediction refinement with optical flow. The subblock size for affine motion compensation is 4x4 for uni-prediction with and 8x8 for bi-prediction. 8-tap interpolation filter is used for 8x8 subblocks. | JVET-O0070 (InterDigital) |
| 4-2.2a | Withdrawn |  |
| 4-2.2b | Withdrawn |  |
| 4-2.3a | Interweaved prediction for luma component. | JVET-O0069 (Bytedance) |
| 4-2.3b | Interweaved prediction for luma component with equal weighting values | JVET-O0069 (Bytedance) |
| 4-2.3c | Test 4-2.3a + 4-2.1a PROF is applied on the two auxiliary prediction individually. | JVET-O0069 (Bytedance) |
| 4-2.3d | Test 4-2.3b + 4-2.1a PROF is applied on the two auxiliary prediction individually. | JVET-O0069 (Bytedance) |
| 4-2.4a | Simplifications of interweaved affine motion compensation in test 4-2.3a, reusing bi-prediction processes. | JVET-O0082 (Tencent) |
| 4-2.4b | Simplifications of interweaved affine motion compensation in test 4-2.3b, reusing bi-prediction processes. | JVET-O0082 (Tencent) |
| 4-2.5a | Phase-variant affine MC. Up to 9 rows and 4 columns of MVx/MVy are derived. | JVET-O0051 (MediaTek) |
| 4-2.5c | Phase-variant affine MC. Test 4-2.5a with fallback mode disabled. | JVET-O0051 (MediaTek) |
| 4-2.5d** | Phase-variant affine MC. The input reference samples of a 4x4 block is limited to be 9x9 square block. | JVET-O0051 (MediaTek) |
| 4-2.5e | Test 4-2.5a + 4-2.1a | JVET-O0051 (MediaTek) |
| * The software of Test 4-2.1b/c was re-tagged after T3 to fix 2 bugs to make the software align itself with the planned CE descriptions of CE4-2.1b/c. ** The software of Test 4-2.5d was re-tagged after T3 to remove one line of code that triggers compilation error due to an unused variable. |  |  |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-2.1a | -0.48% | -0.09% | -0.07% | 101% | 100% | -0.32% | -0.04% | -0.37% | 102% | 100% |
| 4-2.1b | -0.40% | -0.02% | 0.02% | 100% | 101% | -0.27% | 0.02% | -0.21% | 101% | 100% |
| 4-2.1c | -0.46% | -0.05% | -0.06% | 100% | 100% | -0.35% | -0.06% | -0.21% | 101% | 100% |
| 4-2.3a | -0.32% | -0.06% | -0.05% | 100% | 100% | -0.14% | -0.02% | -0.26% | 103% | 100% |
| 4-2.3b | -0.29% | -0.03% | -0.07% | 101% | 101% | -0.10% | 0.08% | -0.26% | 103% | 100% |
| 4-2.3c | -0.53% | -0.10% | -0.05% | 103% | 101% | -0.34% | -0.25% | -0.18% | 105% | 102% |
| 4-2.3d | -0.54% | -0.09% | -0.05% | 103% | 102% | -0.33% | -0.13% | -0.25% | 106% | 103% |
| 4-2.4a | -0.32% | -0.05% | -0.03% | 105% | 102% | -0.11% | 0.00% | -0.17% | 108% | 104% |
| 4-2.4b | -0.28% | -0.09% | -0.07% | 106% | 102% | -0.15% | 0.06% | -0.18% | 108% | 103% |
| 4-2.5a | -0.39% | -0.51% | -0.50% | 101% | 99% | -0.18% | -0.16% | -0.22% | 101% | 99% |
| 4-2.5c | -0.36% | -0.48% | -0.53% | 104% | 99% | -0.15% | -0.17% | -0.69% | 108% | 104% |
| 4-2.5d | -0.38% | -0.50% | -0.56% | 102% | 100% | -0.21% | -0.19% | -0.30% | 103% | 101% |
| 4-2.5e | -0.59% | -0.47% | -0.47% | 103% | 100% | -0.38% | -0.31% | -0.56% | 105% | 105% |

Complexity analysis

Basic operations per 4x4 subblock should be provided for complexity analysis. In addition, analysis of additional memory access and local buffer requirement along with a technique under test is requested, normalized to 4x4 subblocks as well.

| Test# | Number of mults | Number of adds | Number of shifts | Additional memory access | Additional local buffer | Others |
| --- | --- | --- | --- | --- | --- | --- |
| VTM-5.0 | 400/800 | 350/700 | 74/172 | 0 | 0 | Results are shown for uni/bi-prediction separately |
| 4-2.1a | 432/864 | 430/860 | 106/236 | 0 | 96/96 bytes | Delta MV: 4x4x2x1 = 32 bytes Gradient: 4x4x2x2 = 64 bytes CU level delta MV calculations per reference picture for both uni and bi-prediction: 4x4x2 additions + 4x4x2 shifts + 4x4x2 clippings |
| 4-2.1b | 432/744 | 430/760 | 106/216 | 0 | 96/96 bytes | Delta MV: 4x4x2x1 = 32 bytes Gradient: 4x4x2x2 = 64 bytes CU level delta MV calculations per reference picture for uni-prediction is the same as 4-2.1a. CU level delta MV calculations per reference picture for bi-prediction: 8x8x2 additions + 8x8x2 shifts + 8x8x2 clippings. |
| 4-2.1c | 432/976 | 430/984 | 106/224 | 0 | 96/96 bytes | Same as 4-2.1b |
| 4-2.3a | 728 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |
| 4-2.3b | 712 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |
| 4-2.3c | 786/864 | 768/860 | 224/236 | 0 | 20 bytes / 0 |  |
| 4-2.3d | 760/864 | 768/860 | 224/236 | 0 | 20 bytes / 0 |  |
| 4-2.4a | 728 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |
| 4-2.4b | 712 / 800 | 626 / 700 | 164 / 172 | 0 | 20 bytes / 0 |  |
| 4-2.5a | 401.5/803 | 370/740 | 94/212 | 2*(CU_Height +L-1) samples for a CU | 28.125 bytes | 4x4-level: 20 “&” operation + 4-sapme of padding CU-level: 12 adds + 1 shift + 12 clipping + 2 absolute operation + 2 comparison |
| 4-2.5c | 401.5/803 | 370/740 | 94/212 | 2*(CU_Height +L-1) samples for a CU | 28.125 bytes | 4x4-level: 20 “&” operation + 4-sapme of padding CU-level: 12 adds + 1 shift + 12 clipping |
| 4-2.5d | 401.5/803 | 370/740 | 94/212 | 0 | 28.125 bytes | 4x4-level: 20 “&” operation + 13-sapme of padding CU-level: 12 adds + 1 shift + 12 clipping + 2 absolute operation + 2 comparison |
| 4-2.5e | 433.5/867 | 450/900 | 126/276 | 2*(CU_Height +L-1) samples for a CU | 28.125 bytes | 4x4-level: 20 “&” operation + 4-sapme of padding CU-level: 12 adds + 1 shift + 12 clipping + 2 absolute operation + 2 comparison |

Subjective evaluation

Following common test condition, sequence CatRobot1 encoded with RA configuration at QP 37 will be used for subjective evaluation. Both proponents and crosscheckers are requested to check the subjective quality of a test and feedback whether this is an observable difference compared with the anchor. The bitstream of the sample should be provided to CE4 participants.

Feedback collected from all CE4 participants are listed below.

- The tester of CE4-2.1 suggest focusing on the rotating color plate, especially the boundaries at color sectors. It is also suggested using a slower playing back speed.

- The tester reported no visual quality issue of CE4-2.4 tests.

- The crosschecker reported no visual quality issue of CE4-2.3 tests.

- The crosschecker reported no visual quality issue of CE4-2.5a.

- One participant reported no observable difference of CE4-2.1a compared with VTM-5.0 anchor.

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-2.1a | T.-D. Chuang (MediaTek) | Y | Y | Y | Slightly higher. Might be noise. |
| 4-2.1b | T.-D. Chuang (MediaTek) | Y | Y | Y | Y |
| 4-2.1c | T.-D. Chuang (MediaTek) | Y | Y | Y | Y |
| 4-2.3a | G. Li (Tencent) | Y | Y | Y | Y |
| 4-2.3b | G. Li (Tencent) | Y | Y | Y | Y |
| 4-2.3c | G. Li (Tencent) | Y | Y | Y | Y |
| 4-2.3d | G. Li (Tencent) | Y | Y | Y | Y |
| 4-2.4a | Y.-W. Chen (Kwai) | Y | Y | Y | Y |
| 4-2.4b | Y.-W. Chen (Kwai) | Y | Y | Y | Y |
| 4-2.5a | H. Hang (Qualcomm) | Y | Y | Y | Crosscheck’s runtime is not accurate |
| 4-2.5c | T.-C. Ma (Kwai) | Y | Y | Y | Y |
| 4-2.5d | H. Hang (Qualcomm) | Y | Y | Y | Crosscheck’s runtime is not accurate |
| 4-2.5e | J. Chen (Alibaba) | Y | Y | Y | Y |
| SW match CE description: Y SW implementation match CE description, N mismatch. Text match SW: Y text match SW, N mismatch. Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results. EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch. |  |  |  |  |  |

Detailed description of tests

- Test 4-2.1: Prediction refinement with optical flow for affine mode (JVET-O0070, InterDigital)

To achieve a finer granularity of motion compensation, this contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, luma prediction sample is refined by adding a difference derived by the optical flow equation. The sub-block-based affine motion compensation is performed to generate sub-block prediction. The spatial gradients of the sub-block prediction are calculated at each sample location using a 3-tap filter [-1, 0, 1]. The sub-block prediction is extended by one pixel on each side for the gradient calculation. The pixels on the extended borders are copied from the nearest integer pixel position in the reference picture. The MV difference between pixel MV and the sub-block MV, as shown in the below figure. The luma prediction refinement is calculated by the optical flow equation based on the MV differences and gradients. Finally, the luma prediction refinement is added to the sub-block prediction.

3 sub-tests are planned in CE. In test a, affine subblock size is 4x4 in both uni- and bi-prediction. In test b, affine subblock size is 4x4 for uni-prediction and 8x8 for bi-prediction. Test c is test b except 8-tap interpolation filter is used for 8x8 sub-block.

Figure. Sub-block MV VSB and pixel Δv(i,j) (red arrow)

- Test 4-2.2: Simplified prediction refinement for affine motion compensation (Withdrawn)

- Test 4-2.3: Interweaved Prediction for Affine Motion Compensation (JVET-O0069, Bytedance)

In this contribution, an interweaved prediction approach is proposed for affine motion compensation (AMC). With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions.

Figure. Interweaved prediction.

Figure. Interweaved prediction is not applied on the shaded regions.

In test 4-2.3a, interweaved prediction will be tested with the unequal weighting values as proposed in JVET-N0261.

In test 4-2.3b, interweaved prediction will be tested with equal weighting values (1/2, 1/2).

In test 4-2.3c, test 4-2.3a will be combined with test 4-2.1a.

In test 4-2.3d, test 4-2.3b will be combined with test 4-2.1a.

- Test 4-2.4: Simplifications of interweaved affine mode (JVET-O0082, Tencent)

In interweaved affine mode, the two predictions are both uni-predicted, and then a weighted averaging process is invoked to generate the final prediction. To keep higher interpolation accuracy and simple control logic, it is proposed to regard interweaved affine mode as a special bi-predicted mode such that the weighted averaging may reuse the current weighted bi-prediction process with interweaved affine weighting table as input.

Moreover, it is proposed to further simplify the weighting process: Always using 2:2 weighting for all pixels except for the shaded region (as shown in the figure below) which keeps using 4:0 weighting as only one prediction is available for such a region.

- Test 4-2.5: Phase-variant affine subblock motion compensation (JVET-O0051, MediaTek)

In this CE test, the phase-variant affine subblock motion compensation (MC) is tested. The conventional two stage horizontal-vertical interpolation is applied. However, unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows and different sample columns in the affine subblock. The used interpolation filter sets in the proposed method shall be the same as those in VTM-5.0, the 6-tap filter for affine MC. If the delta MVs of control point MVs are too small, the fallback mode, the original phase-invariant affine MC in VTM, is applied. The number of filtering operations for each affine subblock in the proposed method is also shall be the same as that in VTM-5.0.

CE4-2.5a requires an 11×9 luma reference sample window and a 9×7 chroma reference sample window for the phase-variant horizontal filtering.

CE4-2.5c is the similar to CE4-2.5a, but the fallback mode is disabled. Phase-variant horizontal filtering and vertical filtering are always applied regardless of |dMvVerX| and |dMvHorY|.

CE4-2.5d is the similar to CE4-2.5a, but it only requires a 9×9 luma reference sample window and a 7×7 chroma reference sample window for the phase-variant horizontal filtering. The 9×9 and 7×7 reference sample windows are enlarged to 11×9 and 9×7 reference sample windows by padding, and the rest filtering processes are not changed.

CE4-2.5e is the combination of the PROF in CE4-2.1a and CE4-2.5a. It uses the phase-variant affine MC described in CE4-2.5a for affine MC and then refined by CE4-2.1a. The gradient value calculation in CE4-2.1a is not modified. The only change of PROF in this combined test is that the MV difference between sample-based affine MV and subblock MV used in PROF is modified as the MV difference between sample-based affine MV and phase-variant equivalent MV. The phase-variant equivalent MV is derived as follows. It can be pre-calculated once per CU.

pvMVDy[x][y] = (dMvHorY×(x–1) – (dMvHorY>>1)) >> 7

pvMVDx[x][y] = (dMvVerX×(pvMVDy[x][y] + ((2y–3) << (scalcShift-1)))) >> (7+scalcShift)

The x and y are the sample position within a 4×4 subblock. The scalcShift is 4 for luma and 5 for chroma.

CE4-3: Combination of LIC and affine MC

Test specification

| Test # | Description | Source |
| --- | --- | --- |
| 4-3.1a | Unidirectional LIC with affine MC | JVET-O0066 (Qualcomm, MediaTek, Huawei) |
| 4-3.1b | Test 4-3.1a + encoder fast algorithm | JVET-O0066 (Qualcomm, MediaTek, Huawei) |
| 4-3.2a | Test 4-3.1a + min/max simplification | JVET-O0086 (Huawei) |
| 4-3.2b | Test 4-3.1b + min/max simplification | JVET-O0086 (Huawei) |

Simulation results

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4-3.1a | -0.46% | -0.39% | -0.50% | 116% | 100% | -0.37% | 0.03% | -0.30% | 119% | 101% |
| 4-3.1b | -0.32% | -0.22% | -0.40% | 105% | 99% | -0.24% | -0.02% | -0.43% | 108% | 102% |
| 4-3.2a | -0.41% | -0.37% | -0.48% | 117% | 100% | -0.31% | -0.12% | -0.40% | 119% | 101% |
| 4-3.2b | -0.24% | -0.23% | -0.23% | 105% | 101% | -0.13% | -0.19% | -0.24% | 107% | 101% |

Crosscheck report

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 4-3.1a | W. Chen (InterDigital) |  |  |  |  |
|  | P. Onno (Canon) | / | Y | Y | Y/unreliable |
| 4-3.1b | W. Chen (InterDigital) |  |  |  |  |
|  | P. Onno (Canon) | / | Y | Y | Y/unreliable |
| 4-3.2a | P. Onno (Canon) | Y | Y | Y | Y/unreliable |
| 4-3.2b | P. Onno (Canon) | Y | Y | Y | Y/unreliable |
| Note: the crosschecking of 4-3.1a and 4-3.1b by the first crosschecker is still on going, and no results have been reported by the time the document is uploaded. The second crosschecker provided full simulation results, but no report on whether SW match CE description. Note: one participant reported a discrepancy between SW implementation and CE description in test 4-3.2, but there is no consensus with the tester through the discussion in CE4 mailing list. SW match CE description: Y SW implementation match CE description, N mismatch. Text match SW: Y text match SW, N mismatch. Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results. EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch. |  |  |  |  |  |

Detailed description of tests

- Test 4-3.1 Combination of LIC and affine MC (JVET-O0066, Qualcomm, MediaTek, Huawei)

In test 4-3.1a, unidirectional LIC method with 16x16 processing units and extension to affine mode was proposed. It is applied to both luma and chroma components and in the following configuration:

- Intra neighbor samples are not used in LIC parameter derivation and are replaced by the reference neighbor samples

- Disable for combined inter/intra prediction (CIIP) and intra block copy (IBC) blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- Applied on 16x16 processing units, where LIC parameters are estimated for the first top left 16x16 unit and are used for other units within the coding unit (CU).

- When combined with affine mode, LIC parameters are derived based on the samples derived on a sub-block basis, and it is limited to the first top-left 16x16 processing unit.

In test 4-3.1b, two encoder-only changes are applied:

- The motion search results of LIC-off is reused to test LIC-on as proposed in JVET-M0182.

- If the LIC usage ratio of neighbouring 4x4 subblocks is larger than or equal to a threshold (TH), then the mean removed sum of absolute differences (MR-SAD) motion search with LIC-on is further tested. As illustrated in Figure 3, assuming the current CU size is W×H, the number of neighbouring 4x4 subblocks inside the region marked in red is n, and LIC is applied to m out of n subblocks. If mn≥TH, then MR-SAD motion search with LIC-on is further tested. TH is set to 0.10.

It is reported that the worst case computational complexity is 1.3 multiplications and 1.6 additions per sample, in case of an 8x8 CU.

- Test 4-3.2 Tests 4-3.1 + min/max simplification (JVET-O0086, Huawei)

It is proposed to apply MinMax method to derive LIC parameters a and b. Additional improvements proposed in JVET-N0716 on top of the MinMax methods relate to the following aspects:

- No template downsampling.

- Template filtering when combined with affine mode.

- Mean values: mean values ymean and xmean can be calculated by performing averaging separately for each side and both results are averaged with each other; then parameter b is calculated as follows:

b=ymean-a*xmean

- Slope value: not only for the smallest luma value A and largest luma value B but also for the second-smallest luma value A' and the second-largest luma value B' are searched as shown in Fig. 1 in JVET-N0716. Then, the trend line can be fitted between the average of the data points at A and A' (by using yA = (A + A') >>  1) and the average of the data points at B and B' (by using yB = (B + B') >> 1), which generally leads to a better data-fit:

a=yB-yAxB-xA

Non-CE4/CE4-related contributions

Affine motion compensation (17)

| JVET-VC number | Title |
| --- | --- |
| JVET-O0123 | Non-CE9: Alignment of BDOF refinement process with PROF |
| JVET-O0252 | CE4-related: Alignment and simplification of PROF and BDOF |
| JVET-O0254 | Non-CE4: Size constrain for inherited affine motion prediction |
| JVET-O0281 | CE4-related: Alignment of displacement bitdepths for BDOF and PROF |
| JVET-O0286 | CE4-related: Extension of phase-variant affine subblock motion compensation in CE4-2.5 |
| JVET-O0313 | CE4-related: On Conditions for enabling PROF |
| JVET-O0438 | Non-CE4: Cleanup for affine AMVR enabling flag in SPS level |
| JVET-O0500 | non-CE4: affine and sub-block modes coding clean-up |
| JVET-O0530 | Non-CE4: Adaptive subblock size for affine motion compensation |
| JVET-O0553 | Non-CE4: On prediction refinement with optical flow |
| JVET-O0580 | CE4-related: Additional tests for prediction refinement with optical flow for affine mode (CE4-2.1) with 4x8 sub-block size |
| JVET-O0583 | CE4-related: Prediction refinement with optical flow for chroma components for affine mode |
| JVET-O0593 | CE4-related: Harmonization of BDOF and PROF |
| JVET-O0594 | CE4-related: Prediction sample padding unification for BDOF and PROF |
| JVET-O0595 | CE4-related: Modified adaptive motion vector resolution (AMVR) for affine mode |
| JVET-O0624 | CE4-related: Unified padding for BDOF and PROF |
| JVET-O0664 | CE4-related: Simplifications on PROF |

LIC (6)

| JVET-VC number | Title |
| --- | --- |
| JVET-O0190 | Non-CE4: LIC model parameters using forward mapped luma samples in LMCS |
| JVET-O0257 | CE4-related: Simplified local illumination compensation |
| JVET-O0457 | CE4-related: LIC flag inherited per component |
| JVET-O0470 | CE4-related: encoder simplification of local illumination compensation |
| JVET-O0531 | Non-CE4: On LIC for Affine |
| JVET-O0605 | CE4-related: On exclusively applying LIC and PROF |

Motion vector coding (17)

| JVET-VC number | Title |
| --- | --- |
| JVET-O0164 | Non-CE4: Simplification on AMVP list construction |
| JVET-O0253 | Non-CE4: Simplification of HMVP in AMVP mode |
| JVET-O0284 | CE4-related: Interaction between symmetric MVD and mvd_l1_zero_flag |
| JVET-O0414 | Non-CE4 : Symmetric-MVD control considering the reference picture type |
| JVET-O0424 | Non-CE4: On TMVP reference frame selection using HMVP |
| JVET-O0434 | Non-CE4: Simplification of TMVP |
| JVET-O0439 | Non-CE4: Modification of the temporal merging candidate for the regular merge mode |
| JVET-O0440 | Non-CE4: A modified method to derive the temporal merge candidate for long-term reference picture |
| JVET-O0441 | Non-CE4: Short-tap interpolation for bi-prediction |
| JVET-O0456 | Non-CE4: Grouping the syntax elements of motion vector differences |
| JVET-O0468 | Non-CE4 : simplified SMVD mode coding |
| JVET-O0567 | Non-CE4: Cleanups on MVD ranges |
| JVET-O0572 | Non-CE4: On availability of default reference pictures used in SMVD |
| JVET-O0579 | Non-CE4 : SMVD processing when mvd_l1_zero_flag is set equal to 1 |
| JVET-O0587 | Non-CE4: Simplification of MVP list construction |
| JVET-O0607 | CE4-related: On handling the interaction of the mvd_l1_zero_flag and sym_mvd_flag |
| JVET-O0652 | CE4-related: Line buffer size reduction method for switchable interpolation filter |

BCW modifications (4)

| JVET-VC number | Title |
| --- | --- |
| JVET-O0126 | Non-CE4: Align coding of BPWA index with normal truncated rice binarization process |
| JVET-O0366 | Non-CE4 : Simplifications on BCW index derivation process |
| JVET-O0367 | Non-CE4 : The proposed BCW index derivation for pairwise candidate |
| JVET-O0516 | non-CE4: BCW for pairwise candidate |

Merge modifications (59)

| JVET-VC number | Title |
| --- | --- |
| JVET-O0128 | Non-CE4: Simplification of triangle mode MV context storage |
| JVET-O0163 | Non-CE4: Simplification on SbTMVP |
| JVET-O0167 | Non-CE4: On SbTMVP base motion data derivation |
| JVET-O0217 | Non-CE4: Simplification of motion vector storing process for triangle merge mode |
| JVET-O0220 | Non-CE4: On maximum number of subblock merge candidates |
| JVET-O0221 | Non-CE4: On merge data syntax |
| JVET-O0222 | Non-CE4: Modification of triangle merge mode enabling criterion |
| JVET-O0249 | Non-CE4: Merge Modes Signaling |
| JVET-O0263 | Non-CE4: Cleanups on syntax design for sub-block coding tools |
| JVET-O0265 | Non-CE4: Simplified motion field storage for TPM |
| JVET-O0278 | CE4-related: On signalling of merge flags |
| JVET-O0279 | CE4-related: Simplification of motion vector storing process for triangle prediction mode |
| JVET-O0280 | CE4-related: SIMD implementation for weighted sample prediction process of triangle prediction mode |
| JVET-O0283 | CE4-related: Simplification for SbTMVP |
| JVET-O0285 | CE4-related: On merging candidate list construction |
| JVET-O0305 | Non-CE4: Undefined condition of merge mode syntax |
| JVET-O0309 | Non-CE4: Merge mode signalling overhead reduction |
| JVET-O0314 | Non-CE4: Modification of MV storage in triangle merge mode |
| JVET-O0328 | Non-CE4: On Sub-block Merge Candidates List |
| JVET-O0329 | Non-CE4: Simplification of motion vector storage process for triangle merge mode |
| JVET-O0330 | Non-CE4: Syntax element â€˜regular_merge_flagâ€™ signalling |
| JVET-O0361 | Non-CE4: Remove the redundancy among the flag coding for merge modes |
| JVET-O0362 | Non-CE4: Modified enabling condition for triangle mode |
| JVET-O0378 | Non-CE4: Simplification of MV context storage for triangle mode |
| JVET-O0379 | Non-CE4: Modified mode decision process for triangle mode |
| JVET-O0384 | CE4-related: CIIP Simplification |
| JVET-O0388 | Non-CE4: On the context of triangle merge index |
| JVET-O0411 | Non-CE4: Simplification of bi-prediction MV generation for triangle partition mode storage |
| JVET-O0415 | Non-CE4 : Fix of POC distance condition for MMVD |
| JVET-O0418 | CE4-related: Simplification of motion vector storage operation for triangle merge mode |
| JVET-O0420 | CE4-related: Reduce scaling operation number for MMVD |
| JVET-O0421 | Non-CE4: On CIIP Merge mode with no residual |
| JVET-O0422 | Non-CE4: On mode flags signalling for Merge |
| JVET-O0423 | Non-CE4: On SbTMVP derivation using HMVP |
| JVET-O0431 | Non-CE4: On Merge data signaling |
| JVET-O0489 | Non-CE4: Geometrical partitioning for inter blocks |
| JVET-O0501 | non-CE4: temporal merge modes simplification |
| JVET-O0513 | Non-CE4: Adaptive blending filtering for TPM |
| JVET-O0522 | Non-CE4: CIIP using triangular partitions |
| JVET-O0534 | Non-CE4: Redundancy removal for merge mode signaling |
| JVET-O0537 | Non-CE: Weighted intra and inter prediction mode |
| JVET-O0552 | Non-CE4: Extension of triangle partition mode to P-slices |
| JVET-O0560 | Merge Mode Syntax |
| JVET-O0585 | Non-CE4 : Merge syntax for removing redundant syntax signaling or fall back merge mode |
| JVET-O0586 | Non-CE4: Redundancy removal in merge modes signaling |
| JVET-O0588 | CE4-related: On SbTMVP motion shift derivation |
| JVET-O0589 | Non-CE4: Simplification of HMVP in merge list construction |
| JVET-O0601 | CE4-related: On motion vector population under triangle prediction mode |
| JVET-O0604 | CE4-related: Simplification on block location validation in SbTMVP |
| JVET-O0629 | Non-CE4: Simplification of motion field for triangle partition |
| JVET-O0633 | Non-CE4: On MMVD candidate flag coding |
| JVET-O0639 | Non-CE4: Context reduction for merge mode |
| JVET-O0645 | Non-CE4/8: Blending-off Switch for TPM Mode |
| JVET-O0646 | Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode |
| JVET-O0653 | Non-CE4: Motion field storage simplification for triangular prediction mode |
| JVET-O0660 | Simplification of MV storage for blending block in triangle mode |
| JVET-O0661 | Non-CE4/8: Simplification of CIIP on IBC mode |
| JVET-O0666 | Non-CE4: Fix for missing MV in 4xN and Nx4 triangle partition mode |
| JVET-O0795 | CE4-related: On the context variables and syntax elements of merge index |

Recommendations

Discuss CE4 tests and review all Non-CE4/CE4-related contributions.