| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0025-v4 |
| --- | --- |

| Title: | CE5: Summary Report on Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Ching-Yeh Chen | Tel: Email: | anorkin@netflix.com chingyeh.chen@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Core Experiment 5 on Loop Filtering. There are two categories of the tests: deblocking filter (CE5-1, CE5-2, and CE5-3) and adaptive loop filter (CE5-4). The subjective testing for the deblocking filter category is planned to be performed at the meeting.

The corresponding compression performance of each coding tool evaluated in CE5 is summarized in this contribution. In addition, analysis of complexity of the proposals and cross-checking results are provided.

The software basis for this CE was VTM-5.0. Configurations and test conditions in JVET-N1010 [1] for SDR sequences are used. Results for additional configuration with ALF turned off are also reported. For the subjective viewing of deblocking proposals, additional encodes with different QPs have been generated, including two sequences in 4:4:4 chroma format.

# Detailed description of tests

#### CE5-1 Sub-PU deblocking and longer tap filter

Proposal in this sub-CE investigate changes to sub-PU deblocking and longer deblocking filter of VVC.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.1 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-O0083 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

#### CE5-1.1: Modification of sub-PU deblocking and longer tap filter JVET-O0083

This test proposes two modifications to the current deblocking filter design in VTM.

The first modification disables application of longer tap filter for implicit TU boundaries.

The second modification disables the filtering for first and last sub-PU edges for a block which uses sub-PU tools. This reportedly has an effect that the longer tap filter can directly be applied to the coding unit edge without any further checks.

#### CE5-2 Deblocking at 4x4 block boundaries

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-2.1 (without chroma) | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-O0060 | Hyeongmun Jang hm.jang@lge.com |
| CE5-2.2 (with chroma) | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra Anand.meher.kotra@huawei.com Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-O0060 | Hyeongmun Jang hm.jang@lge.com |
| CE5-2.3 (without chroma) | Hyeongmun Jang hm.jang@lge.com JVET-O0091 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE5-2.4 (with chroma) | Hyeongmun Jang hm.jang@lge.com JVET-O0091 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE5-2.5 Chroma on 4x4 chroma grid | Hyeongmun Jang hm.jang@lge.com JVET-O0092 | Misra Kiran misrak@sharplabs.com |

#### CE5-2.1 Deblocking for 4xN, Nx4, 8xN and Nx8 blocks JVET-O0060

As shown in the figure above, the proposed method enforces the Luma weak filter with one or no sample modification, when any of the blocks sharing the edge has a size of 4 samples (i.e. width = 4 samples for vertical edge, height = 4 samples for horizontal edge).

The different steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition is evaluated to true i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge. If true then

- Check if the one of the blocks has a block width of 4 samples, if true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8)≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

#### CE5-2.2 Deblocking for 4xN, Nx4, 8xN and Nx8 blocks JVET-O0060

This test would check the CE5-2.1 + chroma deblocking performed on the 4x4 chroma grid.

#### CE5-2.3 Parallel deblocking filter JVET-O0091

Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is studied in terms of subjective quality and computational complexity. Chroma deblocking is performed in the same way as in the current deblocking. [Cross-checker argues that chroma is also modified to be filtered more often, see cross-checkers notes]

Filter length decision process for luma is as follows:

- If the distance of Q side is smaller than or equal to 4, filter length is set equal to 0.

- Otherwise, if the distance of the each side is smaller than or equal to 8, filter length is equal to 3.

- Otherwise if the distance of the each side is smaller than or equal to 16, filter length is equal to 5.

- Otherwise, filter length is set equal to 7.

In the proposal, different filter lengths than in VTM5.0 be derived in some cases as shown in the example below.

Example filter length for VTM-5.0 (different filter lengths).

The filter length of the proposed method is only decided based on the distance between neighboring blocks as shown in the figure below.

Same filter length for blocks of the same size by the proposed method.

#### CE5-2.4 Parallel deblocking filter JVET-O0091

This test would study the CE5-2.3 + chroma deblocking changes as in JVET-N0463.

- If the distance of Q side is smaller than or equal to 2, filter length is set equal to 0

- Otherwise, if the distance of the each side is smaller than or equal to 4, filter length is set equal to 1.

- Otherwise, filter length is set equal to 3.

To support various color formats, distance condition for filter length decision is adaptively applied. For example, chroma filter length decision for 4:4:4 chroma format is applied as bellow.

- If the distance of Q side is smaller than or equal to 4, filter length is set equal to 0

- Otherwise, if the distance of the each side is smaller than or equal to 8, filter length is equal to 1.

- Otherwise, filter length is set equal to 3.

#### CE5-2.5 Deblocking chroma on 4x4 chroma grid JVET-O0092

Chroma deblocking is performed on the 4x4 chroma grid (current VTM chroma deblocking is done on the 8x8 chroma grid).

In this proposal, three parts are modified compared with deblocking filtering of VTM-5.0.

- Enable filtering for chroma sub-pu edge.

- Enable filtering for chroma edge whose boundary strength is not 0.

- Perform filtering on 4x4 chroma sample grid.

Reportedly, filtering process is applied at edges which are aligned with 8x8 grid for luma and edges which are aligned with 4x4 grid for chroma component, which makes filtering process aligned between luma and chroma component for 4:2:0 format.

#### CE5-3 MV difference threshold in deblocking decisions

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-3.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-O0061 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

#### CE5-3.1 Sub-sample MV threshold for deblocking decisions JVET-O0061

It has been shown in JVET-N0359 that block artifacts can occur between two blocks for a magnitude difference in horizontal or vertical motion vector component smaller than one luma sample.

The test investigates the effect of different sub-sample thresholds in deblocking decisions. It is proposed to set the MV component difference threshold for determination of boundary strength to 2 in units of quarter luma samples to enable more efficient reduction of blocking artifacts.

#### CE5-4 Adaptive loop filter JVET-O0090

In the JVET-N0415 proposal, which was adopted for VTM5.0, ALF process can now either use filter sets of previously received APS (temporal filter sets) or use predefined luma filter sets. However it is not possible to signal more than one new luma filter set in a given APS. In ALF tool, the filter coefficients of one new luma filter set can be predicted by using the coefficients of one of the predefined luma filter set. For chroma filter, it is possible to choose one temporal filter or signaling one new filter, but it is not possible to switch between chroma filters at CTU level.

In JVET-N0243, alternative luma filter sets are signaled at tile group level, so that more than one luma filter set can be signaled at once per tile group (i.e. they could be put in a same APS). JVET-N0243 also contained some syntax reduction mechanism that enables to further merge filter indexes between successive filter sets. Filters from different filter sets defined at tile group level can be mixed in a CTU specific filter set, through an additional signaling in the CTU data that enables to indicate an alternative luma filter set index for each luma filter index.

In JVET-N0243, alternative chroma filters can be also signaled at tile group level, so that more than one chroma filter can be signaled at once per tile group (i.e. they could be put in the same APS). One alternative chroma filter can be thus selected per chroma component and per CTU through an additional signaling in the CTU data that enables to indicate one alternative chroma filter index per chroma component processed by ALF.

In JVET-N0243, there are no pre-trained fixed filter sets, nor temporal filter sets, which are introduced in VTM5.0 with adoption of JVET-N0415. The following tests would allow clarifying gains of each sub-tool of JVET-N0243 and their potential overlap with adopted JVET-N0415.

Five tests were planned to be performed (according to the CE5 description).

- CE5-4.1 will provide results of mixing different luma filters at CTU level coming from the different filters signaled in the APS(s).

- CE5-4.2 will provide results when enabling the signaling of additional luma filter sets in APS.

- CE5-4.3 will provide combined results of CE5-4.1 and CE5-4.2.

- CE5-4.4 will provide results when enabling alternative chroma filters in VTM5.0 at CTU level.

- CE5-4.5 will provide results of the combination of tests CE5-4.1, CE5-4.2 and CE5-4.4.

In the final CE5-4 results, the proponents of CE5-4 decided to withdraw the CE5-4.1 and CE5-4.3 tests. Accordingly, the tests CE5-4.2, CE5-4.4 and CE5-4.5 were renamed are CE5-4.1, CE5-4.2 and CE5-4.3. The following names and subtests are reported in this summary report.

The proposed modifications are summarized in the following.

CE5-4.1:

- Allow one additional luma filters set in one APS 
(totally up to two luma filter sets in one APS)

- Signal the filter selection from different luma filter sets in one APS for each class at CTB level

CE5-4.2:

- Allow at most seven additional chroma filters in one APS 
(totally up to eight chroma filters in one APS)

- Signal the filter selection from different chroma filters in one APS for each chroma component at CTB level

CE5-4.3:

- The combination of CE5-4.1 + CE5-4.2

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE5-4.1 | VTM5.0 + additional luma filter sets signalled in APS JVET-O0090 | Canon | Nan Hu nanh@qti.qualcomm.com |
| CE5-4.2 | VTM5.0 + alternative chroma filters + CTU chroma filter selection JVET-O0090 | Canon | Nan Hu nanh@qti.qualcomm.com |
| CE5-4.3 | Combination of test 5-4.3 and 5-4.4 JVET-O0090 | Canon | Nan Hu nanh@qti.qualcomm.com |

# Test results

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-1.1 | Sub-PU deblocking and longer tap filter | JVET-O0083 |
| CE5-2.1 | Deblocking for 4xN, Nx4, 8xN and Nx8 blocks | JVET-O0060 |
| CE5-2.2 | CE5.2.1 + chroma deblocking performed on the 4x4 chroma grid | JVET-O0060 |
| CE5-2.3 | Parallel deblocking filter | JVET-O0091 |
| CE5-2.4 | Parallel deblocking filter + chroma deblocking changes as in JVET-N0463 | JVET-O0091 |
| CE5-2.5 | Deblocking chroma on 4x4 chroma grid | JVET-O0092 |
| CE5-3.1 | MV difference threshold in deblocking decisions | JVET-O0061 |
| CE5-4.1 | VTM5.0 + additional luma filter sets signaled in APS | JVET-O0090 |
| CE5-4.2 | VTM5.0 + alternative chroma filters + CTB chroma filter selection | JVET-O0090 |
| CE5-4.3 | Combination of test 5-4.1 and 5-4.2 | JVET-O0090 |

## VTM5.0 CTC (ALF on)

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT |
| CE5-1.1 | -0.01 | 0.00 | 0.00 | 100% | 100% |  | 0.01 | -0.07 |  | -0.03 |  | 100% |  | 100% |  |
| CE5-2.1 | 0.03 | 0.00 | 0.00 | 102% | 100% |  | -0.08 | -0.03 |  | -0.05 |  | 100% |  | 100% |  |
| CE5-2.2 | 0.03 | 0.52 | 0.46 | 100% | 101% |  | -0.09 | 0.82 |  | 0.77 |  | 99% |  | 99% |  |
| CE5-2.3 | -0.06 | -0.14 | -0.13 | 102% | 97% |  | -0.01 | -0.33 |  | -0.41 |  | 100% |  | 101% |  |
| CE5-2.4 | -0.04 | 0.10 | -0.14 | 101% | 100% |  | -0.01 | 0.57 |  | 0.44 |  | 100% |  | 101% |  |
| CE5-2.5 | 0.00 | 0.52 | 0.46 | 103% | 99% |  | 0.01 | 1.02 |  | 0.91 |  | 100% |  | 100% |  |
| CE5-3.1 | - | - | - | - | - |  | 0.06 | 0.01 |  | -0.05 |  | 100% |  | 99% |  |
| CE5-4.1 | 0.01 | 0.00 | -0.01 | 101% | 98% |  | -0.08 | 0.09 |  | 0.06 |  | 101% |  | 101% |  |
| CE5-4.2 | 0.08 | -0.88 | -1.23 | 100% | 99% |  | 0.09 | -1.96 |  | -2.36 |  | 100% |  | 100% |  |
| CE5-4.3 | 0.08 | -0.88 | -1.23 | 100% | 100% |  | 0.01 | -1.91 |  | -2.28 |  | 101% |  | 100% |  |

|  | Low Delay B |  |  |  |  |  |  |  |  | Low Delay P |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y |  | U |  | V |  | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT |
| CE5-1.1 | 0.04 | 0.19 |  | 0.10 |  | 100% |  | 102% |  | 0.03 | 0.14 |  | 0.04 |  | 100% |  | 101% |  |
| CE5-2.1 | -0.11 | 0.08 |  | -0.01 |  | 100% |  | 99% |  | -0.15 | 0.10 |  | 0.21 |  | 102% |  | 102% |  |
| CE5-2.2 | -0.09 | 0.02 |  | -0.19 |  | 97% |  | 101% |  | -0.16 | 0.13 |  | -0.16 |  | 98% |  | 99% |  |
| CE5-2.3 | -0.01 | 0.17 |  | -0.16 |  | 102% |  | 99% |  | -0.04 | 0.10 |  | -0.14 |  | 101% |  | 100% |  |
| CE5-2.4 | 0.00 | 0.19 |  | -0.28 |  | 102% |  | 100% |  | -0.04 | 0.37 |  | -0.25 |  | 101% |  | 99% |  |
| CE5-2.5 | 0.02 | 0.67 |  | 0.02 |  | 102% |  | 97% |  | -0.01 | 0.66 |  | 0.19 |  | 101% |  | 97% |  |
| CE5-3.1 | 0.03 | 0.03 |  | -0.08 |  | 99% |  | 101% |  | -0.02 | -0.02 |  | -0.07 |  | 102% |  | 99% |  |
| CE5-4.1 | -0.15 | 0.28 |  | 0.04 |  | 104% |  | 98% |  | -0.19 | 0.26 |  | 0.04 |  | 105% |  | 100% |  |
| CE5-4.2 | 0.08 | -2.43 |  | -2.78 |  | 101% |  | 100% |  | 0.04 | -2.33 |  | -2.59 |  | 100% |  | 101% |  |
| CE5-4.3 | -0.12 | -2.45 |  | -2.64 |  | 104% |  | 100% |  | -0.17 | -2.51 |  | -2.62 |  | 104% |  | 99% |  |

## VTM5.0 CTC + ALF off

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT |
| CE5-1.1 | -0.01 | 0.00 | 0.00 | 100% | 99% |  | 0.02 | 0.01 |  | -0.08 |  | 100% |  | 99% |  |
| CE5-2.1 | -0.01 | 0.00 | 0.00 | 96% | 101% |  | -0.20 | -0.04 |  | -0.07 |  | 98% |  | 100% |  |
| CE5-2.2 | -0.01 | 0.26 | 0.09 | 100% | 101% |  | -0.19 | 1.40 |  | 1.34 |  | 98% |  | 101% |  |
| CE5-2.3 | -0.03 | -0.10 | -0.08 | 100% | 98% |  | 0.04 | -0.34 |  | -0.46 |  | 100% |  | 100% |  |
| CE5-2.4 | -0.03 | 0.20 | 0.03 | 100% | 98% |  | 0.03 | 1.27 |  | 1.03 |  | 100% |  | 100% |  |
| CE5-2.5 | 0.00 | 0.27 | 0.09 | 100% | 98% |  | 0.01 | 1.91 |  | 1.69 |  | 100% |  | 99% |  |
| CE5-3.1 | - | - | - | - | - |  | 0.02 | 0.03 |  | -0.01 |  | 99% |  | 100% |  |

|  | Low Delay B |  |  |  |  |  |  |  |  | Low Delay P |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y |  | U |  | V |  | EncT |  | DecT | Y |  | U |  | V |  | EncT |  | DecT |
| CE5-1.1 | 0.04 | -0.11 |  | -0.13 |  | 100% |  | 100% |  | 0.07 | 0.08 |  | -0.10 |  | 100% |  | 100% |  |
| CE5-2.1 | -0.17 | -0.10 |  | -0.30 |  | 95% |  | 100% |  | -0.33 | -0.10 |  | -0.03 |  | 96% |  | 100% |  |
| CE5-2.2 | -0.16 | -0.08 |  | -0.37 |  | 100% |  | 100% |  | -0.32 | 0.07 |  | -0.39 |  | 100% |  | 101% |  |
| CE5-2.3 | 0.02 | 0.14 |  | -0.25 |  | 99% |  | 97% |  | 0.03 | -0.01 |  | -0.19 |  | 99% |  | 97% |  |
| CE5-2.4 | 0.01 | 0.94 |  | 0.32 |  | 98% |  | 96% |  | 0.06 | 0.52 |  | -0.20 |  | 99% |  | 95% |  |
| CE5-2.5 | 0.06 | 1.31 |  | 0.60 |  | 99% |  | 95% |  | 0.08 | 1.30 |  | 0.60 |  | 100% |  | 94% |  |
| CE5-3.1 | 0.04 | 0.18 |  | -0.09 |  | 101% |  | 100% |  | -0.12 | -0.02 |  | -0.15 |  | 98% |  | 100% |  |

# Complexity Analysis

## Deblocking filtering complexity

### Luma deblocking complexity

| Tests | Samples from block boundary modified | Samples from block boundary for deblocking decision | Max number of operations for filtering per line (add/mult/compar/shift) | Max number of operations for filtering per *sample* (add/mult/compar/shift) | Max. num. oper., filtering and decsion (Per *Sample*) | Max oper. for decis. per line (add/mult/compar/shift) | Number of line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.1 | All 4x4: 1+1 | All 4x4: 3+3 | All 4x4: 16 (6/2/5/3) | All 4x4: 4 (6/2/5/3)/4 | ~4 + 1.1875= 5.1875 | All 4x4: 4.75 (2.1875/0.5/1.5625/0.5) | same as VTM-5.0 | N |
| CE5-2.2 | All 4x4: 1+1 | All 4x4: 3+3 | All 4x4: 16 (6/2/5/3) | All 4x4: 4 (6/2/5/3)/4 | ~4 + 1.1875= 5.1875 | All 4x4: 4.75 (2.1875/0.5/1.5625/0.5) | same as VTM-5.0 | N |
| CE5-2.3 | All 4x4: 0 Other : same as VTM-5.0 | All 4x4: 0 Other : same as VTM-5.0 | All 4x4: 0 Other : same as VTM-5.0 | All 4x4: 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.4 | All 4x4: 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | All 4x4 : 0 Other : same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.5 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-3.1 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |

### Chroma deblocking complexity

| Tests | samples from block boundary modified | samples from block boundary for deblocking decision | Max number of operations for filtering per line (add/mult/compar/shift) | Max number of operations for filtering per *sample* (add/mult/compar/shift) | Max number of operations including filter and decision per *sample* (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Number of line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1 | longer tap filters on 7 + 7, 3 + 7 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | #### same as VTM-5.0 | same as VTM-5.0 |  | same as VTM-5.0 |
| CE5-2.1 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.2 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.3 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.4 | all 2x2 : 0 same as VTM-5.0 | all 2x2 : 0 same as VTM-5.0 | all 2x2 : 0 same as VTM-5.0 | all 2x2 : 0 same as VTM-5.0 | all 2x2 : 0 same as VTM-5.0 | all 2x2 : 0 same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-2.5 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |
| CE5-3.1 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | same as VTM-5.0 | N |

### Parallel processing (deblocking proposals)

| Tests | Smallest unit size needed to perform proposed deblocking operations separately from any other units |
| --- | --- |
| CE5-1.1 | Same as VTM-5.0 |
| CE5-2.1 | Luma: 4x4, Chroma: 8x8 |
| CE5-2.2 | Luma: 4x4, Chroma: 4x4 |
| CE5-2.3 | Luma: 4x4, Chroma: 4x4 |
| CE5-2.4 | Luma 4x4, Chroma: 2x2(4:2:0)/4x4(4:4:4) |
| CE5-2.5 | Luma 8x8, Chroma: 4x4 |
| CE5-3.1 | Same as VTM-5.0 |

### Decoding complexity on additional sequences, including 4:4:4 content (deblocking proposals)

Decoding times on subjective test sequences in ALF-off configuration have been measured. This allows to evaluate the complexity increase on 4:4:4 content among the proposal in CE5-2. Note that the decoding time may be higher than on average since the high QP (i.e. low bitrate) encodes have been used.

| Sequence | CE5-1.1 | CE5-3.1 |
| --- | --- | --- |
| FoodMarket QP37 |  |  |
| FoodMarket QP32 |  |  |
| Campfire QP37 |  |  |
| Campfire QP32 |  |  |
| Cactus QP37 |  |  |
| Cactus QP32 |  |  |
| KristenAndSara QP37 |  |  |
| KristenAndSara QP32 |  |  |
| Red Kayak* QP37 |  |  |
| Red Kayak* QP32 |  |  |
| Johnny QP37 |  |  |
| Johnny QP32 |  |  |

| Sequence | CE5-2.1 NoALF | CE5-2.2 NoALF | CE5-2.3 NoALF | CE5-2.4 NoALF | CE5-2.5 |
| --- | --- | --- | --- | --- | --- |
| Red Kayak* QP37 |  | 101% |  | 100% |  | 101% |  | 100% |  | 103% |  |
| Red Kayak* QP32 |  | 100% |  | 102% |  | 100% |  | 100% |  | 100% |  |
| Cactus QP37 |  | 102% |  | 101% |  | 104% |  | 104% |  | 101% |  |
| Cactus QP32 |  | 99% |  | 101% |  | 102% |  | 102% |  | 99% |  |
| FourPeople QP37 |  | 102% |  | 102% |  | 105% |  | 105% |  | 103% |  |
| FourPeople QP32 |  | 100% |  | 101% |  | 106% |  | 106% |  | 98% |  |
| KristenAndSara QP37 |  | 98% |  | 101% |  | 102% |  | 102% |  | 102% |  |
| KristenAndSara QP32 |  | 104% |  | 102% |  | 108% |  | 108% |  | 102% |  |
| Kimono 4:4:4 QP37 |  | 99% |  | 101% |  | 99% |  | 98% |  | 96% |  |
| Kimono 4:4:4 QP32 |  | 100% |  | 101% |  | 101% |  | 101% |  | 96% |  |
| Lupo 4:4:4 QP37 |  | 100% |  | 101% |  | 101% |  | 102% |  | 105% |  |
| Lupo 4:4:4 QP32 |  | 104% |  | 99% |  | 106% |  | 107% |  | 104% |  |

## ALF Complexity Analysis

| Test # | Time complexity N = num. classes | Filter sets | Filters storage (bits) | CTB level storage |
| --- | --- | --- | --- | --- |
| VTM5.0 |  |  | 3060 bits |  |
| CE5-4.1 | O (N) | x2 luma filter set | 6060 bits | 25 indices (1 bit each) |
| CE5-4.2 | O (1) | x8 chroma filter | 3480 bits | 2 indices (3 bits each) |
| CE5-4.3 | O (N) | x2 luma filter set + x8 chroma filter | 6480 bits | 25 luma indices (1 bit each) + 2 indices (3 bits each) |

# Cross-checker report

| Test # | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Visual quality observations | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1 | C.-M. Tsai | Y | Y | Y | N | N |  |  |
| CE5-2.1 | H. Jang | Y | N (slightly different 2~3% enc./dec. time) | Y | N | Y | N |  |
| CE5-2.2 | H. Jang | Y | N (slightly different 2~3% enc./dec. time) | Y | N | Y | N |  |
| CE5-2.3 | K. Andersson | Y | N. ~2% increase in decoding time vs Anchor for RA/LD/LDP. | Y | Y. The proposal has many changes to VVC deblocking unrelated to 4x4 grid deblocking. The changes reportedly reduce possibility to remove large block artifacts due to restriction to only apply short filters for large transform blocks when prediction subblocks are used. |  |  |  |
| CE5-2.4 | K. Andersson | Y | N. ~2% increase in decoding time vs Anchor for RA/LD/LDP. | Y | Y. The proposal has many changes to VVC deblocking unrelated to 4x4 grid deblocking. The changes reportedly reduce possibility to remove large block artifacts due to restriction to only apply short filters for large transform blocks when prediction subblocks are used. |  |  |  |
| CE5-2.5 | K. Misra | Y |  | Y | N | N | N/A |  |
| CE5-3.1 | C.-M. Tsai | Y | Y | Y | N | N | N/A |  |

| Test | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-4.1 | N. Hu | Y | Y | Y | N | N |  |
| CE5-4.2 | N. Hu | Y | Y | Y | N | N |  |
| CE5-4.3 | N. Hu | Y | Y | Y | N | N |  |

CE5-2.3 and CE5-2.4 (Cross-checker’s comment):

The proposals have more changes than just applying deblocking to luma on a 4x4 grid. Deblocking filter length in VVC is based on the transform size and then restricted when prediction subblocks are used to enable use of long deblocking filters also when prediction subblocks are used in combination with large transforms. CE5-2.3 and CE5-2.4 instead determine the deblocking filter length based on the distance to closest prediction or transform boundary. This restricts the deblocking filter length for transform block boundaries that are adjacent to prediction subblock boundaries to short deblocking filters which is reportedly insufficient to handle block artifacts from large transforms.

The proposals also have more changes than just applying deblocking for deblocking chroma on a smaller grid. For chroma, VVC only deblocks a boundary of a chroma component if it is intra or if an adjacent block has non-zero transform coefficients when also the block size is at least 8 on each side of the boundary while the proposal deblocks chroma transform boundaries without that size restriction.

# CE5 related contributions

## Deblocking related contributions

JVET-O0158 Non-CE5: Deblocking based on DMVR refined motion (K. Andersson, J. Enhorn)

JVET-O0159 Non-CE5: Deblocking tC table defined for 10-bit video (K. Andersson, J. Enhorn)

JVET-O0287 CE5-related: Simplification of subblock deblocking (C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei))

JVET-O0413 Chroma deblocking filter adjustments for 4:4:4 and 4:2:2 (Y. Morigami, M. Ikeda, T. Suzuki (Sony))

JVET-O0566 Non-CE5: Consistent Deblocking for Chroma Components (J. Xu, J. Wang, L. Zhang, W. Zhu, Y. Wang (Bytedance))

JVET-O0578 Non-CE5: Long-tap deblocking filter on vertical tile boundary (H.Jang, S.Lee, N.Park, J.Nam, S.Kim, J.Lim (LGE))

JVET-O0582 Non-CE5: Deblock filtering process for BDPCM block (H. Jang, J. Nam, S. Kim, J. Lim (LGE))

JVET-O0637 Non-CE5: On chroma line selection for gradient computation in deblocking (K. Misra, P. Cowan, A. Segall (Sharp Labs Of America))

JVET-O0643 Modification on Delta QP for Deblocking (Y. Han, G. Van Der Auwera, M. Coban, M. Karczewicz (Qualcomm))

JVET-O0656 Non-CE5: Boundary strength derivation for transform skipped block (S. Iwamura, S. Nemoto, A. Ichigaya (NHK))

JVET-O0803 Non-CE5: Unified design for longer tap deblocking line buffer reduction (A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei))

JVET-O0494 AHG12: On filtering of independently coded region (R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)) (Also is in ALF related contributions)

JVET-O0142 AHG12: On turning off ALF filtering at brick and slice boundaries (Y.-K. Wang, J. Chen, Hendry (Futurewei)) (Also is in ALF related contributions)

## ALF related contributions

JVET-O0047 Non-CE5: Alternative signalling for ALF clipping parameters (A. M. Kotra, V. Stepin, S. Esenlik, S. Ikonin, B. Wang, H. Gao, E. Alshina (Huawei))

JVET-O0058 Non-CE5: Simplification on ALF clipping parameter coding (S.-C. Lim, J. Kang, H. Lee, J. Lee, H. Y. Kim (ETRI))

JVET-O0064 Non-CE5: Modification of clipping value signalling for adaptive loop filter (N. Hu, H. E. JVET-O0064 Egilmez, V. Seregin, M. Karczewicz (Qualcomm))

JVET-O0067 Non-CE5: Modified coding method for ALF clipping parameters (L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance))

JVET-O0142 AHG12/Non-CE5: On turning off ALF filtering at brick and slice boundaries (Y.-K. Wang, J. Chen, Hendry (Futurewei))

JVET-O0188 Non-CE5: Non-linear ALF simplifications (S. Ikonin, V. Stepin, E. Alshina (Huawei))

JVET-O0216 Non-CE5: Modification of ALF coefficient signalling (A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei))

JVET-O0247 AHG17: Multiple ALF APS for chroma (V. Seregin, N. Hu, M. Coban, M. Karczewicz (Qualcomm))

JVET-O0288 CE5-related: On the syntax constraints of ALF APS (O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek))

JVET-O0289 CE5-related: Redundancy removal in ALF coefficient coding (O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek))

JVET-O0290 CE5-related: Modified syntax design of ALF clipping parameters signalling (O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek))

JVET-O0300 Non-CE5: Clean-up of ALF Syntax Parameters (S. Paluri, J. Lim, S. Kim (LGE))

JVET-O0301 Non-CE5: Simplification of ALF Clipping Coefficients in the APS (S. Paluri, J. Lim, S. Kim (LGE))

JVET-O0302 Non-CE5: Simplification of ALF Coefficients in the APS (S. Paluri, J. Lim, S. Kim (LGE))

JVET-O0374 CE5-related: Context reduction for ALF related syntax element (L. Xu, X. Cao, F. Chen, L. Wang (Hikvision))

JVET-O0386 Non-CE5: A simplification of clipping thresholds for Non-Linear ALF (K. Unno, K. Kawamura, S. Naito (KDDI))

JVET-O0425 Non-CE5: Supplementary results on alternative filter sets for Adaptive Loop Filter (JVET-O0090) (J. Taquet, P. Onno, C. Gisquet, G. Laroche (Canon))

JVET-O0427 Non-CE5: Remove differential coding for ALF coefficients (X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU))

JVET-O0430 Non-CE5: Modification of clipping value signalling for ALF (X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU))

JVET-O0437 Non-CE5: Unification of non-linear ALF luma/chroma clipping parameters (X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU))

JVET-O0499 Non-CE5: Parameter alignment for pipelined ALF (A. Wieckowski, K. Suehring, R. Skupin, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI))

JVET-O0532 Non-CE5: Simplification on clipping value calculation for adaptive loop filter (N. Hu, V. Seregin, M. Karczewicz (Qualcomm))

JVET-O0625 Non-CE5: Unified padding method for samples at variant boundaries in ALF (H. Liu, L. Zhang, K. Zhang, Z. Deng, N. Zhang, J. Xu (Bytedance))

JVET-O0636 Cross-Component Adaptive Loop Filter for chroma, (K. Misra, F. Bossen, A. Segall (Sharp Labs of America))

JVET-O0648 Non-CE5: On Exp-Golomb Coding of ALF Filter Coefficients and Clip Values (H. E. Egilmez, N. Hu, V. Seregin, M. Karczewicz (Qualcomm))

JVET-O0654 AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter (N. Hu, V. Seregin, M. Karczewicz (Qualcomm))

JVET-O0662 AHG12/Non-CE5: Modified ALF filtering for Slice, Brick and Virtual boundaries (A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina (Huawei))

JVET-O0665 Non-CE5: Syntax redundancy removal in adaptive loop filter (X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU))

JVET-O0669 Non-CE5: A Simplification of ALF Coefficient Signalling (H. E. Egilmez, N. Hu, V. Seregin, M. Karczewicz (Qualcomm))

JVET-O0494 AHG12: On filtering of independently coded region (R. Skupin, Y. Sanchez, K. Sühring, T. Schierl (HHI)) (Also is in deblocking related contributions)

JVET-O0142 AHG12: On turning off ALF filtering at brick and slice boundaries (Y.-K. Wang, J. Chen, Hendry (Futurewei)) (Also is in deblocking related contributions)

## Cross-checks of non-CE5 contributions

JVET-O0786 Crosscheck of JVET-O0216 (Non-CE5: Modification of ALF coefficient signalling), (C.-Y. Lai (MediaTek)

JVET-O0807 Crosscheck of JVET-O0582 (Non-CE5 : Deblock filtering process for BDPCM block), (C.-M. Tsai (MediaTek)

JVET-O0809 Crosscheck of JVET-O0803 (Non-CE5: Unified design for longer tap deblocking line buffer reduction), (C.-M. Tsai (MediaTek))

JVET-O0831 Cross-check of JVET-O0288 (CE5-related: On the syntax constraints of ALF APS), (S.-C. Lim, J. Kang, H. Lee, J. Lee (ETRI))

JVET-O0832 Cross-check of JVET-O0290 (CE5-related: Modified syntax design of ALF clipping parameters signalling), (S.-C. Lim, J. Kang, H. Lee, J. Lee (ETRI))

JVET-O0839 Crosscheck of JVET-O0064: Non-CE5: Modification of clipping value signalling for adaptive loop filter, (L. Zhang (Bytedance))

JVET-O0846 Crosscheck of JVET-O0566 (Non-CE5: Consistent Deblocking for Chroma Components), (M. Ikeda (Sony))

JVET-O0847 Crosscheck of JVET-O0656 (Non-CE5: Boundary strength derivation for transform skipped block), (K. Kondo, M. Ikeda (Sony))

JVET-O0858 Crosscheck of JVET-O0654 (AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter), (H. Liu (Bytedance))

JVET-O0865 Cross-check of JVET-O0625: "Non-CE5: Unified padding method for samples at variant boundaries in ALF", (F. Le Léannec (Interdigital))

JVET-O0892 Crosscheck of JVET-O0067 (Non-CE5: Modified coding method for ALF clipping parameters), (N. Hu (Qualcomm))

JVET-O0893 Crosscheck of JVET-O0302 (Non-CE5: Simplification of ALF Coefficients in the APS), (N. Hu (Qualcomm))

JVET-O0894 Crosscheck of JVET-O0499 (Non-CE5: Parameter alignment for pipelined ALF), (N. Hu (Qualcomm))

JVET-O0895 Crosscheck of JVET-O0427 (Non-CE5: Remove differential coding for ALF coefficients), (N. Hu (Qualcomm))

JVET-O0896 Crosscheck of JVET-O0430 (	Non-CE5: Modification of clipping value signalling for ALF), (N. Hu (Qualcomm))

JVET-O0897 Crosscheck of JVET-O0437 (Non-CE5: Unification of non-linear ALF luma/chroma clipping parameters), (N. Hu (Qualcomm))

JVET-O0923 Crosscheck of JVET-O0665 (Non-CE5: Syntax redundancy removal in adaptive loop filter), (S. Ye (Hikvision))

JVET-O0939 Crosscheck of JVET-O0654 (AHG12/Non-CE5: Unification of boundary handling for adaptive loop filter), (S. Paluri (LGE))

JVET-O0950 Crosscheck of JVET-O0578 Non-CE5: Long-tap deblocking filter on vertical tile boundary, (J. Ye, X. Li (Tencent))

JVET-O0983 Crosscheck of JVET-O0532 (Non-CE5: Simplification on clipping value calculation for adaptive loop filter), (K. Unno (KDDI))

JVET-O0985 Crosscheck of JVET-O0425 (Non-CE5: Supplementary results on alternative filter sets for Adaptive Loop Filter), (K. Unno (KDDI))

JVET-O0751 Crosscheck of JVET-O0413 (Chroma deblocking filter adjustments for 4:4:4 and 4:2:2), (T. Chujoh (Sharp))

# References

- “JVET common test conditions and software reference configurations”, JVET-N1010.

- “Algorithm description for Versatile Video Coding and Test Model 3 (VTM3)”, JVET-N1002.