| 11Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0028-v3 |
| --- | --- |

| Title: | CE8: Summary Report on Screen Content Coding Tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s)or Contact(s): | Xiaozhong Xu Yung-Hsuan Chao Yu-Chen Sun Jizheng Xu | Tel: Email: | xiaozhongxu@tencent.com yunghsua@qti.qualcomm.com yuchen.s@alibaba-inc.com xujizheng@bytedance.com |
| Source: | CE8 coordinators |  |  |

Abstract

This contribution provides a summary report of Core Experiment 8 on screen content coding tools. 20 tests have been tested in CE8 in between JVET-N and JVET-O meetings, to study and evaluate technologies related to screen content coding. In this report, coding performance and complexity of these tests are reported and analyzed. In particular, test results against VTM-5.0 anchors (CTC settings) are provided to show the coding efficiency and complexity trade-off of each tool. Some of the tests are also evaluated under low QPs or 444 testing conditions. Crosschecking results for the performed tests are integrated in this contribution.

- Introduction

As is agreed in Geneva meeting, the following tests in table 1 are performed in CE8.

Table 1: Summary of tests performed in CE8

| Test | Tester | Document | Tool description | Cross checker |
| --- | --- | --- | --- | --- |
| CE8-1.1 | S.-T. Hsiang (MediaTek) | JVET-O0053 | Coding BVDs and MVDs | J. Li (Panasonic) |
| CE8-1.2a | M. Salehifar (LGE) | JVET-O0114 | MVD coding with 3 additional context coded bins | Y. Han (Qualcomm) |
| CE8-1.2b | M. Salehifar (LGE) | JVET-O0114 | MVD coding with 1 additional context coded bin | Y. Han (Qualcomm) |
| CE8-1.3a | J. Nam (LGE) | JVET-O0093 | Context modelling and binarization process for BVD coding | Y. Sun (Hikvision) |
| CE8-1.3b | J. Nam (LGE) | JVET-O0093 | Context modeling and binarization process for both BVD and MVD coding | Y. Sun (Hikvision) |
| CE8-1.4 | J. Li (Panasonic) Y. Han (Qualcomm) H. Jang (LGE) | JVET-O0117 | Default IBC mode in case of invalid vector | Y.-W. Chen (Kwai) |
| CE8-1.5a | J. Li (Panasonic) Y. Han (Qualcomm) H. Jang (LGE) | JVET-O0117 | CE8-1.4 + CU based Chroma BV | Y.-W. Chen (Kwai) |
| CE8-1.5b | J. Li (Panasonic) Y. Han (Qualcomm) H. Jang (LGE)H. Jang (LGE) | JVET-O0117 | CE8-1.4 + Subblock based Chroma BV with default BV | Y.-W. Chen (Kwai) |
| CE8-1.6b | J. Xu (ByteDance) | JVET-O0073 | Dedicated IBC buffer with 96x128 in 8-bit precision | X. Xu (Tencent) |
| CE8-1.6c | J. Xu (ByteDance) | JVET-O0073 | Dedicated IBC buffer with 96x128 in 10-bit precision | X. Xu (Tencent) |
| CE8-1.7 | L. Zhang (ByteDance) X. Li (Tencent) S. H. Wang (PKU) | JVET-O0078 | Single HMVP table for all CUs inside the shared merge list region for IBC | C.-C. Chen (MediaTek) |
| CE8-2.1 | Y.-H. Chao (Qualcomm) Y.-C. Sun (Alibaba) W. Zhu (ByteDance) | JVET-O0119 | Palette mode in HEVC SCC + palette flag signaling improvement | J. Lai (MediaTek) |
| CE8-2.2 | Y.-C. Sun (Alibaba) | JVET-O0059 | Palette mode and with neighboring pixel copy combination | W. Zhu (ByteDance) |
| CE8-2.3 | W. Zhu (ByteDance) | JVET-O0071 | Palette PLT mode and IBC combination | Y.-C. Sun (Alibaba) |
| CE8-2.4 | Y.-H. Chao (Qualcomm) | JVET-O0120 | Line-based CG Palette mode | Y.-C. Sun (Alibaba) |
| CE8-2.5 | Y.-C. Sun (Alibaba) W. Zhu (ByteDance) | JVET-O0072 | Combination test of CE8-2.2 and CE8-2.3 | C.-H. Hung (Qualcomm) |
| CE8-3.1a | Santiago de Luxán (HHI) T.-C. Ma (Kwai Inc.) | JVET-O0097 | Enable TS for ISP TUs at the TU level | Y.-H. Chao (Qualcomm) |
| CE8-3.2 | T. Tsukuba, Y. Yagasaki (Sony) | JVET-O0081 | Chroma TS support | T. Nguyen (HHI) |
| CE8-4.2 | Y.-H. Chao (Qualcomm) | JVET-O0121 | Dual tree disabled for YUV4:4:4 format | W. Zhu (ByteDance) |

- Description of CE tools

The detailed description of each tools is provided in this section.

  - CE8-1.1 (JVET-O0053)

In this test, the proposed method for coding the MVDs and BVDs is tested. The proposed method supports to employ an increased number of context-coded bins for entropy coding the prefix part of a binarized absolute value of a BVD or MVD component.

  - CE8-1.2 (JVET-O0114)

In this test, unified MVD and BVD coding method will be evaluated. Considering different characteristics between MVDx and MVDy, as well as existence of bigger values for MVD (also less skewed data) in IBC case, gt_x flags up to 15 will be tested in terms of prefix.

  - CE8-1.3 (JVET-O0093)

In this test, BVD coding methods for only IBC mode are proposed. Especially, syntax elements for block vector coding are used by separating those of motion vector coding. Aadditional context modeling and different binarization methods will be tested. During the plenary discussion, it was desirable that same design is used for MVD and BVD coding. So, the context modeling and binarization method which is applicable to MVD as well as BVD will be also tested.

  - CE8-1.4 (JVET-O0117)

In this test, default processing method which uses a default value of 1 << (intenalBitFDepth – 1) as prediction samples for IBC mode in order to prevent un-defined decoder behavior is tested.

  - CE8-1.5 (JVET-O0117)

In this test, Subblock based BV derivation with default BV are proposed. CU based Chroma BV with default IBC mode in case of invalid vector is tested.

  - CE8-1.6 (JVET-O0073)

In this test, a dedicated IBC reference buffer with lower bit-depth than internal bit-depth is tested. Different buffer sizes and bit-depths will be evaluated to get a trade-off among memory requirement, coding efficiency and decoder check/encoder constrains. Lower QPs will also be used in the test, when the precision of the buffer is low.

  - CE8-1.7 (JVET-O0078)

HMVP buffer update process is modified such that the HMVP buffer will not update with decoded motion or block vectors for IBC when the current block is inside a shared merging candidate list region. In other words, the decoded motion or block vector will not be put into the corresponding HMVP buffer for update if it is inside a shared merging candidate list region.

  - CE8-2.1 (JVET-O0119)

Palette coding as in HEVC SCC.

For dual tree enabled in SPS, palette coding is applied separately for luma tree and chorma tree in intra slice and jointly for luma and chroma in inter slice. For dual tree disabled in SPS, palette coding is applied jointly for both intra and inter slice.

The test also includes the palette flag signaling improvement in JVET-N0258, which was adopted in Geneva meeting to the palette base code.

  - CE8-2.2 (JVET-O0059)

Palette Improvements:

Palette mode and with neighboring pixel copy combination

In this test, the method combining palette mode and neighboring pixel copy is tested. The decoder first derives the prediction block based on the intra prediction information. Then, the decoder decodes a palette and an index map. Using the decoding palette information, the decoder refines the prediction block and reconstructs the block. During the presentation of this CE test during the Marrakech meeting, it was commented that the whole intra prediction process is relatively complicated. To address the comments, in this test, simplified intra prediction (neighboring pixel copy only) will be tested. In the simplified intra prediction (neighboring pixel copy) and palette combination, only horizontal and vertical neighboring pixel copy intra prediction modes are used. The new combined mode is disabled for CUs with small CU size less than 256 in order to reduce implementation complexity. To signal the combined mode, a flag, intra_palette mode is first encoded. If the flag is turned on, a pred_dir flag is then signaled indicating whether horizontal or vertical intra prediction in used. On the decoder side, the decoder first decodes a palette and an index map. A specific index (0) is reserved to indicate the sample is reconstructed from intra prediction. When decoder reconstruct the palette coded block, samples with the index, 0, are reconstructed by the horizontal or vertical nearest samples across CU boundary, and samples with other indices are reconstructed by the colors in the palette. Figure 1 shows an example.

​​​​

(a) vertical neighboring pixel copy combination (b) horizontal neighboring pixel copy combination

Figure 1. An example of palette mode and with horizontal/vertical neighboring pixel copy combination.

  - CE8-2.3 (JVET-O0071)

In this test, a combined IBC/Palette mode is tested. A special palette index indicates that the predictor for the current sample is copied from IBC prediction. Otherwise the sample is from palette predictor.

  - CE8-2.4 (JVET-O0120)

In this CE, a line-based CG palette mode, which simplify the palette mode in HEVC, is tested. Like the coefficient group (CG) used in transform coefficient coding, a CU is divided into multiple segments, where encoding/parsing of index runs, palette index values, and quantized colors are done sequentially. In this test, different context modeling and parsing orders within each line CG will be evaluated.

In this CE, a line-based CG palette mode, which reduces the buffer to store index values, index maps in HEVC palette mode, is tested. A palette mode coded CU is divided into multiple line segments (line coefficient group (CG)) of m samples based on the traverse scan mode, as shown in Figure 2. The coefficient parsing, i.e., parsing the index runs, palette index values, and quantized colors, are done sequentially for each line CG.

The parsing of the palette index run in each line CG is as follows: a context coded bin (index_flag) is parsed for each sample indicating if the sample is of the same run mode as the previously scanned sample, i.e., if the previous scanned sample and the current sample are both of run type COPY_ABOVE or if the previous scanned sample and the current sample are both of run type INDEX with the same index value. If the pixel and the previous sample are of different mode, a context coded bin is parsed indicating the run type, i.e., INDEX or COPY_ABOVE. Same as the HEVC palette, decoder doesn’t have to parse run type if the sample is in the first row (horizontal traverse scan) or in the first column (vertical traverse scan) since the INDEX mode is used by default. Also, decoder doesn’t have to parse run type if the previously parsed run type is COPY_ABOVE.

The index values (for index runs of INDEX mode) and quantized escape colors are bypass coded and grouped apart from encoding/parsing of context coded bins, i.e., index_flag and run_type, to improve throughput within each line CG.

​​

Figure 2: line CG segmentation for horizontal and vertical traverse scans

  - CE8-3.1 (JVET-O0097)

When ISP was proposed for VVC in JVET-M0102, Transform Skip (TS) was restricted to blocks. Since ISP is not applied on 4×4 blocks, there was no interaction between both tools. For this reason, in the current VVC design, Transform Skip (TS) is not allowed in CUs using ISP. The experimental results shown in JVET-N0401 show that it is possible to get a coding gain improvement in classes F and TGM by allowing this interaction without having a negative impact on CTC. It is therefore proposed to enable the transform skip mode on CUs using ISP either at the TU or the CU level.

In the test, the impact of coding efficiency if the transform skip flag is enabled with ISP mode in TU level is tested.

  - CE8-3.2 (JVET-O0081)

In this test, chroma transform skip (TS) is introduced and evaluated. The motivation is to unify TS and MTS signaling between luma and chroma by relocating transform_skip_flag and mts_idx into residual_coding part. One context model is added for chroma transform skip. No context model and no binarization are changed for the mts_idx.

Residual coding semantics

transform_skip_flag[ x0 ][ y0 ][ cIdx ] specifies whether a transform is applied to the associated transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that no transform is applied to the current transform block. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr. transform_skip_flag[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the decision whether transform is applied to the current transform block or not depends on other syntax elements. When transform_skip_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

tu_mts_idx[ x0 ][ y0 ][ cIdx ] specifies which transform kernels are applied to the residual samples along the horizontal and vertical direction of the associated transform block. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb and equal to 2 for Cr.

When tu_mts_idx[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

If ChromaArrayType is larger than 0, tu_mts_idx[ x0 ][ y0 ][ 1] and tu_mts_idx[ x0 ][ y0 ][ 2] shall be equal to 0.

Due to the adoption of JVET-N0193 (CE6: Reduced Secondary Transform (RST) (CE6-3.1)), RST for chroma is possible. Thus, following modification is applied for chroma TS support: when chroma TS is applied, both primary transform and secondary transform for chroma is skipped. numSigCoeff for st_idx is not incremented when chroma TS is applied.

  - CE8-4.2 (JVET-O0121)

In this CE test, disabling dual tree (--DualITree=0) is tested for YUV4:4:4 sequences.

----------------

- CE results summary

Test configurations

Each individual tool under CE8 shall be enabled and tested in all cases (class A~E, F and TGM).

The mandatory common test conditions (CTC) as specified in the JVET-N1010 document [?] [21] shall be followed to perform the simulations in this CE. For CE8-1.6 and CE8-3 related tests, additional results with low QP settings should be provided, as noted earlier in this document.

For test on YUV4:4:4 sequences, additional results with dual tree disabled (--DualITree=0) should be provided for palette related tests and for other CE tests which claim to have benefit disabling dual tree.

Test sequences

The mandatory test sequences (YUV4:2:0 format) as described in JVET-N1010 shall be used for these tests. In addition, the following 4 TGM sequences (YUV4:2:0 format) shall be tested. IBC, BDPCM and HashME are enabled for class TGM.

| Category | Resolution | Sequence name | fps | Bit depth | Frames to be encoded | Intra | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4:2:0 TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 | 8 8 8 8 | 0-299* 0-599 0-599 0-599 | M M M M | M M M M | O O O O |
| *Note that only the first 300 frames of this sequence are used. M refers to mandatory test in the configuration; O refers to optional test in the configuration |  |  |  |  |  |  |  |  |

The following test sequences in YUV4:4:4 format (based on JCTVC-U1015 + AOV4:4:4) shall be used when providing results for YUV4:4:4 tests. IBC, BDPCM and HashME are enabled for all YUV4:4:4 sequences.

| Category | Sequence name | Resolution | Bit depth | Frame rate | Frames to be encoded |
| --- | --- | --- | --- | --- | --- |
| TGM | sc_flyingGraphics_1920x1080_60_8bit* sc_desktop_1920x1080_60_8bit sc_console_1920x1080_60_8bit ChineseEditing_1920x1080_60_8bit | 1920x1080 1920x1080 1920x1080 1920x1080 | 8 8 8 8 | 60 60 60 60 | 0-299* 0-599 0-599 0-599 |
| TGM | sc_web_browsing_1280x720_30_8bit sc_map_1280x720_60_8bit sc_programming_1280x720_60_8bit sc_SlideShow_1280x720_20_8bit | 1280x720 1280x720 1280x720 1280x720 | 8 8 8 8 | 30 60 60 20 | 0-299 0-599 0-599 0-499 |
| A | ArenaOfValor_1920x1080_60_8bit_444 sc_robot_1280x720_30_8bit | 1920x1080 1280x720 | 8 8 | 60 30 | 0-599 0-299 |
| M | Basketball_Screen_2560x1440_60p_8b444* MissionControlClip2_2560x1440_60p_8444* MissionControlClip3_1920x1080_60p_8b444 | 2560x1440 2560x1440 1920x1080 | 8 8 8 | 60 60 60 | 322-621* 120-419* 0-599 |
| CC | EBURainFruits_1920x1080_50_10bit* Kimono1_1920x1080_24_10bit* | 1920x1080 1920x1080 | 10 10 | 50 24 | 0-249* 0-119* |
| *Note that the frames used are different from the original number of frames in the sequence TGM: Text and graphics with motion; M: mixed content; A: animation; CC: camera-captured content |  |  |  |  |  |

YUV4:4:4 Test sequences are available from the following locations (please contact JCT-VC/JVET chairs for login information):

- ArenaOfValor YUV4:4:4: ftp.ient.rwth-aachen.de.

- Other YUV4:4:4 sequences: ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/upload/screen_content/ScExt-TestSequences

When the random access configuration is used, the period for intra random access frames for each test sequence needs to be specified, as follows:

- IntraPeriod: Specifies the intra refresh period in the random access configuration. The intra refresh period is dependent on the frame rate of the test sequence. A value 16 shall be used for test sequences with a frame rate equal to 20fps, 32 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

The followings are summary tables of the tests in this CE.

Table 3: CE8 test results with VTM-5.0 CTC

|  |  | AI Over VTM-5.0 |  |  |  |  | RA Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC | VTM+IBC | -0.62% | -0.62% | -0.65% | 146% | 100% | 0.01% | -0.32% | -0.25% | 107% | 100% |
|  | CE8-1.1 | -0.67% | -0.69% | -0.69% | 149% | 100% | -0.04% | -0.28% | -0.35% | 107% | 98% |
|  | CE8-1.2a | -0.64% | -0.64% | -0.68% | 148% | 103% | -0.01% | -0.31% | -0.23% | 108% | 103% |
|  | CE8-1.2b | -0.64% | -0.62% | -0.67% | 148% | 103% | 0.00% | -0.23% | -0.28% | 108% | 103% |
|  | CE8-1.3a | -0.64% | -0.63% | -0.67% | 147% | 98% | 0.01% | -0.24% | -0.22% | 106% | 100% |
|  | CE8-1.3b | -0.64% | -0.63% | -0.67% | 147% | 98% | 0.17% | -0.18% | -0.07% | 105% | 100% |
|  | CE8-1.4 | -0.62% | -0.62% | -0.65% | 149% | 99% | 0.01% | -0.32% | -0.25% | 107.00% | 99% |
|  | CE8-1.5a | -0.64% | -0.68% | -0.73% | 150% | 99% | 0.00% | -0.38% | -0.29% | 107% | 99% |
|  | CE8-1.5b | -0.64% | -0.67% | -0.70% | 150% | 99% | 0.00% | -0.31% | -0.28% | 106% | 99% |
|  | CE8-1.6b | -0.56% | -0.57% | -0.57% | 151% | 100% | 0.03% | -0.24% | -0.21% | 109% | 100% |
|  | CE8-1.6c | -0.57% | -0.57% | -0.60% | 151% | 98% | 0.03% | -0.25% | -0.21% | 109% | 99% |
|  | CE8-1.7 | -0.62% | -0.59% | -0.64% | 144% | 99% | 0.01% | -0.28% | -0.18% | 105% | 89% |
|  | CE8-2.1 | 0.04% | 0.03% | 0.02% | 108% | 105% | 0.03% | -0.06% | -0.08% | 102% | 102% |
|  | CE8-2.2 | 0.04% | 0.03% | 0.03% | 110% | 108% | 0.03% | 0.00% | -0.06% | 101% | 101% |
|  | CE8-2.3 | 0.04% | 0.03% | 0.04% | 107% | 100% | 0.03% | -0.02% | -0.06% | 103% | 98% |
|  | CE8-2.4 | 0.04% | 0.03% | 0.01% | 112% | 103% | 0.03% | -0.06% | -0.09% | 101% | 101% |
|  | CE8-2.5 | 0.04% | 0.03% | 0.02% | 107% | 99% | 0.03% | -0.02% | -0.05% | 103% | 97% |
|  | CE8-3.1a | 0.00% | 0.00% | 0.02% | 100% | 100% | -0.02% | -0.05% | -0.05% | 100% | 100% |
|  | CE8-3.2 | 0.02% | 0.04% | 0.02% | 101% | 100% | 0.03% | -0.01% | 0.15% | 102% | 100% |
|  | CE8-4.2 |  |  |  |  |  |  |  |  |  |  |
| Class F | CE8-1.1 | -0.50% | -0.29% | -0.41% | 101% | 100% | -0.40% | -0.31% | -0.25% | 101% | 99% |
|  | CE8-1.2a | -0.31% | -0.37% | -0.30% | 102% | 107% | -0.26% | -0.35% | -0.09% | 101% | 104% |
|  | CE8-1.2b | -0.24% | -0.27% | -0.21% | 102% | 106% | -0.20% | -0.17% | -0.10% | 101% | 105% |
|  | CE8-1.3a | -0.34% | -0.24% | -0.28% | 101% | 97% | -0.30% | -0.17% | -0.13% | 101% | 102% |
|  | CE8-1.3b | -0.34% | -0.24% | -0.28% | 101% | 97% | -0.16% | -0.15% | -0.04% | 100% | 101% |
|  | CE8-1.4 | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.00% | 0.00% | 0.00% | 98% | 97% |
|  | CE8-1.5a | -0.04% | -0.07% | -0.06% | 101% | 99% | 0.00% | -0.10% | -0.02% | 98% | 97% |
|  | CE8-1.5b | -0.06% | -0.13% | -0.14% | 101% | 98% | -0.08% | -0.09% | -0.07% | 98% | 97% |
|  | CE8-1.6b | 0.14% | 0.16% | 0.21% | 102% | 101% | 0.13% | 0.15% | 0.27% | 101% | 99% |
|  | CE8-1.6c | 0.13% | 0.07% | 0.08% | 102% | 98% | 0.11% | 0.16% | 0.12% | 101% | 98% |
|  | CE8-1.7 | 0.00% | 0.01% | 0.08% | 100% | 99% | 0.00% | 0.08% | 0.09% | 94% | 83% |
|  | CE8-2.1 | -0.72% | -0.56% | -0.83% | 107% | 108% | -0.55% | -0.83% | -0.77% | 107% | 107% |
|  | CE8-2.2 | -0.75% | -0.76% | -0.75% | 109% | 107% | -0.67% | -0.89% | -0.87% | 104% | 102% |
|  | CE8-2.3 | -0.76% | -0.63% | -0.75% | 106% | 101% | -0.60% | -0.79% | -0.74% | 105% | 99% |
|  | CE8-2.4 | -0.81% | -0.73% | -0.94% | 109% | 107% | -0.65% | -1.01% | -0.76% | 111% | 107% |
|  | CE8-2.5 | -0.78% | -0.76% | -0.82% | 106% | 101% | -0.72% | -0.91% | -0.83% | 106% | 98% |
|  | CE8-3.1a | 0.02% | 0.05% | -0.05% | 101% | 99% | -0.01% | 0.12% | 0.10% | 100% | 101% |
|  | CE8-3.2 | 0.02% | -0.10% | -0.09% | 101% | 100% | 0.05% | -0.09% | -0.11% | 102% | 100% |
|  | CE8-4.2 |  |  |  |  |  |  |  |  |  |  |
| SCC TGM | CE8-1.1 | -1.19% | -1.07% | -1.06% | 100% | 102% | -1.14% | -0.89% | -0.94% | 100% | 100% |
|  | CE8-1.2a | -0.81% | -0.79% | -0.77% | 101% | 105% | -0.82% | -0.74% | -0.75% | 101% | 106% |
|  | CE8-1.2b | -0.67% | -0.61% | -0.63% | 102% | 105% | -0.59% | -0.60% | -0.63% | 102% | 106% |
|  | CE8-1.3a | -0.83% | -0.82% | -0.83% | 99% | 99% | -0.46% | -0.38% | -0.40% | 100% | 100% |
|  | CE8-1.3b | -0.83% | -0.82% | -0.83% | 99% | 99% | -0.38% | -0.38% | -0.41% | 100% | 101% |
|  | CE8-1.4 | 0.00% | 0.00% | 0.00% | 99% | 99% | 0.00% | 0.00% | 0.00% | 98% | 98% |
|  | CE8-1.5a | -0.16% | -0.21% | -0.19% | 98% | 99% | 0.01% | -0.06% | -0.11% | 98% | 97% |
|  | CE8-1.5b | -0.33% | -0.43% | -0.47% | 99% | 98% | -0.14% | -0.20% | -0.27% | 98% | 97% |
|  | CE8-1.6b | 0.68% | 0.71% | 0.71% | 101% | 101% | 0.47% | 0.44% | 0.43% | 103% | 101% |
|  | CE8-1.6c | 0.52% | 0.54% | 0.53% | 101% | 100% | 0.25% | 0.25% | 0.23% | 102% | 102% |
|  | CE8-1.7 | -0.02% | -0.01% | -0.01% | 100% | 101% | -0.07% | -0.03% | -0.08% | 100% | 101% |
|  | CE8-2.1 | -4.29% | -4.53% | -3.89% | 102% | 102% | -1.71% | -2.54% | -2.15% | 102% | 102% |
|  | CE8-2.2 | -5.02% | -4.95% | -4.32% | 103% | 96% | -2.02% | -2.78% | -2.43% | 102% | 101% |
|  | CE8-2.3 | -4.65% | -4.69% | -4.04% | 101% | 98% | -1.84% | -2.56% | -2.20% | 101% | 98% |
|  | CE8-2.4 | -4.35% | -4.70% | -4.04% | 104% | 99% | -1.60% | -2.46% | -2.04% | 104% | 101% |
|  | CE8-2.5 | -5.29% | -5.12% | -4.47% | 103% | 97% | -2.13% | -2.80% | -2.45% | 102% | 98% |
|  | CE8-3.1a | -0.02% | 0.02% | 0.03% | 102% | 102% | -0.04% | 0.01% | -0.09% | 100% | 100% |
|  | CE8-3.2 | 0.02% | -0.57% | -0.85% | 102% | 101% | -0.03% | -0.72% | -0.98% | 103% | 99% |
|  | CE8-4.2 |  |  |  |  |  |  |  |  |  |  |

Table 4: CE8 test results with VTM-5.0 CTC low QP

|  |  |  | AI Over VTM-5.0 |  |  |  |  |  | RA Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| CTC | CE8-1.6b | 0.02% |  | 0.01% | 0.01% | 100% | 98% |  |  |  |  |  |  |
|  | CE8-2.1 | -0.21% |  | -0.36% | -0.43% | 107% | 105% | -0.01% |  | -0.03% | -0.01% | 104% | 107% |
|  | CE8-2.4 | -0.28% |  | -0.42% | -0.47% | 108% | 102% | -0.01% |  | -0.03% | 0.00% | 104% | 106% |
|  | CE8-3.1a | 0.00% |  | 0.00% | 0.01% | 101% | 100% |  |  |  |  |  |  |
|  | CE8-3.2 | 0.03% |  | -0.05% | -0.08% | 102% | 100% | 0.03% |  | -0.10% | -0.08% | 104% | 100% |
| Class F | CE8-1.6b | 0.78% |  | 0.67% | 0.67% | 101% | 100% | 0.85% |  | 0.94% | 0.87% | 100% | 96% |
|  | CE8-2.1 | -0.80% |  | -1.06% | -1.06% | 106% | 102% | -0.69% |  | -0.90% | -1.08% | 102% | 99% |
|  | CE8-2.4 | -1.06% |  | -1.22% | -1.26% | 109% | 103% | -0.90% |  | -1.00% | -1.15% | 103% | 98% |
|  | CE8-3.1a | -0.02% |  | -0.04% | -0.06% | 101% | 103% | -0.07% |  | 0.00% | -0.04% | 100% | 101% |
|  | CE8-3.2 | -0.21% |  | -0.89% | -0.83% | 102% | 100% | -0.19% |  | -1.56% | -1.33% | 103% | 100% |
| SCC TGM | CE8-1.6b | 0.08% |  | 0.87% | 0.82% | 99% | 98% | -1.73% |  | 0.70% | 0.69% | 102% | 100% |
|  | CE8-2.1 | -11.07% |  | -10.22% | -9.79% | 102% | 89% | -5.30% |  | -4.79% | -4.71% | 92% | 96% |
|  | CE8-2.4 | -11.20% |  | -10.66% | -10.25% | 104% | 88% | -6.54% |  | -5.08% | -5.03% | 94% | 95% |
|  | CE8-3.1a | -0.08% |  | -0.03% | -0.02% | 102% | 103% |  |  |  |  |  |  |
|  | CE8-3.2 | -2.21% |  | -4.07% | -4.05% | 103% | 98% | -6.00% |  | -2.88% | -2.95% | 104% | 98% |

Table 5: CE8 test results with VTM-5.0 444 materials

|  |  | AI Over VTM-5.0 |  |  |  |  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Dual Tree ON | CE8-2.1 | -2.13% | -6.81% | -7.13% | 106% | 98% | -1.71% | -6.85% | -6.93% | 100% | 99% |
|  | CE8-2.2 | -2.37% | -7.02% | -7.31% | 110% | 101% | -1.99% | -7.16% | -7.29% | 103% | 100% |
|  | CE8-2.3 | -2.25% | -6.83% | -7.14% | 108% | 100% | -1.82% | -6.93% | -7.01% | 102% | 102% |
|  | CE8-2.4 | -2.34% | -7.25% | -7.45% | 109% | 96% | -1.93% | -7.16% | -7.11% | 102% | 99% |
|  | CE8-2.5 | -2.45% | -7.03% | -7.33% | 109% | 101% | -2.09% | -7.24% | -7.39% | 103% | 103% |
|  | CE8-3.2 | -0.19% | -1.96% | -2.19% | 104% | 99% | -0.43% | -2.68% | -2.77% | 103% | 100% |
|  | CE8-4.2 | -2.55% | -3.12% | -3.15% | 160% | 93% | -1.01% | -1.85% | -1.80% | 108% | 100% |
| Dual Tree OFF | CE8-2.1 | -5.76% | -9.88% | -10.46% | 102% | 97% | -4.15% | -7.90% | -8.62% | 99% | 97% |
|  | CE8-2.2 | -6.19% | -10.40% | -10.98% | 104% | 99% | -4.50% | -8.32% | -9.05% | 98% | 94% |
|  | CE8-2.3 | -6.13% | -10.37% | -10.96% | 104% | 100% | -4.49% | -8.46% | -9.06% | 101% | 102% |
|  | CE8-2.4 | -5.94% | -10.28% | -10.93% | 103% | 95% | -4.24% | -8.47% | -8.96% | 101% | 98% |
|  | CE8-2.5 | -6.47% | -10.85% | -11.39% | 105% | 101% | -4.72% | -8.79% | -9.44% | 102% | 103% |
|  | CE8-3.2 |  |  |  |  |  |  |  |  |  |  |
|  | CE8-4.2 |  |  |  |  |  |  |  |  |  |  |

- Crosschecker reports

Crosscheckers reported that the software/results in CE8 are generally in good shape, no outstanding issues were identified except that some crosscheckers reported that they did not have a good study of the software. Some minor differences were found, which should be due to the identified IBC related bugs in VTM-4.0.

Table 10: crosscheck reports that are associated with the tests in this CE.

| # |  | Cross-checker |  | Software review/studied? (Y/N) |  | Any significant inconsistencies between the description and the provided software? (Y/N) |  | Results available (Full/Partial) |  | Resutls matched? (Y/N) |  | Runtime behavior matched? (Y/N) |  | Additional comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE8-1.1 |  | J. Li |  | Y |  | N |  | Partial |  | Y |  | N |  | xcheck run-time is not reliable |  |
|  |  | (Panasonic) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.2a |  | Y. Han |  |  |  |  |  |  |  | Y |  | N |  | Runtime difference less than 2%. Crosschecker results show less increase. |  |
|  |  | (Qualcomm) |  | Y |  | N |  | Full |  |  |  |  |  |  |  |
| CE8-1.2b |  | Y. Han |  | Y |  | N |  | Full |  | Y |  | N |  | Runtime difference less than 2%. Crosschecker results show less increase. |  |
| CE8-1.3a |  | Y. Sun |  | Y |  | N |  | Full |  | Y |  | Y |  | Runtime difference less than 5% in CTC, less than 1% in SCC sequence |  |
|  |  | (Hikvision) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.3b |  | Y. Sun |  | Y |  | N |  | Full |  | Y |  | Y |  | Runtime difference less than 5% in CTC, less than 1% in SCC sequence |  |
|  |  | (Hikvision) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.3c |  |  |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
| CE8-1.4a |  | Y.-W. Chen |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Kwai) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.5a |  | Y.-W. Chen |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Kwai) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.5b |  | Y.-W. Chen |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Kwai) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.6b |  | X. Xu |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Tencent) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.6c |  | X. Xu |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Tencent) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-1.7 |  | Y. -C. Lin (MediaTek) |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
| CE8-2.1 |  | J. Lai |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (MediaTek) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-2.2 |  | W. Zhu |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (ByteDance) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-2.3 |  | Y.-C. Sun |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Alibaba) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-2.4 |  | Y.-C. Sun |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Alibaba) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-2.5 |  | Y.-H. Chao |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (Qualcomm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-3.1a |  | Y.-H. Chao |  | Y |  | N |  | Full |  | Y |  | N |  | xcheck run-time is not reliable |  |
|  |  | (Qualcomm) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-3.2 |  | T. Nguyen |  | Y |  | N |  | 420 CTC: Y 420 LQP: N 444 CTC: N 444 LQP: N |  | Y |  | N |  | Cross-checkers runtimes not reliable |  |
|  |  | (HHI) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE8-4.2 |  | W. Zhu |  | Y |  | N |  | Full |  | Y |  | Y |  |  |  |
|  |  | (ByteDance) |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Complexity Analysis

To be added by proponents.

- Input contributions for CE8 and CE8 related topics

CE contributions

- JVET-O0053, CE8-1.1: Unified MVD coding for inter and IBC	S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0059, CE8: Palette mode with neighboring pixel copy (CE8-2.2), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba)

- JVET-O0071, CE8-2.3: Compound Palette Mode, W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-O0072, CE8-2.5: Combined test of CE8-2.2 and CE8-2.3, W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu (Bytedance), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba)

- JVET-O0073, CE8-1.6: Dedicated IBC Buffer, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-O0078, CE8-1.7: Single HMVP table for all CUs inside the shared merge list region for IBC, S.H. Wang (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU), L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance), X. Xu, X. Li, G. Li, S. Liu (Tencent)

- JVET-O0081, CE8-3.2: Chroma Transform Skip, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki (Sony)

- JVET-O0093, CE8-1.3: BVD coding method, J. Nam, H. Jang, J. Choi, J. Heo, J. Lim, S. Kim (LGE)

- JVET-O0097, CE8-3.1: Enable Transform Skip in CUs using ISP, S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI), T.-C. Ma, X.-Y. Xiu, Y.-W. Chen, X. Wang (Kwai)

- JVET-O0114, CE8: Unified MVD and Block Vector Difference Coding (CE8-1.2), M. Salehifar, S. Paluri, S. Kim (LGE)

- JVET-O0117, CE8-1.4 and CE8-1.5: Default Processing for IBC Mode, Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm), J. Li, C. Lim (Panasonic), H. Jang, J. Nam, J. Lim (LGE)

- JVET-O0119, CE8-2.1: Palette mode in HEVC, Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu (Bytedance)

- JVET-O0120, CE8-2.4: Line-based CG Palette Mode, Y.-H. Chao, C.-H. Hung, W.-J. Chien, T. Hsieh, M. Karczewicz (Qualcomm)

- JVET-O0121, CE8-4.2: Dual tree disabled for YUV4:4:4 format, Y.-H. Chao, C.-H. Hung, H. Wang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

CE-related and non-CE contributions

The non-CE8/CE8 related contributions are listed with topic-wise categorization as follows:

  - IBC related contributions

- JVET-O0074, CE8-1.6-related: IBC Dedicated Buffer with 120x128 8-bit Setting, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-O0127, CE8-related: IBC modifications, J. Li, C.S. Lim (Panasonic)

- JVET-O0162, Non-CE8: On IBC motion vector list construction, H. Chen, H. Yang (Huawei)

- JVET-O0248, CE8-related: Dedicated IBC reference buffer without bitstream restrictions, H. Gao, A.Karabutov, S. Esenlik, S. Ikonin, B. Wang, A. M. Kotra, E. Alshina (Huawei), ,

- JVET-O0258, Non-CE8: Adaptive single/dual tree with IBC simplification， W. Zhu, J. Xu, L. Zhang (Bytedance)

- JVET-O0325, CE8-related: Simple BVD coding scheme without adding more context, Y. Sun, F. Chen, L. Wang (Hikvision)

- JVET-O0326, CE8-related: JVET-O0325 combined with CE8 BVD coding scheme, Y. Sun, F. Chen, L. Wang (Hikvision)

- JVET-O0327, CE8-related: Default filling method for IBC mode, S. Ye, Y. Sun, F. Chen, L. Wang (Hikvision)

- JVET-O0331, Non-CE8: On IBC BVP Candidate List Derivation, H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)

- JVET-O0442, [Non-CE8] IBC prediction improvement, A. Singh, A. Konda (Samsung)

- JVET-O0443, [Non-CE8] Flipping of reference blocks for Intra Block Copy (IBC), A. Singh, A. Konda (Samsung)

- JVET-O0455, Non-CE8: On IBC merge mode, Y.-J. Chang, Y. Zhang, C.-C. Chen, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-O0480, Non-CE8: On signaling of IBC block vector predictor list size, X. Xu, X. Li, S. Liu (Tencent)

- JVET-O0482, CE8-related: Modulo operations for IBC block vectors, X. Xu, X. Li, S. Liu (Tencent)

- JVET-O0564, CE8-related: Signalling of chroma IBC prediction modes, W. Zhu, L. Zhang, J. Xu, K. Zhang (Bytedance)

- JVET-O0565, CE8-related: Default chroma IBC mode with vertical/ horizontal copying, W. Zhu, J. Xu, L. Zhang (Bytedance)

- JVET-O0568, Non-CE8: An Alternative VPDU Memory Reuse for IBC, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)

- JVET-O0574, Non-CE8: Fixes of IBC BV candidate list, L. Zhang, J. Xu, K. Zhang, H. Liu, W. Zhu (Bytedance)

- JVET-O0602, Non-CE3/Non-CE8: Remove MIP/IBC mode storage from line buffer, H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-O0614, CE8-related: Combination of a simplified IBC mode and the palette mode in CE8-2.5, Y.-C. Sun, X. Zuo, T.-S. Chang, J. Lou (Alibaba)

- JVET-O0626, Non-CE8: Simplified IBC BV candidate list construction process, L. Zhang, K. Zhang, H. Liu, J. Xu, Z. Deng, N. Zhang (Bytedance)

- JVET-O0651, Non-CE8: Intra prediction mode derivation for DM chroma block with corresponding IBC/PCM luma, L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-O0661, Non-CE4/8: Simplification of CIIP on IBC mode, Y. Zhang, C.-C. Chen, Y.-J. Chang, W.-J. Chien, M. Karczewicz (Qualcomm)

- JVET-O0682, CE8-related: An Intra-Affine mode for screen content coding, J. Cao, Z.-R. Li, J. Wang, F. Liang (SYSU), Y.-F. Yu, Y. Liu (OPPO)

- JVET-O0706, CE8-related: 4x4 sub-block based dual-tree chroma IBC block-vector derivation, H. Yang, Y. He (InterDigital)

  - Palette related contributions

- JVET-O0282, CE8-releated: Bug Fix in CE8-2.1, C.-H. Hung, Y.-H. Chao, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)

- JVET-O0259, CE8-related: Palette mode with 8 bits entries, W. Zhu, J. Xu, L. Zhang, H.-C. Chuang (Bytedance)

- JVET-O0600, CE8-related: Palette mode excluding small blocks, H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)

- JVET-O0914, CE8-related: Improved color generation method for palette mode, C.-H. Hung, Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)

  - Transform skip related contributions

- JVET-O0405, Non-CE8: Minimum Allowed QP for Transform Skip Mode, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)

- JVET-O0919, Non-CE8: Minimum QP for Transform Skip Mode, M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban (Qualcomm)

  - BDPCM related contributions

- JVET-O0165, CE8-related: SPS flag for BDPCM. F. Henry (Orange), G. Clare (bcom)

- JVET-O0166, CE8-related: BDPCM for chroma, F. Henry (Orange), G. Clare (bcom)

- JVET-O0200, CE8-related: RDPCM intra mode mapping, maximum RDPCM block size, M. Coban, Y. Han, M. Karczewicz (Qualcomm)

- JVET-O0205, CE8-related: Improvements on BDPCM, S. Yoo, J. Choi, J. Nam, J. Heo, J. Lim, S. Kim (LGE)

- JVET-O0206, CE8-related: Unified condition between BDPCM and transform skip， S. Yoo, J. Nam, J. Choi, J. Lim, S. Kim (LGE)

- JVET-O0296, CE8-related: Intra prediction mode for BDPCM, M.-S. Chiang, O. Chubach, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0315, CE8-related: Intra prediction mode alignment for BDPCM, W. Zhu, L. Zhang, J. Xu (Bytedance)

- JVET-O0342, Non-CE8: Unification of intra prediction mode of BDPCM block, J. Lee, J. Kang, H. Lee, S.-C. Lim, H. Y. Kim (ETRI)

- JVET-O0376, CE8-related: A SPS level flag for BDPCM and JCCR, L. Xu, X. Cao, F. Chen (Hikvision)

- JVET-O0463, Non-CE8: Intra prediction mode of BDPCM, D. Kim, J. Jung, G. Ko, J. Son, J. Kwak (WILUS)

- JVET-O0542, Non-CE8: Modified intra mode associated with BPDCM, X. Zhao, X. Li, X. Xu, S. Liu (Tencent)

  - Other SCC related contributions

- JVET-O0483, AHG11: SCC test sequences for VVC standardization and suggestion for class F update, X. Xu, X. Li, S. Liu (Tencent), A. Tourapis (Apple), J. Xu (ByteDance

- JVET-O0563, Non-CE8: disabling TPM blending, W. Zhu, L. Zhang, J. Xu, K. Zhang (Bytedance)

- JVET-O0597, CE3/8-related: On signaling of PDPC enabling/disabling flag in SPS, X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang (Kwai Inc.)

- JVET-O0645, Non-CE4/8: Blending-off Switch for TPM Mode, C.-C. Chen, Y. Zhang, K. Reuze, Y.-J. Chang, W.-J. Chien2, M. Karczewicz (Qualcomm)

- JVET-O0646, Non-CE4/8: Combination of Blending-off Switch (JVET-O0645) and Simplified Motion Storage (JVET-O0411) for TPM Mode, C.-C. Chen, K. Reuze, Y. Zhang, W.-J. Chien2, M. Karczewicz (Qualcomm)