| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0030-v4 |
| --- | --- |

| Title: | CE10: Summary Report on Neural Network based Filter for Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yiming Li Shan Liu Kei Kawamura | Tel: Email: | userlym@whu.edu.cn shanl@tencent.com ki-kawamura@kddi.com |
| Source: | CE10 coordinators |  |  |

_____________________________

- Abstract

This contribution provides a summary report of Core Experiment 10 on neural network based filter for video coding. 21 tests have been tested in CE10 in between JVET-N and JVET-O meetings, to study and evaluate technologies related to neural network based filter. In this report, coding performance and complexity of these tests are reported and analyzed. Crosschecking results for the performed tests are integrated in this contribution.

Introduction

As is agreed in Geneva meeting, the following tests in table 1 are performed in CE10.

Table 1: Summary of tests performed in CE10

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE10-1.2a | in-loop/ post filter | CNNLF as in-loop filter | JVET-N0110 | Y.-L. Hsiao (Mediatek) | R. Yang (Intel) |
| CE10-1.7a | in-loop/ post filter | NN Filter as in-loop filter | JVET-N0480 | H. Yin (Intel) | Y.-L. Hsiao (Mediatek) |
| CE10-1.7b |  | NN Filter as post filter |  |  | Y.-L. Hsiao (Mediatek) |
| CE10-1.10a | bit-depth | 6-bit(int) weights | JVET-N0712 | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-1.10b |  | 14-bit(int) weights |  |  | H. Yin (Intel) |
| CE10-1.11a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-1.11b |  | CNNF as post filter |  |  | H. Yin (Intel) |

| CE10-2.1a | bit-depth | 32-bit(float) weights | JVET-N0133 | Shuai Wan (NPU) | T. Ouyang (Wuhan Univ) |
| --- | --- | --- | --- | --- | --- |
| CE10-2.1b |  | 8-bit(int) weights |  |  |  |
| CE10-2.2a | in-loop/ post filter | CNNF as in-loop filter |  |  | C.-Y Lin (Hikvision) |
| CE10-2.2b |  | CNNF as post filter |  |  |  |
|  | generalization capability (QP) |  |  |  |  |
| CE10-2.3b |  | CTC QP+5 |  |  | T. Ouyang (Wuhan Univ) |
| CE10-2.5a | bit-depth | 32-bit(float) weights | JVET-N0254 | Y. Wang (Wuhan Univ.) | M. Wang (NPU) |
| CE10-2.5b |  | 8-bit(int) weights |  |  | M. Wang (NPU) |
| CE10-2.7a | generalization capability (QP) | CTC QP-5 |  |  | M. Wang (NPU) |
| CE10-2.7b |  | CTC QP+5 |  |  | M. Wang (NPU) |
| CE10-2.8a | bit-depth | 4-bit(int) weights | JVET-N0513 | Y. Dai(USTC) H. Zhao(USTC) |  |
| CE10-2.8b |  | 6-bit(int) weights |  |  |  |
| CE10-2.8c |  | 8-bit(int) weights |  |  | C.-Y Lin (Hikvision) |
| CE10-2.8d |  | 16-bit(int) weights |  |  |  |
| CE10-2.10a | bit-depth | 8-bit(int) weights | JVET-N0710 | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-2.10b |  | 16-bit(int) weights |  |  | H. Yin (Intel) |
| CE10-2.11a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-2.11b |  | CNNF as post filter |  |  | H. Yin (Intel) |

It comprises 2 categories,

- CE 10.1: Topics on top of sequence-adaptive (two-pass) methods

- CE 10.2: Topics on top of sequence-independent (one-pass) methods

Each category investigated the following problems:

- Test [bit-depth] The impact of the bit-depth of the parameters.

- Test [in-loop/post filtering] The difference between in-loop and post processing filter.

- The generalization capability of the NN filter when the test QP different from the training QP. (The difference should be no smaller than 5)

Description of CE tools

The detailed description of each tools is provided in this section.

CE10-1 Topics on top of sequence-dependent methods

- CE10-1.2 (JVET-O0056)

The proposed CNNLF is applied after the adaptive loop filter (ALF). To reduce the signalling overhead of CNN parameters and CNNLF decoding time, a neural network of only three layers is used and shared by luma and chroma. As shown in Figure 1, the parameters of a layer, MxNxPxQ, are explained as follows.

- M: horizontal spatial filter size in one channel
- N: vertical spatial filter size in one channel
- P: number of input channels
- Q: number of filters in one layer

The operations in each layer include convolution, batch normalization, adding bias, and a rectified linear unit (ReLU) per filter. An exception is for the last layer, where no ReLU is applied.

Figure 1.  The 3-layer CNN shared by luma and chroma

The parameters of CNN are signalled in the I-picture of each RAS of roughly 1 sec. The parameters are trained using only pictures of temporal levels 0 and 1 in the RAS. To improve coding efficiency, when the encoder decides not to apply CNNLF for the I-picture, the CNN parameters are not signalled, and the remaining pictures in this RAS are all set as CNNLF-off. A multi-level on/off control is applied at picture, CTB, and 32x32 block levels for each color component.

- CE10-1.7 (JVET-O0063)

The proposed ACNNLF is applied after the adaptive loop filter (ALF). The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the Figure 2, the 1st layer includes convolution with ReLu, the 2nd layer includes convolution without ReLu;

- Dynamic number of ACNNLFs for luma and dynamic number of ACNNLFs for chroma adaptively trained with video data, the number of ACNNLFs for luma and chroma are signalled in APS (Adaptation Parameter Set);

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the APS and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 2. ACNNLF for luma

The parameters of ACNNLF are signaled in APS. In RA configuration, only the I-frame contains ACNNLF parameters of this RAS. The on/off control is applied at the APS for luma and chroma components, and the acnnlf_luma_frame_enable_flag and acnnlf_chroma_frame_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

- CE10-1.10&1.11 (JVET-O0131)

The weight is used for sum of residual signals by the CNN filter and the signals after deblocking filter. In the introduction, a benefit of an outside control of CNN model is mentioned.

The basic weight is derived from the distance from the boundary. We reuse the boundary definition and its strength derivation from the deblocking filter operation. When the boundary strength is larger than 0, weight is further modified by the block types like intra or inter coding. Figure 3 shows the actual weights for both intra and inter blocks. Thick and thin lines illustrates the block and pixel boundary.

Figure 3. Derived weights for intra (left) and inter (right) blocks when the BS is significant

CE10-2 Topics on top of sequence-independent methods

- CE10-2.1&2.2&2.3 (JVET-O0079)

The proposed CNNLF includes six inputs: three reconstructed components and three auxiliary inputs, i.e. one QPmap and two CUmaps for luma component and chroma component. The overview structure is shown in Figure 4. Due to different components have different partition information, the three components will be processed jointly and separately to fuse its own auxiliary information. The network is shared by luma and chroma so the three components will be filtered simultaneously.

Figure 4. Main structure of the proposed CNNLF

- CE10-2.5&2.7 (JVET-O0101)

The network structure of DRNLF is shown in Figure 6, where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN. So the input has 4 channels in total.

Figure 6. Network structure of DRNLF.

The main body of the proposed network consists of series of DRU. The structure of DRU is shown in Figure 7. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 7. Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied for both intra and inter pictures. Additional flag is signaled for each CTUs to indicate the on/off of the DRNLF.

- CE10-2.8&2.9 (JVET-O0347)

The proposed CNN-based filter is located between DF and SAO. A flag which indicates whether CTU is filtered by NN filter is transmitted to decoder. Residue learning technique is integrated into network to accelerate training. To reduce memory usage, parameters of several layers are shared, which means we can achieve the performance of deeper network with fewer parameters. As shown in Figure 8, layers of the same color represent the same parameters.

Figure 8. The structure of the proposed network

- CE10-2.10&2.11 (JVET-O0132)

The filter structure is shown in Figure 9. This filter has four layers with 3x3 taps. The input of “sum” block is residual signal from the left signal and reconstructed signal just after deblocking filter. The output of “sum” block is filtered pixels. Actual output is weighted sum of after-filtered and before-filtered pixels based on the distance of edge.

Figure 9. The structure of proposed CNNF

Filter coefficients are pre-defined in both the encoder and the decoder so that additional side information is not required. A representation of filter coefficients is fixed point value (integer value). The filter is applied for both intra and inter pictures.

CE results summary

It is agreed that VTM version VTM-5.0 shall be used for these experiments, and the test conditions of mandatory test follows the CTC specified in the JVET-N1010 document. For optional test, only the first intra period should be tested.

Specifically, this CE shall use the following these test conditions:

- Test Condition1: Common Test Conditions (CTC).

- Test Condition2: CTC, but short test, which only need to test the first intra period.

- Test Condition3: Based on Test Condition2, test QP=CTC QP +/- 5

The followings are summary tables of the tests in this CE.

Table 1: CE10 test results with VTM-5.0+ Test Condition 1 (CTC)

|  |  | AI Over VTM-5.0 |  |  |  |  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC Full test | CE10-1.2a |  |  |  |  |  | -1.20% | -14.17% | -14.11% | 100% | 117% |
|  | CE10-1.7a |  |  |  |  |  | -0.80% | -10.56% | -9.93% | 100% | 129% |
|  | CE10-1.7b |  |  |  |  |  | -0.95% | -10.78% | -10.39% | 100% | 158% |
|  | CE10-1.11a |  |  |  |  |  | -0.04% | -15.54% | -15.17% | 99% | 190% |
|  | CE10-1.11b |  |  |  |  |  | 0.25% | -5.04% | -6.01% | 100% | 213% |
|  | CE10-2.2a | -3.93% | -7.89% | -7.29% | 134% | 74517% | -0.26% | -3.86% | -2.91% | 139% | 132513% |
|  | CE10-2.2b | -3.05% | -9.02% | -10.47% | 136% | 74083% | -1.89% | -11.91% | -8.90% | 139% | 135869% |
|  | CE10-2.8c | -0.51% | -0.30% | -0.60% | 92% | 188% |  |  |  |  |  |
|  | CE10-2.11a |  |  |  |  |  | -1.10% | -2.94% | -3.35% | 127% | 56667% |
|  | CE10-2.11b |  |  |  |  |  | -0.32% | -0.66% | -0.81% | 100% | 64536% |

Table 2: CE10 test results with VTM-5.0+ Test Condition 2

|  |  |  |  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# |  | Description | Y | U | V | EncT | DecT |
| Test Condition2 | CE10-1.10a |  | 6-bit(int) weights | 0.04% | -0.97% | -2.30% | 100% | 127% |
|  | CE10-1.10b |  | 14-bit(int) weights | 0.23% | -15.24% | -13.66% | 100% | 190% |
|  | CE10-2.1a |  | 32-bit(float) weights | -0.46% | -4.11% | -2.80% | 142% | 134684% |
|  | CE10-2.1b |  | 8-bit(int) weights | 0.79% | -5.06% | -4.73% | 192% | 306401% |
|  | CE10-2.5a |  | 32-bit(float) weights | -1.01% | -5.01% | -4.24% | 105% | 5652% |
|  | CE10-2.5b |  | 8-bit(int) weights | -0.84% | -4.13% | -2.92% | 107% | 5515% |
|  | CE10-2.8c |  | 8-bit(int) weights |  |  |  |  |  |
|  | CE10-2.10a |  | 8-bit(int) weights | -1.00% | -2.98% | -3.46% | 128% | 57777% |
|  | CE10-2.10b |  | 16-bit(int) weights | -1.16% | -3.21% | -3.59% | 129% | 59072% |

Table 3: CE10 test results with VTM-5.0+ Test Condition 3

|  |  |  |  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# |  | Description | Y | U | V | EncT | DecT |
| Test Condition3 | CE10-2.2a |  | CTC QP for testing | -0.46% | -4.11% | -2.80% | 142% | 134684% |
|  | CE10-2.3b |  | CTC QP+5 | -1.52% | -6.15% | -4.31% | 166% | 154379% |
|  | CE10-2.5a |  | CTC QP for testing | -1.01% | -5.01% | -4.24% | 105% | 5652% |
|  | CE10-2.7a |  | CTC QP-5 | -0.63% | -3.83% | -3.34% | 105% | 5026% |
|  | CE10-2.7b |  | CTC QP+5 | -1.15% | -5.01% | -4.47% | 111% | 5525% |

Network analysis

For the understanding of network, e.g., complexity, the following information is provided.

Table 4: CE10 test results with complexity analysis

| Information in the Inference Stage Combining with Codec |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Details |  |  |  |  |  |  |  |
|  | Total Conv. Layers | Total FC Layers | Framework | Param. Num | Param. Precision | Mem.P(MB) | Mem.T(MB)(e.g., 4K input) |
| CE10-1.2 | 3 | 0 | NA | 1506 | 6(I), 32(I) | 0.001282 | 0.344 (128x128, CTU based) |
| CE10-1.7 | 2 | 0 | NA | 732x3 (Luma), 402x3 (Chroma) | 6(I), 32(I) | 0.0028 | 0.0448 |
| CE10-1.10 | 3 | 0 | Caffe | 1478 | CE10-1.10a: 6(I), 15(I) CE10-1.10b: 14(I), 15(I) | CE10-1.10a: 0.001152 CE10-1.10b: 0.002592 | 0.344 (128x128, CTU based) |
| CE10-2.1 | 39 | 36 | Tensorflow | 519112 | 32(F) | 4.93 | H*W*2.68*10e-3 |
| CE10-2.5 | 21 | 0 | PyTorch | 22371 | CE10-2.5a: 32(F), CE10-2.5b: 8(I) | CE10-2.5a: 0.09, CE10-2.5b: 0.02 | 196.23(block based, 80000pixels) |
| CE10-2.8 | 11 | 0 | cuDNN | 7521 | 8(I)32(I) | 0.01 | 3659(4K as input) |
| CE10-2.10 | 4 | 0 | Caffe | 56481 | CE10-2.10a: 8(I), 15(I) CE10-2.10b: 16(I), 15(I) | CE10-1.10a: 0.056762 CE10-1.10b: 0.112922 | 10.063 (128x128, CTU based) |

Crosschecker reports

Crosscheckers reported that the software/results in CE10 are listed.

Table 5: crosscheck reports that are associated with the tests in this CE.

| # | Tester | Cross-checker | Software review/studied? (Y/N) | Any significant inconsistencies between the description and the provided software? (Y/N) | Results available (Full/Partial) | Resutls matched? (Y/N) | Runtime behavior matched? (Y/N) | Additional comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE10-1.2a | Y.-L. Hsiao (MediaTek) | H. Yin (Intel) | N | N | Full | Y | Y |  |
| CE10-1.7a | H. Yin (Intel) | Y.-L. Hsiao (MediaTek) | N | N | Full | Y | Y |  |
| CE10-1.7b | H. Yin (Intel) | Y.-L. Hsiao (MediaTek) | N | N | Full | Y | Y |  |
| CE10-1.10a | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | N | Y | Only Decoder MD5 result match check, no check for encoder: Only ArenaOfValor QP22 and QP27 got different MD5 |
| CE10-1.10b | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | Y | Y | Only Decoder MD5 result match check, no check for encoder |
| CE10-1.11a | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | N | N | Only Decoder MD5 result match check, no check for encoder: runtime error |
| CE10-1.11b | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | N | Y | Only Decoder MD5 result match check, no check for encoder: Only Tango2 QP27 got different MD5 |
| CE10-2.1a | S. Wan(NPU) | T. Ouyang(Wuhan Univ.) | N | N | Partial | Y | N | 1.Minor different results caused by system inconsistencies; 2. Test on all sequences except ClassA1 and ClassA2 due to running out of memory. |
| CE10-2.1b | S. Wan(NPU) | T. Ouyang(Wuhan Univ.) | N | N | Partial | Y | N | 1. Test on all sequences except ClassA1 and ClassA2 due to running out of memory. |
| CE10-2.2a | M. Wang(NPU) | C. Lin (Hikvision) | N | N | Full | Y | N | Encoding time and decoding time for test are longer. This different time behavior may be related to tensorflow's instruction optimaization (AVX) |
| CE10-2.2b | M. Wang(NPU) | C. Lin (Hikvision) | N | N | Full | Y | N | Encoding time and decoding time for test are longer. This different time behavior may be related to tensorflow's instruction optimaization (AVX) |
| CE10-2.3b | S. Wan(NPU) | T. Ouyang(Wuhan Univ.) | N | N | Partial | Y | N | 1.Minor different results caused by system inconsistencies; 2. Test on all sequences except ClassA1 and ClassA2 due to running out of memory. |
| CE10-2.5a | Y. Wang（Wuhan Univ.） | M. Wang (NPU) | Y | N | Full | Y | Y | Minor difference due to the float operation in different platform |
| CE10-2.5b | Y. Wang（Wuhan Univ.） | M. Wang (NPU) | Y | N | Full | Y | Y | Minor difference due to the float operation in different platform |
| CE10-2.7a | Y. Wang（Wuhan Univ.） | M. Wang (NPU) | Y | N | Full | Y | Y | Minor difference due to the float operation in different platform |
| CE10-2.7b | Y. Wang（Wuhan Univ.） | M. Wang (NPU) | Y | N | Full | Y | Y | Minor difference due to the float operation in different platform |
| CE10-2.8c | H. Zhao (USTC) | C. Lin(Hikvision) |  |  |  |  |  |  |
| CE10-2.10a | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | Y | N | Only Decoder MD5 result match check, no check for encoder |
| CE10-2.10b | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | Y | N | Only Decoder MD5 result match check, no check for encoder |
| CE10-2.11a | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | N | N | Only Decoder MD5 result match check, no check for encoder: runtime error |
| CE10-2.11b | K. Kawamura (KDDI) | H. Yin (Intel) | N | N | Partial | Y | N | Only Decoder MD5 result match check, no check for encoder |

Input contributions for CE10 and CE10 related topics

CE contributions

- JVET-O0056, CE10-1.2: Convolutional neural network loop filter, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)

- JVET-O0063, CE10-1.7: Adaptive convolutional neural network loop filter, H. Yin, R. Yang, X. Fang, S. Ma (Intel)

- JVET-O0079, CE10: Integrated in-loop filter based on CNN (Tests 2.1, 2.2 and 2.3), S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou (NPU), Y.-Z. Ma, J.-Y. Huo (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO)

- JVET-O0101, CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7), Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen(Wuhan Univ.), L. Zhao, S. Liu, X. Li(Tencent)

- JVET-O0131, CE10-1.10/CE10-1.11: Evaluation results of CNN-based filtering with on-line learning model, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)

- JVET-O0132, CE10-2.10/CE10-2.11: Evaluation results of CNN-based filtering with off-line learning model, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)

- JVET-O0347, CE10:Neural Network based Filter for Video Coding, H. Zhao, Y. Dai, D. Liu, N. Yan, H. Li (USTC), X. Chen, H. Yang (Huawei)

CE-related and non-CE contributions

- JVET-O0157, Non-CE10: A CNN based in-loop filter for intra frame, L. Xu, J. Zhu, K. Kazui (Fujitsu)

- JVET-O0348, CE10-Related: Classifier Based Adaptive Convolutional Neural Network Loop Filter, H. Yin, R. Yang, X. Fang, S. Ma (Intel)