| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O0041-v1 |
| --- | --- |

| Title: | AHG17: On signalling of frame field information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Futurewei) 10180 Telesis Court San Diego, CA, 92121, USA Karsten Suehring (HHI) Einsteinufer 37, 10587 Berlin, Germany Chad Fogg (MoveiLabs) | Tel: Email: | +1-908-803-3888 ye-kui.wang@futurewei.com {firstname.lastname}@hhi.fraunhofer.de chadfogg@gmail.com |
| Source: | Futurewei Technologies, Inc., Fraunhofer HHI, MovieLabs |  |  |

_____________________________

# Abstract

This contribution proposes to signal the frame field information using a separate SEI message instead of in the picture timing information SEI message, and it is further proposed that, when field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS.

# Introduction

Currently, the frame field information and picture timing information are both signalled in the picture timing SEI message.

However, the following was observed:

- There are cases wherein only one of the picture timing information and the frame field information is needed.

- When there are multiple layers, the frame field information is typically the same for all the layers while the picture timing information is different. This is in the authors' understanding why during the development of layered HEVC the frame field information SEI message was specified to separately signal the frame field information and the picture timing information.

It is therefore proposed to signal the frame field information using a separate SEI message instead of in the picture timing information SEI message.

# Proposal

The proposed text changes for separately signal the frame field information are provided below.

General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( NalUnitType = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 145 ) |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 168 ) // Same value as for the SEI message in HEVC |  |
| frame_field_info( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| else /* NalUnitType = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 132 ) |  |
| decoded_picture_hash( payloadSize ) |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| frame_field_info_present_flag | u(1) |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| cpb_removal_delay_minus1[ sps_max_sub_layers_minus1 ] | u(v) |
| for( i = TemporalId; i < sps_max_sub_layers_minus1; i++ ) { |  |
| sub_layer_delays_present_flag[ i ] | u(1) |
| if( sub_layer_delays_present_flag[ i ] ) { |  |
| cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| dpb_output_delay | u(v) |
| } |  |
| } |  |

Frame-field information SEI message syntax

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

Picture timing SEI message semantics

The picture timing SEI message provides CPB removal delay and DPB output delay information for the access unit associated with the SEI message.

If nal_hrd_parameters_present_flag or vcl_hrd_parameters_present_flag of the buffering period SEI mesage applicable for the current access unit is equal to 1, the variable CpbDpbDelaysPresentFlag is set equal to 1. Otherwise, CpbDpbDelaysPresentFlag is set equal to 0.

The presence of picture timing SEI messages is specified as follows:

–	If CpbDpbDelaysPresentFlag is equal to 1, a picture timing SEI message shall be associated with the current access unit.

–	Otherwise (CpbDpbDelaysPresentFlag is equal to 0), there may or may not be a picture timing SEI message associated with the current access unit.

NOTE 1 – When CpbDpbDelaysPresentFlag is equal to 0, a picture timing SEI message containing only the frame field information syntax elements pic_struct, source_scan_type, and duplicate_flag may be present.

frame_field_info_present_flag equal to 1 specifies that the syntax elements pic_struct, source_scan_type, and duplicate_flag are present in the picture timing SEI message. frame_field_info_present_flag equal to 0 specifies that the syntax elements pic_struct, source_scan_type, and duplicate_flag are not present in the picture timing SEI message.

pic_struct indicates whether a picture should be displayed as a frame or as one or more fields and, for the display of frames when fixed_pic_rate_within_cvs_flag is equal to 1, may indicate a frame doubling or tripling repetition period for displays that use a fixed frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65. The interpretation of pic_struct is specified in Table D.2. Values of pic_struct that are not listed in Table D.2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore reserved values of pic_struct.

When present, it is a requirement of bitstream conformance that the value of pic_struct shall be constrained such that exactly one of the following conditions is true:

–	The value of pic_struct is equal to 0, 7 or 8 for all pictures in the CVS.

–	The value of pic_struct is equal to 1, 2, 9, 10, 11 or 12 for all pictures in the CVS.

–	The value of pic_struct is equal to 3, 4, 5 or 6 for all pictures in the CVS.

When fixed_pic_rate_within_cvs_flag is equal to 1, frame doubling is indicated by pic_struct equal to 7, which indicates that the frame should be displayed two times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65, and frame tripling is indicated by pic_struct equal to 8, which indicates that the frame should be displayed three times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65.

NOTE 2 – Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2 and 4:4:4 chroma formats are shown in Figure D.1, Figure D.2 and Figure D.3, respectively.

Association indicators for fields (pic_struct equal to 9 through 12) provide hints to associate fields of complementary parity together as frames. The parity of a field can be top or bottom, and the parity of two fields is considered complementary when the parity of one field is top and the parity of the other field is bottom.

When frame_field_info_present_flag is equal to 1, it is a requirement of bitstream conformance that the constraints specified in the third column of Table D.2 shall apply.

NOTE 3 – When frame_field_info_present_flag is equal to 0, then in many cases default values may be inferred or indicated by other means. In the absence of other indications of the intended display type of a picture, the decoder should infer the value of pic_struct as equal to 0 when frame_field_info_present_flag is equal to 0.

source_scan_type equal to 1 indicates that the source scan type of the associated picture should be interpreted as progressive. source_scan_type equal to 0 indicates that the source scan type of the associated picture should be interpreted as interlaced. source_scan_type equal to 2 indicates that the source scan type of the associated picture is unknown or unspecified. source_scan_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall interpret the value 3 for source_scan_type as equivalent to the value 2.

The following applies to the semantics of source_scan_type:

–	If general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the value of source_scan_type shall be equal to 0 when present, and should be inferred to be equal to 0 when not present.

–	Otherwise, if general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 1 when present and should be inferred to be equal to 1 when not present.

–	Otherwise, when general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 2 when present and should be inferred to be equal to 2 when not present.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate of a previous picture in output order.

NOTE 4 – The duplicate_flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other such duplication and picture rate interpolation methods. This flag would commonly be used when a video feed is encoded as a field sequence in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 5 – When field_seq_flag is equal to 1 and duplicate_flag is equal to 1, this should be interpreted as an indication that the access unit contains a duplicated field of the previous field in output order with the same parity as the current field unless a pairing is otherwise indicated by the use of a pic_struct value in the range of 9 to 12, inclusive.

Table D.2 – Interpretation of pic_struct

| Value | Indicated display of picture | Restrictions |
| --- | --- | --- |
| 0 | (progressive) Frame | field_seq_flag shall be equal to 0 |
| 1 | Top field | field_seq_flag shall be equal to 1 |
| 2 | Bottom field | field_seq_flag shall be equal to 1 |
| 3 | Top field, bottom field, in that order | field_seq_flag shall be equal to 0 |
| 4 | Bottom field, top field, in that order | field_seq_flag shall be equal to 0 |
| 5 | Top field, bottom field, top field repeated, in that order | field_seq_flag shall be equal to 0 |
| 6 | Bottom field, top field, bottom field repeated, in that order | field_seq_flag shall be equal to 0 |
| 7 | Frame doubling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 8 | Frame tripling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 9 | Top field paired with previous bottom field in output order | field_seq_flag shall be equal to 1 |
| 10 | Bottom field paired with previous top field in output order | field_seq_flag shall be equal to 1 |
| 11 | Top field paired with next bottom field in output order | field_seq_flag shall be equal to 1 |
| 12 | Bottom field paired with next top field in output order | field_seq_flag shall be equal to 1 |

Figure D.1 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields

Figure D.2 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields

Figure D.3 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields

...

Frame-field information SEI message semantics

The frame-field information SEI message may be used to indicate how the associated picture should be displayed, the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

When field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS.

pic_struct indicates whether a picture should be displayed as a frame or as one or more fields and, for the display of frames when fixed_pic_rate_within_cvs_flag is equal to 1, may indicate a frame doubling or tripling repetition period for displays that use a fixed frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65. The interpretation of pic_struct is specified in Table D.2. Values of pic_struct that are not listed in Table D.2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore reserved values of pic_struct.

When present, it is a requirement of bitstream conformance that the value of pic_struct shall be constrained such that exactly one of the following conditions is true:

–	The value of pic_struct is equal to 0, 7 or 8 for all pictures in the CVS.

–	The value of pic_struct is equal to 1, 2, 9, 10, 11 or 12 for all pictures in the CVS.

–	The value of pic_struct is equal to 3, 4, 5 or 6 for all pictures in the CVS.

When fixed_pic_rate_within_cvs_flag is equal to 1, frame doubling is indicated by pic_struct equal to 7, which indicates that the frame should be displayed two times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65, and frame tripling is indicated by pic_struct equal to 8, which indicates that the frame should be displayed three times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65.

NOTE 2 – Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2 and 4:4:4 chroma formats are shown in Figure D.1, Figure D.2 and Figure D.3, respectively.

Association indicators for fields (pic_struct equal to 9 through 12) provide hints to associate fields of complementary parity together as frames. The parity of a field can be top or bottom, and the parity of two fields is considered complementary when the parity of one field is top and the parity of the other field is bottom.

When frame_field_info_present_flag is equal to 1, it is a requirement of bitstream conformance that the constraints specified in the third column of Table D.2 shall apply.

NOTE 3 – When frame_field_info_present_flag is equal to 0, then in many cases default values may be inferred or indicated by other means. In the absence of other indications of the intended display type of a picture, the decoder should infer the value of pic_struct as equal to 0 when frame_field_info_present_flag is equal to 0.

source_scan_type equal to 1 indicates that the source scan type of the associated picture should be interpreted as progressive. source_scan_type equal to 0 indicates that the source scan type of the associated picture should be interpreted as interlaced. source_scan_type equal to 2 indicates that the source scan type of the associated picture is unknown or unspecified. source_scan_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall interpret the value 3 for source_scan_type as equivalent to the value 2.

The following applies to the semantics of source_scan_type:

–	If general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the value of source_scan_type shall be equal to 0 when present, and should be inferred to be equal to 0 when not present.

–	Otherwise, if general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 1 when present and should be inferred to be equal to 1 when not present.

–	Otherwise, when general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 2 when present and should be inferred to be equal to 2 when not present.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate of a previous picture in output order.

NOTE 4 – The duplicate_flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other such duplication and picture rate interpolation methods. This flag would commonly be used when a video feed is encoded as a field sequence in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 5 – When field_seq_flag is equal to 1 and duplicate_flag is equal to 1, this should be interpreted as an indication that the access unit contains a duplicated field of the previous field in output order with the same parity as the current field unless a pairing is otherwise indicated by the use of a pic_struct value in the range of 9 to 12, inclusive.

Table D.2 – Interpretation of pic_struct

| Value | Indicated display of picture | Restrictions |
| --- | --- | --- |
| 0 | (progressive) Frame | field_seq_flag shall be equal to 0 |
| 1 | Top field | field_seq_flag shall be equal to 1 |
| 2 | Bottom field | field_seq_flag shall be equal to 1 |
| 3 | Top field, bottom field, in that order | field_seq_flag shall be equal to 0 |
| 4 | Bottom field, top field, in that order | field_seq_flag shall be equal to 0 |
| 5 | Top field, bottom field, top field repeated, in that order | field_seq_flag shall be equal to 0 |
| 6 | Bottom field, top field, bottom field repeated, in that order | field_seq_flag shall be equal to 0 |
| 7 | Frame doubling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 8 | Frame tripling | field_seq_flag shall be equal to 0 fixed_pic_rate_within_cvs_flag shall be equal to 1 |
| 9 | Top field paired with previous bottom field in output order | field_seq_flag shall be equal to 1 |
| 10 | Bottom field paired with previous top field in output order | field_seq_flag shall be equal to 1 |
| 11 | Top field paired with next bottom field in output order | field_seq_flag shall be equal to 1 |
| 12 | Bottom field paired with next top field in output order | field_seq_flag shall be equal to 1 |

Figure D.1 – Nominal vertical and horizontal sampling locations of 4:2:0 samples in top and bottom fields

Figure D.2 – Nominal vertical and horizontal sampling locations of 4:2:2 samples in top and bottom fields

Figure D.3 – Nominal vertical and horizontal sampling locations of 4:4:4 samples in top and bottom fields

# Patent rights declaration(s)

Futurewei Technologies, Inc., Fraunhofer HHI, and MovieLabs do not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the contributors).