| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0042 |
| --- | --- |

| Title: | Replicating pic_struct into separate elements for future extensibility |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Fogg | Tel: Email: | chadfogg@gmail.com |
| Source: | MovieLabs |  |  |

_____________________________

# Abstract

This proposal replicates the compound functionality of pic_struct into separate elements (pic_output_period, bottom_pic_flag, field_pairig_idc) to meet the extensibility needs of future video applications. It is not proposed to replace the existing pic_struct in SEI messages inherited from AVC and HEVC, which would be supported for legacy applications and transcoding.

# Background

In 2016, DVB requested the ability to signal frame repetition periods greater than the three currently supported in pic_struct, in order for example to indicate 24 frame/sec film content in 120 Hz constant frame rate display bitstreams, and instead settled on a recommendation of using duplicate_flag to signal coded pictures that are essentially repeats of previous pictures in output order.  Future version of DVB, ATSC, and other system specifications are expected to support frame rates between 240 and 300 Hz, yet 24 frame/sec film content is expected to continue to be mixed within sequences coded for such high target frame rates.

The proposed pic_output_period performs similar function to AV1's frame_presentation_time, and is essentially an explicitly signalled version of the internally derived variable DeltaToDivisor in the current VVC draft [1].  The proposed field_pairing_idc covers the current special pic_struct cases of value 9,10,11, and 12 that were introduced to HEVC for field sequences. The proposed element bottom_pic_flag (from AVC) indicates the parity of the coded field.

These three new elements proposed in this document replicate the functionality of pic_struct in the pic_timing SEI message in the current VVC draft [1] (and the frame_field_info SEI message being proposed in JVET-O0042 at this meeting), as illustrated in the table below:

| pic_struct | Indicated display output of current picture | field_seq_flag | bottom_pic_flag | pic_output_period | field_ pairing_idc |
| --- | --- | --- | --- | --- | --- |
| 0 | (progressive) Frame | 0 | - | 1 | - |
| 1 | Top field | 1 | 0 | 1 | 1 |
| 2 | Bottom field | 1 | 1 | 1 | 1 |
| 3 | Top field, bottom field, in that order | 0 | - | 2 | - |
| 4 | Bottom field, top field, in that order | 0 | - | 2 | - |
| 5 | Top field, bottom field, top field repeated, in that order | 0 | - | 3 | - |
| 6 | Bottom field, top field, bottom field repeated, in that order | 0 | - | 3 | - |
| 7 | Frame doubling | 0 | - | 2 (*) | - |
| 8 | Frame tripling | 0 | - | 3 (*) | - |
| 9 | Top field paired with previous bottom field in output order | 1 | 0 | 1 | 2 |
| 10 | Bottom field paired with previous top field in output order | 1 | 1 | 1 | 2 |
| 11 | Top field paired with next bottom field in output order | 1 | 0 | 1 | 3 |
| 12 | Bottom field paired with next top field in output order | 1 | 1 | 1 | 3 |

(*) fixed_pic_rate_within_cvs_flag is required to be equal to 1.

# Proposal

Add pic_output_period, field_pairing_idc, and bottom_pic_flag to the VVC [1] slice header in clause 7.3.5.1 after the other output-related elements (pic_output_flag, no_output_of_prior_pics_flag..) as shown by the red highlighted text:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period_minus1 | ue(v) |
| if( field_seq_flag == 1 ){ |  |
| bottom_pic_flag | u(1) |
| field_pairing_idc | u(2) |
| } |  |
| if( ( NalUnitType != IDR_W_RADL && NalUnitType != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| [..] |  |
| } |  |

Add the following semantics to clause 7.4.6.1:

pic_output_period_minus1 indicates the number of picture periods that the current decoded picture occupies in the output process. When not present, pic_output_period_minus1 is assumed to be equal to 0. When pic_output_period_minus1 is present in the bitstream, the value DeltaToDivisor is equal to pic_ouput_period_minus1 + 1. If pic_struct is also present in the bitstream, then the value of pic_output_period and pic_struct should have identical DeltaToDivisor derivations.

bottom_pic_flag equal to 1 indicates that the current field picture has bottom parity. bottom_pic_flaq equal to 0 indicates the current field picture has top parity.

field_pairing_idc indicates the pairing of the current field picture according to the following table:

| field_pairing_idc | Interpretation |
| --- | --- |
| 0 | field pairing is not specified |
| 1 | field is not paired |
| 2 | field is paired with previous field opposite to bottom_pic_flag parity in output order |
| 3 | field is paired with next field opposite to bottom_pic_flag parity in output order |

# References

JVET-N1001:  Versatile Video Coding ([working] Draft 5).

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.