| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0042-r2 |
| --- | --- |

| Title: | Replicating pic_struct into separate elements for future extensibility |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Fogg | Tel: Email: | chadfogg@gmail.com |
| Source: | MovieLabs |  |  |

_____________________________

# Abstract

This proposal breaks the compound functionality of pic_struct into separate elements (pic_output_period, bottom_pic_flag, field_paired_with_next_opposite_field_flag, output_fields_from_frame_flag, top_field_first_flag) to meet the extensibility needs of future video applications. It is not proposed to replace the existing pic_struct in SEI messages inherited from AVC and HEVC, which would be supported for legacy applications and transcoding, though equivalent functionality is proposed via the new elements.  Some of the pic_struct modes in HEVC may now be depreciated in industrial use, and therefore functionality may be undesirable to carry forward in a more compact set of new syntax constructs for VVC. Powerpoint slides are also included in version 2 (revision 1) of this proposal. The element pic_output_period is essentially a downloadable DeltaToDivisor that has been derived from pic_struct table in HEVC (and copied into the VVC working draft). The bottom_pic_flag originates from AVC slice header to indicate the current coded field parity (conveyed in pic_struct 1, 2, 9, 10, 11, 12 scenarios). field_paired_with_next_opposite_field_flag identifies whether the previous or next field of opposite parity is a paired with the current field (intended for systems that encourage fields to occur in pairs, i.e. no dangling fields). top_field_first originates from MPEG-2, which indicates the field (top or bottom) of a frame is that is first output (in pic_struct 3, 4, 5, 6 scenarios).  Compared to the 4-bit pic_struct bitstream element, any possible field sequence combination of bottom_pic_flag and field_paired_with_next_opposite_field_flag, or frame sequence combination of output_fields_from_frame_ flag and top_field_first_flag consumes 2 bits.  The effective length of pic_output_period starts at 3-bits if pic_output_period is coded with an unsigned exp-Golumb code, i.e. u(e). Should pic_output_period be specified with a variable length code, or the interpreted picture period of 0 not be desired to be expressed via pic_output_period (such as by changing the bitstream element to pic_output_period_minus1), then the nominal length of pic_output_period (or pic_output_period_minus1) is 1-bit. These new elements can be further conditionally signalled by presence or enabled flags at a higher layer, such as picture parameter set (PPS) or sequence parameter set (SPS).

# Background

In 2016, DVB requested the ability to signal frame repetition periods greater than the three frames currently supported in pic_struct (equal to 8), in order for example to indicate 24 frame/sec film content in 120 Hz constant frame rate display bitstreams. DVB instead settled on a recommendation to use duplicate_flag to signal coded pictures that are essentially repeats of previous decoded pictures in output order.  Future version of DVB, ATSC, and other system specifications are expected to support frame rates between 240 and 300 Hz, yet 24 frame/sec film content is expected to continue to be mixed within sequences coded for such high target frame rates. For example, 24 frame/sec film clips can be spliced or edited into the same bitstream with electronic camera captured content at 120 frame/sec or any capture rate in between such as 60 frames/sec, while all the timing and output rate indicators signal 120 frames/sec. Thus the repetition output process traditionally conveyed by pic_struct (and the derived DeltaToDivisior variable) would effectively fill-in or pad the output sequence of pictures so that no picture gaps are present.

The proposed pic_output_period performs similar function to AV1's frame_presentation_time, and is essentially an explicitly signalled version of the internally derived variable DeltaToDivisor in the current VVC draft [1].  The proposed field_paired_with_next_opposite_field_flag covers the current special pic_struct cases of value 9,10,11, and 12 that were introduced to HEVC for field sequences. The proposed element bottom_pic_flag (from AVC) indicates the parity of the coded field.

These new elements proposed in this document replicate the functionality of pic_struct in the pic_timing SEI message in the current VVC draft [1] (and the frame_field_info SEI message being proposed in JVET-O0041 at this meeting), as illustrated in the table below:

| pic_struct | Indicated Display of Picture | field_seq_flag | output_picture_period | bottom_pic_flag | field_paired_with_next_opposite_field_flag | output_fields_from_frame_flag | output_top_field_first_flag |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (progressive) Frame | 0 | 1 | - | - | 0 | - |
| 1 | Top field | 1 | 1 | 0 | 0 or 1 | - | - |
| 2 | Bottom field | 1 | 1 | 1 | 0 or 1 | - | - |
| 3 | Top field, bottom field, in that order | 0 | 2 | - | - | 1 | 1 |
| 4 | Bottom field, top field, in that order | 0 | 2 | - | - | 1 | 0 |
| 5 | Top field, bottom field, top field repeated, in that order | 0 | 3 | - | - | 1 | 1 |
| 6 | Bottom field, top field, bottom field repeated, in that order | 0 | 3 | - | - | 1 | 0 |
| 7 | Frame doubling | 0 | 2 | - | - | 0 | - |
| 8 | Frame tripling | 0 | 3 | - | - | 0 | - |
| 9 | Top field paired with previous bottom field in output order | 1 | 1 | 0 | 0 | - | - |
| 10 | Bottom field paired with previous top field in output order | 1 | 1 | 1 | 0 | - | - |
| 11 | Top field paired with next bottom field in output order | 1 | 1 | 0 | 1 | - | - |
| 12 | Bottom field paired with next top field in output order | 1 | 1 | 1 | 1 | - | - |
| X | repeat more than 3 pictures |  | > 3 |  |  |  |  |

(*) fixed_pic_rate_within_cvs_flag is required to be equal to 1.

# Proposal

This proposal is organized into sets of syntax and semantic options.

## Essential syntax

The following essential syntax additions to not replicate all the capabilities of the current pic_struct concept in VVC inherited from HEVC.  The picture output period and coded field parity are considered necessary for interpretation of decoder output.

Add pic_output_period and bottom_pic_fiag to the slice header as indicated in the red highlighted text:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period | ue(v) |
| if( field_seq_flag == 1 ) |  |
| bottom_pic_flag | u(1) |
| if( ( NalUnitType != IDR_W_RADL && NalUnitType != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| [..] |  |
| } |  |

Add the following semantics to clause 7.4.6.1:

pic_output_period indicates the number of picture periods that the current decoded picture occupies in the output process. When not present, pic_output_period is assumed to be equal to 1. When pic_output_period  is present in the bitstream, the value DeltaToDivisor is equal to pic_ouput_period. If pic_struct is also present in the bitstream, it shall a DeltaToDivisor derivation equal to pic_output_period. pic_output_period shall be equal to 0 if the coded picture is not intended to be output (when pic_output_flag == 0).

bottom_pic_flag equal to 1 indicates that the current field picture has bottom parity. bottom_pic_flaq equal to 0 indicates the current field picture has top parity.

## Comprehensive syntax

Add pic_output_period, field_pairing_idc, frame_output_idc, and bottom_pic_flag to the VVC [1] slice header in clause 7.3.5.1 after the other output-related elements (pic_output_flag, no_output_of_prior_pics_flag..) as shown by the red highlighted text:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period | ue(v) |
| if( field_seq_flag == 1 ){ |  |
| bottom_pic_flag | u(1) |
| field_paired_with_next opposite_field_flag | u(1) |
| } else { |  |
| output_fields_from_frame_flag | u(1) |
| if( output_fields_from_frame_flag == 1 ) |  |
| output_top_field_first_flag | u(1) |
| } |  |
| if( ( NalUnitType != IDR_W_RADL && NalUnitType != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| [..] |  |
| } |  |

Add the following semantics to clause 7.4.6.1:

pic_output_period indicates the number of picture periods that the current decoded picture occupies in the output process. When not present in the bitstream, pic_output_period is assumed to be equal to 1 for the current picture. When pic_output_period  is present in the bitstream, the value DeltaToDivisor is set equal to pic_ouput_period. If pic_struct is also present in the bitstream from an SEI message, then the value of pic_output_period and pic_struct shall have identical DeltaToDivisor derivations. pic_output_period shall be equal to 0 if the coded frame is not intended to be output.

bottom_pic_flag equal to 1 indicates that the current field picture has bottom parity. bottom_pic_flaq equal to 0 indicates the current field picture has top parity.

field_paired_with_next_opposite_field _flag equal to 1 indicates the current field picture is paired with the next field of opposite parity to bottom_pic_flag in output order. field_paired_with_next_opposite_field_flag equal to 0 indicates that the current field picture is paired with the previous field of opposite parity to bottom_pic_flag in output order.

output_fields_from_frame_flag equal to 1 indicates that the decoded frame is output as two field pictures.  field_output_from_flag_flag equal to 0 indicates that the decoded frame is output as a frame picture.

output_top_field_first_flag equal to 1 indicates the top field of the decoded frame is output first, and the bottom field is output second. output_top_field_first_flag equal to 0 indicates that the bottom field is output first, and the top field is output second.

## Alternative bitstream element lengths for pic_ouput_period

A pre-determined length that accommodates 24 frames/sec content embedded in 300 frames/sec transport, given that ceil( log2 ( 300 / 24 )) = 4 bits in the slice_header in clause 7.3.5.1 of the VVC draft [1]:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| [..] |  |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period | u(4) |
| if( field_seq_flag == 1 ){ |  |
| [..] |  |
| } |  |

Alternatively, a variable length indicated in higher level syntax such as sequence parameter set (SPS) or picture parameter set (PPS).  Here, PPS modification in VVC [1] clause 7.3.5.1 is proposed due to the existing location of the output process-related output_flag_precense_flag:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| pic_output_period_length | u(5) |
| single_tile_in_pic_flag | u(1) |
| [..] |  |
| } |  |

..and then the modified slice header as such:

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| [..] |  |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period | u(v) |
| if( field_seq_flag == 1 ){ |  |
| [..] |  |
| } |  |

With the associated semantics added to clause 7.4.3.4

## Alternative pic_output_period syntax element and semantics

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| [..] |  |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_output_period_minus1 | u(v) |
| if( field_seq_flag == 1 ){ |  |
| [..] |  |
| } |  |

pic_output_period_minus1 indicates the number of picture periods that the current decoded picture occupies in the output process. PicOutputPeriod is equal to pic_output_period_minus1 + 1. When not present in the bitstream, pic_output_period_minus1 is assumed to be equal to 0 for the current picture. When pic_output_period_minus1 is present in the bitstream, the value DeltaToDivisor is set equal to PicOutputPeriod. If pic_struct is also present in the bitstream from an SEI message, then the value of PicOutputPeriod and pic_struct shall have identical DeltaToDivisor derivations. pic_output_period shall be equal to 0 if the coded frame is not intended to be output.

## Alternatives between essential and comprehensive

Out of the identified features of pic_struct, the most important is the parity of the coded field picture (bottom_pic_flag).  The remaining elements in descending rank of importance are: pic_output_period, field_paired_with_next_output_field_flag

## No changes to current VVC draft or JVET-O0041

Although picture output period is essential information for the output process to maintain a constant output picture rate as required by most common applications of MPEG-2, AVC, and HEVC, the coded repeated output frames and fields can be marked by the duplicate_flag = 1 introduced in the pic_timing() SEI message in HEVC, and inherited in the current working draft [1] of VVC.

# References

[1] JVET-N1001:  Versatile Video Coding ([working] Draft 5).

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.