| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0046-v1 |
| --- | --- |

| Title: | AHG16: Changing cu_qp_delta signalling to facilitate VDPU-level processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou Tim Hellman | Email: | brian.heng@broadcom.com minhua.zhou@broadcom.com tim.hellman@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution claims that the existing syntax definition for cu_qp_delta, in which QP delta transmission is delayed until the first transform unit with coded coefficients, inhibits VDPU-level decode processing (64x64 decoder pipeline). It notes that a QP value is necessary for filtering operations, and therefore, for CU sizes larger than 64x64, earlier VDPUs cannot be filtered until cu_qp_delta is received. It claims that this is the only syntax element in the entire standard that introduces such a delay.

The contribution recommends changing the delta QP syntax as follows:

- For any CU larger than 64x64, send the cu_qp_delta syntax with the first transform unit, regardless of whether that first TU has any non-zero CBF or not.

In this way, QP can be determined when a VDPU is parsed, rather than depending on delta QP information carried in future VDPUs. The experimental results revealed that the proposed delta QP signalling method is equally efficient when compared to the current method implemented in VTM5.0.

# Introduction

VVC uses the concept of a 64x64 Virtual Decoder Pipeline Unit (VDPU). In particular, maximum transform sizes, luma/chroma syntax interleaving, etc… have all been designed such that a decoder can process 64x64 blocks (VDPUs) at a time, even for large CU sizes up to 128x128. As was emphasized at that time, enabling the use of a 64x64 pipeline allows for significant cost reductions when compared with the buffering requirements of a 128x128 pipeline.

However, it is asserted that the current delta QP design breaks this 64x64 pipeline concept. Specifically, in VVC the delta QP (cu_qp_delta) is transmitted in the first transform unit (TU) of a quantization group that has coded coefficients (tu_cbf = 1). As shown in Figure 1, for large CUs this means that delta QP might not be sent until the final TU. And yet, the delta QP value sent in that final TU applies to the entire CU.

The lack of a QP value means that earlier TUs in a CU cannot be processed to completion, because QP values are needed for in-loop filtering. Specifically, the deblocking filter requires the QP to determine filtering thresholds.

It is asserted that this hobbles decoder architectures that process at the VDPU level. Workarounds for this problem are possible, but these solutions are awkward, and essentially require buffering an entire CTU of information. With 128x128 CTUs in VVC, this amount of buffering is significant. Therefore, the current delta QP design nullifies much of the cost reductions achieved by enabling 64x64 VDPU processing in in the first place. No other syntax element causes such a processing delay. A preferable algorithm would allow the QP value to be known at the time a VDPU is parsed.

Figure 1: Delta QP transmission in a large CU.

# Proposed Solution

For all CUs larger than 64x64, this contribution proposes to transmit the cu_qp_delta syntax with the first TU, regardless of whether that first TU has any non-zero CBF or not.  For all CUs 64x64 and smaller, the cu_qp_delta syntax is unchanged.

The required change for this modification would be to add the following highlighted conditions to the transform unit syntax table:

if( (cbWidth > 64 ¦ ¦ cbHeight > 64 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) {

if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) {

cu_qp_delta_abs

if( cu_qp_delta_abs )

cu_qp_delta_sign_flag

}

}

Moving the delta QP earlier for large CUs has no impact on predicted QP or on the behavior of quantization groups.  The impact on coding efficiency is as follows:

- For inter-predicted CUs, the CU coded block flag (cu_cbf=0) can be used to indicate when none of TUs are coded. When cu_cbf=0, the cu_qp_delta syntax is not present with either the existing syntax or the proposed syntax above. So, the change above has no impact on inter CU coding efficiency.

- For intra-predicted CUs, the cu_cbf is not used. Therefore, for 128x128, 128x64, and 64x128 intra CUs with no coded TUs, the new approach would send a delta QP value, whereas the old approach would not. However, as will be shown in Section 3, this has almost no impact on overall coding efficiency.

# Experimental Results

The common test conditions cannot be used for testing cu_qp_delta, because the CTC specify a constant QP with delta QP syntax disabled (cu_qp_delta_enabled_flag = 0). In an effort to test the maximum impact of the above change, in the following tests the delta QP syntax was enabled, and delta QP was sent with every CU (down to 4x4 CU level, i.e. setting MaxCuDQPSubdiv to 10). The QP was held constant (i.e. delta QP equal to 0 is signalled), so no specific rate control algorithm was used here. By using the VTM5.0 reference software, the following two tests were carried out:

- Test 1) VTM5.0 with MaxCuDQPSubdiv set equal to 10

- Test 2) Proposed method with MaxCuDQPSubdiv set equal to 10

Table 1 shows the summary BD-rate differences when comparing the proposed method with the current delta QP signalling method implemented in the VTM5.0. On average no BD-rate difference was observed. Tiny bit-rate difference can be found in some of class F sequences in the attached spreadsheet.

Table 2 summarizes the delta QP signalling overhead when setting MaxCuDQPSubdiv to 10 and fixing the delta QP to 0 in VTM5.0. When compared to the VTM5.0 CTC anchor, the average BD-rate overhead is 0.05% in AI, 0.13% in RA and 0.24% in LD_B. respectively.  Note that both the test and reference in Table 2 use unmodified VTM5.0 code (not the proposed method) to illustrate the overall overhead from coding delta QP in VTM5.0.

Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 w/ MaxCuDQPSubdiv = 10 (Test1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 83% | 91% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 93% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 96% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 93% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM5.0 w/ MaxCuDQPSubdiv = 10 (Test1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 94% | 92% |
| Class A2 | 0.00% | 0.00% | 0.00% | 94% | 96% |
| Class B | 0.00% | 0.00% | 0.00% | 95% | 93% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 96% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 90% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 97% | 88% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM5.0 w/ MaxCuDQPSubdiv = 10 (Test1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 87% | 77% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 94% | 93% |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 93% | 106% |

Table 1. VTM5.0 with MaxCuDQPSubdiv 10 (test 1) vs. proposed method (test 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.07% | 107% | 103% |
| Class A2 | 0.03% | 0.04% | -0.01% | 105% | 97% |
| Class B | 0.04% | -0.03% | 0.02% | 102% | 101% |
| Class C | 0.07% | -0.03% | -0.01% | 97% | 86% |
| Class E | 0.09% | -0.03% | -0.01% | 105% | 99% |
| Overall | 0.05% | -0.01% | 0.01% | 103% | 97% |
| Class D | 0.15% | 0.25% | -0.02% | 103% | 94% |
| Class F (optional) | 0.09% | 0.04% | 0.10% | 114% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.04% | 0.14% | 106% | 104% |
| Class A2 | 0.08% | -0.09% | 0.08% | 104% | 100% |
| Class B | 0.14% | 0.00% | -0.07% | 103% | 98% |
| Class C | 0.18% | 0.01% | -0.10% | 101% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | -0.01% | -0.01% | 103% | 99% |
| Class D | 0.32% | -0.05% | 0.25% | 102% | 103% |
| Class F (optional) | 0.13% | -0.03% | 0.10% | 101% | 108% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.27% | -0.12% | 116% | 101% |
| Class C | 0.28% | 0.04% | 0.02% | 115% | 134% |
| Class E | 0.37% | 0.67% | 0.22% | 94% | 93% |
| Overall | 0.24% | 0.29% | 0.01% | 110% | 109% |
| Class D | 0.37% | -0.38% | 0.59% | 96% | 97% |
| Class F (optional) | 0.18% | -0.34% | -0.23% | 112% | 101% |

Table 2. VTM5.0 CTC (anchor) vs. VTM5.0 with MaxCuDQPSubdiv 10 (test1)

# Conclusions

This contribution recommends solving the issue described above so that QP for a VDPU can be determined when parsing that VDPU. It is asserted that this will enable the use of a 64x64 decoder pipeline, without requiring additional delays, buffering, and/or other awkward workarounds to determine the QP for earlier TUs in large CUs. The problem can be solved using the change proposed above, or through some equivalent means.

The authors would like to thank Bytedance for cross-checking.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 4)”, JVET-N1001-v7, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.

# Text changes

In syntax table 7.3.7.10 Transform unit syntax, add the conditions highlighted in yellow.

| if( (cbWidth > 64 ¦ ¦ cbHeight > 64 ¦ ¦ tu_cbf_luma[ x0 ][ y0 ] ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) && treeType != DUAL_TREE_CHROMA ) { |  |
| --- | --- |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_abs | ae(v) |
| if( cu_qp_delta_abs ) |  |
| cu_qp_delta_sign_flag | ae(v) |
| } |  |
| } |  |