| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0047-v1 |
| --- | --- |

| Title: | Non-CE5: Alternative signalling for ALF clipping parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Stepin Viktor Semih Esenlik Sergey Ikonin Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com stepin.victor@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

The current contribution proposes an alternative way of signalling the Adaptive loop filter (ALF) clipping parameters. In VTM-5.0, the clipping parameters are signalled using Kth order Exponential golomb codes.  As the clipping parameter is only an index, whose values can range from 0 to 3 inclusive, it is reported that signalling using Kth order Exponential golomb codes is inefficient and therefore a simpler scheme using fixed length code (or) truncated unary code is proposed.

The objective results with “fixed length code” signalling scheme are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, -0.02%, 0.00%, -0.04% is achieved.

The objective results with “truncated unray code” signalling scheme are as follows: Over VTM5.0 Anchor (AI, RA, LDB, LDP): Luma BD-Rate of -0.02%, -0.03%, -0.01%, -0.04% is achieved.

# Introduction

VTM-5.0 [1] uses non-linear ALF [2], wherein a simple clipping function is used to reduce the impact of neighboring sample values when they are too different when compared to the current sample value which is filtered. The clipping parameters are specified for each ALF filter and one clipping value is signalled per filter coefficient. As a result 12 clipping values can be signalled in the bitstream per Luma filter and up to 6 clipping values for the Chroma filter.  Only 4 fixed values are used for clipping. Therefore the clipping parameter which is signalled in the bitstream is an index which points to one of the 4 fixed values. Moreover the clipping parameters are signalled using Kth order Exponential golomb codes.

As shown in the syntax table (Table 1) below, the syntax elements alf_luma_clip_idx and alf_chroma_clip_idx use the signalling type of uek(v) which is a Kth order Exponential golomb code.

| if( alf_luma_clip_flag ) { |  |
| --- | --- |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| if( alf_chroma_clip_flag ) { |  |
| --- | --- |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) |
| } |  |
| } |  |

Table 1.  alf_luma_clip_idx and alf_chroma_clip_idx are signalled using Kth order Exponential golomb codes in VTM-5.0

# Proposed Solution

Since the range of values of the syntax element alf_luma_clip_idx and alf_chroma_clip_idx  is 0 to 3 inclusive, it is reported that signalling using Kth order Exponential golomb codes is inefficient and therefore the current proposal proposes to use a simple and more efficient signalling scheme such as “fixed length code of 2 bits” (or) “truncated unary code”. The syntax table with fixed length code is as shown in table 2 below. The syntax elements alf_luma_clip_min_eg_order_minus1, alf_luma_clip_eg_order_increase_flag[ i ], alf_chroma_clip_min_eg_order_minus1, alf_chroma_clip_eg_order_increase_flag[ i ] need not be signalled any more and the syntax elements alf_luma_clip_idx and alf_chroma_clip_idx use the signalling type of u(2) which is a fixed length code of 2 bits.

| if( alf_luma_clip_flag ) { |  |
| --- | --- |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| if( alf_chroma_clip_flag ) { |  |
| --- | --- |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | u(2) |
| } |  |
| } |  |

Table 2.  alf_luma_clip_idx and alf_chroma_clip_idx are signalled using fixed length code of two bits

Table 3 below shows the syntax using truncated unary code.  The maximum value of the truncated unary binarization tu(v) is set equal to 3.

| if( alf_luma_clip_flag ) { |  |
| --- | --- |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | tu(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

| if( alf_chroma_clip_flag ) { |  |
| --- | --- |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | tu(v) |
| } |  |
| } |  |

Table 3. alf_luma_clip_idx and alf_chroma_clip_idx are signalled truncated unary code with maxVal=3

The advantage of using fixed length code or truncated unary code for signalling the clipping parameters is that, some of the unnecessary syntax elements related to Kth order Exponential golomb codes are not signalled anymore and the number of bits required to signal the “alf clipping parameters” is reduced both in the average and worst case scenarios.

# Average and worst case bit rate reduction per filter set

For a given filter set, the number of filter classes for Luma are 25 and the Max. number of Luma filter coefficients per filter class is 12. The number of Chroma coefficients per filter set is 6, as there is only one Chroma filter signalled per filter set. Therefore in current VTM-5.0, assuming in average case that the 1st order exponential golomb code is selected to be used for signaling clipping parameters,  then the total number of bits required to signal clipping parameters is around 924 bits, whereas with the fixed length coding scheme the number of bits required to signal the clipping parameters is around 612 bits, which gives around 30% bit-rate reduction. In the worst case, when more filter sets are transmitted in the adaptation parameter set (APS), then the bit-rate reduction by using fixed length code is reported to be even more substantial.

# Simulation Results

The proposed signalling scheme is implemented on top of VTM-5.0 [1] and the simulation is performed under common test conditions as described in JVET-N1010 [3].

## Results with fixed length code (2 bits)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class E | -0.02% | -0.01% | -0.02% | 100% | 99% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.02% | 0.01% | -0.02% | 100% | 101% |
| Class F | -0.02% | -0.01% | -0.01% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | -0.04% | 100% | 100% |
| Class A2 | -0.01% | -0.05% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Class C | -0.04% | -0.11% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.05% | 100% | 100% |
| Class D | -0.04% | 0.00% | 0.04% | 100% | 100% |
| Class F | -0.03% | 0.05% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.21% | 0.02% | 101% | 103% |
| Class C | 0.03% | 0.10% | -0.21% | 100% | 100% |
| Class E | -0.02% | 0.39% | -0.30% | 100% | 100% |
| Overall | 0.00% | 0.22% | -0.14% | 100% | 101% |
| Class D | -0.07% | -0.55% | 1.51% | 100% | 100% |
| Class F | 0.04% | 0.05% | 0.82% | 101% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.12% | 0.12% | 100% | 102% |
| Class C | -0.01% | -0.05% | 0.09% | 100% | 99% |
| Class E | -0.08% | 0.27% | -0.64% | 100% | 101% |
| Overall | -0.04% | 0.10% | -0.08% | 100% | 101% |
| Class D | -0.01% | -0.41% | -0.13% | 101% | 100% |
| Class F | 0.09% | 0.35% | -0.31% | 100% | 101% |

## Results with truncated unary coding

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Class B | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Class C | -0.03% | -0.01% | -0.01% | 101% | 100% |
| Class E | -0.03% | -0.01% | -0.02% | 101% | 100% |
| Overall | -0.02% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.03% | 0.01% | -0.01% | 101% | 99% |
| Class F | -0.01% | 0.00% | 0.00% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.21% | 0.04% | 100% | 101% |
| Class A2 | -0.04% | 0.05% | 0.01% | 100% | 102% |
| Class B | -0.01% | 0.00% | -0.05% | 100% | 101% |
| Class C | -0.06% | -0.08% | -0.10% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.06% | -0.03% | 100% | 101% |
| Class D | -0.03% | 0.02% | 0.08% | 100% | 101% |
| Class F | -0.02% | 0.06% | 0.07% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.18% | 0.09% | 100% | 106% |
| Class C | -0.04% | -0.25% | -0.27% | 100% | 101% |
| Class E | 0.04% | 0.11% | -0.13% | 100% | 101% |
| Overall | -0.01% | 0.02% | -0.08% | 100% | 103% |
| Class D | -0.06% | -0.14% | 0.31% | 100% | 104% |
| Class F | 0.03% | 0.18% | -0.44% | 101% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.08% | 0.04% | 100% | 102% |
| Class C | -0.01% | -0.34% | -0.06% | 100% | 103% |
| Class E | -0.06% | 0.35% | -0.21% | 101% | 104% |
| Overall | -0.04% | 0.01% | -0.06% | 100% | 103% |
| Class D | 0.02% | -0.07% | -0.31% | 100% | 116% |
| Class F | -0.02% | -0.24% | 0.07% | 101% | 104% |

# Conclusion

An alternative way of signalling ALF clipping parameters is proposed. It is reported that using fixed length code (2 bits) or truncated unary code to signal the clipping parameters is more efficient than using kth order exponential golomb codes. Objective results show that a minor Luma BD-rate gain is achieved with slightly reduced encoding and decoding complexity. It is suggested to adopt the fixed length coding scheme (2 bits) for signalling the ALF clipping parameters. Specification text changes are attached along with the contribution.

# References

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear adaptive loop filter, JVET-N0242, J. Taquet, P. Onno, C. Gisquet, G. Laroche

- “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).