| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0048-v1 |
| --- | --- |

| Title: | CE3: Simplification on Intra sub-partition coding mode (tests 3-1.2.1 and 3-1.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | B. Wang, A. M. Kotra, S. Esenlik H. Gao, E. Alshina (Huawei) | Tel: Email: | biao.wang@huawei.com, |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

Intra sub-partition (ISP) coding mode can result into 1xN (N>=16) or 2xM (M>=8) sub-partitions. These shapes can lead to inefficient memory access. This contribution provides two alternative restrictions, i.e. CE3-1.2.1 and CE3-1.2.2. In CE3-1.2.1, partitions that could result into 1xN and 2xM sub-partitions are prohibited. While in CE3-1.2.2, only the partitions that could result into 1xN sub-partitions are disallowed. The two restrictions makes memory access more efficient for processing ISP sub-partitions. Compared to VTM5.0 with all intra configuration, CE3-1.2.1 and CE3-1.2.2 bring coding change of 0.07% and 0.01%, respectively. The encoding and decoding time is respectively reduced to 98% and 99% for CE3-1.2.1, and for CE3-1.2.2 the runtime is reduced to 99% for both encoding and decoding sides.

# Problem Definition

In ISP coding tool, 4x4 luma block is not allowed to be split. 4x8 and 8x4 luma block can split into two sub-blocks, each with 16 samples. All other luma blocks can split into 4 sub-blocks. The splitting can be either horizontal or vertical. The vertical splitting for small blocks could results into sub-partitions with size 1xN (N>=16) and 2xM (M>=8).  For example, when a luma intra block is 4x16 and has a vertical splitting. The four sub-partition size is 1x16, as shown in Fig. 1.

Such a sub-partition is not friendly to hardware implementation. In hardware, data are generally stored row by row in memory. When data is fetched, memory will return all data in one row, regardless how much data in a row is effectively used. The size of the data return in a row is usually a number equals to power of two (i.e. 8, 16, etc.). Suppose samples are fetched in a row with 8 samples, a block size with 1x16 would require 16 memory fetches, while in each fetch only 1 sample out of 8 is effectively used. As a consequence, a block with size 1x16 will only achieve 1/8 (i.e. 12.5%) of available memory bandwidth. When the memory burst size is larger (e.g N > 8), the ratio of achieved memory bandwidth to available memory bandwidth would even lower.

Figure 1: 1x16 sub-blocks from vertical splitting of a 4x16 block

# Proposed Solutions

In this contribution, two alternative solutions are provided, corresponding to two sub-CE tests.

The first alternative (i.e. CE3-1.2.1) prohibits the partitions that could result into 1xN or 2xM sub-partitions. Furthermore, for 8xN blocks, vertical splitting is allowed but the number of the sub-block is restricted to 2. Therefore, the sub-block size from 8xN vertical splitting will be 4xN instead of 2xN.

On the purpose of comparison, the original rule of splitting for different block sizes in ISP is listed in Table 1, while the splitting rule of CE3-1.2.1 is listed in Table 2. The change of splitting rule to original ISP coding tool is highlighted in boldface.

Table 1: original rule of splitting in ISP

| Block Size | Number of Sub-Partitions | Partition direction |
| --- | --- | --- |
| 4×4 | Not divided |  |
| 4×8 | 2 | Allow both horizontal and vertical |
| 4×N (N>8) | 4 | Allow both horizontal and vertical |
| 8×4 | 2 | Allow both horizontal and vertical |
| 8×N (N>4) | 4 | Allow both horizontal and vertical |
| All other cases allowed in ISP | 4 | Allow both horizontal and vertical |

Table 2: proposed rule of splitting to prevent 1xN and 2xM sub-partitions in CE3-1.2.1

| Block Size | Number of Sub-Partitions | Partition direction |
| --- | --- | --- |
| 4×4 | Not divided |  |
| 4×8 | 2 | Allow only horizontal partition |
| 4×N (N>8) | 4 | Allow only horizontal partition |
| 8×4 | 2 | Allow both horizontal and vertical |
| 8×N (N>4) | 4 | Allow only horizontal partition |
| 8×N (N>4) | 2 | Allow only vertical partition |
| All other cases allowed in ISP | 4 | Allow both horizontal and vertical partition |

The second alternative (i.e. CE3-1.2.2) prohibits the partitions that could result into 1xN sub-partitions only. The splitting rule of CE3-1.2.2 is listed in Table 3. The change of splitting rule to original ISP coding tool is highlighted in boldface.

Table 3: proposed rule of splitting to prevent 1xN sub-partitions in CE3-1.2.2

| Block Size | Number of Sub-Partitions | Partition direction |
| --- | --- | --- |
| 4×4 | Not divided |  |
| 4×8 | 2 | Allow both horizontal and vertical |
| 4×N (N>8) | 4 | Allow only horizontal partition |
| 8×4 | 2 | Allow both horizontal and vertical |
| 8×N (N>4) | 4 | Allow both horizontal and vertical |
| All other cases allowed in ISP | 4 | Allow both horizontal and vertical partition |

# Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [1]. AI and RA results are reported.

## Simulation result for CE3-1.2.1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.02% | 98% | 100% |
| Class A2 | 0.02% | -0.10% | -0.05% | 99% | 99% |
| Class B | 0.03% | 0.01% | 0.10% | 99% | 99% |
| Class C | 0.10% | 0.05% | 0.05% | 98% | 98% |
| Class E | 0.21% | 0.19% | 0.19% | 98% | 98% |
| Overall | 0.07% | 0.02% | 0.07% | 98% | 99% |
| Class D | 0.13% | 0.27% | 0.16% | 98% | 96% |
| Class F | 0.13% | 0.09% | 0.12% | 97% | 98% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.34% | 0.04% | 99% | 99% |
| Class A2 | 0.05% | 0.00% | 0.06% | 99% | 99% |
| Class B | 0.04% | -0.04% | -0.20% | 99% | 98% |
| Class C | 0.00% | -0.19% | -0.15% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.13% | -0.09% | 99% | 98% |
| Class D | 0.07% | -0.02% | 0.10% | 98% | 96% |
| Class F | 0.08% | 0.03% | 0.15% | 98% | 97% |

## Simulation result for CE3-1.2.2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.04% | 99% | 100% |
| Class A2 | -0.01% | -0.07% | -0.02% | 99% | 100% |
| Class B | 0.00% | -0.06% | 0.03% | 99% | 98% |
| Class C | 0.02% | -0.02% | 0.09% | 99% | 100% |
| Class E | 0.04% | 0.00% | -0.03% | 99% | 98% |
| Overall | 0.01% | -0.02% | 0.03% | 99% | 99% |
| Class D | 0.02% | 0.22% | -0.07% | 99% | 95% |
| Class F | 0.03% | 0.06% | 0.07% | 97% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.27% | 0.09% | 99% | 99% |
| Class A2 | 0.02% | 0.05% | -0.03% | 99% | 99% |
| Class B | 0.02% | -0.02% | -0.14% | 99% | 98% |
| Class C | 0.00% | -0.08% | -0.04% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.07% | -0.04% | 99% | 98% |
| Class D | 0.04% | 0.14% | -0.08% | 99% | 97% |
| Class F | -0.01% | 0.01% | 0.02% | 99% | 98% |

# Conclusion

The current contribution document reports the results of CE3-1.2.1 and CE3-1.2.2 as defined in [2]. Test CE3-1.2.1 prevents partitions that would result into 1xN or 2xM sub-partitions. Test CE3-1.2.2 only prohibits partitions that would result into 1xN sub-partitions. Both solutions make the process of sub-partition in ISP more hardware friendly. Compared to VTM5.0 in AI configuration, the decoding time is reduced to 99% and marginal coding change of 0.13% and 0.03% are observed for CE3-1.2.1 and CE3-1.2.2, respectively.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Draft Specification Changes on top of [3]

## Changes of CE3-1.2.1

7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| … |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY && cbWidth != 4) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |

7.4.8.5 Coding unit semantics

…

…

…

Table 7-8 – Name association to IntraSubPartitionsSplitType

| IntraSubPartitionsSplitType | Name of IntraSubPartitionsSplitType |
| --- | --- |
| 0 | ISP_NO_SPLIT |
| 1 | ISP_HOR_SPLIT |
| 2 | ISP_VER_SPLIT |

The variable NumIntraSubPartitions specifies the number of transform block subpartitions an intra luma coding block is divided into. NumIntraSubPartitions is derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, NumIntraSubPartitions is set equal to 1.

- Otherwise, if one of the following conditions is true, NumIntraSubPartitions is set equal to 2:

  - cbWidth is equal to 4 and cbHeight is equal to 8 and IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT,

  - cbWidth is equal to 8 and cbHeight is equal to 4.

  - cbWidth is equal to 8 and cbHeight is not equal to 4 and IntraSubPartitionsSplitType is equal  to ISP_VER_SPLIT

- Otherwise, NumIntraSubPartitions is set equal to 4.

## Changes of CE3-1.2.2

7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| … |  |
| … |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY && (cbWidth != 4 ¦¦ cbHeight = = 8 )) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |

# References

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-N1023, G. Van der Auwera, L. Li, and A. Filippov

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, B. Bross, J. Chen, and S. Liu