| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0050-v1 |
| --- | --- |

| Title: | CE3-2.1.1 and CE3-2.1.2: Removing 2x2, 2x4, and 4x2 chroma CBs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com |
| Source: | MediaTek Inc., Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes to remove small chroma intra coding blocks (CBs) by constraining the partitioning of the chroma intra CBs in single coding tree. In single coding tree, a smallest chroma intra prediction unit (SCIPU) is defined as a coding tree node whose chroma block size is larger than or equal to 16 chroma samples and has at least one child luma block smaller than 64 luma samples. It is required that in each SCIPU, all CBs are inter, or all CBs are non-inter, i.e, either intra or intra block copy (IBC). In case of a non-inter SCIPU, it is further required that chroma of the non-inter SCIPU shall not be further split and luma of the SCIPU is allowed to be further split. In this way, the smallest chroma intra CB size is 16 chroma samples, and 2x2, 2x4, and 4x2 chroma CBs are removed. In CE3-2.1.1, no additional syntax is signalled, and whether a SCIPU is non-inter can be derived by the prediction mode of the first luma CB in the SCIPU. In CE3-2.1.2, the type of a SCIPU is inferred to be non-inter if the current slice is an I-slice or the current SCIPU has a 4x4 luma partition in it after further split one time (because no inter 4x4 is allowed in VVC); otherwise, the type of the SCIPU (inter or non-inter) is indicated by one flag before parsing the CUs in the SCIPU. It is reported that CE3-2.1.1 results in 0.07%/-0.08%/-0.11% and 0.14%/0.16%/0.07% Y/Cb/Cr BD-rates with negligible runtime changes for VTM5.0-RA and VTM5.0-LB, respectively. It is reported that CE3-2.1.2 results in 0.09%/-0.01%/-0.02% and 0.19%/0.19%/0.10% Y/Cb/Cr BD-rates with negligible runtime changes for VTM5.0-RA and VTM5.0-LB, respectively. It is asserted that the worst case hardware processing throughput for chroma intra prediction can be increased to 4X of that in VTM5.0.

# Introduction

In typical hardware video encoders and decoders, processing throughput drops when a picture has more small intra blocks because of sample processing data dependency between neighbouring intra blocks. The predictor generation of an intra block requires top and left boundary reconstructed samples from neighbouring blocks. Therefore, intra prediction has to be sequentially processed block by block.

In HEVC, the smallest intra CU is 8x8 luma samples. The luma component of the smallest intra CU can be further split into four 4x4 luma intra prediction units (PUs), but the chroma components of the smallest intra CU cannot be further split. Therefore, the worst case hardware processing throughput occurs when 4x4 chroma intra blocks or 4x4 luma intra blocks are processed.

In VTM5.0, in single coding tree, since chroma partitions always follows luma and the smallest intra CU is 4x4 luma samples, the smallest chroma intra CB is 2x2. Therefore, in VTM5.0, the smallest chroma intra CBs in single coding tree is 2x2. The worst case hardware processing throughput for VVC decoding is only 1/4 of that for HEVC decoding. Moreover, the reconstruction process of a chroma intra CB becomes much more complex than that in HEVC after adopting tools including cross-component linear model (CCLM), 4-tap interpolation filters, position-dependent intra prediction combination (PDPC), and combined inter intra prediction (CIIP). It is challenging to achieve high processing throughput in hardware decoders. In this contribution, two methods that improve the worst case hardware processing throughput are proposed.

# Proposed methods

The goal of the proposed methods is to disallow chroma intra CBs smaller than 16 chroma samples by constraining the partitioning of chroma intra CBs.

In single coding tree, a SCIPU is defined as a coding tree node whose chroma block size is larger than or equal to TH chroma samples and has at least one child luma block smaller than 4TH luma samples, where TH is set to 16 in this contribution. It is required that in each SCIPU, all CBs are inter, or all CBs are non-inter, i.e, either intra or IBC. In case of a non-inter SCIPU, it is further required that chroma of the non-inter SCIPU shall not be further split and luma of the SCIPU is allowed to be further split. In this way, the smallest chroma intra CB size is 16 chroma samples, and 2x2, 2x4, and 4x2 chroma CBs are removed.

Two SCIPU examples are shown in Figure 1. In Figure 1 (a), one chroma CB of 8x4 chroma samples and three luma CBs (4x8, 8x8, 4x8 luma CBs) form one SCIPU because the ternary tree (TT) split from the 8x4 chroma samples would result in chroma CBs smaller than 16 chroma samples. In Figure 1 (b), one chroma CB of 4x4 chroma samples (the left side of the 8x4 chroma samples) and three luma CBs (8x4, 4x4, 4x4 luma CBs) form one SCIPU, and the other one chroma CB of 4x4 samples (the right side of the 8x4 chroma samples) and two luma CBs (8x4, 8x4 luma CBs) form one SCIPU because the binary tree (BT) split from the 4x4 chroma samples would result in chroma CBs smaller than 16 chroma samples.

- (b)

Figure 1.  SCIPU examples

In CE3-2.1.1, no additional syntax is signalled, and whether a SCIPU is non-inter can be derived by the prediction mode of the first luma CB in the SCIPU.

In CE3-2.1.2, the type of a SCIPU is inferred to be non-inter if the current slice is an I-slice or the current SCIPU has a 4x4 luma partition in it after further split one time (because no inter 4x4 is allowed in VVC); otherwise, the type of the SCIPU (inter or non-inter) is indicated by one signalled flag before parsing the CUs in the SCIPU.

By applying the above two methods, the worst case hardware processing throughput occurs when 4x4, 2x8, or 8x2 chroma blocks, instead of a 2x2 chroma blocks, are processed. The worst case hardware processing throughput is the same as that in HEVC and is 4X of that in VTM5.0.

# Experiment results

The proposed methods have been implemented on top of VTM5.0 [1]. The common test conditions [2] are used for evaluation. The results of CE3-2.1.1and CE3-2.1.2 are summarized in Table 1 and Table 2, respectively.

Table 1.  Results of CE3-2.1.1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.14% | 0.16% | 100% | 101% |
| Class A2 | 0.07% | 0.12% | 0.18% | 100% | 99% |
| Class B | 0.05% | -0.06% | -0.20% | 100% | 98% |
| Class C | 0.12% | -0.22% | -0.42% | 99% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.08% | -0.11% | 100% | 100% |
| Class D | 0.27% | 0.74% | 0.73% | 100% | 100% |
| Class F | 0.16% | 0.30% | 0.33% | 99% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.21% | 0.23% | 100% | 101% |
| Class C | 0.27% | 0.13% | 0.06% | 100% | 101% |
| Class E | 0.11% | 0.13% | -0.17% | 101% | 98% |
| Overall | 0.14% | 0.16% | 0.07% | 100% | 100% |
| Class D | 0.23% | 0.68% | 1.17% | 98% | 101% |
| Class F | 0.25% | 0.07% | -0.42% | 97% | 98% |

Table 2.  Results of CE3-2.1.2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.12% | 0.08% | 99% | 101% |
| Class A2 | 0.08% | 0.12% | 0.16% | 100% | 101% |
| Class B | 0.08% | 0.04% | 0.00% | 100% | 101% |
| Class C | 0.17% | -0.10% | -0.28% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | -0.01% | -0.02% | 99% | 101% |
| Class D | 0.32% | 0.90% | 0.77% | 99% | 102% |
| Class F | 0.18% | 0.41% | 0.38% | 99% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | 0.19% | 0.42% | 100% | 104% |
| Class C | 0.40% | 0.13% | 0.00% | 100% | 102% |
| Class E | 0.03% | 0.27% | -0.29% | 101% | 105% |
| Overall | 0.19% | 0.19% | 0.10% | 101% | 103% |
| Class D | 0.37% | -0.04% | 0.91% | 100% | 101% |
| Class F | 0.39% | 0.39% | -0.37% | 98% | 102% |

The AI results when single tree is used for I slices is shown in Table 3. Y, Cb, and Cr BD-rates are -0.09%, -0.88%, and -1.11%, respectively, and encoding time is reduced by 10% because of removing small intra chroma blocks. For AI condition, CE3-2.1.1 and CE3-2.1.2 have the identical results, since the SCIPU type is always non-inter in I slices.

Table 3.  Results of CE3-2.1.1 and CE3-2.1.2 when single tree is used for I slices

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.07% | -0.01% | 93% | 102% |
| Class A2 | 0.04% | -0.75% | -0.33% | 90% | 100% |
| Class B | -0.04% | -1.04% | -1.53% | 90% | 100% |
| Class C | -0.25% | -0.95% | -1.56% | 89% | 105% |
| Class E | -0.15% | -1.47% | -1.71% | 91% | 102% |
| Overall | -0.09% | -0.88% | -1.11% | 90% | 102% |
| Class D | -0.10% | -1.67% | -1.61% | 88% | 99% |
| Class F (optional) | -0.22% | 0.13% | -0.62% | 93% | 102% |

# Conclusions

In this proposal, two methods (CE3-2.1.1 and CE3-2.1.2) are proposed to guarantee the smallest chroma intra CB size equal to 16 chroma samples. The average luma BD-rate is 0.1% to 0.2% compared with VTM5.0. CE3-2.1.1 has slightly better coding efficiency than CE3-2.1.2.

# References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies. Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).