| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0051-v1 |
| --- | --- |

| Title: | CE4-2.5: Phase-variant affine subblock motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chen-Yen Lai, Ching-Yeh Chen,Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, jenny.lai, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, phase-variant affine subblock motion compensation (MC) is proposed. The conventional two-stage horizontal-vertical interpolation is applied. Unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows in the affine subblock for horizontal, and different phases of filters can be applied to different sample columns in the affine subblock for vertical filtering. The used interpolation filter sets in the proposed method are the same as those in VTM5.0. The number of filtering operations for each affine subblock in the proposed method is also the same as that in VTM5.0. Four methods are proposed in this contribution. CE4-2.5a is the phase-variant affine MC with fallback mode, which falls back to block-based MC when the affine motion is small, and at most 11x9 reference samples are required for a 4×4 luma subblock MC. CE4-2.5c is the phase-variant affine MC without fallback mode and at most 11x9 reference samples for a 4×4 luma subblock MC. CE4-2.5d is the phase-variant affine MC with fallback mode and at most a 9×9 reference samples for a 4×4 luma subblock MC. CE4-2.5e is a combined test of CE4-2.5a and the prediction refinement with optical flow (PROF) in CE4-2.1a. Simulation results reportedly show that, for CE4-2.5a, the average luma BD-rates are -0.39% and -0.18% in RA and LB, respectively. For CE4-2.5c, the average luma BD-rates are -0.36% and -0.15% in RA and LB, respectively. For CE4-2.5d, the average luma BD-rates are -0.38% and -0.21% in RA and LB, respectively. For CE4-2.5e, the average luma BD-rates are -0.59% and -0.38% in RA and LB, respectively.

# Introduction

In VTM5.0, when affine mode is applied, the coding unit (CU) is divided into 4×4 subblocks. The motion vector (MV) of each subblock is derived by using an affine motion model. For affine mode MC, the 4×4 block-based translational motion model MC is applied. The 4×4 block-based translational motion model MC can be divided into two stages. The first stage is to filter an (4+L–1)×(4+L–1) reference block window with (4+L–1) rows of horizontal filtering, where the L is the filter tap length of the interpolation filer. In VTM5.0, the L is six for luma component and four for chroma components. For each row, four times of horizontal filtering are performed to generate 4×1 horizontally filtered samples. Therefore, in the first stage, in total 4×(4+L–1) times of horizontal filtering are performed, and 4×(4+L–1) horizontally filtered samples are generated. The horizontal filter phases of all the sample rows in the (4+L–1)×(4+L–1) reference block window are the same. In the second stage, the 4×(4+L–1) filtered samples are further vertically filtered for the four sample columns. For each column, four times of vertical filtering are performed to generate 1×4 vertically filtered samples. Therefore, in the second stage, in total 4×4 times of vertical filtering are performed, and 4×4 vertically filtered samples are generated as the MC results. The vertical filter phases of all the sample columns in the horizontally filtered block of 4×(4+L–1) samples are also the same. The horizontal filter phase and the vertical filter phase are derived from the MV of the 4×4 subblock. In each 4×4 subblock, the translational motion model is applied using phase-invariant MC.

# Proposed methods

## CE4-2.5a

To better approximate the affine motion model in an affine subblock, phase-variant MC is applied to the subblock. In the proposed method, the affine coded block is also divided into 4×4 subblocks, and a subblock MV is derived for each subblock as done in VTM5.0. The MC of each subblock is divided into two stages. The first stage is to perform (4+L–1) rows of horizontal filtering to generate a 4×(4+L–1) horizontally filtered samples. However, different from the translational MC, the filter phase of each sample row is different. For each sample row, MVx is derived as follows.

MVx = (subblockMVxAdj + mvdRow[rowIdx]) >>7                   		(Equation 1)

mvdRow[rowIdx] = dMvVerX×(rowIdx – L/2 – 1)                   		(Equation 2)

The filter phase of each sample row is derived from the MVx. Let subblockMVx be the x component of the subblock MV as derived in VTM5.0. The subblockMVxAdj is equal to subblockMVx plus an adjustment term of “isLuma ? ((8-subblockMVy&15)×dMvVerX)>>4 : ((16-subblockMVy&31)×dMvVerX)>>5”, where the subblockMVy is the y component of the subblock MV and FractionalPart(subblockMVy) is the fractional part of subblockMVy. This adjustment term only needs to be calculated once per 4x4 subblock or can be pre-calculated with different values of fractional sample part of subblockMVy (0, 1, 2, … to 15 for luma; 0, 1, 2, … to 31 for chroma) and stored in a buffer once per CU. The rowIdx is the sample row index. The dMvVerX is (cuBottomLeftCPMVx – cuTopLeftCPMVx) << (7 – log2LumaCbHeight), where cuBottomLeftCPMVx is the x component of the CU bottom left control point motion vector (CPMV), cuTopLeftCPMVx is the x component of the CU top left CPMV, and LumaCbHeight is the log2 of the height of the luma coding block (CB). The values stored in mvdRow buffer are pre-calculated once per CU. The values are clipped within one inter sample, e.g. ± (1<<(7+4)) for luma and ± (1<<(7+5)) for chroma. Therefore, the maximum horizontal position difference of the reference samples of the top-row and the bottom-row is two samples. At most a (4+L–1+2)×(4+L–1) reference sample window is required (e.g., 11×9 reference sample window for luma).

After the horizontal filtering, a 4×(4+L–1) horizontally filtered samples are generated. Figure 1 shows the concept of the proposed horizontal filtering. The gray points are the samples of the reference samples, and the orange points denote the horizontally filtered samples. A blue tube of 6×1 samples denotes applying one time of 6-tap horizontal filtering for luma, as shown in Figure 1 and Figure 2. Each sample row needs four times of horizontal filtering. The filter phases on a sample row are the same. However, the filter phases on different rows are different. Skewed 4×9 samples are generated.

Figure 1.  Phase-variant horizontal filtering

Figure 2.  Illustration of applying one time of 6-tap horizontal filtering

In the second stage, the 4×(4+L–1) horizontally filtered samples (orange samples in Figure 1) are further vertically padded and filtered. For each sample column, MVy is derived as follows.

MVy = (subblockMVy + mvdColumn[columnIdx]) >>7                    	(Equation 3)

mvdColumn[columnIdx] = dMvHorY×(columnIdx–1) – (dMvHorY>>1)		(Equation 4)

The filter phase of each sample column is derived from the MVy. The columnIdx is the sample column index. The dMvHorY is (cuTopRightCPMVy – cuTopLeftCPMVy) << (7 – log2LumaCbWidth), where cuTopRightCPMVy is the y component of the CU top right CPMV, cuTopLeftCPMVy is the y component of the CU top left CPMV, and log2LumaCbWidth is the log2 of the width of the luma CB. The values stored in mvdColumn buffer are pre-calculated once per CU. The values are clipped within one inter sample, e.g. ± (1<<(7+4)) for luma and ± (1<<(7+5)) for chroma.

After the vertical filtering, 4×4 affine subblock prediction samples are generated. Figure 3 shows the concept of the proposed vertical filtering. The light orange points are the horizontally filtered samples from first stage. The green points are the vertically padded samples from the top row and the bottom row. The red points are the vertically filtered samples as the final prediction samples.

Figure 3. Non-uniform phase vertical filtering

In this proposal, the used interpolation filter sets are the same as those in VTM5.0. The only difference is that the horizontal filter phases are different for different sample rows and the vertical filter phases are different for different sample columns. As for the number of filtering operations for each affine subblock in the proposed method, it is the same as that in VTM5.0.

In the proposed method, a fallback mode is conditionally applied. When |dMvVerX| < 64, the phase-invariant horizontal filtering in VTM5.0 is applied. When |dMvHorY| < 64, the phase-invariant vertical filtering in VTM5.0 is applied.

## CE4-2.5c

CE4-2.5c is the similar to CE4-2.5a, but the fallback mode is disabled. Phase-variant horizontal filtering and vertical filtering are always applied regardless of |dMvVerX| and |dMvHorY|.

## CE4-2.5d

CE4-2.5d is the similar to CE4-2.5a, but it only requires a 9×9 luma reference sample window and a 7×7 chroma reference sample window for the phase-variant horizontal filtering. The 9×9 and 7×7 reference sample windows are enlarged to 11×9 and 9×7 reference sample windows by padding, and the rest filtering processes are not changed.

## CE4-2.5e

CE4-2.5e is the combination of the PROF in CE4-2.1a and CE4-2.5a. It uses the phase-variant affine MC described in CE4-2.5a for affine MC and then refined by CE4-2.1a. The gradient value calculation in CE4-2.1a is not modified. The only change of PROF in this combined test is that the MV difference between sample-based affine MV and subblock MV used in PROF is modified as the MV difference between sample-based affine MV and phase-variant equivalent MV. The phase-variant equivalent MV is derived as follows. It can be pre-calculated once per CU.

pvMVDy[x][y] = (dMvHorY×(x–1) – (dMvHorY>>1)) >> 7

pvMVDx[x][y] = (dMvVerX×(pvMVDy[x][y] + ((2y–3) << (scalcShift-1)))) >> (7+scalcShift)

The x and y are the sample position within a 4×4 subblock. The scalcShift is 4 for luma and 5 for chroma.

# Simulation results

Four tests are evaluated in this contribution.

- CE4-2.5a: phase-variant affine MC with fallback mode and at most 11x9 reference samples for a 4×4 luma subblock MC

- CE4.2.5c: phase-variant affine MC without fallback mode and at most 11x9 reference samples for a 4×4 luma subblock MC

- CE4-2.5d: phase-variant affine MC with fallback mode and at most a 9×9 reference samples for a 4×4 luma subblock MC

- CE4-2.5e: Combination of CE4-2.5a and CE4.2.1a (PROF).

The common test conditions (CTC) [1] are used for evaluation. The results of CE4-2.5a, CE4-2.5c, 4-2.5d, and CE4-2.5e are shown in Table 1, Table 2, Table 3, and Table 4, respectively. Simulation results reportedly show that, for CE4-2.5a, the average luma BD-rates are -0.39% and -0.18% in RA and LB, respectively. For CE4-2.5c, the average luma BD-rates are -0.36% and -0.15% in RA and LB, respectively. For CE4-2.5d, the average luma BD-rates are -0.38% and -0.21% in RA and LB, respectively. For CE4-2.5e, the average luma BD-rates are -0.59% and -0.38% in RA and LB, respectively.

Table 1.  Results of the CE4-2.5a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.24% | -0.04% | 102% | 100% |
| Class A2 | -0.53% | -0.56% | -0.53% | 101% | 101% |
| Class B | -0.60% | -0.74% | -0.83% | 101% | 99% |
| Class C | -0.26% | -0.39% | -0.43% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.51% | -0.50% | 101% | 99% |
| Class D | -0.19% | -0.31% | -0.20% | 100% | 101% |
| Class F | -0.25% | -0.27% | -0.37% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.33% | -0.11% | 101% | 99% |
| Class C | -0.14% | -0.07% | -0.48% | 102% | 101% |
| Class E | 0.02% | 0.02% | -0.07% | 101% | 99% |
| Overall | -0.18% | -0.16% | -0.22% | 101% | 99% |
| Class D | -0.18% | 0.01% | 0.99% | 98% | 99% |
| Class F | -0.31% | -0.32% | -0.68% | 100% | 98% |

Table 2.  Results of the CE4-2.5c

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.21% | -0.09% | 105% | 99% |
| Class A2 | -0.48% | -0.58% | -0.60% | 105% | 101% |
| Class B | -0.56% | -0.77% | -0.79% | 104% | 98% |
| Class C | -0.22% | -0.26% | -0.47% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.48% | -0.53% | 104% | 99% |
| Class D | -0.23% | -0.24% | -0.17% | 102% | 104% |
| Class F | -0.19% | -0.29% | -0.32% | 105% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.27% | -0.95% | 108% | 103% |
| Class C | -0.15% | -0.32% | -0.59% | 105% | 105% |
| Class E | 0.12% | 0.20% | -0.40% | 113% | 104% |
| Overall | -0.15% | -0.17% | -0.69% | 108% | 104% |
| Class D | -0.12% | -0.24% | 0.33% | 101% | 101% |
| Class F | -0.20% | -0.75% | -0.76% | 106% | 97% |

Table 3.  Results of the CE4-2.5d

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.22% | -0.02% | 103% | 101% |
| Class A2 | -0.53% | -0.49% | -0.61% | 102% | 101% |
| Class B | -0.59% | -0.80% | -0.91% | 102% | 100% |
| Class C | -0.24% | -0.34% | -0.51% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.50% | -0.56% | 102% | 100% |
| Class D | -0.19% | -0.14% | -0.07% | 100% | 100% |
| Class F | -0.25% | -0.29% | -0.37% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.16% | -0.13% | 102% | 100% |
| Class C | -0.15% | -0.34% | -0.50% | 102% | 103% |
| Class E | -0.05% | -0.05% | -0.33% | 104% | 101% |
| Overall | -0.21% | -0.19% | -0.30% | 103% | 101% |
| Class D | -0.16% | 0.33% | 0.58% | 106% | 109% |
| Class F | -0.27% | -0.53% | -0.98% | 99% | 97% |

Table 4.  Results of the CE4-2.5e

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.25% | -0.03% | 103% | 101% |
| Class A2 | -1.04% | -0.47% | -0.49% | 103% | 101% |
| Class B | -0.74% | -0.69% | -0.74% | 103% | 98% |
| Class C | -0.38% | -0.38% | -0.44% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.59% | -0.47% | -0.47% | 103% | 100% |
| Class D | -0.64% | -0.53% | -0.40% | 103% | 101% |
| Class F | -1.00% | -0.67% | -0.79% | 103% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.25% | -0.61% | 104% | 105% |
| Class C | -0.27% | -0.38% | -0.54% | 103% | 104% |
| Class E | -0.36% | -0.31% | -0.53% | 107% | 108% |
| Overall | -0.38% | -0.31% | -0.56% | 105% | 105% |
| Class D | -0.46% | -0.56% | -0.28% | 102% | 106% |
| Class F | -1.40% | -1.04% | -1.31% | 100% | 100% |

# Conclusions

In this contribution, phase-variant affine subblock MC is proposed. Different phases of filters are applied to different sample rows for horizontal filtering and different sample columns for vertical filtering. The BD-rate is roughly -0.4% when compared with the current phase-invariant affine subblock MC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).