| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0052-v1 |
| --- | --- |

| Title: | CE7-1: TB-level constraints on context-coded bins for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Shih-Ta Hsiang, Zhi-Yi Lin, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | +886-3-5670766 {peter.chuang, shih-ta.hsiang, zhi-yi.lin, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com +1 858 658 3973 {mcoban, martak} @qti.qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Inc. |  |  |

_____________________________

# Abstract

In the VTM5.0 coefficient coding, a coefficient subblock-level maximum number of context-coded bins is imposed. The averaged maximum number of context-coded bins per 16 samples is increased by 28% compared with that in HEVC. In this contribution, two methods are proposed to use a transform block (TB) level constraint for maximum number of context-coded bins instead of coefficient subblock-level constraint. In CE7-1.1, for a luma TB, the maximum number of context-coded bins is equal to tb_width*tb_height*1.75. For a chroma TB, the maximum number of context-coded bins is equal to tb_width*tb_height*1.25. The tb_width and tb_height are the width and height of a TB after coefficient zero-out. In CE7-1.2, for a luma TB, the maximum number of context-coded bins is equal to subblock_size*lastPosSubblock*1.75, where lastPosSubblock is the number of subblocks traversed in the scanning of the non-zero-out region of transform until reaching the subblock with the last coefficient position. For a chroma TB, the maximum number of context-coded bins is equal to subblock_size*lastPosSubblock*1.25. The supplementary result of CE7-1.2b is also provided. In CE7-1.2b, the maximum number of context-coded bins of a luma TB is the same as CE7-1.2, but the maximum number of context-coded bins of a chroma TB is equal to subblock_size*lastPosSubblock*1.5. The averaged maximum number of context-coded bins per 16 samples is only increased by 1.3% compared with that in HEVC in both CE7-1.1 and CE7-1.2. Simulation results reportedly show that, in CE7-1.1, the average luma BD-rates are -0.01%, 0.00%, and -0.01% in AI, RA, and LB cases, respectively. In CE7-1.2, the average luma BD-rates are 0.00%, 0.02%, and 0.01% in AI, RA, and LB cases, respectively. In CE7-1.2b, the average luma BD-rates are -0.01%, 0.00%, and 0.02% in AI, RA, and LB cases, respectively.

# Introduction

In HEVC, for a 4x4 coefficient subblock, the maximum number of context-coded bins per 16 samples is 24 or 25, including 15 or 16 significant coefficient flags depending on whether the last significant coefficient is in the subblock, eight greater than one (gt1) flags, and one greater than two (gt2) flag. In VTM5.0, for a 4x4 coefficient coding subblock, at most 32 context-coded bins can be used. For a 2x2 chroma coefficient coding subblock, at most eight context-coded bins can be used. The averaged maximum number of context-coded bins per 16 samples in VTM5.0 is 32 bins. Compared with that in HEVC, the worst-case number of context-coded bins is increased by 28%. During the discussion in the last JVET meeting, it was commented that limiting context coded bins at 4x4 coefficient subblock level is not desirable. Imposing such a limit at a higher level may be better. In this contribution, it is proposed to use TB-level constraints. The proposed methods are described in Section 2, and the results are reported in Section 3.

# Proposed TB-level constraints on context-coded bins

## CE7-1.1: TB-level constraints based on the non-zero-out region size of TB

In CE7-1.1, the area of the TB and the colour component are used to derive the maximum number of context-coded bins for a TB. For a luma TB, the maximum number of context-coded bins is equal to tb_width*tb_height*1.75. For a chroma TB, the maximum number of context-coded bins is equal to tb_width*tb_height*1.25. The tb_width and tb_height are the width and height of a TB after coefficient zero-out. In transform coefficient coding, a variable, remBinsPass1, is first set to the maximum number of context-coded bins and will be decreased by one when a context-coded bin is signalled. When the remBinsPass1 is larger than or equal to four, the sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag can be coded by using context-coded bins. When the remBinsPass1 is smaller than 4, the coefficients are coded with the syntax element of dec_abs_level by using Golomb-Rice code and bypass-coded bins. The remBinsPass1 is reset for every TB and not reset for non-first coefficient subblocks in decoding order. The transition of using context-coded bins for the sig_coeff_flag, abs_level_gt1_flag, par_level_flag, and abs_level_gt3_flag to using bypass-coded bins for the rest coefficients only happens at most once per TB, unlike VTM5.0 that the might happen for each coefficient subblock. For a coefficient subblock, if the remBinsPass1 is smaller than 4, the entire coefficient subblock is coded by using bypass-coded bins.

## CE7-1.2: TB-level constraints based on the last coded subblock position of TB

In CE7-1.2, instead of imposing a fixed limit for each coefficient group (CG), a TB-level limit for total number of context-coded bins is used. The limit is set to number of coded CGs in a TB times the CG size times 1.75 for luma component or 1.25 for chroma components. The last significant coefficient position is used to determine the number of CGs scanned starting from the last position. This can be described by:

lastPosSubblock = (scanPosLast >> log2CGSize) + 1

regBinTULimit = lastPosSubblock * CGsize * (isLuma? 1.75 : 1.25),

where scanPosLast is the last position in scan order, regBinTULimit is the initial total number of regular coded bins at TB level.

## CE7-1.2b: TB-level constraints based on the last coded subblock position of TB with more context-coded bins for chroma components

In CE7-1.2b, based on CE7-1.2, the factor of chroma component is changed from 1.25 to 1.5 for better coding performance. It can be described by:

lastPosSubblock = (scanPosLast >> log2CGSize) + 1

regBinTULimit = lastPosSubblock * CGsize * (isLuma? 1.75 : 1.5),

where scanPosLast is the last position in scan order, regBinTULimit is the initial total number of regular coded bins at TB level.

## Analysis

By using the proposed methods, the averaged maximum number of context-coded bins per 16 samples is 25.33 in both CE7-1.1 and CE7-1.2 and is only 1.3% larger than that in HEVC. The averaged maximum number of context-coded bins per 16 samples in CE7-1.2b is 26.67 and is 6.7% larger than that in HEVC. Compared with VTM5.0, the worst case number of context-coded bins for coefficient coding is reduced by 27% for CE7-1.1 and CE7-1.2 and by 21% for CE7-1.2b. The analysis is summarized in Table 1.

Table 1.  The averaged maximum number of context-coded bins per 16 samples in HEVC, VTM5.0, and the proposed methods

| Number of maximum context-coded bins | 4x4 luma subblock | 4x4 chroma subblock | 2x2 chroma subblock | Maximum number of context-coded bins in one 64-sample luma block and two 16-sample chroma blocks | Averaged maximum number of context-coded bins per 16 samples |
| --- | --- | --- | --- | --- | --- |
| HEVC | 24 or 25 | 24 or 25 | N/A | 25*4 + 25*2 = 150 | 25.00 (100%) |
| VTM5.0 | 32 | 32 | 8 | 32*4 + 32*2 = 192 | 32.00 (128%) |
| CE7-1.1/CE7-1.2 | 28 | 20 | 5 | 28*4 + 20*2 = 152 | 25.33 (101.3%) |
| CE7-1.2b | 28 | 24 | 6 | 28*4 + 24*2 = 160 | 26.67 (106.7%) |

# Simulation results

The proposed methods are implemented on VTM5.0. The common test conditions (CTC) [1] are used for evaluation. The CTC QP (QP = 22, 27, 32, 37) and low QP (QP = 2, 7, 12, 17) conditions are tested. The results of CE7.1.1 and CE7.1.2 in CTC QP are shown in Table 2 and Table 3, respectively. The results of the CE7-1.1 and CE7-1.2 in low QP are shown in Table 4 and Table 5, respectively. Also the results of the CE7-1.2b in CTC QP and low QP are shown in Table 6 and Table 7.

Table 2.  Results of CE7-1.1 in CTC (QP = 22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.14% | 0.02% | 104% | 103% |
| Class A2 | 0.01% | -0.07% | -0.03% | 101% | 104% |
| Class B | -0.02% | -0.13% | -0.02% | 102% | 105% |
| Class C | -0.02% | -0.05% | 0.01% | 102% | 104% |
| Class E | -0.03% | -0.15% | -0.12% | 101% | 104% |
| Overall | -0.01% | -0.06% | -0.03% | 102% | 104% |
| Class D | -0.01% | -0.01% | -0.13% | 96% | 96% |
| Class F | -0.01% | 0.18% | 0.02% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.19% | 0.06% | 103% | 105% |
| Class A2 | 0.05% | 0.12% | 0.18% | 103% | 105% |
| Class B | -0.01% | -0.08% | -0.12% | 103% | 99% |
| Class C | -0.03% | -0.11% | 0.02% | 100% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | 0.02% | 102% | 100% |
| Class D | 0.00% | -0.04% | -0.04% | 98% | 98% |
| Class F | -0.01% | 0.01% | 0.12% | 100% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.07% | 0.54% | 102% | 105% |
| Class C | -0.03% | 0.07% | -0.07% | 103% | 101% |
| Class E | 0.04% | -0.67% | 0.59% | 100% | 98% |
| Overall | -0.01% | -0.17% | 0.35% | 102% | 102% |
| Class D | 0.03% | -0.19% | 1.18% | 98% | 98% |
| Class F | -0.06% | -0.44% | -0.54% | 98% | 98% |

Table 3.  Results of CE7-1.2 in CTC (QP = 22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.07% | 0.02% | 103% | 102% |
| Class A2 | 0.01% | 0.00% | -0.04% | 102% | 101% |
| Class B | -0.02% | -0.07% | 0.00% | 102% | 103% |
| Class C | -0.01% | -0.01% | 0.05% | 102% | 104% |
| Class E | -0.02% | -0.07% | -0.07% | 101% | 105% |
| Overall | 0.00% | -0.02% | 0.00% | 102% | 103% |
| Class D | -0.01% | 0.07% | -0.03% | 97% | 96% |
| Class F | 0.01% | 0.09% | 0.00% | 102% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.14% | 0.14% | 103% | 103% |
| Class A2 | 0.08% | 0.21% | 0.12% | 103% | 105% |
| Class B | 0.00% | -0.06% | -0.29% | 103% | 97% |
| Class C | -0.02% | -0.12% | -0.15% | 100% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.04% | -0.09% | 103% | 99% |
| Class D | 0.05% | 0.24% | 0.02% | 99% | 95% |
| Class F | 0.00% | 0.09% | 0.21% | 100% | 93% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.20% | 0.15% | 100% | 99% |
| Class C | 0.00% | 0.14% | -0.44% | 100% | 100% |
| Class E | 0.06% | 1.06% | 1.50% | 100% | 99% |
| Overall | 0.01% | 0.39% | 0.29% | 100% | 99% |
| Class D | 0.01% | -0.40% | 0.95% | 99% | 98% |
| Class F | -0.06% | 0.24% | -0.13% | 98% | 97% |

Table 4.  Results of CE7-1.1 in low QP conditions (QP = 2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | -0.13% | 99% | 98% |
| Class A2 | 0.07% | 0.00% | 0.01% | 99% | 98% |
| Class B | 0.06% | -0.10% | -0.09% | 99% | 101% |
| Class C | 0.13% | 0.03% | 0.04% | 99% | 99% |
| Class E | 0.04% | -0.04% | -0.02% | 99% | 101% |
| Overall | 0.06% | -0.06% | -0.04% | 99% | 99% |
| Class D | 0.13% | 0.03% | 0.02% | 99% | 98% |
| Class F | 0.11% | 0.07% | 0.09% | 99% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.24% | -0.13% | 100% | 96% |
| Class A2 | 0.32% | 0.26% | 0.27% | 99% | 95% |
| Class B | 0.03% | -0.04% | -0.04% | 99% | 99% |
| Class C | 0.09% | 0.18% | 0.16% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.04% | 0.06% | 100% | 98% |
| Class D | 0.05% | 0.33% | 0.25% | 100% | 98% |
| Class F | 0.11% | 0.16% | 0.00% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.04% | -0.03% | 100% | 99% |
| Class C | 0.11% | 0.28% | 0.26% | 101% | 99% |
| Class E | -0.11% | -0.32% | -0.29% | 100% | 100% |
| Overall | 0.03% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.08% | 0.29% | 0.29% | 101% | 99% |
| Class F | 0.11% | 0.34% | 0.36% | 100% | 100% |

Table 5.  Results of CE7-1.2 in low QP conditions (QP = 2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.24% | -0.16% | 99% | 97% |
| Class A2 | 0.11% | 0.01% | 0.01% | 98% | 95% |
| Class B | 0.08% | -0.13% | -0.12% | 99% | 97% |
| Class C | 0.13% | 0.01% | 0.02% | 99% | 98% |
| Class E | 0.04% | -0.04% | 0.01% | 99% | 97% |
| Overall | 0.08% | -0.08% | -0.05% | 99% | 97% |
| Class D | 0.13% | 0.01% | 0.02% | 99% | 99% |
| Class F | 0.12% | 0.04% | 0.02% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.26% | -0.13% | 100% | 98% |
| Class A2 | 0.35% | 0.30% | 0.32% | 100% | 97% |
| Class B | 0.06% | 0.06% | 0.07% | 99% | 98% |
| Class C | 0.10% | 0.23% | 0.20% | 102% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.09% | 0.11% | 100% | 98% |
| Class D | 0.06% | 0.31% | 0.19% | 100% | 98% |
| Class F | 0.21% | 0.20% | 0.09% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.06% | 0.07% | 100% | 99% |
| Class C | 0.11% | 0.32% | 0.28% | 101% | 99% |
| Class E | -0.06% | -0.18% | -0.09% | 100% | 100% |
| Overall | 0.05% | 0.09% | 0.10% | 100% | 99% |
| Class D | 0.09% | 0.28% | 0.30% | 101% | 98% |
| Class F | 0.14% | 0.36% | 0.39% | 100% | 99% |

Table 6.  Results of CE7-1.2b in CTC (QP = 22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.00% | 101% | 103% |
| Class A2 | -0.02% | -0.07% | -0.08% | 100% | 97% |
| Class B | -0.01% | -0.09% | -0.03% | 101% | 101% |
| Class C | -0.01% | -0.03% | -0.01% | 100% | 91% |
| Class E | -0.03% | -0.01% | -0.08% | 101% | 95% |
| Overall | -0.01% | -0.05% | -0.04% | 101% | 97% |
| Class D | -0.02% | 0.16% | -0.08% | 100% | 94% |
| Class F | -0.02% | 0.04% | -0.02% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.20% | -0.02% | 103% | 102% |
| Class A2 | 0.02% | 0.07% | 0.01% | 103% | 101% |
| Class B | -0.01% | -0.03% | -0.16% | 102% | 104% |
| Class C | 0.01% | -0.21% | 0.03% | 99% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.05% | 101% | 100% |
| Class D | 0.03% | 0.21% | 0.18% | 100% | 98% |
| Class F | -0.01% | 0.09% | 0.07% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.27% | 0.24% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.07% | 100% | 107% |
| Class E | 0.12% | 0.53% | 0.15% | 100% | 105% |
| Overall | 0.02% | 0.24% | 0.16% | 100% | 103% |
| Class D | 0.05% | 0.19% | 0.92% | 100% | 102% |
| Class F | 0.08% | 0.20% | 0.46% | 100% | 97% |

Table 7.  Results of CE7-1.2b in low QP conditions (QP = 2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.17% | -0.15% | 100% | 96% |
| Class A2 | -0.02% | 0.00% | 0.00% | 102% | 100% |
| Class B | -0.02% | -0.11% | -0.10% | 101% | 97% |
| Class C | 0.08% | 0.01% | 0.03% | 100% | 93% |
| Class E | 0.03% | -0.04% | -0.03% | 98% | 95% |
| Overall | 0.01% | -0.06% | -0.05% | 100% | 96% |
| Class D | 0.10% | 0.03% | 0.03% | 100% | 88% |
| Class F | 0.07% | 0.01% | 0.02% | 99% | 91% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.25% | -0.14% | 105% | 101% |
| Class A2 | 0.12% | 0.09% | 0.10% | 101% | 97% |
| Class B | -0.01% | -0.09% | -0.09% | 101% | 101% |
| Class C | 0.05% | 0.00% | 0.02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.06% | -0.03% | 102% | 99% |
| Class D | 0.04% | 0.12% | 0.02% | 100% | 95% |
| Class F | 0.12% | 0.11% | 0.03% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | -0.10% | 103% | 107% |
| Class C | 0.05% | 0.06% | 0.03% | 100% | 105% |
| Class E | -0.08% | -0.29% | -0.27% | 101% | 100% |
| Overall | 0.00% | -0.09% | -0.10% | 102% | 104% |
| Class D | 0.05% | 0.02% | 0.13% | 101% | 100% |
| Class F | 0.03% | 0.12% | 0.02% | 100% | 98% |

# CABAC bin statistic analysis

CABAC bin to bit ratio for overall bitstream as well as transform coefficient coding (trf) specific syntax elements is analyzed. Transform coefficient coding specific syntax elements considered are context coded sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag syntax elements and bypass coded abs_remainder and dec_abs_level syntax elements.

The following data are collected at the decoder side:

- The peak (based on weighted bin rate) context coded bin rate, bypass bin rate and bit rate of a coded sequence.

- The total context coded bin rate, bypass bin rate and bit rate over a coded sequence.

- The total context coded bin rate, bypass bin rate and bit rate for transform coefficient coding specific syntax elements over a coded sequence.

In weighted bin rate computations, bypass coded bins are counted as 0.25 context coded bins.

Results will be added.

# Conclusions

In this contribution, two methods are proposed to use TB-level constraints for maximum number of context-coded bins instead of coefficient subblock-level. The worst case CABAC throughput is effectively improved to be similar to that in HEVC with negligible coding efficiency loss.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).