| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0054-v1 |
| --- | --- |

| Title: | CE9-1.1: BDOF early termination |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com +81-50-3750-2744 {kenji.kondo, Masaru.Ikeda, teruhiko.s}@sony.com |
| Source: | MediaTek Inc., Sony Corporation |  |  |

_____________________________

# Abstract

In this contribution, CE9-1.1 test is reported. In a hardware decoder of VTM5.0, the critical path of inter prediction contains the cascaded operations of decoder motion vector refinement (DMVR) and bidirectional optical flow (BDOF). For each luma subblock of size min(cbWidth,16)×min(cbHeight,16) in DMVR, sum of absolute differences (SADs) between 1/2-subsampled 10-bit list 0 (L0) and list 1 (L1) prediction samples generated by bilinear interpolation are calculated for motion refinement. For each 4x4 luma block and luma subblock of size min(cbWidth,16)×min(cbHeight,16) in BDOF, SADs between full-sampled 14-bit L0 and L1 prediction samples generated by eight-tap interpolation are calculated for early termination. In order to reduce this critical path, when both DMVR and BODF are applied, it is proposed to reuse the SAD calculated for each subblock in DMVR to conditionally disable BDOF for the corresponding subblock, and it is also proposed to remove 4x4 block early termination in BDOF. In this way, the SAD calculations in BDOF are totally removed when DMVR is applied, and therefore it is asserted that the critical path in hardware decoding is effectively reduced. It is reported that the luma BD-rate is -0.02% for RA with 2% decoding time reduction when the anchor is VTM5.0.

# Introduction

In VTM5.0, DMVR is adopted to refine motion vectors (MVs) at the decoder for a coding unit (CU) when the CU is coded with regular merge/skip mode and bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and the bi-prediction with CU weights (BCW) selects equal weights. When DMVR is applied, one luma coding block (CB) is divided into several independently processed subblocks of size min(cbWidth,16)×min(cbHeight,16). DMVR refines MVs of each subblock by minimizing the SAD between 1/2-subsampled 10-bit L0 and L1 prediction samples generated by bilinear interpolation. For each subblock, integer △MV search around the initial MVs (i.e., the MVs of the selected regular merge/skip candidate) is first performed using SAD, and then fractional △MV derivation is performed to obtain the final MVs.

BDOF refines the luma prediction samples for a CU when the CU is coded with bi-prediction, one reference picture is before the current picture and the other reference picture is after the current picture in display order, and BCW selects equal weights. The eight-tap interpolation is used for generating the initial L0 and L1 prediction samples according to the input MVs (e.g., final MVs of DMVR in case of enabling DMVR). Next, a two-level early termination process is performed. The first early termination is at subblock level, and the second early termination is at 4×4 block level and is checked when the first early termination does not occur. At each level, the SAD between full-sampled 14-bit L0 and L1 prediction samples in each subblock/4x4 block is calculated first. If the SAD is smaller than one threshold, BDOF is not applied to the subblock/4x4 block. Otherwise, BDOF parameters are derived and used to generate the final luma sample predictors for each 4x4 block. In BDOF, the subblock size is the same as that in DMVR, i.e., min(cbWidth,16)×min(cbHeight,16).

When the CU is coded with regular merge/skip mode, one reference picture is before the current picture and the other reference picture is after the current picture in display order, the temporal distance between the current picture and one reference picture is equal to that between the current picture and the other reference picture, and BCW selects equal weights, DMVR and BDOF are both applied. The flow of cascading DMVR and BDOF processes is shown in Fig. 1. This is the critical path of inter prediction in a hardware decoder of VTM5.0.

Figure 1.  The flow of cascading DMVR and BDOF processes in VTM5.0. The DMVR SAD operations and BDOF SAD operations are different and not shared.

# Proposed method

As shown in Fig. 1, the SAD operations are required in DMVR and BDOF processes. In order to reduce the latency and operations in this critical path, when DMVR and BDOF are both applied, it is proposed to reuse the subblock SAD calculated in DMVR for the subblock early termination in BDOF, and it is also proposed to remove the 4x4 block early termination in BDOF. The threshold per sample in the subblock early termination is set to the threshold per sample in 4x4 block early termination divided by 32, where 32 is derived based on the prediction sample bitdepth difference and subsampling ratio difference in SAD operations between DMVR and BDOF.

# Experimental results

The proposed method has been implemented on VTM5.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the simulation results of the proposed method.

Table 1.  Simulation results of the proposed method

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | -0.01% | 100% | 100% |
| Class A2 | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | 0.04% | 0.02% | 100% | 97% |
| Class C | -0.03% | 0.02% | -0.08% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.02% | 100% | 98% |
| Class D | -0.01% | 0.00% | -0.04% | 99% | 100% |
| Class F | -0.01% | 0.01% | 0.01% | 100% | 95% |

# Conclusions

This contribution proposes to reuse the SAD calculated in DMVR for early termination in BDOF, in order to reduce the required operations when cascading DMVR and BDOF processes. Simulation results show that the luma BD-rate is -0.02% for RA with 2% decoding time reduction.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).