| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0057-v1 |
| --- | --- |

| Title: | CE4: Switchable interpolation filter (CE4-1.1, CE4-1.2, CE4-1.3, CE4-1.4, CE4-1.5, CE4-1.6, CE4-1.7) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Henkel, I. Zupancic, B. Bross, M. Winken, H. Schwarz, D. Marpe, T. Wiegand Fraunhofer HHI Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49-30-31002-562 anastasia.henkel@ hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports results for the CE4 core experiments CE4-1 which investigate the different aspects of the switchable interpolation filters for half-sample (half-pel) positions as proposed in JVET-N0309. The switching of the half-pel luma interpolation filter is done depending on the motion vector accuracy. In addition to the existing quarter-pel, full-pel, and 4-pel AMVR modes, a new half-pel accuracy AMVR mode is introduced (CE4-1.1).  Only in case of half-pel motion vector accuracy, one (CE4-1.2) or two (CE4-1.3) alternative half-pel luma interpolation filters can be signalled. In case of skip/merge mode using a spatial merging candidate, the information which interpolation filter is applied for the half-pel position is inherited from the neighbouring block. Additional tests evaluate the performance when this inheritance is disabled (CE4-1.4, CE4-1.5) and only disabled in case of DMVR (CE4-1.6, CE4-1.7).  In summary, the following results are reported for the common test conditions relative to VTM-5.0:

CE4-1.1: 	RA: 	-0.05% (Y), -0.12% (Cb), -0.16% (Cr)	103% enc.time	100% dec.time

LB: 	 0.01% (Y),  0.04% (Cb), -0.26% (Cr)	103% enc.time	100% dec.time

CE4.1.2:	RA:	-0.26% (Y), -0.81% (Cb), -0.84% (Cr)	101% enc.time	99% dec.time

LB:		-0.10% (Y), -0.02% (Cb), -0.27% (Cr)	103% enc.time	100% dec.time

CE4.1.3:	RA:	-0.34% (Y), -0.98% (Cb), -0.93% (Cr)	104% enc.time	99% dec.time

LB:		-0.12% (Y), -0.03% (Cb), -0.05% (Cr)	106% enc.time	99% dec.time

CE4.1.4:	RA:	-0.12% (Y), -0.63% (Cb), -0.53% (Cr)	102% enc.time	99% dec.time

LB:		-0.06% (Y), -0.04% (Cb), -0.09% (Cr)	103% enc.time	99% dec.time

CE4.1.5:	RA:	-0.18% (Y), -0.70% (Cb), -0.69% (Cr)	105% enc.time	100% dec.time

LB:		-0.04% (Y), -0.12% (Cb), -0.23% (Cr)	108% enc.time	101% dec.time

CE4.1.6:	RA:	-0.26% (Y), -0.85% (Cb), -0.85% (Cr)	102% enc.time	99% dec.time

CE4.1.7:	RA:	-0.33% (Y), -0.94% (Cb), -0.90% (Cr)	105% enc.time	99% dec.time

It can be seen that each aspect except the combination with DMVR contributes to the overall BD-rate.

# Switched half-pel interpolation filter

## Half-pel AMVR mode (CE4-1.1)

An additional AMVR mode for non-affine non-merge inter-coded CUs is proposed which allows signaling of motion vector differences at half-pel accuracy.  The existing AMVR scheme of the current VVC draft is extended straightforward in the following way: Directly following the syntax element amvr_flag, if amvr_flag == 1, there is a new context-modeled binary syntax element hpel_amvr_flag which indicates usage of the new half-pel AMVR mode if hpel_amvr_flag == 1. Otherwise, i.e. if hpel_amvr_flag == 0, the selection between full-pel and 4-pel AMVR mode is indicated by the syntax element amvr_precision_flag as in the current VVC draft.

The AMVR signalling is extended as follows:

VTM-5.0										Proposed

The purpose of this test is to investigate the gain that comes from HPEL AMVR extension without alternative luma half-pel interpolation filters.

## Alternative luma half-pel interpolation filters with merging (CE4-1.2, CE4-1.3)

In JVET-N0309, alternative luma half-pel interpolation filters are proposed where the following two 6-tap filters with smoothing characteristics, denoted as FlatTop and Gauss, are tested in CE4-1:

|  | Filter | Coefficients h[i] |
| --- | --- | --- |
|  | HEVC | [-1 4 -11 40 40 -11 4 -1] |
|  | Flat Top | [ -3 4 31 31 4 -3 ] |
|  | Gauss | [ 3 9 20 20 9 3 ] |

### Explicit signalling

For a non-affine and non-merge inter-coded CU which uses half-pel motion vector accuracy (i.e., the half-pel AMVR mode), an alternative luma half-pel interpolation filter is used. For test 1.2, The Gauss luma half-pel interpolation filter is used. For test 1.3, a switching between the two alternative half-pel interpolation filters is made based on the value of a new syntax element hpel_if_idx. The syntax element hpel_if_idx is only signaled in case of half-pel AMVR mode as follows:

| AMVR mode | hpel_if_idx | Interpolation filter |
| --- | --- | --- |
| QPEL, FPEL, 4PEL | not present (inferred to 2) | HEVC |
| HPEL | 0 | FlatTop |
| HPEL | 1 | Gauss |

### Implicit Signalling

In case of skip/merge mode using a spatial merging candidate, the information which interpolation filter is applied for the half-pel position is inherited from the neighbouring block.

## Alternative luma half-pel interpolation filters without merging (CE4-1.4, CE4-1.5)

In these two tests, the impact of the implicit signalling is investigated. The tests 1.4 and 1.5 correspond to tests 1.2 and 1.3, with the distinction that in tests 1.4 and 1.5 the information which interpolation filter is applied for the half-pel position is not inherited from the neighbouring block in case of skip/merge mode using a spatial merging candidate.

## Alternative luma half-pel interpolation filters with merging and without DMVR (CE4-1.6, CE4-1.7)

The tests 1.6 and 1.7 correspond to tests 1.2 and 1.3, with the distinction that in tests 1.6 and 1.7 the DMVR process is not applied to CUs which are encoded in spatial skip/merge mode and which have inherited the information from the neighbouring block that an alternative interpolation filter is applied for the half-pel position.

# Simulation Results

According to the common test conditions in JVET-N1010 [1], simulations were performed for random-access (RA) and low-delay B (LB) configuration with VTM-5.0 software.  The main parameters of the simulation environment are shown in Table 1.

Table 1 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

Note that for the RA configuration, the half-pel AMVR mode as well as the switched half-pel interpolation filter are disabled for the non-reference pictures (i.e., pictures having temporal_id == 4).

Furthermore, the half-pel AMVR mode and the switched half-pel interpolation filter are tested in all configurations only if the best RD cost of a full-pel AMVR mode is smaller or equal than the best available RD cost for the current partition multiplied by a factor T, where T is set as shown in Table 2.

Table 2 Values of factor T used in the experiments

| Test | T |
| --- | --- |
| CE4-1.2, CE4-1.4, CE4-1.6 | 1.25 |
| CE4-1.3, CE4-1.5, CE4-1.7 | 1.35 |

## CE4-1.1

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.20% | 0.01% | 103% | 100% |
| Class A2 | -0.08% | -0.16% | -0.10% | 103% | 99% |
| Class B | -0.05% | -0.07% | -0.28% | 102% | 102% |
| Class C | -0.04% | -0.09% | -0.16% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.12% | -0.16% | 103% | 100% |
| Class D | -0.04% | 0.06% | 0.24% | 102% | 101% |
| Class F | -0.01% | 0.05% | -0.02% | 101% | 101% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,04% | -0,23% | 104% | 99% |
| Class C | 0,05% | -0,10% | -0,15% | 103% | 100% |
| Class E | 0,02% | 0,35% | -0,46% | 103% | 102% |
| Overall | 0,01% | 0,04% | -0,26% | 103% | 100% |
| Class D | 0,00% | -0,07% | 0,17% | 103% | 99% |
| Class F | 0,05% | -0,11% | -0,19% | 100% | 99% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,02% | 0,10% | 0,15% | 101% | 98% |
| Class C | -0,01% | -0,10% | 0,05% | 102% | 101% |
| Class E | 0,15% | 0,79% | 0,28% | 100% | 101% |
| Overall | 0,02% | 0,21% | 0,15% | 101% | 100% |
| Class D | 0,00% | -0,08% | -0,23% | 102% | 101% |
| Class F | 0,03% | 0,28% | -0,13% | 100% | 100% |

## CE4-1.2

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.80% | -0.53% | 101% | 98% |
| Class A2 | -0.22% | -1.27% | -0.87% | 102% | 98% |
| Class B | -0.33% | -0.88% | -1.17% | 101% | 100% |
| Class C | -0.25% | -0.40% | -0.63% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.81% | -0.84% | 101% | 99% |
| Class D | -0.10% | -0.26% | -0.03% | 101% | 100% |
| Class F | -0.06% | -0.05% | -0.06% | 101% | 100% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | 0.23% | -0.25% | 103% | 100% |
| Class C | -0.16% | -0.30% | -0.37% | 103% | 101% |
| Class E | 0.02% | -0.08% | -0.18% | 101% | 100% |
| Overall | -0.10% | -0.02% | -0.27% | 103% | 100% |
| Class D | -0.08% | -0.18% | 0.88% | 102% | 98% |
| Class F | -0.10% | -0.52% | -0.30% | 102% | 102% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,67% | -1,19% | -0,87% | 101% | 97% |
| Class C | -0,45% | -0,56% | -0,07% | 102% | 100% |
| Class E | -0,48% | 1,12% | -0,12% | 102% | 102% |
| Overall | -0,55% | -0,40% | -0,42% | 102% | 99% |
| Class D | -0,38% | -0,38% | -0,66% | 102% | 100% |
| Class F | -0,26% | 0,62% | -0,29% | 102% | 101% |

## CE4-1.3

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -1.11% | -0.62% | 104% | 101% |
| Class A2 | -0.31% | -1.35% | -1.09% | 106% | 98% |
| Class B | -0.42% | -1.04% | -1.25% | 104% | 101% |
| Class C | -0.30% | -0.53% | -0.66% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.98% | -0.93% | 104% | 99% |
| Class D | -0.18% | -0.30% | -0.16% | 103% | 100% |
| Class F | -0.10% | -0.09% | -0.11% | 102% | 101% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | 0.06% | -0.05% | 107% | 98% |
| Class C | -0.18% | -0.20% | -0.06% | 107% | 100% |
| Class E | 0.02% | 0.05% | -0.03% | 102% | 99% |
| Overall | -0.12% | -0.03% | -0.05% | 106% | 99% |
| Class D | -0.19% | -0.15% | 0.58% | 105% | 99% |
| Class F | -0.06% | -0.15% | -0.55% | 103% | 99% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,77% | -1,20% | -1,19% | 103% | 98% |
| Class C | -0,53% | -0,62% | -0,30% | 105% | 101% |
| Class E | -0,54% | -0,01% | -0,13% | 103% | 101% |
| Overall | -0,63% | -0,71% | -0,62% | 104% | 99% |
| Class D | -0,45% | -0,12% | -0,36% | 104% | 100% |
| Class F | -0,23% | -0,06% | 0,31% | 104% | 101% |

## CE4-1.4

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.78% | -0.26% | 102% | 99% |
| Class A2 | -0.12% | -1.02% | -0.70% | 103% | 98% |
| Class B | -0.13% | -0.60% | -0.78% | 102% | 100% |
| Class C | -0.14% | -0.24% | -0.30% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.63% | -0.53% | 102% | 99% |
| Class D | -0.04% | -0.18% | 0.15% | 102% | 101% |
| Class F | -0.05% | -0.01% | -0.04% | 101% | 100% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,05% | 0,04% | 0,21% | 104% | 100% |
| Class C | -0,04% | -0,03% | -0,35% | 103% | 99% |
| Class E | -0,11% | -0,19% | -0,25% | 102% | 99% |
| Overall | -0,06% | -0,04% | -0,09% | 103% | 99% |
| Class D | -0,08% | -0,35% | 0,18% | 101% | 97% |
| Class F | -0,01% | -0,04% | -0,37% | 102% | 99% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,32% | -0,39% | -0,84% | 102% | 98% |
| Class C | -0,31% | -0,36% | -0,01% | 102% | 100% |
| Class E | -0,33% | 0,36% | -0,44% | 99% | 98% |
| Overall | -0,32% | -0,19% | -0,46% | 101% | 99% |
| Class D | -0,24% | -0,12% | 0,85% | 101% | 98% |
| Class F | -0,10% | 0,34% | 0,87% | 102% | 100% |

## CE4-1.5

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.61% | -0.38% | 105% | 100% |
| Class A2 | -0.18% | -1.19% | -0.81% | 106% | 99% |
| Class B | -0.19% | -0.68% | -0.94% | 106% | 101% |
| Class C | -0.19% | -0.43% | -0.51% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.18% | -0.70% | -0.69% | 105% | 100% |
| Class D | -0.10% | -0.19% | -0.02% | 104% | 100% |
| Class F | -0.07% | -0.07% | -0.06% | 103% | 101% |

|  |  |  | Low Delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,10% | -0,09% | -0,14% | 109% | 100% |
| Class C | -0,05% | -0,16% | -0,02% | 108% | 101% |
| Class E | 0,06% | -0,09% | -0,66% | 106% | 102% |
| Overall | -0,04% | -0,12% | -0,23% | 108% | 101% |
| Class D | -0,13% | -0,18% | 0,52% | 106% | 100% |
| Class F | -0,07% | -0,03% | -0,02% | 105% | 101% |

|  |  |  | Low Delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,34% | -0,56% | -0,98% | 105% | 99% |
| Class C | -0,37% | -0,49% | -0,32% | 106% | 102% |
| Class E | -0,39% | 0,18% | 0,16% | 103% | 101% |
| Overall | -0,36% | -0,35% | -0,48% | 105% | 100% |
| Class D | -0,28% | -0,15% | 0,08% | 104% | 101% |
| Class F | -0,14% | 0,11% | 0,79% | 104% | 101% |

## CE4-1.6

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.91% | -0.57% | 102% | 100% |
| Class A2 | -0.22% | -1.29% | -0.89% | 103% | 98% |
| Class B | -0.34% | -0.92% | -1.19% | 102% | 101% |
| Class C | -0.24% | -0.39% | -0.61% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.85% | -0.85% | 102% | 99% |
| Class D | -0.11% | -0.29% | -0.08% | 102% | 99% |
| Class F | -0.06% | -0.10% | -0.08% | 101% | 100% |

## CE4-1.7

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM_5_0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -1.03% | -0.58% | 105% | 98% |
| Class A2 | -0.29% | -1.33% | -1.02% | 106% | 98% |
| Class B | -0.43% | -1.00% | -1.22% | 104% | 100% |
| Class C | -0.29% | -0.52% | -0.66% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.94% | -0.90% | 105% | 99% |
| Class D | -0.16% | -0.29% | -0.19% | 104% | 100% |
| Class F | -0.11% | -0.10% | -0.08% | 102% | 98% |

- References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors), “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).