| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0058-v1 |
| --- | --- |

| Title: | Non-CE5: Simplification on ALF clipping parameter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Jungwon Kang Hahyun Lee Jinho Lee Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr hykim5@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution presents a simplification on clipping parameter coding in the adaptive loop filter (ALF). In WD 5.0, K-th order Exponential Golomb code is used to code ALF clipping parameter for both luma and chroma in the non-linear ALF. The proposed methods use fixed length code and truncated unary code for ALF clipping parameters for both luma and chroma. The test 1 of the proposed method replaces K-th order Exponential Golomb code with fixed length code. And, the test 2 of the proposed method replaces K-th order Exponential Golomb code with truncated unary code. Both tests of the proposed method remove 4 syntax elements and related semantics used to derive the order K of K-th order Exponential Golomb code. Compared to VTM-5.0, it is reported that the test 1 of the proposed method shows average BD-rates of -0.01%, -0.02%, and 0.00% for AI, RA, and LDB cases, respectively. And, the test 2 of the proposed method shows average BD-rates of -0.02%, -0.02%, and 0.01% for AI, RA, and LDB cases, respectively.

Introduction

In WD 5.0 [1], the adaptive loop filter (ALF) parameters are signalled in adaptation parameter set (APS) [2]. In one APS, up to 25 sets of luma filter coefficients and clipping value indices, and up to 1 set of chroma filter coefficients and clipping value indices are signalled for the non-linear ALF [3]. The clipping value indices, which are decoded from the APS, allow determining clipping values for sample value differences between the center sample and non-center samples. And, for each clipping value index, the value of clipping value index is in the range of 0 to 3.

As the clipping parameters are signalled per filter coefficient, up to 12 clipping indices for 1 set of luma filter and up to 6 clipping indices for 1 set of chroma filter are signalled by using K-th order Exponential Golomb code. In the worst case, 12 * 25 clipping indices for luma filters and 6 clipping indices for chroma filter should be signalled. As K-th order Exponential Golomb code is not efficient for syntax element which has a small range of values, this contribution proposes to use fixed length code and truncated unary code for ALF clipping parameters for both luma and chroma.

Proposed Method

For ALF clipping parameter coding for both luma and chroma, the test 1 of the proposed method replaces K-th order Exponential Golomb code with fixed length code u(2). And, the test 2 of the proposed method replaces K-th order Exponential Golomb code with truncated unary code tu(v) with maximum value 3.

Both tests of the proposed method can remove the following syntax elements and related semantics used to derive the order K of K-th order Exponential Golomb code.

- alf_luma_clip_min_eg_order_minus1

- alf_luma_clip_eg_order_increase_flag[ i ]

- alf_chroma_clip_min_eg_order_minus1

- alf_chroma_clip_eg_order_increase_flag[ i ]

Test 1 (fixed length code instead of K-th order Exponential Golomb code)

The following shows the proposed draft text changes based on Draft 5 v8 when using the test 1 of the proposed method:

7.3.5.3	Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)u(2) |
| } |  |
| } |  |
| … |  |

7.4.6.3	Adaptive loop filter data semantics

…

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

…

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

…

Test 2 (truncated unary code instead of K-th order Exponential Golomb code)

The following shows the proposed draft text changes based on Draft 5 v8 when using the test 2 of the proposed method:

7.3.5.3	Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| … |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)tu(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)tu(v) |
| } |  |
| } |  |
| … |  |

7.4.6.3	Adaptive loop filter data semantics

…

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

…

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

…

Experiment results

The experiments were performed based on VTM-5.0 software [4] with VTM configuration files under JVET common test condition [5]. And, the experiments were conducted on the platform of Intel Xeon E5-2690 v2 (3.0GHz) and 64GB RAM with GCC 7.3.1 compiler under CentOS Linux.

The experimental results of the proposed methods compared to VTM-5.0 anchor are summarized in Table I and Table II. For the test 1, the average BD-rates of -0.01%, -0.02%, and 0.00% are observed for AI, RA, and LDB cases, respectively. And, for the test 2, the average BD-rates of -0.02%, -0.02%, and 0.01% are observed for AI, RA, and LDB cases, respectively. Thus, it can be noted that the proposed methods introduce a small coding performance improvement. And, the detailed results can be found in accompanied Excel files.

Table I. Experimental result of the proposed method (Test 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | 0.00% | 0.00% | 98% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class E | -0.02% | -0.01% | -0.02% | 100% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.02% | 0.01% | -0.02% | 100% | 100% |
| Class F | -0.02% | -0.01% | -0.01% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | -0.04% | 100% | 101% |
| Class A2 | -0.01% | -0.05% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Class C | -0.04% | -0.11% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.05% | 100% | 100% |
| Class D | -0.04% | 0.00% | 0.04% | 100% | 100% |
| Class F | -0.03% | 0.05% | 0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.21% | 0.02% | 100% | 100% |
| Class C | 0.03% | 0.10% | -0.21% | 101% | 100% |
| Class E | -0.02% | 0.39% | -0.30% | 100% | 100% |
| Overall | 0.00% | 0.22% | -0.14% | 100% | 100% |
| Class D | -0.07% | -0.55% | 1.51% | 101% | 100% |
| Class F | 0.04% | 0.05% | 0.82% | 101% | 100% |

Table II. Experimental result of the proposed method (Test 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.03% | -0.01% | -0.01% | 100% | 100% |
| Class E | -0.03% | -0.02% | -0.03% | 100% | 100% |
| Overall | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.03% | 0.01% | -0.02% | 100% | 100% |
| Class F | -0.02% | -0.01% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.15% | 0.01% | 100% | 101% |
| Class A2 | -0.03% | -0.04% | -0.02% | 100% | 100% |
| Class B | -0.01% | 0.01% | -0.11% | 100% | 100% |
| Class C | -0.04% | 0.00% | -0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.07% | 100% | 100% |
| Class D | -0.02% | -0.06% | 0.07% | 100% | 100% |
| Class F | -0.03% | 0.03% | 0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.26% | 0.13% | 100% | 100% |
| Class C | 0.00% | 0.08% | -0.12% | 100% | 100% |
| Class E | 0.07% | 0.09% | -0.14% | 100% | 100% |
| Overall | 0.01% | 0.15% | -0.02% | 100% | 100% |
| Class D | -0.05% | -0.39% | 0.37% | 101% | 100% |
| Class F | 0.01% | -0.18% | -0.20% | 100% | 100% |

Conclusions

In this contribution, a simplification on clipping parameter coding in the adaptive loop filter (ALF) is proposed. The proposed methods use fixed length code and truncated unary code for ALF clipping parameters for both luma and chroma. The test 1 of the proposed method replaces K-th order Exponential Golomb code with fixed length code. And, the test 2 of the proposed method replaces K-th order Exponential Golomb code with truncated unary code. Both tests of the proposed method remove 4 syntax elements and related semantics used to derive the order K of K-th order Exponential Golomb code. The experimental result reportedly shows that the test 1 of the proposed method shows average BD-rates of -0.01%, -0.02%, and 0.00% for AI, RA, and LDB cases, respectively, and the test 2 of the proposed method shows average BD-rates of -0.02%, -0.02%, and 0.01% for AI, RA, and LDB cases, respectively. Therefore, it is recommended to adopt this proposal to clean up the current non-linear adaptive loop filter design.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1001, Mar. 2019

- W. Wan, P. Yin, Hendry, S. Paluri, J. Zhao, J. Lim, S. Kim, W. Choi, K. Choi, and K. Choi, “AHG17: Design for signalling reshaper model,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0805, Mar. 2019

- J. Taquet, P. Onno, C. Gisquet, G. Laroche, “CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0242, Mar. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).