| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0059 |
| --- | --- |

| Title: | CE8: Palette mode with neighboring pixel copy (CE8-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tsui-Shan Chang, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd |  |  |

_____________________________

- Abstract

This document proposed palette mode with neighboring pixel copy (CE8-2.2) and reports the results on top of VTM-5.0. The CE results show that:

For 4:2:0 test sequences,

- the proposed method provides 5.02%, 2.02%, and 0.40% BD-rate gains for luma in TGM sequences, with 103%, 102% and 107% in encoding time, and 96%, 101% and 103% in decoding time under AI, RA, and LD configurations, respectively.

- the proposed method provides 0.75%, 0.67%, and 0.32% BD-rate gains for luma in class F sequences, with 109%, 104% and 110% for encoding time, and 107%, 102% and 106% in decoding time under AI, RA, and LD configurations, respectively.

For 4:4:4 test sequences,

- when DualITree turned off, the proposed method achieves 17.84%, 11.72%, and 7.29% BD-rate gain in TGM 1080p sequences, and 6.19%, 4.50%, and 2.34% BD-rate gains overall for luma, with 104%, 98% and 105% in encoding time, and 99%, 94% and 98% in decoding time under AI, RA, and LD configurations, respectively.

- when DualITree turned on, the proposed method achieves 7.35%, 6.17%, and 5.87% BD-rate gain in TGM 1080p sequences, and 2.37%, 1.99%, and 1.98% BD-rate gains overall for luma with 110%, 103% and 107s% encoding time, and 101%, 100% and 100% in decoding time under AI, RA, and LD configurations, respectively.

CE Description

In test CE8-2.2, the method combining palette mode and neighboring pixel copy is tested. When the palette mode is used for a CU and the number of pixels in the CU is greater than or equal to 64, the decoder will first decode a flag indicating whether neighboring pixel copy is enabled. For CUs in which number of pixels is less than 64, the proposed pixel copy disallowed to increase decoder throughput. If the proposed pixel copy is enabled, the decoder then decodes another direction flag to indicate whether horizontal or vertical intra prediction in used. Finally, the decoder decodes a palette and an index map. A specific index (0) is reserved to indicate the sample is reconstructed from intra prediction. When decoder reconstruct the palette coded block, samples with the index, 0, are reconstructed by the horizontal or vertical nearest samples across CU boundary, and samples with other indices are reconstructed by the colors in the palette. Figure 1 shows an example.

(a) vertical intra prediction combination 		 (b) horizontal intra prediction combination

Figure 1. An example of palette mode and horizontal/vertical intra prediction combination.

Simulation results

Test sequences 4:2:0

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The DualITree is turned on for all Classes. The IBC, HashME, and RDPCM are turned on only for Class F and Class TGM. Table 1 shows the results of CE8.2.2 compared with VTM5.0 on test sequences 4:2:0.

Table 1: Results of the CE8.2.1 on test sequences 4:2:0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.02% | 111% | 106% |
| Class A2 | 0.05% | 0.10% | 0.02% | 112% | 109% |
| Class B | 0.05% | 0.01% | 0.09% | 110% | 109% |
| Class C | 0.06% | 0.02% | 0.04% | 108% | 110% |
| Class E | 0.05% | 0.06% | -0.03% | 110% | 106% |
| Overall | 0.04% | 0.03% | 0.03% | 110% | 108% |
| Class D | 0.04% | 0.20% | 0.00% | 107% | 106% |
| Class F | -0.75% | -0.76% | -0.75% | 109% | 107% |
| TGM | -5.02% | -4.95% | -4.32% | 103% | 96% |
|  | Random Access |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.10% | 0.12% | 102% | 102% |
| Class A2 | 0.05% | 0.07% | 0.04% | 102% | 102% |
| Class B | 0.04% | 0.02% | -0.20% | 101% | 102% |
| Class C | 0.02% | -0.01% | -0.08% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | -0.06% | 101% | 101% |
| Class D | 0.06% | -0.28% | 0.01% | 102% | 103% |
| Class F | -0.67% | -0.89% | -0.87% | 104% | 102% |
| TGM | -2.02% | -2.78% | -2.43% | 102% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.39% | 0.21% | 105% | 105% |
| Class C | 0.05% | 0.24% | 0.02% | 106% | 108% |
| Class E | 0.06% | 0.12% | -0.03% | 104% | 106% |
| Overall | 0.04% | 0.27% | 0.09% | 105% | 106% |
| Class D | 0.09% | -0.14% | 0.91% | 104% | 103% |
| Class F | -0.32% | -1.07% | -1.12% | 110% | 106% |
| TGM | -0.40% | -0.54% | -0.51% | 107% | 103% |

Test sequences 4:4:4

The proposed method is evaluated according to the CE8 description [3] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The IBC, HashME, and RDPCM are turned on for all classes. Table 2 shows the results with DualITree turned off while Table 3 shows the results with DualITree turned on.

Table 2: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -17.84% | -24.29% | -23.89% | 101% | 91% |
| TGM 720p | -4.56% | -10.59% | -12.59% | 104% | 96% |
| Animation | 0.05% | -0.28% | -0.22% | 104% | 103% |
| Mixed content | -1.13% | -5.32% | -6.14% | 107% | 106% |
| Camera-Captured content | 0.01% | 0.01% | 0.04% | 105% | 105% |
| Overall | -6.19% | -10.40% | -10.98% | 104% | 99% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -11.72% | -18.55% | -18.15% | 91% | 87% |
| TGM 720p | -4.94% | -9.93% | -12.49% | 99% | 94% |
| Animation | 0.04% | -0.18% | -0.13% | 103% | 98% |
| Mixed content | -0.33% | -3.65% | -4.40% | 103% | 99% |
| Camera-Captured content | -0.01% | 0.25% | 0.18% | 101% | 100% |
| Overall | -4.50% | -8.32% | -9.05% | 98% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.29% | -12.27% | -12.00% | 104% | 97% |
| TGM 720p | -2.02% | -6.73% | -7.78% | 103% | 98% |
| Animation | 0.06% | -0.28% | 0.01% | 105% | 96% |
| Mixed content | 0.67% | -2.73% | -3.38% | 104% | 100% |
| Camera-Captured content | -0.03% | 0.04% | -0.12% | 103% | 100% |
| Overall | -2.34% | -5.65% | -5.97% | 104% | 98% |

Table 3: Results of the CE8.2.1 on test sequences 4:4:4, DualITree = 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -7.35% | -14.56% | -14.45% | 115% | 98% |
| TGM 720p | -1.67% | -6.95% | -8.47% | 109% | 100% |
| Animation | 0.03% | -1.09% | -0.76% | 109% | 108% |
| Mixed content | 0.10% | -5.73% | -5.52% | 109% | 104% |
| Camera-Captured content | 0.06% | 0.03% | 0.02% | 105% | 102% |
| Overall | -2.37% | -7.02% | -7.31% | 110% | 101% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -6.17% | -14.30% | -13.84% | 96% | 92% |
| TGM 720p | -1.02% | -7.33% | -8.80% | 106% | 104% |
| Animation | 0.05% | -0.47% | -0.43% | 108% | 107% |
| Mixed content | -0.45% | -6.73% | -5.93% | 106% | 102% |
| Camera-Captured content | 0.06% | 0.11% | -0.05% | 104% | 104% |
| Overall | -1.99% | -7.16% | -7.29% | 103% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -5.87% | -11.47% | -10.97% | 106% | 99% |
| TGM 720p | -1.33% | -6.08% | -7.58% | 105% | 98% |
| Animation | 0.00% | -0.31% | -0.51% | 109% | 103% |
| Mixed content | -0.35% | -6.80% | -6.19% | 107% | 100% |
| Camera-Captured content | 0.05% | 0.04% | -0.02% | 108% | 108% |
| Overall | -1.98% | -6.07% | -6.25% | 107% | 100% |

- Reference

- Y.-H. Chao, V. Seregin, M. Karczewicz, Y.-C. Sun, T.-S. Chang, J. Lou, “CE8: Palette Mode in HEVC (CE8-2.1)”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang, “CE8-related: Palette Mode Coding”, JVET-N0258, 14th meeting, Geneva, CH, March 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-N1028, 14th meeting, Geneva, CH, March 2019.

- Y.-C. Sun, T.-S. Chang, J. Lou, Y.-H. Chao, V. Seregin, M. Karczewicz, R. Chernyak, S. Ikonin, J. Chen “CE8: Palette Mode Improvements in HEVC (CE8-2.2)”, JVET-N0404, 14th meeting, Geneva, CH, March 2019.

- Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).