| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0060-v1 |
| --- | --- |

| Title: | CE5-2.1 and CE5-2.2: Deblocking on 4x4 sample grids |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jack Enhorn, Zhi Zhang, Rickard Sjöberg, | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
|  | Anand Meher Kotra, Semih Esenlik, Biao Wang, Han Gao, Elena Alshina | Email: | Anand.meher.kotra@huawei.com |
|  | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Ericsson, Huawei Technologies. Co., Ltd, MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE5-2.1 and CE5-2.2, which is a joint proposal from Ericsson, Huawei and MediaTek Inc. VTM5.0 does not apply horizontal deblocking filter to the current coding block (CB) and internal vertical edges if the left side of the current CB is not lying on 8x8 sample grid. Similarly, the vertical deblocking filter is not applied to the current CB and internal horizontal edges if the top side of the current CB is not lying on 8x8 sample grid. CE5-2.1 applies luma deblocking on 4x4 sample grids to not only allow deblocking of Nx4, 4xN blocks aligned on 8x8 sample grids (N can be upto 64) but also for other blocks that are not aligned on 8x8 sample grids. For a given edge, if at least one of the blocks sharing the edge has a size of 4 samples (orthogonally), then only weak filtering decision and operations are performed. Furthermore, the number of samples to-be-read is three and samples to-be-modify is one for parallel friendly processing. CE5-2.2 also applies luma deblocking on 4x4 sample grids, but the chroma deblocking is also performed on 4x4 sample grids.

Subjective analysis shows that when compared to the Anchor (VTM5.0), the proposed test improves the subjective quality of sequences.

Objective results of the proposed methods are reported as follows:

CE5-2.1: luma deblocking on 4x4 sample grids + chroma deblocking on 8x8 sample grids
Over VTM5.0 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, -0.20%, -0.17%, -0.33% is achieved with negligible encoding time and decoding time.
Over VTM5.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.03%, -0.08%, -0.11%, -0.15% is achieved with negligible encoding time and decoding time.

CE5-2.2: luma deblocking on 4x4 sample grids + chroma deblocking on 4x4 sample grids
Over VTM5.0 Anchor with ALF OFF (AI, RA, LDB, LDP): Luma BD-Rate of -0.01%, -0.19%, -0.16%, -0.32% is achieved with negligible encoding time and decoding time.
Over VTM5.0 Anchor with ALF ON (AI, RA, LDB, LDP): Luma BD-Rate of 0.03%, -0.09%, -0.09%, -0.16% is achieved with negligible encoding time and decoding time.

# Introduction

VTM5.0 performs deblocking on 8 x 8 sample grids. Therefore deblocking is only performed for all the block edges which overlap with the 8 x 8 sample grids. Due to the VTM5.0 partitioning scheme (Binary-tree (BT), Ternary-tree (TT), Intra Sub Partitions, Sub Block Transforms), some long CU edges which may not overlap with the 8 x 8 sample grids but still have sufficient blocking artifacts. The scenario is depicted in the Fig. 1 below. The edge shown in red color belongs to a 4 x 16 block and current VTM5.0 deblocking does not perform deblocking on this edge, as it does not overlap with an 8 x 8 sample grid.

Figure 1.  VTM5.0 performs deblocking on 8 x 8 sample grid.

# Proposed Method

## CE5-2.1: luma deblocking on 4x4 sample grids + chroma deblocking on 8x8 sample grids

As shown in Fig. 2, the proposed method enforces the luma weak filtering with one or no sample modification, when any of the blocks sharing the edge has a size of 4 samples. (i.e. width = 4 samples for vertical edge, height = 4 samples for horizontal edge).

The different steps performed for a vertical edge are illustrated below:

- Check if filter on/off condition (i.e. d < Beta, where d = dpq0 + dpq3 and dpq0 = dp0 + dq0 and dpq3 = dp3 + dq3) is evaluated to true for the respective edge. If true, then

- Check if the one of the blocks has a block width of 4 samples. If true, then

- Enforce weak filter with maximum one sample modification. Therefore the following condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8)≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

Figure 2.  Weak filter with one sample modification for 4xN, Nx4 and 8xN, Nx8 blocks.

## CE5-2.2: luma deblocking on 4x4 sample grids + chroma deblocking on 4x4 sample grids

CE5-2.2 is built on top of CE5-2.1, but the chroma deblocking is performed on 4x4 sample grids without changing VTM5.0 chroma deblocking decisions and filtering operations.

# Complexity evaluation

Complexity of the proposal is evaluated based on the following criteria described in the CE5 document, JVET-N1025 [1].

## Deblocking complexity of CE5-2.1

Luma deblocking complexity:

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - For blocks size ≥ 8 samples: it is the same as in VTM5.0. Except when the edge is shared with a block whose size is 4 samples. Then only one sample is modified

  - For block size < 8 samples: one sample is modified

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size ≥ 8 samples: 4 samples are used (same as in VTM5.0)

  - For block size < 8 samples: Three samples are only used.

- Number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

  - For all 4x4: 16 operation (6 for addition, 2 for multiplication, 5 for multiplication, and 3 for comparison)

- Number of line buffers:

  - 4 lines (same as VTM5.0)

- Parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

  - All edges overlapping with 4 x 4 grid can be deblocked in parallel

- Whether proposals increase the worst case complexity and if so, by how much: No

Chroma deblocking complexity:

- Chroma filtering is not changed

## Deblocking complexity of CE5-2.2

Luma deblocking complexity:

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - For blocks size ≥ 8 samples: it is the same as in VTM5.0. Except when the edge is shared with a block whose size is 4 samples. Then only one sample is modified

  - For block size < 8 samples: one sample is modified

- Number of samples from the block boundary that are needed for deblocking decisions:

  - For blocks size ≥ 8 samples: 4 samples are used (same as in VTM5.0)

  - For block size < 8 samples: Three samples are only used.

- Number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

  - All 4x4: 16 operation (6 for addition, 2 for multiplication, 5 for multiplication, and 3 for comparison)

- Number of line buffers:

  - 4 lines (same as VTM5.0)

- Parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

  - All edges overlapping with 4 x 4 grid can be deblocked in parallel

- Whether proposals increase the worst case complexity and if so, by how much: No

Chroma deblocking complexity:

- Number of samples from the block boundary that can be modified by deblocking (P+Q):

  - Same as VTM5.0

- Number of samples from the block boundary that are needed for deblocking decisions:

  - Same as VTM5.0

- Number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

  - Same as VTM5.0

- Number of line buffers: Same as VTM5.0

- Parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

  - All edges overlapping with 4 x 4 grid can be deblocked in parallel

- Whether proposals increase the worst case complexity and if so, by how much: No

# Subjective Evaluation

To illustrate the benefit of 4x4 deblocking we show two examples.

Fig. 3a below shows a blocking artifact, observed on VTM-5.0 for the following configuration: FourPeople Sequence, POC 241, configuration is LDP, QP 37, with ALF OFF. The size of the coding unit is 32 x 16 and is further split using tripe tree portioning. The artifact results because the horizontal coding unit edge does not overlap with the 8 x 8 sample grid.

As depicted in Fig. 3b, since deblocking is performed on 4 x 4 sample gird in CE 5-2.1, the corresponding coding unit edge overlaps on the sample grid and therefore deblocked. As a result the blocking artifact is not observable any more.

Edge not deblocked

Fig 3a. Anchor, Horizontal edge of CU with size (32 x 4) is not delocked as it does not overlap on 8 x 8 sample grid

Edge deblocked

Fig 3b. CE 5-2.1, Horizontal edge of CU with size (32 x 4) deblocked, as it overlaps on 4 x 4 sample grid

Below is another example of visual artefacts due to deblocking on an 8x8 grid when artefacts not overlap 8x8 grid, Cactus at POC 0 in LDB configuration and at QP 37:

Figure 4a: VTM-5.0 Cactus POC 0 QP 37, Left ALF OFF, Right: ALF ON.

Then when instead deblocking is applied on an 4x4 grid as in CE5-2.1 and CE5-2.2 visual artefacts can be reduced as seen in Figure 4b and 4c. Most effect comes from deblocking luma on 4x4 grid.

Figure 4b: CE5-2.1 Cactus POC 0 QP 37, Left: ALF OFF, Right: ALF ON.

Figure 4c: CE5-2.2 Cactus POC 0 QP 37, Left: ALF OFF, Right: ALF ON.

# Experimental results

The proposed method is implemented on top of VTM5.0 [2] and the simulation is performed under common test conditions as described in JVET-N1010 [3]. Table 1 and Table 2 show the CE5-2.1 results for both ALF ON and ALF OFF. Table 3 and Table 4 show the CE5-2.2 results for both ALF ON and ALF OFF. Decoding time was measured on same machine for anchor and test and without yuv output. Encoding time was measured on machines with similar capabilities for anchor and test.

## YCbCr 4:2:0

Table 1.   Objective results of CE5-2.1 on top of VTM5.0 for CTC configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,00% | 0,01% | 102% | 98% |
| Class A2 | 0,03% | 0,00% | 0,00% | 101% | 100% |
| Class B | 0,06% | 0,00% | 0,00% | 99% | 99% |
| Class C | 0,05% | 0,00% | 0,00% | 107% | 101% |
| Class E | -0,03% | 0,00% | 0,00% | 100% | 101% |
| Overall | 0,03% | 0,00% | 0,00% | 102% | 100% |
| Class D | 0,05% | 0,00% | 0,01% | 99% | 104% |
| Class F | 0,00% | 0,00% | 0,00% | 100% | 102% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,04% | -0,13% | 0,08% | 101% | 101% |
| Class A2 | -0,16% | -0,03% | -0,05% | 101% | 98% |
| Class B | -0,04% | -0,05% | -0,04% | 101% | 100% |
| Class C | -0,12% | 0,05% | -0,15% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,08% | -0,03% | -0,05% | 100% | 100% |
| Class D | -0,02% | 0,03% | 0,26% | 98% | 102% |
| Class F | -0,05% | 0,06% | 0,03% | 102% | 99% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,09% | 0,15% | 0,39% | 101% | 98% |
| Class C | -0,13% | 0,17% | -0,10% | 101% | 96% |
| Class E | -0,12% | -0,17% | -0,56% | 96% | 103% |
| Overall | -0,11% | 0,08% | -0,01% | 100% | 99% |
| Class D | -0,01% | 0,08% | 0,62% | 104% | 101% |
| Class F | -0,09% | 0,30% | 0,18% | 99% | 102% |
|  | Low Delay P |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,08% | 0,11% | 0,21% | 100% | 101% |
| Class C | -0,15% | -0,30% | 0,10% | 104% | 103% |
| Class E | -0,26% | 0,62% | 0,34% | 104% | 101% |
| Overall | -0,15% | 0,10% | 0,21% | 102% | 102% |
| Class D | -0,04% | -0,08% | -0,17% | 98% | 100% |
| Class F | -0,11% | 0,11% | -0,26% | 100% | 104% |

Table 2. Objective results of CE5-2.1 on top of VTM5.0 for CTC with ALF switched off configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.01% | 96% | 101% |
| Class A2 | -0.01% | 0.00% | 0.00% | 96% | 101% |
| Class B | 0.02% | 0.00% | 0.00% | 94% | 101% |
| Class C | 0.02% | 0.01% | 0.00% | 99% | 102% |
| Class E | -0.13% | 0.00% | 0.00% | 96% | 100% |
| Overall | -0.01% | 0.00% | 0.00% | 96% | 101% |
| Class D | 0.01% | 0.00% | 0.00% | 91% | 101% |
| Class F | -0.02% | 0.00% | 0.00% | 95% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.08% | -0.07% | 96% | 100% |
| Class A2 | -0.40% | -0.04% | -0.07% | 98% | 100% |
| Class B | -0.12% | -0.07% | -0.15% | 99% | 99% |
| Class C | -0.17% | 0.03% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | -0.04% | -0.07% | 98% | 100% |
| Class D | -0.13% | 0.12% | -0.13% | 100% | 99% |
| Class F | -0.06% | 0.01% | 0.04% | 109% | 102% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.05% | -0.38% | 95% | 99% |
| Class C | -0.20% | 0.04% | -0.20% | 95% | 100% |
| Class E | -0.20% | -0.37% | -0.31% | 96% | 101% |
| Overall | -0.17% | -0.10% | -0.30% | 95% | 100% |
| Class D | -0.06% | 0.36% | -0.49% | 96% | 103% |
| Class F | -0.17% | 0.29% | -0.47% | 95% | 98% |
|  |  |  | Low Delay P |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.34% | -0.52% | 97% | 99% |
| Class C | -0.28% | -0.12% | -0.20% | 95% | 100% |
| Class E | -0.46% | 0.35% | 1.01% | 94% | 99% |
| Overall | -0.33% | -0.10% | -0.03% | 96% | 100% |
| Class D | -0.16% | -0.33% | -0.11% | 95% | 100% |
| Class F | -0.15% | 0.30% | -0.10% | 91% | 100% |

Table 3.   Objective results of CE5-2.2 on top of VTM5.0 for CTC configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.52% | 0.31% | 100% | 101% |
| Class A2 | 0.02% | 0.51% | 0.66% | 99% | 100% |
| Class B | 0.06% | 0.68% | 0.63% | 99% | 101% |
| Class C | 0.05% | 0.59% | 0.46% | 105% | 101% |
| Class E | -0.03% | 0.18% | 0.12% | 96% | 100% |
| Overall | 0.03% | 0.52% | 0.46% | 100% | 101% |
| Class D | 0.04% | 0.57% | 0.54% | 102% | 102% |
| Class F | 0.01% | 0.54% | 0.61% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,05% | 0,53% | 0,28% | 98% | 98% |
| Class A2 | -0,14% | 0,37% | 0,86% | 99% | 99% |
| Class B | -0,07% | 1,13% | 1,00% | 98% | 99% |
| Class C | -0,12% | 0,98% | 0,78% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,09% | 0,82% | 0,77% | 99% | 99% |
| Class D | -0,05% | 1,30% | 1,31% | 102% | 100% |
| Class F | -0,02% | 1,25% | 1,45% | 104% | 100% |
|  | Low Delay B |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,07% | -0,02% | -0,24% | 98% | 99% |
| Class C | -0,11% | -0,20% | -0,22% | 98% | 104% |
| Class E | -0,09% | 0,39% | -0,08% | 94% | 100% |
| Overall | -0,09% | 0,02% | -0,19% | 97% | 101% |
| Class D | 0,00% | 0,47% | 0,48% | 97% | 101% |
| Class F | -0,01% | 0,72% | 0,53% | 96% | 97% |
|  | Low Delay P |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,11% | 0,02% | -0,05% | 97% | 100% |
| Class C | -0,13% | -0,23% | -0,11% | 100% | 100% |
| Class E | -0,28% | 0,79% | -0,40% | 98% | 96% |
| Overall | -0,16% | 0,13% | -0,16% | 98% | 99% |
| Class D | -0,10% | 0,12% | 0,28% | 99% | 103% |
| Class F | -0,05% | 0,82% | 1,05% | 98% | 101% |

Table 4. Objective results of CE5-2.2 on top of VTM5.0 for CTC with ALF switched off configurations

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.37% | 0.02% | 101% | 102% |
| Class A2 | -0.02% | 0.07% | 0.32% | 101% | 101% |
| Class B | 0.02% | 0.51% | 0.31% | 100% | 102% |
| Class C | 0.02% | 0.31% | 0.07% | 102% | 103% |
| Class E | -0.13% | -0.12% | -0.39% | 94% | 96% |
| Overall | -0.01% | 0.26% | 0.09% | 100% | 101% |
| Class D | 0.00% | 0.34% | 0.11% | 99% | 97% |
| Class F | -0.01% | 0.44% | 0.50% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,15% | 0,51% | 0,04% | 98% | 100% |
| Class A2 | -0,39% | 0,83% | 1,45% | 98% | 100% |
| Class B | -0,13% | 1,87% | 1,73% | 97% | 102% |
| Class C | -0,15% | 1,91% | 1,76% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,19% | 1,40% | 1,34% | 98% | 101% |
| Class D | -0,13% | 2,43% | 2,23% | 99% | 103% |
| Class F | -0,04% | 1,56% | 1,86% | 107% | 102% |
|  |  |  | Low Delay B |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,11% | 0,14% | -0,11% | 98% | 100% |
| Class C | -0,18% | 0,27% | -0,26% | 101% | 100% |
| Class E | -0,21% | -0,90% | -0,93% | 101% | 99% |
| Overall | -0,16% | -0,08% | -0,37% | 100% | 100% |
| Class D | -0,08% | 0,63% | 0,12% | 101% | 103% |
| Class F | -0,17% | 0,37% | 0,76% | 101% | 103% |
|  |  |  | Low Delay P |  |  |
|  | Over VTM5.0 with ALF switched off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,27% | 0,14% | -0,47% | 98% | 100% |
| Class C | -0,26% | 0,21% | -0,18% | 101% | 102% |
| Class E | -0,46% | -0,24% | -0,52% | 103% | 101% |
| Overall | -0,32% | 0,07% | -0,39% | 100% | 101% |
| Class D | -0,18% | 0,44% | -0,04% | 99% | 101% |
| Class F | -0,14% | 0,17% | 0,30% | 98% | 101% |

## Additional decoding time measurement on YCbCR 4:4:4

Decoding time comparison in Table 5 was generated by geometrical average of 10 decodings.

Table 5: Decoding time measurement for 4:4:4 subjective test cases

|  | CE5-2.1No ALF | CE5-2.1 ALF | CE5-2.2No ALF | CE5-2.2 ALF |
| --- | --- | --- | --- | --- |
| Kimono (RA) | 100% | 101% | 101% | 100% |
| EBULupoAndCandleLight (LDB) | 102% | 101% | 100% | 101% |

# Conclusion

This contribution presents the results for CE5-2.1 and CE5-2.2. The proposed method applies deblocking on 4x4 grid to ensure that all blocks whose size is 4xN, Nx4, 8xN, Nx8 are deblocked and not only those that are aligned with the 8x8 sample grid. To ensure parallel processing, the method uses constrained version of the VTM5.0 weak filter. This filter only modifies one sample on either side of the edge, when one of the blocks sharing the edge has a size of 4 samples. The proposed method substantially improves the subjective quality when compared to VTM5.0. Objective results show a Luma BD-Rate gain of upto 0.33% for LDP configuration (ALF off). It is suggested to adopt this technique to the VVC draft specification and the VTM reference software.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)," Document of Joint Video Experts Team, JVET-N1001.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- C.-Y. Chen and A. Norkin, "Description of Core Experiment 5 (CE5): Loop filtering," Document of Joint Video Experts Team, JVET-N1025.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).