| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0061-v1 |
| --- | --- |

| Title: | CE5-3.1 Sub-sample MV threshold for deblocking decisions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports results of CE5-3.1. VTM-5.0 fails to remove blocking artifacts that origins from differences smaller than one sample for at least one motion vector component on respective side of a prediction block boundary since the threshold for deblocking decision is one sample. In this core experiment a threshold of half-sample can combat most visual artifacts occurring from small differences in motion in neighbouring blocks.

BDR results of 0.06%/0.03%/-0.02% for RA/LDB/LDP with ALF on and 0.02%/0.04%/-0.12% for RA/LDB/LDP with ALF off are reported.

Encoding time of 100%/99%/102% for RA/LDB/LDP with ALF on and 99%/101%/98% for RA/LDB/LDP with ALF off are reported.

Decoding time of 99%/101%/99% for RA/LDB/LDP with ALF on and 100%/100%/100% for RA/LDB/LDP with ALF off are reported.

# Introduction

When identical reference frames are used in motion compensation for both blocks adjacent to a prediction block boundary the motion vector components of the blocks are compared. If the difference is equal or larger than 1 (in luma samples) the boundary strength is set to 1 and deblocking can be applied to the boundary.

# Problem description

Consider a gradient in a reference picture. Then consider that this region is referenced by two blocks, block P and block Q, in the current picture, see Figure 2, where block P derives the prediction block by motion compensation the top half of the region and block Q derives the prediction block by motion compensation of the bottom half of the region.

Figure 1: A region of a reference picture				Figure 2: Block P and block Q in current picture

In figure 3 the result of applying motion compensation using VTM interpolation filter with a difference in the horizontal component of the motion vectors of less than one pixel for block P and Q when the vertical component of the motion vectors are identical. It can be seen that even a sub-sample difference in a motion vector component between block P and Q can result in boundary artifacts.

Figure 3: Illustration of two prediction blocks after motion compensation with a given difference in only the horizontal component of the motion vectors. Top left: 1/4 luma sample, Top right: 1/2 luma sample, Bottom left: 3/4 luma sample, Bottom right: 1 luma

# Proposed solution

It is proposed to set the MV component difference threshold for determination of boundary strength to 2 in units of quarter luma samples to enable more efficient reduction of blocking artifacts. This threshold is less conservative than the threshold used in JVET-N0359 [1] but it was found to be sufficient when conduction the core experiment [5].

# Specification change

The change in the VVC specification [2] is to modify the MV threshold for deblocking decision to a half-sample which is indicated by 2 (marked yellow) in units of quarter pel in section 8.8.3.5 shown below.

8.8.3.5	Derivation process of boundary filtering strength

..

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the motion vectors used in the prediction of the two coding subblocks is greater than or equal to 2 in units of quarter luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 2 in units of quarter luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 2 in units of quarter luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 2 in quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 2 in units of quarter luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 2 in units of quarter luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 2 in units of quarter luma samples.

..

# Results

## Subjective examples

On example where the VTM-5.0 deblocking is not applied for boundaries with motion differences smaller than one sample is shown on the left side of Figure 4 for a region of Johnny coded with LDB configuration when ALF is turned off. Using a MV threshold of half-sample enables deblocking of such boundaries as shown on the right side of Figure 4. The example is generated by decoding using VTM-5.0 until POC 74 and then for POC 75 apply CE5-3.1 deblocking.

In Figure 5 on the left side another example from Johnny is shown for the case when ALF is used. On the right side of Figure 5 the artifact is removed when CE5-3.1 deblocking is used. The example is generated by decoding using VTM-5.0 until POC 72 and then for POC 73 apply CE5-3.1 deblocking.

Figure 4: Johnny LDB for POC 75 ALF OFF, Left: VTM-5.0, Right: CE5-3.1

Figure 5: Johnny LDB for POC 73 ALF ON, Left: VTM-5.0, Right: CE5-3.1

## Objective results over CTC

The objective performance of the proposal is shown below in comparison to VTM-5.0 [3] on both anchor and test. The objective results show a small loss for random access and low delay B and a small gain for LDP. Encoding is measured on simulations run on a cluster with machines with same capability. Decoding time is measured by running anchor and test on the same machine and without YUV output.

|  | Random access Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | -0,14% | -0,01% | 100% | 96% |
| Class A2 | 0,09% | 0,05% | -0,04% | 100% | 100% |
| Class B | 0,08% | 0,06% | 0,01% | 99% | 101% |
| Class C | 0,02% | 0,04% | -0,15% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,06% | 0,01% | -0,05% | 100% | 99% |
| Class D | 0,08% | -0,08% | 0,04% | 106% | 100% |
| Class F | 0,01% | 0,01% | 0,03% | 98% | 100% |
|  | Low delay B Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | 0,02% | 0,03% | 100% | 103% |
| Class C | 0,04% | 0,12% | 0,17% | 101% | 100% |
| Class E | 0,07% | -0,09% | -0,59% | 96% | 101% |
| Overall | 0,03% | 0,03% | -0,08% | 99% | 101% |
| Class D | 0,05% | -0,25% | 0,96% | 98% | 99% |
| Class F | 0,02% | 0,04% | -0,41% | 100% | 100% |
|  | Low delay P Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | -0,13% | -0,10% | 100% | 99% |
| Class C | -0,03% | -0,17% | 0,31% | 101% | 99% |
| Class E | -0,03% | 0,36% | -0,52% | 105% | 101% |
| Overall | -0,02% | -0,02% | -0,07% | 102% | 99% |
| Class D | 0,04% | 0,38% | 0,15% | 99% | 99% |
| Class F | 0,03% | 0,84% | 0,60% | 103% | 100% |

## Objective results without ALF

|  | Random access Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,02% | -0,02% | 99% | 102% |
| Class A2 | 0,05% | 0,08% | -0,01% | 99% | 99% |
| Class B | 0,02% | -0,01% | -0,02% | 98% | 100% |
| Class C | 0,01% | 0,08% | 0,02% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0,02% | 0,03% | -0,01% | 99% | 100% |
| Class D | 0,04% | 0,08% | 0,05% | 99% | 99% |
| Class F | 0,02% | -0,02% | -0,01% | 102% | 99% |
|  | Low delay B Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,04% | 0,10% | -0,34% | 99% | 100% |
| Class C | -0,02% | 0,22% | -0,01% | 101% | 100% |
| Class E | 0,11% | 0,24% | 0,21% | 103% | 98% |
| Overall | 0,04% | 0,18% | -0,09% | 101% | 100% |
| Class D | 0,01% | 0,39% | -0,25% | 99% | 96% |
| Class F | 0,01% | 0,33% | -0,33% | 101% | 99% |
|  | Low delay P Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,11% | -0,18% | -0,84% | 100% | 100% |
| Class C | -0,09% | -0,12% | 0,01% | 97% | 99% |
| Class E | -0,20% | 0,39% | 0,80% | 96% | 101% |
| Overall | -0,12% | -0,02% | -0,15% | 98% | 100% |
| Class D | -0,08% | -0,36% | 0,17% | 97% | 98% |
| Class F | -0,06% | -0,38% | -0,79% | 97% | 101% |

# Conclusions

We recommend adoption of a half-sample MV component difference threshold for determining boundary strength for deblocking to VVC draft. This ensures that blocking artifacts that origins from neighbouring blocks with small differences in motion also are deblocked.

# References

- K. Andersson, J. Enhorn, “CE11-related: On MV threshold for deblocking”, Document of Joint Video Experts Team, JVET-N0359.

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 5)", Document of Joint Video Experts Team, JVET-N1001.

- VTM-5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

- C-Y. Chen, A. Norkin, "Description of Core Experiment 5 (CE5): Loop filtering", Document of Joint Video Experts Team, JVET-N1025.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).