| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0062 |
| --- | --- |

| Title: | CE6-1.2: Transform Candidate Ordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Per Wennersten, Jacob Ström Torshamnsgatan 23 16480 Stockholm, Sweden | Tel: Email: | +46 10 714 64 37 {christopher.hollmann, davood.saffar, per.wennersten, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution provides results for CE6-1.2, which tested different algorithms for ordering the MTS transform candidates. The results can be summarized as follows:

|  | BDR-Y AI | BDR-Y RA | BDR-Y LD | EncT AI | EncT RA | EncT LD |
| --- | --- | --- | --- | --- | --- | --- |
| CE6-1.2a | 0.00% | 0.01% | 0.01% | 98% | 99% | 99% |
| CE6-1.2b | 0.00% | 0.00% | 0.05% | 100% | 98% | 98% |
| CE6-1.2c | 0.00% | 0.00% | 0.06% | 98% | 101% | 98% |
| CE6-1.2d | 0.00% | 0.01% | -0.05% | 100% | 98% | 102% |
| CE6-1.2e | 0.00% | 0.00% | -0.01% | 102% | 97% | 100% |

It should be noted that the possibility to carry out this test has been somewhat limited due to the way that LFNST is implemented; it is no longer possible to affect the search with the parameter MTSIntraMaxCand, and a way of sorting the candidates proposed in the CE has already been implemented as part of the LFNST search.

# Description of software

This document provides results for the software agreed to be tested in CE6-1.2 at the 14th JVET meeting in Geneva, Switzerland, in March 2019. The software is based on proposal JVET-N0491 [2] and described in the CE description [1] as follows:

This proposal applies three changes: removing the combination (DCT-8, DCT-8) from the MTS tool, replacing the size-32 DCT-8 with the existing DCT-2, and sorting the combinations (DCT-8, DST-7) and (DST-7, DCT-8) based on the intra prediction mode (for intra-predicted blocks) or the shape of the block (for inter-predicted blocks if inter MTS is enabled). Additionally, it is proposed to reduce the number of fully evaluated transform candidates in the CTC to reduce the encoder run time. The goal of applying the sorting algorithm is to regain the loss introduced by reducing the number of fully evaluated transform candidates from 3 to 2. This is done by setting the configuration parameter --MTSIntraMaxCand to 2.

This CE focusses on the sorting algorithm and the encoder speed-up without the first two aspects changing the transform modes above. Also, this CE only evaluates the candidate ordering for intra-predicted blocks, the method for inter-predicted blocks is not changed.

The following table shows an overview of the sorting algorithm. Note that the order and coding of the remaining candidates is not changed.

| Test | Condition | First candidate | Second candidate |
| --- | --- | --- | --- |
| CE6-1.2a/d | Intra mode > 34 | (DCT-8, DST-7) | (DST-7, DCT-8) |
|  | Intra mode <= 34 | (DST-7, DCT-8) | (DCT-8, DST-7) |
| CE6-1.2b/e | Width > Height | (DCT-8, DST-7) | (DST-7, DCT-8) |
|  | Width <= Height | (DST-7, DCT-8) | (DCT-8, DST-7) |

A total of 5 tests were evaluated:

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-1.2a | The sorting algorithm as proposed in JVET-N0491 based on the intra mode, compared against CTC anchor | C. Hollmann (Ericsson) | X. Zhao (Tencent) |
| CE6-1.2b | The sorting algorithm without the described intra prediction mode dependency, compared against CTC anchor | C. Hollmann (Ericsson) | X. Zhao (Tencent) |
| CE6-1.2c | Reducing the number of fully evaluated transform candidates to 2 without the sorting algorithm, compared against CTC anchor | C. Hollmann (Ericsson) | X. Zhao (Tencent) |
| CE6-1.2d | Test CE6-1.2a versus test CE6-1.2c as an anchor (which uses 2 candidates) | C. Hollmann (Ericsson) | X. Zhao (Tencent) |
| CE6-1.2e | Test CE6-1.2b versus test CE6-1.2c as an anchor (which uses 2 candidates) | C. Hollmann (Ericsson) | X. Zhao (Tencent) |

Notes on the software:

- VTM-5.0 includes a low-frequency non-separable secondary transform (LFNST), and as part of that adoption, the encoder search algorithm was changed, rendering the configuration parameter MTSIntraMaxCand non-functional if LFNST is enabled. The software includes a hard-coded limitation that emulates the parameter. Since the investigation in this CE was at least partly build on the possibility to change MTSIntraMaxCand, this has limited the possibility to carry out the CE test as planned. For LDB simulations, as LFNST is disabled in the Common Test Conditions (CTC), the parameter is used. This issue was reported on the JVET bug tracker [3].

- Furthermore, due to the way the LFNST search was implemented, the overall impact of the proposed changes in the CE was limited. The encoder search for MTS modified by the LFNST implementation now already implements a sorting of the two MTS candidates (DCT-8, DST-7) and (DST-7, DCT-8) based on the intra mode with a very similar condition to the one proposed in this CE, (>= 34 instead of > 34 in CE6-1.2a).

# Results

Detailed results can be found in the accompanying Excel sheets. Note that the results for tests A and D, and B and E, respectively, can be found in the same files. A summary of the results is shown in the abstract.

## CE6-1.2a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.02% | 96% | 100% |
| Class A2 | 0.00% | -0.04% | 0.00% | 97% | 100% |
| Class B | 0.00% | 0.01% | -0.01% | 97% | 99% |
| Class C | 0.00% | -0.01% | 0.03% | 101% | 101% |
| Class E | 0.00% | 0.02% | 0.01% | 99% | 100% |
| Overall | 0.00% | -0.01% | 0.01% | 98% | 100% |
| Class D | 0.00% | 0.02% | 0.03% | 98% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | -0.07% | 97% | 98% |
| Class A2 | 0.03% | 0.10% | 0.01% | 98% | 96% |
| Class B | 0.02% | -0.07% | -0.10% | 99% | 98% |
| Class C | -0.01% | -0.03% | 0.07% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.03% | 99% | 98% |
| Class D | -0.01% | -0.04% | -0.04% | 101% | 100% |
| Class F | -0.02% | 0.04% | 0.02% | 105% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.33% | -0.16% | 99% | 100% |
| Class C | 0.02% | 0.05% | -0.42% | 101% | 102% |
| Class E | -0.03% | 0.32% | 0.68% | 98% | 96% |
| Overall | 0.01% | 0.23% | -0.03% | 99% | 100% |
| Class D | 0.03% | 0.22% | 1.11% | 103% | 104% |
| Class F | -0.08% | -0.01% | 0.59% | 100% | 99% |

## CE6-1.2b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.03% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.03% | 99% | 102% |
| Class C | 0.00% | -0.02% | 0.03% | 105% | 102% |
| Class E | 0.00% | 0.02% | -0.02% | 92% | 97% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.04% | 0.05% | 100% | 100% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.08% | 98% | 96% |
| Class A2 | 0.03% | -0.10% | -0.01% | 98% | 98% |
| Class B | 0.01% | -0.01% | -0.18% | 98% | 97% |
| Class C | -0.03% | -0.03% | 0.04% | 98% | 90% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.07% | 98% | 95% |
| Class D | 0.01% | -0.01% | 0.03% | 97% | 91% |
| Class F | -0.01% | 0.03% | -0.02% | 102% | 86% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.18% | 0.03% | 99% | 101% |
| Class C | 0.02% | 0.31% | 0.12% | 100% | 102% |
| Class E | 0.15% | -0.23% | 0.48% | 95% | 97% |
| Overall | 0.05% | 0.12% | 0.17% | 98% | 100% |
| Class D | 0.01% | 0.00% | 0.51% | 98% | 96% |
| Class F | 0.01% | -0.47% | -0.57% | 98% | 99% |

## CE6-1.2c

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.01% | 99% | 99% |
| Class A2 | 0.00% | -0.04% | -0.01% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class C | -0.01% | -0.03% | 0.04% | 102% | 100% |
| Class E | 0.00% | 0.00% | -0.01% | 92% | 100% |
| Overall | 0.00% | -0.02% | 0.01% | 98% | 99% |
| Class D | 0.00% | 0.00% | 0.03% | 98% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.03% | 99% | 100% |
| Class A2 | 0.04% | -0.07% | 0.03% | 100% | 102% |
| Class B | 0.01% | 0.00% | -0.12% | 100% | 99% |
| Class C | -0.04% | -0.03% | -0.03% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.05% | 101% | 101% |
| Class D | -0.01% | -0.04% | 0.00% | 100% | 108% |
| Class F | -0.01% | 0.02% | 0.03% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.31% | 0.02% | 97% | 93% |
| Class C | 0.05% | -0.04% | -0.28% | 100% | 103% |
| Class E | 0.13% | -0.56% | 1.22% | 96% | 99% |
| Overall | 0.06% | -0.02% | 0.22% | 98% | 98% |
| Class D | -0.01% | -0.28% | 0.50% | 100% | 96% |
| Class F | 0.03% | 0.49% | 0.40% | 98% | 99% |

## CE6-1.2d

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 98% | 101% |
| Class A2 | 0.00% | -0.01% | 0.01% | 98% | 101% |
| Class B | 0.00% | 0.01% | -0.01% | 99% | 100% |
| Class C | 0.00% | 0.01% | -0.02% | 99% | 101% |
| Class E | 0.00% | 0.02% | 0.01% | 107% | 99% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | -0.04% | 98% | 98% |
| Class A2 | 0.00% | 0.18% | -0.02% | 98% | 95% |
| Class B | 0.00% | -0.06% | 0.02% | 99% | 99% |
| Class C | 0.03% | -0.01% | 0.10% | 97% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.02% | 98% | 97% |
| Class D | 0.00% | -0.01% | -0.04% | 101% | 93% |
| Class F | 0.00% | 0.02% | -0.01% | 107% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.02% | -0.18% | 102% | 109% |
| Class C | -0.03% | 0.08% | -0.14% | 101% | 99% |
| Class E | -0.15% | 0.87% | -0.53% | 102% | 97% |
| Overall | -0.05% | 0.25% | -0.25% | 102% | 102% |
| Class D | 0.03% | 0.51% | 0.59% | 103% | 109% |
| Class F | -0.11% | -0.48% | 0.18% | 102% | 100% |

## CE6-1.2e

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 102% | 103% |
| Class A2 | 0.00% | 0.03% | 0.02% | 101% | 100% |
| Class B | 0.00% | -0.02% | -0.03% | 101% | 103% |
| Class C | 0.00% | 0.01% | -0.02% | 103% | 102% |
| Class E | 0.00% | 0.01% | -0.02% | 100% | 96% |
| Overall | 0.00% | 0.01% | -0.01% | 102% | 101% |
| Class D | 0.00% | 0.04% | 0.02% | 103% | 103% |
| Class F | 0.00% | 0.00% | -0.01% | 102% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | -0.05% | 99% | 96% |
| Class A2 | -0.01% | -0.03% | -0.04% | 98% | 96% |
| Class B | -0.01% | 0.00% | -0.06% | 98% | 97% |
| Class C | 0.01% | 0.00% | 0.08% | 95% | 88% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.02% | 97% | 94% |
| Class D | 0.02% | 0.03% | 0.03% | 98% | 85% |
| Class F | 0.01% | 0.01% | -0.05% | 104% | 88% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-1.2c |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.12% | 0.00% | 102% | 109% |
| Class C | -0.03% | 0.34% | 0.40% | 100% | 99% |
| Class E | 0.02% | 0.32% | -0.71% | 99% | 98% |
| Overall | -0.01% | 0.14% | -0.04% | 100% | 103% |
| Class D | 0.02% | 0.28% | 0.03% | 98% | 99% |
| Class F | -0.02% | -0.93% | -0.96% | 100% | 100% |

# References

[1]	X. Zhao, H. E. Egilmez. Description of Core Experiment 6 (CE6): Transforms and transform signaling, JVET-N1026, March 2019, Geneva, Switzerland

[2]	C. Hollmann, D. Saffar, P. Wennersten, J. Ström. CE6-related: Transform Candidate Ordering, JVET-N0491, March 2019, Geneva, Switzerland

[3] JVET Bug Tracker. #287: Fast encoder search for MTS defect, May 2019, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/287

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).