| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, July 2019 | Document: JVET-O0063 |
| --- | --- |

| Title: | CE10-1.7: Adaptive convolutional neural network loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hujun Yin, Rongzhen Yang, Xiaoran Fang, Shoujiang Ma, 2200 Mission College Blvd, Santa Clara, CA, 95054, USA | Tel: Email: | +1 408 410 5081 {Hujun.yin, Rongzhen.yang} @intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

Adaptive Convolutional Neural Network Loop Filter (ACNNLF) was proposed in JVET-M0566, continually developed in JVET-N0480 and farther evaluated in this document as Core Experiment 10-1.7  (JVET-N1030) for two tests: in-loop filtering (as test CE10-1.7a) and post filtering (out-loop) (as test CE10-1.7b).

The proposed ACNNLFs method applies three 2-layer convolutional neural network (CNN) for each luma and chroma. The parameters of those CNN networks are signaled in the I frame of each RAS (Random Access Segment). The training of three CNNs is performed by using the recursive procedure of training data processing defined in JVET-M0566.

Compared with VTM-5.0-RA, the proposed ACNNLF (post filtering mode) achieves -3.24%, 0.04%, and -1.02% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.67%, -14.11%, and -17.09% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.73%, -21.49%, and -19.26% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.28%, -3.00%, and -1.28% BD-rates for Y, U, and V, respectively, for Class C video sequences.

# Introduction

The tested ACNNLF has the following properties:

- Two layer CNN, as shown in the figure 2;

- Up to 3 ACNNLFs for luma and 3 ACNNLFs for chroma adaptively trained with video data;

- The encoder selects the best ACNNLF for each of luma and chroma in each CTB from the trained set of ACNNLFs, to perform encoding. The weights of the trained set of ACNNLFs are signaled in the tile group header and the index of selected ACNNLF is signaled in the coded stream in CTU level;

The decoder receives the ACNNLF weights (up to 3 sets for luma and 3 set for chroma) and the CTU level selected ACNNLF and uses the selected ACNNLF to perform the filtering process for each luma and chroma block during frame reconstruction.

Figure 1. ACNNLF for luma

The parameters of ACNNLF are signaled in the tile group header. In RA configuration, only the I-slice contains ACNNLF parameters of this RAS. The on/off control is applied at the tile group header level for luma and chroma components, and the acnnlf_luma_slice_enable_flag and acnnlf_chroma_slice_enable_flag syntax elements are used to indicate ACNNLF enable/disable. CTB level information indicates which ACNNLF is selected or if there is no ACNNLF applied in the CTB.

Two defined CE tests as in-loop filtering (as test CE10-1.7a) and post filtering (out-loop) (as test CE10-1.7b) are presented in the contribution. In the in-loop filtering, the ACNNLF is applied after the adaptive loop filter (ALF) and then the reconstructed frames are cached in the buffer; in post filtering (out-loop), the ACNNLF is applied to the frames outside of in-loop filters, i.e., the reconstructed frames before PSNR evaluation.

Compared to previous JVET-N0480, the quantization aware training processing are performed on luma, which improves the performance gain of luma from -0.58% to -0.80% (In-Loop filter) and -0.95% (as post filter).

# Experimental results

The following sections describe the settings and results for the experiments using VTM 5.0 following the template recommended in JVET-M1006 [5].

## Training stage

Online training based on the video sequence itself is performed for each Random Access Segment. To align with CE10-1.2 (JVET-O0056), pictures with temporal ID equal to 0 or 1 in the RAS are used for training, QP values {22, 27, 32, 37} are used.

Table 1. ACNNLF Training Setup

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | 0.0055 |
|  |  | optimizer: | ADAM |
|  |  | batch size: | 128 |
|  |  | epoch: | 148* |
|  |  | loss function: | L1 |
|  |  | training GPU: | GTX 1080 Ti |
|  |  | training time: | 15 minutes* |
|  |  | framework: | TensorFlow |
| Optional | Other Settings: |  |  |
|  |  | Patch Size | 128x128 |
|  |  | Dataset | CTC Sequences |
|  |  | QP set for generating training dataset | 22, 27, 32, 37 |

* Measured with A1/A2 video streams, per RAS with training procedure defined in figure 2

## Inference stage

In inference stage, at encoder, all three ACNNLFs are tested to choose the best ACNNLF. At decoder, only the signaled ACNNLF for each CTB is used reconstruct the frame. Currently the ACNNLF inference module is aligned with M0159 with 6 bit fixed point for weights and 32 bits for other parameters.

Table 2. ACNNLF Inference Stage Setup

|  | Network Details |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Param. Num | Mem.P(MB) | Mem.T(MB) |
| ACNNLF | 2 | 0 | 732x3 (Luma) 402x3 (Chroma) | 0.0028 | 0.0448 |

## Experimental Results

The proposed ACNNLF is implemented on top of VTM-5.0. The simulations were performed following the JVET common test conditions [3].  Only RA video sequence are tested, Table 4 shows the results of the proposed scheme on Random Access for Class A1/A2, B and C, as the test CE10-1.7a (in-loop filter mode):

Table 4.  Experimental Results of the CE10-1.7a (In-Loop Filtering)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.23% | 0.56% | -0.36% | 100% | 143% |
| Class A2 | -1.12% | -14.35% | -17.24% | 100% | 144% |
| Class B | -0.58% | -21.28% | -18.41% | 100% | 133% |
| Class C | 0.24% | -2.66% | -1.00% | 100% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.80% | -10.56% | -9.93% | 100% | 129% |

Table 5 shows the results of the proposed scheme on Random Access for Class A1/A2, B and C, as the test CE10-1.7b (post filter mode):

Table 5.  Experimental Results of the CE10-1.7b (Post Filtering)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -3.24% | 0.04% | -1.02% | 100% | 153% |
| Class A2 | -0.67% | -14.11% | -17.09% | 100% | 188% |
| Class B | -0.73% | -21.49% | -19.26% | 100% | 186% |
| Class C | 0.28% | -3.00% | -1.28% | 100% | 117% |
| Class E |  |  |  |  |  |
| Overall | -0.95% | -10.78% | -10.39% | 100% | 158% |

# Conclusions

This contribution presents an ACNNLF design with 3 classes of CNN based loop filters, where each filter has only 2 CNN layers and 732 parameters for luma. The 3 ACNNLFs are adaptively trained with video sequence data. The best ACNNLF is selected for luma and chroma respectively for each CTB at encoder and indicated to decoder in coded stream with 2 bit indicator at CTB level. Compared with VTM-5.0-RA, the proposed ACNNLF in post filtering test achieves -3.24%, 0.04%, and -1.02% BD-rates for Y, U, and V, respectively, for Class A1 video sequences; -0.67%, -14.11%, and -17.09% BD-rates for Y, U, and V, respectively, for Class A2 video sequences; -0.73%, -21.49%, and -19.26% BD-rates for Y, U, and V, respectively, for Class B video sequences; and 0.28%, -3.00%, and -1.28% BD-rates for Y, U, and V, respectively, for Class C video sequences. Further research on ACNNLF to improve gain and reduce the complexity is planned.

# References

- Ying Ming Li, Shan Liu, and Kei Kawamura, " Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding" JVET-N1030, March 2019

- H. Yin, R. Yang, X. Fan, S.Ma, and Y.Yu, “Adaptive convolutional neural network loop filter”, Document of Joint Video Experts Team, JVET-M0566. Jan 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010

- S. L. Yiming Li, "Methodology and reporting template for neural network coding tool testing," JVET-M1006, 2019

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).