| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0064-v1 |
| --- | --- |

| Title: | Non-CE5: Modification of clipping value signalling for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Hilmi E. Egilmez, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Technology Inc. |  |  |

_____________________________

# Abstract

In VTM-5.0, clipping parameters of non-linear adaptive loop filter are exponential-Golomb coded with adaptive orders. At decoder side, filter coefficients are reconstructed at first, then only clipping parameters for non-zero coefficients are parsed. This may cause delay when parsing a bitstream. To remove the filter coefficient dependency, it is proposed that clipping parameters are always signalled. In addition, it is also proposed that the exponential-Golomb codes are replaced by fixed-length codes or truncated unary codes. Compared to VTM-5.0, experimental results show that proposed methods achieve similar coding efficiency and encoding/decoding time under common test condition.

# Introduction

In VTM-5.0, non-linear adaptive loop filter (ALF) can be applied by clipping the difference between current pixel and a neighbour pixel to some value. There are four possible clipping values for one channel. For each non-zero filter coefficient, the clipping parameter, which can be 0, 1, 2 or 3, is signalled by using an exponential-Golomb code with adaptive order. To parse the clipping parameters, a decoder has to reconstruct the coefficients of ALF first. This filter coefficient dependency may increase delay when parsing a bitstream.

# Proposed methods

Two methods are proposed to simplify the clipping parameters signalling of ALF.

- Removing filter coefficient dependency in clipping parameter signalling: clipping parameters are signalled for both zero and non-zero ALF coefficients, therefore, reconstruction of coefficients is not needed before parsing clipping parameters.

- Exponential-Golomb codes are replaced with fixed-length (2 bits) codes or truncated unary codes (maximum value is 3) for clipping parameters signalling.

# Experimental results

The proposed methods are implemented in VTM-5.0. The simulations were conducted under JVET common test conditions [1].

## Results of removing filter coefficient dependency for clipping parameter signalling

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | -0.03% | 0.00% | 100% | 102% |
| Class B | 0.01% | 0.03% | -0.05% | 100% | 101% |
| Class C | 0.00% | 0.04% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.02% | 100% | 101% |
| Class D | 0.01% | -0.12% | 0.02% | 99% | 98% |
| Class F | 0.00% | 0.05% | 0.00% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.23% | -0.02% | 101% | 97% |
| Class C | 0.04% | -0.05% | -0.12% | 101% | 102% |
| Class E | -0.01% | 0.03% | -0.33% | 101% | 102% |
| Overall | 0.02% | 0.09% | -0.13% | 101% | 100% |
| Class D | -0.03% | -0.27% | 0.57% | 101% | 102% |
| Class F | 0.01% | -0.22% | -0.03% | 100% | 102% |

## Results of replacing exponential-Golomb codes with fixed-length codes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | -0.01% | 0.00% | 0.00% | 100% | 103% |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 99% | 100% |
| Class E | -0.02% | -0.01% | -0.02% | 99% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.02% | 0.01% | -0.02% | 99% | 98% |
| Class F | -0.02% | -0.01% | -0.01% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | -0.04% | 102% | 101% |
| Class A2 | -0.01% | -0.05% | -0.02% | 101% | 102% |
| Class B | -0.01% | 0.00% | -0.02% | 99% | 99% |
| Class C | -0.04% | -0.11% | -0.12% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | -0.05% | 100% | 100% |
| Class D | -0.04% | 0.00% | 0.04% | 99% | 99% |
| Class F | -0.03% | 0.05% | 0.01% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.21% | 0.02% | 100% | 99% |
| Class C | 0.03% | 0.10% | -0.21% | 100% | 102% |
| Class E | -0.02% | 0.39% | -0.30% | 100% | 101% |
| Overall | 0.00% | 0.22% | -0.14% | 100% | 101% |
| Class D | -0.07% | -0.55% | 1.51% | 100% | 101% |
| Class F | 0.04% | 0.05% | 0.82% | 100% | 99% |

## Results of replacing exponential-Golomb codes with truncated unary codes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 100% |
| Class C | -0.03% | -0.01% | -0.01% | 99% | 99% |
| Class E | -0.03% | -0.02% | -0.03% | 100% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.01% | -0.02% | 99% | 97% |
| Class F | -0.02% | -0.01% | 0.00% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.01% | -0.11% | 101% | 101% |
| Class C | -0.04% | 0.00% | -0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.06% | 0.07% | 98% | 99% |
| Class F | -0.03% | 0.03% | 0.04% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.08% | -0.12% | 100% | 100% |
| Class E | 0.07% | 0.09% | -0.14% | 100% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.39% | 0.37% | 100% | 103% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Results of removing filter coefficient dependency for clipping parameter signalling and replacing exponential-Golomb codes with fixed-length codes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | -0.01% | 0.00% | 0.00% | 100% | 105% |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 99% | 100% |
| Class E | -0.02% | -0.01% | -0.02% | 99% | 100% |
| Overall | -0.01% | 0.00% | -0.01% | 100% | 101% |
| Class D | -0.02% | 0.01% | -0.02% | 99% | 95% |
| Class F | -0.02% | -0.01% | -0.01% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 101% | 100% |
| Class A2 | -0.01% | -0.02% | 0.01% | 100% | 101% |
| Class B | 0.00% | 0.03% | -0.01% | 100% | 101% |
| Class C | -0.03% | -0.04% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.04% | 100% | 101% |
| Class D | -0.02% | -0.03% | 0.09% | 99% | 98% |
| Class F | -0.03% | 0.04% | 0.03% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.21% | 0.01% | 100% | 99% |
| Class C | -0.01% | 0.39% | 0.00% | 101% | 101% |
| Class E | 0.04% | 0.10% | 0.13% | 100% | 102% |
| Overall | -0.01% | 0.24% | 0.04% | 100% | 100% |
| Class D | -0.05% | -0.04% | 1.22% | 101% | 101% |
| Class F | 0.00% | 0.01% | -0.07% | 100% | 102% |

## Results of removing filter coefficient dependency for clipping parameter signalling and replacing exponential-Golomb codes with truncated unary codes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.01% | 99% | 95% |
| Class E | -0.03% | -0.02% | -0.03% | 100% | 101% |
| Overall | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.03% | 0.01% | -0.01% | 100% | 98% |
| Class F | -0.01% | 0.00% | 0.00% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | 0.01% | 103% | 102% |
| Class A2 | -0.02% | -0.03% | -0.03% | 102% | 102% |
| Class B | -0.02% | 0.02% | -0.09% | 99% | 100% |
| Class C | -0.03% | 0.01% | -0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.05% | 100% | 100% |
| Class D | -0.01% | -0.05% | 0.12% | 99% | 98% |
| Class F | -0.03% | 0.02% | 0.04% | 98% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.17% | 0.01% | 99% | 97% |
| Class C | 0.00% | -0.25% | -0.19% | 100% | 101% |
| Class E | 0.08% | 0.14% | -0.48% | 100% | 100% |
| Overall | 0.01% | 0.02% | -0.18% | 100% | 99% |
| Class D | -0.02% | -0.16% | 0.43% | 100% | 100% |
| Class F | -0.05% | -0.08% | 0.44% | 99% | 101% |

# Updated working draft

The working draft was updated on top of JVET-N1001-v8.

## Removing filter coefficient dependency for clipping parameter signalling and replacing exponential-Golomb codes with fixed-length codes

The following text is modified with removing filter coefficient dependency for clipping parameter signaling and replacing exponential-Golomb codes with fixed-length codes

7.3.5.3 Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| …. |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| ….. |  |
| ….. |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)u(2) |
| } |  |
| } |  |
| ….. |  |
| } |  |

7.4.6.3 Adaptive loop filter data semantics

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

## Removing filter coefficient dependency for clipping parameter signaling and replacing exponential-Golomb codes with truncated unary codes

The following text is modified with removing filter coefficient dependency for clipping parameter signaling and replacing exponential-Golomb codes with truncated unary codes

7.3.5.3 Adaptive loop filter data syntax

| alf_data( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| …. |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v) tu(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| ….. |  |
| ….. |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v) tu(v) |
| } |  |
| } |  |
| ….. |  |
| } |  |

7.4.6.3 Adaptive loop filter data semantics

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive. The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive. The maximum value maxVal of the truncated unary binarization tu(v) is set equal to 3.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Technology Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).