| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0066-v1 |
| --- | --- |

| Title: | CE4-3.1a and CE4-3.1b: Unidirectional local illumination compensation with affine prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Nan Hu, Marta Karczewicz Chia-Ming Tsai, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Huanbang Chen, Xiang Ma, Haitao Yang | Tel: Email: | vseregin@qti.qualcomm.com {chia-ming.tsai, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com chenhuanbang@huawei.com |
| Source: | Qualcomm Incorporated, MediaTek Inc, Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents test results for the unidirectional local illumination compensation (LIC) tool, which is tested in CE4. In CE4-3.1a and CE4-3.1b, LIC is applied to regular inter mode, regular merge, merge with motion vector difference (MMVD) and affine modes, and it is disabled for other inter tools and bi-directional prediction. LIC is applied using 16x16 processing units and is not applied for the block with less than 64 luma samples. In CE4-3.1b, encoder-only optimization is applied to further reduce encoding time.

Test results reportedly show the following BD rate reduction compared to VTM-5.0.

CE4-3.1a: Unidirectional LIC with affine
RA BD-rates: -0.46% (Y), -0.39% (U), -0.50% (V) with 116% encoder and 100% decoder runtime
LB BD-rates: -0.37% (Y), 0.03% (U), -0.30 (V) with 119% encoder and 101% decoder runtime

CE4-3.1b: CE4-3.1a + LIC encoder-only optimization
RA BD-rates: -0.32% (Y), -0.22% (U), -0.40% (V) with 105% encoder and 99% decoder runtime
LB BD-rates: -0.24% (Y), -0.02% (U), -0.43% (V) with 108% encoder and 102% decoder runtime

# Introduction

Unidirectional LIC method with 16x16 processing units and extension to affine mode was proposed in JVET-N0343, JVET-N0205, and JVET-N0171. The encoder optimization was proposed in JVET-N0074.

LIC is applied to both luma and chroma components and in the following configuration:

- Intra neighbor samples are not used in LIC parameter derivation and are replaced by the reference neighbor samples

- Disable for combined inter/intra prediction (CIIP) and intra block copy (IBC) blocks

- Disable LIC for the blocks with less than 64 luma samples.

- No temporal inheritance of LIC flag

- No pruning based on LIC flag in merging candidate list generation

- It is not applied to bi-prediction

- LIC is applied on 16x16 processing units, where LIC parameters are estimated for the first top left 16x16 unit and are used for other units within the coding unit (CU) as shown on the Figure 1.

- LIC is applied to sub-block mode, where LIC parameters are derived based on the samples derived on a sub-block basis as depicted on the Figure 2, but it is limited to the first top-left 16x16 processing unit.

Figure1. LIC with 16x16 processing units.

Figure 2. Reference samples in sub-block mode.

LIC flag is included as a part of motion information in addition to motion vectors (MVs) and reference indices. When merge candidate list is constructed, LIC flag is inherited from the neighbor blocks for merge candidates, but LIC flag is not used for motion vector pruning for simplification purpose.

LIC flag is not stored in the motion vector buffer of a reference picture, so LIC flag is always set to false for temporal motion vector predictor (TMVP). LIC flag is also set to false for bi-directional merge candidates, such as pair-wise average candidate, and zero motion candidates.

LIC flag is context coded with a single context, when LIC tool is not applicable, LIC flag is not signaled.

When in-loop luma reshaping is used, the inverse reshaping is applied to the neighbor samples of the current CU prior to LIC parameter derivation, since the current CU neighbors are in the reshaped domain, but the reference picture samples are in the original (non-reshaped) domain.

To derive LIC linear model parameters, linear least square method is utilized, it requires the following operations per CU:

- Multiplications: 2 * min(width, height) + 4

- Additions: 4 * min(width, height) + 4

- Shifts: 12

To apply linear model 1 multiplication and 1 addition are used per sample, which can be done at the reconstruction stage when prediction is added to the residual.

In the worst case, such as 8x8 block, it requires 1.3 multiplications and 1.6 additions per sample in a block. LIC has smaller complexity comparing to other tools already existed in VVC.

In CE4-3.1b, to further reduce encoding time, following two encoder-only changes are applied:

- The motion search results of LIC-off is reused to test LIC-on as proposed in JVET-M0182.

- If the LIC usage ratio of neighbouring 4x4 subblocks is larger than or equal to a threshold (TH), then the mean removed sum of absolute differences (MR-SAD) motion search with LIC-on is further tested. As illustrated in Figure 3, assuming the current CU size is W×H, the number of neighbouring 4x4 subblocks inside the region marked in red is n, and LIC is applied to m out of n subblocks. If mn≥TH, then MR-SAD motion search with LIC-on is further tested. TH is set to 0.10.

Figure 3. Calculate the LIC usage ratio of neighbouring 4x4 subblocks to conditionally test MR-SAD motion search with LIC-on. The current CU is marked in blue. The searched neighbouring 4x4 subblocks are marked in red.

# Test results

The proposed method was implemented on top of VTM-5.0 software [6]. Table 1 shows the results of CE4-3.1a, and Table 2 shows the results of CE4-3.1b. The simulations were performed following the JVET common test conditions (CTC) [7]. The gain of the LIC method is much higher for the sequences that have rich local illumination change, such as FoodMarket, CatRobot, RitualDance.

Table 1. Results of CE4-3.1a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.66% | -0.55% | -0.63% | 122% | 100% |
| Class A2 | -0.39% | -0.37% | -0.39% | 114% | 101% |
| Class B | -0.46% | -0.21% | -0.52% | 113% | 99% |
| Class C | -0.36% | -0.49% | -0.46% | 117% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.46% | -0.39% | -0.50% | 116% | 100% |
| Class D | -0.13% | -0.25% | -0.06% | 116% | 103% |
| Class F | -0.67% | -0.35% | -0.46% | 110% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.03% | -0.34% | 121% | 96% |
| Class C | -0.42% | -0.15% | -0.35% | 127% | 102% |
| Class E | -0.07% | 0.35% | -0.18% | 107% | 107% |
| Overall | -0.37% | 0.03% | -0.30% | 119% | 101% |
| Class D | -0.16% | -0.06% | 0.28% | 118% | 100% |
| Class F | -1.77% | -1.52% | -1.48% | 116% | 99% |

Table 2. Results of CE4-3.1b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.27% | -0.53% | 107% | 101% |
| Class A2 | -0.33% | -0.14% | -0.32% | 108% | 101% |
| Class B | -0.32% | -0.18% | -0.39% | 103% | 97% |
| Class C | -0.20% | -0.30% | -0.36% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.32% | -0.22% | -0.40% | 105% | 99% |
| Class D | 0.02% | 0.09% | 0.22% | 105% | 99% |
| Class F | -0.56% | -0.26% | -0.29% | 101% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  |  |  | Over VTM5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | 0.01% | -0.37% | 109% | 102% |
| Class C | -0.24% | -0.01% | -0.42% | 110% | 108% |
| Class E | -0.01% | -0.10% | -0.54% | 102% | 97% |
| Overall | -0.24% | -0.02% | -0.43% | 108% | 102% |
| Class D | 0.06% | -0.31% | -0.01% | 104% | 91% |
| Class F | -1.73% | -1.06% | -1.73% | 103% | 95% |

# Reference

- V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, and M. Karczewicz, "CE1: Unidirectional illumination compensation (CE1-5.1)," Document of Joint Video Experts Team, JVET-N0343.

- K. Abe, T. Toma, J. Li, C.-W. Kuo, and V. Drugeon, "CE1: Pipeline restriction and DBF modification on LIC (test 1.5.6, test 1.5.7)," Document of Joint Video Experts Team, JVET-N0205.

- H. Chen, X. Ma, H. Yang, and J. Chen, "CE1-related: Combination of LIC and affine," Document of Joint Video Experts Team, JVET-N0171.

- C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, "CE1-5.5: Simplification of local illumination compensation," Document of Joint Video Experts Team, JVET-N0074.

- C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE10-related: Simplification of local illumination compensation,” Document of Joint Video Experts Team, JVET-M0182.

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).