| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0067-v1 |
| --- | --- |

| Title: | Non-CE5: Modified coding method for ALF clipping parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1(858)999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the current VVC design, clipping parameters used in the non-linear adaptive loop filter process are coded in a similar way as filter coefficients. However, there are only four choices allowed for a clipping parameter. Using the Exponential-Golomb code with an adaptive order is not necessary. In addition, whether to signal a clipping parameter is dependent on whether the corresponding filter coefficient is unequal to zero. Such a dependency may impact the throughput. In this contribution, two aspects are proposed: 1) using fixed-length coding for ALF clipping parameters; 2) removing the dependency on values of ALF coefficients. Simulation results reportedly show negligible impact on compression performance.

# Introduction

Non-linear adaptive loop filter (ALF) which clips a difference between the current sample and a neighbour sample is adopted in VTM-5.0 [1]. There are four possible clipping values for one color component. For each non-zero filter coefficient, the clipping parameter, which can be 0, 1, 2 or 3, is signalled by using an Exponential-Golomb binarization method with an adaptive order. Therefore, before parsing the clipping parameters, the filter coefficients must be reconstructed firstly. Such a dependency may impact the throughput when parsing a bitstream.

# Proposed methods

Two methods are proposed to simplify the signaling of clipping parameters used in ALF.

- Replace the Exponential-Golomb binarization method by a fixed-length (2 bits) binarization method.

- Remove the restriction that clipping parameter is only signaled for a non-zero filter coefficient. With the proposed method, the dependency between parsing clipping parameters and the reconstruction of filter coefficients is removed.

# Experimental results

The proposed methods were implemented on top of reference software VTM-5.0 [1], and simulations were performed following the common test conditions [2]. Results are provided for the following two tests:

- Test #1: Method 1).

- Test #2: whole proposal.

The results of Test #1 and Test #2 are shown in Table 1 and Table 2, respectively.

Table 1: Coding performance of Test #1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 101% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class C | -0.02% | -0.02% | -0.02% | 102% | 101% |
| Class E | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.02% | -0.02% | -0.02% | 101% | 102% |
| Class F | -0.02% | -0.02% | -0.02% | 102% | 99% |
|  |  | Random access Main10 |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class F | -0.02% | -0.02% | -0.02% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Class E | -0.02% | -0.02% | -0.02% | 102% | 102% |
| Overall | -0.02% | -0.02% | -0.02% | 101% | 101% |
| Class D | -0.01% | -0.01% | -0.01% | 101% | 104% |
| Class F | -0.04% | -0.04% | -0.04% | 102% | 96% |

Table 2: Coding performance of Test #2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 104% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Class C | -0.02% | -0.02% | -0.02% | 101% | 101% |
| Class E | -0.02% | -0.02% | -0.02% | 101% | 100% |
| Overall | -0.01% | -0.01% | -0.01% | 102% | 100% |
| Class D | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class F | -0.02% | -0.02% | -0.02% | 101% | 98% |
|  |  | Random access Main10 |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class F | -0.02% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Class E | -0.01% | -0.01% | -0.01% | 101% | 102% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class F |  |  |  |  |  |

# Proposed changes to working draft

The suggested specification changes on top of JVET-N1001_v8 is described as follows: deleted text is marked with strikethrough, and newly added text are highlighted in turquoise and grey for removing filter coefficient dependency for clipping parameter signaling and replacing exponential-Golomb codes with fixed-length codes, respectively.

7.3.5.3 Adaptive loop filter data syntax

| alf_data ( adaptation_parameter_set_id ) { | Descriptor |
| --- | --- |
| …. |  |
| if( alf_luma_clip_flag ) { |  |
| alf_luma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 3; i++ ) |  |
| alf_luma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| if( alf_luma_coeff_flag[ sfIdx ] ) { |  |
| for ( j = 0; j < 12; j++ ) { |  |
| if( filtCoeff[ sfIdx ][ j ] ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | uek(v)u(2) |
| } |  |
| } |  |
| } |  |
| } |  |
| ….. |  |
| if( alf_chroma_clip_flag ) { |  |
| alf_chroma_clip_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_clip_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| if( alf_chroma_coeff_abs[ j ] ) |  |
| alf_chroma_clip_idx[ j ] | uek(v)u(2) |
| } |  |
| } |  |
| ….. |  |
| } |  |

7.4.6.3 Adaptive loop filter data semantics

alf_luma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for luma clipping index signalling. The value of alf_luma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_luma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is incremented by 1. alf_luma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for luma clipping index signalling is not incremented by 1.

The order kClipY[ i ] of the exp-Golomb code used to decode the values of alf_luma_clip_idx[ sfIdx ][ j ] is derived as follows:

kClipY[ i ] = ( i = = 0  ?  alf_luma_clip_min_eg_order_minus1 + 1  :  kClipY[ i − 1 ] ) +	(7-75) 
	alf_luma_clip_eg_order_increase_flag[ i ]

alf_luma_clip_idx[ sfIdx ][ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_clip_idx[ sfIdx ][ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_luma_clip_idx[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1 and j = 0..11 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipY[ golombOrderIdxY[ j ] ]		(7-76)

alf_chroma_clip_min_eg_order_minus1 plus 1 specifies the minimum order of the exp-Golomb code for chroma clipping index signalling. The value of alf_chroma_clip_min_eg_order_minus1 shall be in the range of 0 to 6, inclusive.

alf_chroma_clip_eg_order_increase_flag[ i ] equal to 1 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is incremented by 1. alf_chroma_clip_eg_order_increase_flag[ i ] equal to 0 specifies that the minimum order of the exp-Golomb code for chroma clipping index signalling is not incremented by 1.

The order expGoOrderC[ i ] of the exp-Golomb code used to decode the values of alf_chroma_clip_idx[ j ] is derived as follows:

kClipC[ i ] = ( i = = 0  ?  alf_chroma_clip_min_eg_order_minus1 + 1  :  kClipC[ i − 1 ] ) +	(7-84) 
	alf_chroma_clip_eg_order_increase_flag[ i ]	(7-85)

alf_chroma_clip_idx[ j ] specifies the clipping index of the clipping value to use before multiplying by the j-th coefficient of the chroma filter. When alf_chroma_clip_idx[ j ] is not present, it is inferred to be equal 0 (no clipping). It is a requirement of bitstream conformance that the values of alf_chroma_clip_idx[ j ] with j = 0..5 shall be in the range of 0 to 3, inclusive.

The order k of the exp-Golomb binarization uek(v) is derived as follows:

k = kClipC[ golombOrderIdxC[ j ] ]		(7-86)

# Reference

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).